| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET- G0147 |
| --- | --- |

| Title: | AHG8: New Test Sequences for Spherical Video Coding from GoPro |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposed |  |  |
| Author(s) or Contact(s): | Adeel Abbas, David Newman | Tel: Email: | aabbas@gopro.com dnewman@gopro.com |
| Source: | GoPro |  |  |

_____________________________

# Abstract

This contribution provides four new sequences for testing spherical/360 video coding tools. The new sequences are of much higher stitching quality than existing sequences for a moving camera use case, with the hope that they would enable a more thorough testing of future compression technologies.

# Introduction

# The new content has been shot with GoPro Fusion camera [2], which is shown below.

# Sequences

Sequences are provided in yuv420 format at 5760x2880 resolution in Equirectangular (ERP) projection.

| Sequence | Resolution@FPS | Bit Depth | #Frames |
| --- | --- | --- | --- |
| great_wall | 5760x2880@25 | 8 | 250 |
| bran_castle | 5760x2880@30 | 8 | 300 |
| freefall | 5760x2880@30 | 8 | 300 |
| landing | 5760x2880@30 | 8 | 300 |

# Sequence Thumbnails

great_wall

bran_castle

freefall

landing

# References

- E. Alshina, J. Boyce, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360° video,” JVET-F1030, Mar. – Apr. 2017, Hobart, AU

- Fusion Camera from GoPro: https://gopro.com/news/Fusion-Pilot-Program

- David Newman, “Beyond Counting Pixels: Defining Resolution in Spherical” https://gopro.com/news/beyond-counting-pixels--defining-resolution-in-spherical

# Sequence Md5Sums

| Sequence | Md5Sum |
| --- | --- |
| great_wall_5760x2880_25fps_8bit_420_erp.yuv | 27aad313daedbebbb53be6b6e0097de3 |
| bran_castle_5760x2880_30fps_8bit_420_erp.yuv | 324ebf895bb56239d9250d841c7b91fc |
| freefall_5760x2880_30fps_8bit_420_erp.yuv | 10fc77389a161fad7900654f98510b82 |
| landing_5760x2880_30fps_8bit_420_erp.yuv | 5c7631d411e902717e6e76c136ff50aa |

# Sequence Preview Link

# H.264 encoded version of sequences are available here. For best results, it is recommended to download them and play.

# Copyright Information

The sequences provided in this contribution are owned by GoPro.

The following uses are allowed for the proposed sequences:

- Can be published in technical papers, played at technology research and development events.

- Can be used by Standards committees. (e.g., SMPTE, ITU, MPEG, VQEG).

The following uses are NOT allowed for the proposed sequences:

- Do not publish snapshots in product brochures.

- Do not use video for marketing purposes.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

- Do not share with press.