| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13 – 21 July 2017 | Document: JVET-G0153 |
| --- | --- |

| Title: | AHG7: Analysis of HDR metrics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard François (Technicolor) | Tel: Email: | edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution discusses about the metrics specified in JEM Common Test Conditions on HDR (JVET-E1020) for evaluating coding performance of HDR content and extended colour volume content coding. It reports observations made on the metrics currently considered in the AHG7.

# Introduction

In the JVET CfE HDR category (JVET-F1002) and in the AHG7 Common Test Conditions (JVET-F1020), relating to the coding of HDR and extended colour volume materials, the following HDR metrics are considered in addition to the usual PSNR computed in the input Y′, Cb and Cr spaces: wPSNR for Y′, Cb and Cr, tPNSR-Y, deltaE100, PSNR-L100. Three metrics are luma/luminance related (wPSNR-Y, tPNSR-Y, PSNR-L100) and three metrics are chroma/chrominance related (wPSNR-U, wPSNR-V, deltaE100). These metrics are described in Annex A of JVET-F1020.

This contribution provides some observations about the correlation between these HDR metrics.

# Considered metrics and test data

The approach used in this contribution consisted in gathering numerous BD-rate values for different HDR metrics from multiple coding experiments performed on HDR content coding, and in checking the correlation between these values.

The following HDR metrics were considered:

- Luma/Luminance related

  - wPSNR-Y – weighted PSNR of the Y component in BT.2100 PQ representation, where square distortion is weighted by a luma-dependent weight similar to the scaling used for the HDR anchors using a luma-dependent QP adaptation.

  - tPNSR-Y – PSNR computed from the distortion of the linear light Y component, from the CIE XYZ representation, converted to a non-linear value using the PQ reference OETF.

  - PSNR-L100 – PSNR computed from the distortion of the L component from the CIE La*b* representation.

- Chroma/Chrominance related

  - wPSNR-U/V – similar to wPSNR-Y, but for the Cb and Cr components.

  - DE100 – PSNR computed from the CIE deltaE2000 metric, derived from the signal representation in the CIE La*b* space.

  - Avg_wPSNRUV – average of wPSNR-U and wPSNR-V.

The considered data used for the analysis came from various experiments reported in JVET-G0072. These experiments were performed using the JCT-VC HDR test sequences. A total of 135 sets of observations (one set being made of the values for the 6 metrics mentioned above) were used.

# Analysis process

We consider two metrics to be compared. And we note (x, y) the couples of BD-rate values for these metrics from the different video coding experiments.

The correlation coefficient is computed as follows.

A robust linear regression is also performed, based on an M-estimator with the Tukey bisquare estimator [1]. This approach consists in an iterative weighted Least Mean Squares (LMS) estimation, the weight for a given sample depending on the linear regression error for this sample from the previous iteration. First iteration is achieved with all weights set to 1 (which results in ordinary LMS). The weights obtained from this estimation are good indicators to know if a sample is an outlier or can be trusted.

The attached excel file implements this estimation algorithm.

# Analysis results

The results of the correlation analysis for different couples of metrics are plotted in the figures below. For each couple of metrics being examined, the top figure plots the points corresponding to the BD-rate data, the regression line from the ordinary LMS, and the regression line resulting from the robust LMS. The 2 lines distant from the robust LMS regression line by a distance of ,  being the standard deviation resulting from the robust estimation, are also plotted. The bottom figure indicates the average weight obtained for the samples associated with the different used sequences.

## wPSNR-Y – PSNRL100

The correlation coefficient is equal to 0.81, which is rather high. The estimated regression line is defined by the equation:

y = 1.045 * x – 0.001

wPSNR-Y and PSNRL100 metrics look to be well correlated. The regression error standard deviation remains reasonable. All sequences have rather high weights. Starting  has the smallest weight around 0.7.

Figure 1. wPSNRY-PSNRL100.

## wPSNR-Y – tPSNR-Y

The correlation coefficient is equal to 0.61, which is not very high and much smaller than in previous case. The estimated regression line is defined by the equation:

y = 0.471 * x + 0.004

wPSNR-Y and tPSNR-Y metrics do not look to be well correlated. The samples are spread and many samples correspond to contradictory trends (x>0 an y<0, or x<0 and y>0). The estimation error standard deviation is rather large. The weights for various sequences is far from 1, FireEater having a very low average weight.

Figure 2. wPSNRY-tPSNRY.

## wPSNR-U – DE100

The correlation coefficient is equal to 0.90, which is high. The estimated regression line is defined by the equation:

y = 0.823 * x – 0.002

These metrics look to be well correlated. The regression error standard deviation is reasonable, and samples are well spread around the regression line. All sequences, except Starting and FireEater, have weights larger than 0.8.

Figure 3. wPSNRU-DE100.

## wPSNR-V – DE100

The correlation coefficient is equal to 0.92, which is high. The estimated regression line is defined by the equation:

y = 0.696* x – 0.004

These same comment as for the correlation between wPSNR-V and DE100 can be made.

Figure 4. wPSNRV-DE100.

## avg_wPSNRUV – DE100

The correlation coefficient is equal to 0.92, which is high. The estimated regression line is defined by the equation:

y = 0.757* x – 0.002

These same comment as for the correlation between wPSNR-U/V and DE100 can be made.

Figure 5. avg_wPSNRUV-DE100.

# Conclusions

Regarding luma/luminance related metrics, the metrics wPSNR-Y and PSNRL100 seem to be rather well correlated and to show similar trends. This tends to give a rather good confidence to these two metrics, based on quite different models. As they correlate well, one of them could be reasonably removed for further work on HDR.

tPSNR-Y does not correlate well with wPSNR-Y nor with PSNRL100. This may lead to question this metric.

Regarding chroma/chrominance related metrics, the metrics wPSNR-U, wPSNR-V and their average, look well correlated to DE100, and to show quite similar trends. This also gives more confidence for these metrics, and it could be reasonable to remove one of the wPSNR-U/V or DE100 metric.

# References

- Y. Susanti, H. Pratiwi, S. Sulistijowati H., T. Liana, “M estimation, S estimation, and MM estimation in robust regression,” International Journal of Pure and Applied Mathematics, Vol 91,  No. 3,  2014, 349-360. http://www.ijpam.eu/contents/2014-91-3/7/7.pdf