| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macau, CN, 18–25 October 2017 | Document: JVET-H0022 |
| --- | --- |

| Title: | AHG8: New Sequences for Virtual Reality Video from InterDigital |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Yong He, Philippe Hanhart, Eduardo Asbun, Yuwen He, Yan Ye 9710 Scranton Rd, Suite 250 San Diego, CA 92121, USA | Email: | Yong.He@InterDigital.com |
| Source: |  | InterDigital Communications, Inc. |  |  |

____________________________

# Abstract

This contribution makes two 360-degree video sequences available to the JVET.  The video sequences are provided in 8K (8192x4096) resolution.

# Introduction

360° video is a rapidly growing format emerging in the media industry. It is enabled by the rising availability of VR devices and able to provide the viewer with an immersive experience.

# Sequences

The following 8K sequences are provided in equirectangular projection (ERP):

| Sequence | Resolution@FPS | Bit Depth | Frames |
| --- | --- | --- | --- |
| HarborBiking_8192x4096_30fps_8bits_420_erp.yuv | 8192x4096@30 | 8 | 300 |
| KiteFliteWalking_8192x4096_30fps_8bits_420_erp.yuv | 8192x4096@30 | 8 | 300 |

The contributed sequences were captured by InterDigital using GoPro Omni rig, a synchronized camera array with six GoPro Hero4 Black cameras. Kolor Autopano Video Pro is used to stitch the video. No encoding is done after stitching; therefore, the stitched content is provided in uncompressed raw YUV format.

# Sequence Thumbnails

- HarborBiking_8192x4096_30fps_8bits_420_erp.yuv

- KiteFliteWalking_8192x4096_30fps_8bits_420_erp.yuv

Compared to the current CTC sequences Harbor and KiteFlite, HarborBiking and KiteFliteWalking were captured at the same places, but using moving cameras instead of stationary cameras, which should better represent typical use of 360-degree video content.

# Recommended Viewports

The following viewports are recommended for subjective and objective evaluations:

| Sequence | Viewport 1 center (yaw, pitch) | Viewport 2 center (yaw, pitch) |
| --- | --- | --- |
| HarborBiking | 55, 5 | -30, 10 |
| KiteFliteWalking | 10, -10 | -50, -20 |

The corresponding recommended viewport screen captures are shown in the following Figure 3, Figure 4, Figure 5 and Figure 6.

- Recommended Viewport 1 (yaw:55, pitch:5) of HarborBiking

- Recommended Viewport 2 (yaw:-30, pitch:10) of HarborBiking

- Recommended Viewport 1 (yaw:10, pitch:-10) of KiteFliteWalking

- Recommended Viewport 2 (yaw:-50, pitch:-20) of KiteFliteWalking

# Encoding Results

The sequences were encoded with HM-16.16-360Lib-4.0 and JEM-7.0-360Lib-4.0 following the common test conditions defined in [1]. The encoding was done in the padded ERP (PERP) format, using the Random Access configuration, at QP=22, 27, 32, 37. The Excel file that accompanies this document contains the full set of results.

|  |  | HM-16.16 - PERP |  |  |  | JEM-7.0 - PERP |  |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | QP | kbps | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V | kbps | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V |
| Harbor Biking | 22 | 25304 | 41.03 | 51.02 | 50.33 | 21711 | 41.38 | 51.22 | 50.46 | -32.3% | -50.6% | -48.4% |
|  | 27 | 10468 | 38.87 | 49.33 | 48.92 | 8856 | 39.40 | 50.07 | 49.49 |  |  |  |
|  | 32 | 4765 | 36.61 | 47.36 | 47.22 | 3969 | 37.26 | 48.96 | 48.51 |  |  |  |
|  | 37 | 2252 | 34.25 | 45.60 | 45.68 | 1841 | 35.02 | 47.77 | 47.47 |  |  |  |
| KiteFlite Walking | 22 | 56616 | 38.36 | 48.41 | 48.42 | 51568 | 38.63 | 48.32 | 48.34 | -22.3% | -30.3% | -30.7% |
|  | 27 | 26043 | 35.87 | 46.45 | 46.65 | 23356 | 36.30 | 46.79 | 46.96 |  |  |  |
|  | 32 | 12570 | 33.21 | 44.72 | 45.04 | 11175 | 33.74 | 45.63 | 45.87 |  |  |  |
|  | 37 | 5718 | 30.57 | 43.17 | 43.60 | 5059 | 31.14 | 44.55 | 44.76 |  |  |  |
| All |  |  |  |  |  |  |  |  |  | -27.27% | -40.42% | -39.54% |

# Sequence MD5 Sums

Uncompressed 8-bit YUV420 sequences in 8K resolution in equirectangular projection will be uploaded to MPEG content repository. The MD5 of each sequence is listed below:

| Sequence | MD5 |
| --- | --- |
| HarborBiking_8192x4096_30fps_8bits_420_erp.yuv | eb77cf283ad5c842c1bdfade895ba2ad |
| KiteFliteWalking_8192x4096_30fps_8bits_420_erp.yuv | c7a4d54078c04ad79d5c877b40a0a243 |

# Copyright Information

The sequences provided in this contribution are owned by InterDigital Communications, Inc.

The following uses are allowed for the contributed sequences:

- Sequences may be published in technical papers, played at technology research and development events.

- Sequences may be used by Standards committees. (e.g., ITU, MPEG, VQEG).

The following uses are NOT allowed for the contributed sequences:

- Do not publish snapshots in product brochures.

- Do not use video for marketing purposes.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

# References

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360° video”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1030, 7th Meeting, July 2017