| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0077r1 |
| --- | --- |

| Title: | Deblocking Improvements for Large CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu Kiran Misra Andrew Segall | Tel: Email: | +1-360-834-8054 zhuw@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

The contribution proposes a deblocking process designed to reduce the blocking artifacts result from the use of large transforms and block sizes. Compared to the JEM deblocking approach, the process incorporates stronger filters for both luma and chroma.  Additionally, the process includes a control process that considers the block sizes on both sides of the boundary being deblocked.  The stronger filters are used for luma samples that correspond to larger block sizes, while the JEM deblocking filters are still used for luma samples corresponding to the smaller block sizes.  From chroma, the filter is selected uses a different approach, and the stronger filter is applied when chroma samples on either sides of the deblocking boundary belong to a large block. It is reported that the proposed deblocking change improves subjective quality at low bit-rates when compared with the deblocking used in JEM7, and it is proposed to include the technique in formal study.

R1 of the contribution includes:

- Corrections to typographical errors

- A modification to deblocking control process resulting in application of wider stronger filter for large block boundaries when blocks on either side of the boundary make use of Local Illumination Compensation and the CBF is 0 for that block.

# Introduction

The use of large transforms and block sizes results in significant blocking artifacts especially in low bitrate scenarios. Figure 1 below shows the blocking artifacts that result in POC 28 for the FoodMarket sequence (used in MPEG/ITU-T’s CfP on Video Compression with Capability beyond HEVC [1]) when coded with QP 43 using JEM7 [2] in random access configuration.

Figure 1: Blocking artifacts at QP 43 resulting from use of large transforms in JEM

In studying this problem, we have observed that to overcome this issue, it appears necessary to use a stronger filter with more support than what is available in the JEM (and HEVC).

# Proposed Scheme

Several modifications are proposed in this contribution. To facilitate this explanation, we use the following nomenclatures:  First, samples on each side of a block boundary (perpendicular to the boundary edge) are represented as:

… p8 p7 p6 p5 p4 p3 p2 p1 p0 | q0 q1 q2 q3 q4 q5 q6 q7 q8 …

where | represents the block boundary edge.

Thus, in Figure 2a, samples py,x and qy,x correspond to line R[x] when deblocking the vertical edge, while for Figure 2b, samples py,x and qy,x correspond to line R[y] when deblocking the horizontal edge.

| (a) | (b) |
| --- | --- |

Figure 2: Deblocking boundary for (a) vertical edge (b) horizontal edge

## Stronger deblocking filter for luma

For the luma deblocking, we use the stronger filter proposed in JVET-J0028 [3] when a sample belongs to a large block.  The filter is listed below, and a sample belonging to a large block is defined as when the width >= 32 for a vertical edge, and when height>=32 for a horizontal edge.

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (7*q7+2*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (6*q7+q6+2*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (5*q7+q6+q5+2*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (4*q7+q6+q5+q4+2*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (3*q7+q6+q5+q4+q3+2*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (2*q7+q6+q5+q4+q3+q2+2*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

As can be seen, the number of deblocked samples is extended to seven above.  For the samples at small block sides, the strong filter in JEM is used for deblocking, which is given as:.

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

q0′ = ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3

q1′ = ( p0 + q0 + q1 + q2 + 2 )  >>  2

q2′= ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3

Figure 3 shows an illustration where the samples at side P are located in a large block and the samples at side Q are located in a small block. As described, the number of deblocked samples would then be three for side Q and seven for side P.  To be clear, the filter in [1] would then be used for deblocking the samples at the side P and the JEM strong filter would be used for deblocking the samples at the side Q.

Figure 3: An illustration of the proposed deblocking scheme

## Deblocking control for luma

The deblocking decision process is described in thi sub-section.  To begin, we define the following gradient calculation functions:

xCalDQp(R[x]): 		abs(p2 −2*p1+p0)

xCalDQq(R[x]): 		abs(q2 −2*q1+q0)

xCalDQpLargeBlock(R[x]): 	Max(Max(Max(Max(Max(abs(p2−2*p1+p0), 
					abs(p3−2*p2+p1)),
					abs(p4−2*p3+p2)), 
					abs(p5−2*p4+p3)),
					abs(p6 −2*p5+p4)), 
					abs(p7 −2*p6+p5))

xCalDQqLargeBlock(R[x]): 	Max(Max(Max(Max(Max(abs(q2−2*q1+q0), 
					abs(q3−2*q2+q1)),
					abs(q4−2*q3+q2)),
					abs(q5−2*q4+q3)),
					abs(q6 −2*q5+q4)),
					abs(q7 −2*q6+q5))

Wider-stronger luma filter is filters are used only if both Condition1 and Condition2 are TRUE.

Condition 1 and condition 2 are defined as follows.

Condition1 (Large Block Graident Condition) = ( dL < β ) ? TRUE : FALSE

Condition2 (Large Block Strong Filter Condition) = (xUseStrongFilteringLargeBlock(R[0], d0L, bSidePisLargeBlk, bSideQisLargeBlk) && xUseStrongFilteringLargeBlock(R[3], d3L, bSidePisLargeBlk, bSideQisLargeBlk)) ? TRUE:FALSE;

In the condition 1, dL is derived as follows.

First, determine whether P-side makes use of large blocks as follows, as:

bSidePisLargeBlk = ( (edge type is vertical and p0 belongs to CU with width >= 32) | | (edge type is horizontal and p0 belongs to CU with height >= 32) )? TRUE : FALSE

Next, determine whether Q-side makes use of large blocks as follows, as:

bSideQisLargeBlk = ( (edge type is vertical and q0 belongs to CU with width >= 32) | | (edge type is horizontal and q0 belongs to CU with height >= 32) )? TRUE : FALSE

Next, the following variables are derived:

d0P = bSidePisLargeBlk ? XCalDQpLargeBlock(R[0]) : XCalDQp (R[0])

d1P = bSidePisLargeBlk ? XCalDQpLargeBlock(R[3]) : XCalDQp (R[3])

d0Q = bSideQisLargeBlk ? XCalDQqLargeBlock(R[0]) : XCalDQq (R[0])

d1Q = bSideQLargeBlk ? XCalDQqLargeBlock(R[3]) : XCalDQq (R[3])

d0L = d0P + d0Q

d3L = d1P + d1Q

dL = d0L + d3L

In the condition 2, xUseStrongFilteringLargeBlock (R[x], d, bSidePisLargeBlk, bSideQisLargeBlk) computes a Boolean variable, as follows:

((abs((bSidePisLargeBlk ? p7 : p4) - p0) + abs((bSideQisLargeBlk ? q7 : q4) - q0)< (β>> 3)) && (d < (β >> 2)) && (abs(q0- p0) < ((tC * 5 + 1) >> 1)))? TRUE:FALSE

## Strong deblocking filter for chroma

The following strong deblocking filter for chroma is defined as:

p0′= (p2 +2*p1+2*p0+2*q0+q1 +4) >> 3

p1′= (p2+p1+ p0+q0+2) >> 2

p2′= (p4+2*p3+3*p2+ p1+ p0+4) >> 3

q0′= (q2 +2*q1+2*q0+2*p0+p1 +4) >> 3

q1′= (q2+q1+ q0+p0+2) >> 2

q2′= (q4+2*q3+3*q2+ q1+ q0+4) >> 3

## Deblocking control for chroma

The chroma strong filters are used on both sides of the block boundary.  Specifically, the strong filter is used when any of the following conditions is true:

- The edge type is vertical an p0 belongs to CU with width >= 32

- The edge type is horizontal and p0 belongs to CU with height >= 32

- The edge type is vertical and q0 belongs to CU with width >= 32

- The edge type is horizontal and q0 belongs to CU with height >= 32

The above test is the Large Block Condition test for chroma.

## Flowchart of luma and chroma deblocking control process

A flowchart for the luma and chroma deblocking control process is shown below:

Luma deblocking control:

Figure 4: Flowchart of luma deblocking control

Where, text in red corresponds to the proposed modifications, while the text in black corresponds to existing JEM deblocking. Note, the large block condition here is same as with large block condition test for chroma(section 2.4).

Chroma deblocking control:

Figure 5: Flowchart of luma deblocking control

# Experimental Results

Results using the proposed method were generated and viewed.  A still image from the result is shown in Figure 6, which demonstrates the reduction in blocking artifacts as a result of the proposed deblocking modification.

- (b)

Figure 6: Comparison of (a) JEM7 versus (b) JEM7+proposed deblocking modification

Similar improvements were observed in all sequences investigated.

We further refined the deblocking control where “Large Block Strong Filter Condition” is modified as follows:

(Large Block Strong Filter Condition) =

(When block on either side of boundary makes use of Local Illumination Compensation and the CBF of that block is 0) ? TRUE :

( (xUseStrongFilteringLargeBlock(R[0], d0L, bSidePisLargeBlk, bSideQisLargeBlk) &&                                                xUseStrongFilteringLargeBlock(R[3], d3L, bSidePisLargeBlk, bSideQisLargeBlk)) ? TRUE:FALSE );

As result wider stronger filter is used for large block boundaries when blocks on either side of the boundary make use of Local Illumination Compensation and the CBF is 0 for that block. The change further reduces visual artifacts as shown in example below (the region in the red circle):

| (a) | (b) |
| --- | --- |

Figure 7: Comparison of (a) JEM7 versus (b) JEM7+proposed deblocking modification using LIC test

# Conclusion

The contribution proposes use of stronger filters in the deblocking process of luma and chroma. It also proposes a deblocking filter selection process where block sizes on both sides of a boundary are considered when determining the filter to be applied. In visual evaluations, it was observed that the approach resulted in reduced blocking artifacts. And, it is proposed that the approach be studied formally by JVET in a core experiment.

# Reference

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki (editors) “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” JVET-H1002 (v6), October 2017, Macao, China.

- JEM7: https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0/

- T. Suzuki, M. Ikeda, K. Sharman, “Description of SDR and HDR video coding technology proposal by Sony”, doc. JVET-J0028, April 2018.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).