| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0002-v1 |
| --- | --- |

| Title: | JVET AHG report: Draft text and test model algorithm description editing (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Jianle Chen Jill Boyce Seung-Hwan Kim Shan Liu Yan Ye | Email: | benjamin.bross@hhi.fraunhofer.de jianle.chen@huawei.com jill.boyce@intel.com seunghwan3.kim@lge.com shanl@tencent.com yan.ye@interdigital.com |
| Source: | AHG2 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on draft text and test model algorithm description editing (AHG2) between the 11th meeting in Ljubljana, SI (10–18 July 2018) and the 12th meeting in Macao, CN (3–12 October 2018).

# Mandates

At the 11th JVET meeting, AHG2 on draft text and test model algorithm description editing was established with the following mandates:

- Produce and finalize JVET-K1001 Versatile Video Coding (Draft 2).

- Produce and finalize JVET-K1002 Test Model 2 (VTM 2) Algorithm and Encoder Description.

- Gather and address comments for refinement of these documents.

- Coordinate with Test model software development AhG to address issues relating to mismatches between software and text.

# Background

The first draft of Versatile Video Coding (VVC D1) includes a quadtree with nested multi-type tree using binary and ternary splits coding block structure as the initial new coding feature of VVC. At the 11th JVET meeting, it was decided to include more coding features for intra picture-prediction, inter-picture prediction, transform coefficient coding, transform, adaptive loop filtering and a starting basis for high-level syntax in the second draft of Versatile Video Coding (VVC D2) and the VVC Test Model 2 (VTM2) encoding. Draft reference software to implement the VVC decoding process and VTM2 encoding method has also been developed.

The normative decoding process for Versatile Video Coding is specified in the VVC draft 2 text specification document [1]. The VVC Test Model 2 (VTM 2) Algorithm and Encoder Description document provides an algorithm description as well as an encoder-side description of the VVC Test Model 2, which serves as a tutorial for the algorithm and encoding model implemented in the VTM2.0 software [2].

An issue tracker (https://jvet.hhi.fraunhofer.de/trac/vvc) was used to facilitate the reporting of errata with the VVC documents.

# JVET-K1001 VVC specification (Draft 2)

Seven versions of JVET-K1001 were published by the Editing AHG between the 11th meeting in Ljubljana, SI (10–18 July 2018) and the 12th meeting in Macao, CN (3–12 October 2018).

JVET-K1001 has been established based on JVET-J1001 and now contains the following:

- Incorporated JVET-K0230: Separate trees for intra slices (without multi-DMs) with an implicit split to 64x64

- Incorporated JVET-K0556: Prohibit ternary split of something bigger than 64 in width or height (and not send the bit to indicate ternary type at that level)

- Incorporated JVET-K0351 (test c): Keep only the TT restriction (preventing binary split with same orientation in center partition of the ternary split)

- Incorporated JVET-K0554: Implicit splitting at picture boundaries and ensure MinQTSize at boundary splits

- Fixed bug #65 typos and unused variables in section 6.4

- Fixed bug #67 implicit vertical BT split at picture boundary issue

- Incorporated JVET-K0072: Dependent quantization with fallback switch at the picture level and modified entropy coding supporting dependent quantization including:

  - adapted scaling to non-square transform blocks

  - added binarization process for abs_remainder

  - specified CoeffMin and CoeffMax with fixed values

  - added 0-th order Exp-Golomb code parsing process

- Incorporated JVET-K0310: Sign data hiding (can only be used when dependent quantization is disabled)

- Incorporated JVET-K0529: Intra prediction using 3MPM on 67 prediction modes (Planar, DC and 65 angular modes)

- Incorporated JVET-K0122: DC prediction without division

- Incorporated JVET-K0500: Wide-angle intra prediction

- Incorporated JVET-K0063: Position-dependent intra prediction combination

- Incorporated JVET-K0190: Cross-component linear model intra prediction

- Incorporated multiple transform selection (MTS) for both intra and inter, each controlled by an SPS flag

- Incorporated transform skip

- Fixed bug #68 various typos

- Fixed bug #71 various typos

- Fixed bug #72 on CCLM

- Incorporated JVET-K0357: adaptive motion vector resolution (AMVR)

- Incorporated JVET-K0565: affine motion compensation (MC) including:

  - JVET-K0052: Affine merge bug fix

  - JVET-K0184: Affine MC (CE4.1.1a 4x4 fixed subblock size)

  - JVET-K0337: Affine MC coding and models (4.1.3a, affine MVP list construction, and 4.1.3b, MV difference coding, and 4.1.3c, 4/6 parameter model, no slice level switch)

  - JVET-K0367/JVET-K0052/JVET-K0103: Restriction of affine merge mode to CU sizes >= 8x8

- Incorporated 1/16 motion compensation (MC) including:

  - 1/16 MV storage

  - 1/16 merge and affine MVs

  - MVDs in AMVR accuracy (1/4,1,4) shifted to 1/16

  - Inter MVP candidates rounded to AMVR accuracy (1/4,1,4) and shifted to 1/16

  - 1/16 luma and 1/32 chroma interpolation filters

- Incorporated subblock-based temporal merging candidates with 8x8 motion vector storage (JVET-K0346)

- Incorporated JVET-K0371: 4x4 block classification based Adaptive Loop Filter (ALF)

- Fixed bug #75 regarding a bottom and right boundary partition issue

- Fixed bug #90 typos in copying the control point vectors to temporal notion vectors

- Fixed bug #86 in intra reference sample filtering

- Incorporated JVET-K0325: High Level Syntax (HLS) starting point

- Fixed bug #82 on zeroing-out high frequency transform coefficients for larger TUs (>32x32)

- Fixed bug #85 on MTS index coding

The following items have been discussed within the AHG:

- In HEVC, there is a restriction on bi-prediction blocks smaller than 8x8 luma samples. This restriction is currently not in the VVC specification draft and the following input documents have been identified to be related:

JVET-L0104 AHG5: Reducing VVC worst-case memory bandwidth by restricting bi-directional 4x4 inter CUs/Sub-blocks

JVET-L0122 AHG5: Reduction of worst case memory bandwidth

JVET-L0137 CE1-related: Minimum block size restriction

JVET-L0396 CE4-related: Affine restrictions for the worst-case bandwidth reduction

JVET-L0453 Bugfix for restrictions of bi-prediction for small CUs

JVET-L0468 CE4-related: Fixed sub-block size and restriction for ATMVP

- In HEVC, there is a single merge candidate list for all PUs of an 8x8 CU. This was not integrated when the merge candidate list concept was incorporated into the VVC specification draft since there is no partitioning of CUs into PUs. The single merge candidate list was introduced to facilitate parallel merge estimation of the PUs inside an 8x8 CU. Related to that, HEVC allows merge estimation regions to enabled parallel merge list derivation for all blocks inside a merge estimation region. This was not incorporated into the VVC specification draft. The syntax is still in VTM software but it was never tested. Furthermore, adaptation to the current VVC design w.r.t the non-square CUs and affine merge candidate list is needed. The following document has been identified to be related:

JVET-L0216 Non-CE4: Parallel Merge Estimation for VVC

- In HEVC, the merge candidate list pruning is based on partial checks for redundant candidates. This was incorporated into the VVC specification draft. However, the VTM software performs a full pruning of the merge candidate list (meaning each candidate is compared to all other candidates).

- The decision from the last meeting to increase the maximum QP value from 51 to 63 (JVET-K0251) was not yet incorporated into the VVC specification draft since the current draft does not specify QP and QP delta coding and derivation. Due to non-square CUs in VVC, the concept of square quantization groups cannot be carried over from HEVC without modification. The following documents have been identified to be related:

JVET-L0362 Quantization parameter signaling

JVET-L0428 Delta QP and Chroma QP Offset for Separate Tree

JVET-L0553 Fix of Initial QP Signaling

- During the integration of the ALF text, it was noted that the CE description and the meeting notes indicated that the single 7x7 luma shape was adopted. However, the CE software used to generate the results and the VTM implementation uses the adaptive 5x5/7x7 luma filter shape. It was further noted that it does not have big impact on the decoder but it was suggested to discuss CE process and document CE rules more clearly at this meeting.

# JVET-K1002 VVC Test Model 2 (VTM 2) Algorithm and Encoder Description

One version of JVET-K1002 was published by the Editing AHG between the 11th meeting in Ljubljana, SI (10–18 July 2018) and the 12th meeting in Macao, CN (3–12 October 2018).

JVET-K1002 has been established based on JVET-J1002 and now contains the following:

- Incorporated JVET-K0230: Separate trees for intra slices (without multi-DMs) with an implicit split to 64x64

- Incorporated JVET-K0556: Prohibit ternary split of something bigger than 64 in width or height (and not send the bit to indicate ternary type at that level)

- Incorporated JVET-K0351 (test c): Keep only the TT restriction (preventing binary split with same orientation in center partition of the ternary split)

- Incorporated JVET-K0554: Implicit splitting at picture boundaries and ensure MinQTSize at boundary splits

- Incorporated JVET-K0063: Position dependent intra prediction combination (PDPC)

- Incorporated JVET-K0190: CCLM only (test 4.1.8)

- Incorporated JVET-K0122: DC prediction bug fix

- Incorporated JVET-K0529: 67 modes with 3MPM and FLC for non-MPM

- Incorporated JVET-K0500: Wide-angle intra prediction for non-square block

- Incorporated MTS (AMT) modification: Multiple transform selection (MTS)

- Incorporated sub-block based TMVP

- Incorporated adaptive motion vector resolution

- Incorporated 8x8 and 1/16 pel motion field storage

- Incorporated affine motion

Description of the following coding features will be added:

- 1/16 luma and 1/32 chroma interpolation filters

- JVET-K0072: Dependent quantization with modified entropy coding

- Adaptive loop filter

# Recommendations

The AHG recommends to:

- Approve the edited JVET-K1001 and JVET-K1002 documents as JVET outputs,

- Continue to edit the VVC draft and Test Model documents to ensure that all agreed elements of VVC are fully described,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification draft and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC WD and Test Model documents,

- Ensure that, when considering the addition of new feature to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC Working Draft is made available in a timely manner.

# References

- B. Bross, J, Chen, S. Liu, “Versatile Video Coding (Draft 2),” document JVET-K1001, 11th JVET meeting: Ljubljana, SI, 10–18 Jul. 2018.

- J. Chen, Y. Ye, S. Kim, “Test Model 2 (VTM 2) Algorithm and Encoder Description,” document JVET-K0002, 11th JVET meeting: Ljubljana, SI, 10–18 Jul. 2018.