| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0023-v1 |
| --- | --- |

| Title: | CE3: Summary Report on Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Jin Heo Alexey Filippov | Email: | geertv@qti.qualcomm.com jin78.heo@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Introduction

This is the summary report of the third Core Experiment (CE3) [1]. The goal of CE3 is to study intra prediction tools including mode coding for the VVC standard.

The following is the list of defined sub-tests in CE3:

- CE3.1: Multiple reference line prediction (9 tests)

- CE3.2: Intra prediction modes (9 tests)

- CE3.3: Intra reference sample interpolation (7 tests)

- CE3.4: Bidirectional prediction (3 tests)

- CE3.5: Cross-component prediction and separate chroma tree (18 tests)

- CE3.6: Intra mode coding (7 tests)

The CE3 description [1] originally defined 70 tests which were reduced to 53 after tests were withdrawn. This document summarizes the objective results (BD-rates, runtimes), cross-check reports and related input contributions.

# Test conditions and guidelines

The CTC conditions as agreed at the Ljubljana meeting for SDR are followed for the evaluation of objective performance (JVET-K1010). In CE3, it was agreed that it is mandatory to report results with both the ‘All Intra’ and ‘Random Access’ test configurations. One mandatory and one optional anchor are used to study the performance of the proposed technologies. The first anchor is the ‘VVC Test Model’ or VTM, which is mandatory. The second anchor is the ‘Benchmark Model Set’ or BMS, which is optional. The BMS version 2.0.1 software is used with ‘vtm’ configuration files to produce the VTM anchor. In this case, tools are implemented into the BMS version 2.0.1 software. The optional BMS anchor is generated using BMS version 2.1 software and tools are optionally implemented in this software as well. Proposals are compared with respect to objective compression efficiency while complexity is also assessed.

The source codes of the tests were uploaded by proponents into the following CE3 GitLab repository:

https://vcgit.hhi.fraunhofer.de/JVET-K-CE3/VVCSoftware_BMS.git

# Cross-check reporting

Each test is covered by at least one cross-checker and the cross-check reports are included within the respective sub-test sections. Within CE3 it was agreed that cross-checkers answer the following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any significant inconsistencies between the description and the provided software?

- Do the cross-checked results closely match the proponent results for AI and RA conditions (VTM and optional BMS)?

- Were additional results cross-checked?

- Additional comments that may be helpful to the group?

This report summarizes the answers in tables reporting whether the objective test results match in one table column and all other comments in a second table column.

# CE3.1 on ‘Multiple reference line prediction’

## Defined tests [1]

| Test | Add. Ref. Lines | Intra Prediction Modes | Prediction Averaging | Ref. Line Index Signalling | Block Size Restr. | Top CTU Restr. | MTS Restr. | Doc. # |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1.1 | 1,3 | Angular Modes | n.a. | Mode after index | y |  | y |  |
| 1.1.2 | 1,3 | Even Angular Modes | n.a. | Mode after index | y |  | y |  |
| 1.1.3 | 1,3 | Angular Modes | n.a. | Mode after index (MPM only) |  |  |  |  |
| 1.1.4 | 1,3 | Angular Modes | n.a. | Mode after index (MPM only) |  | y |  |  |

| Test | Add. Ref. Lines | Intra Prediction Modes | Prediction Averaging | Ref. Line Index Signaling | Block Size Restr. | Top CTU Restr. | MDIS Restr. | Doc. # |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.2.1 | 1 | Angular Modes | 50/50 extended/nearest | n.a. | y |  |  |  |
| 1.2.2 | 1 | Angular Modes | 50/50 extended/nearest | n.a. | y | y |  |  |
| 1.2.3 | 1 | Angular Modes | 25/75 extended/nearest | n.a. | y | y |  |  |
| 1.2.4 | 1 | Angular Modes | 25/75 extended/nearest | n.a. | y | y | y |  |
| 1.2.5 | 1 | Selected Angular Modes | 25/75 extended/nearest | n.a. | y | y |  |  |

## CE3.1: Test results

### CE3.1: VTM-2.0.1 anchor

|  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.1 | -0.61% | -0.33% | -0.36% | 125% | 99% | -0.33% | -0.17% | -0.13% | 108% | 100% |
| 1.1.2 | -0.47% | -0.22% | -0.23% | 130% | 99% | -0.26% | -0.12% | -0.09% | 108% | 100% |
| 1.1.3 | -0.46% | -0.29% | -0.24% | 102% | 98% | -0.20% | -0.01% | -0.01% | 100% | 100% |
| 1.1.4 | -0.40% | -0.23% | -0.20% | 102% | 98% | -0.18% | -0.03% | 0.02% | 100% | 99% |
| 1.2.1 |  |  |  |  |  |  |  |  |  |  |
| 1.2.2 |  |  |  |  |  |  |  |  |  |  |
| 1.2.3 | -0.18% | -0.13% | -0.10% | 105% | 103% | -0.10% | -0.05% | -0.06% | 102% | 101% |
| 1.2.4 | -0.19% | -0.16% | -0.11% | 105% | 104% | -0.11% | -0.08% | 0.00% | 102% | 101% |
| 1.2.5 | -0.18% | -0.13% | -0.10% | 105% | 103% | -0.10% | -0.05% | -0.06% | 100% | 100% |

### CE3.1: BMS-2.1 anchor (optional)

|  | All Intra Main10 - Over BMS-2.1 |  |  |  |  | Random Access Main10 - Over BMS-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.1 |  |  |  |  |  |  |  |  |  |  |
| 1.1.2 |  |  |  |  |  |  |  |  |  |  |
| 1.1.3 |  |  |  |  |  |  |  |  |  |  |
| 1.1.4 |  |  |  |  |  |  |  |  |  |  |
| 1.2.1 |  |  |  |  |  |  |  |  |  |  |
| 1.2.2 |  |  |  |  |  |  |  |  |  |  |
| 1.2.3 |  |  |  |  |  |  |  |  |  |  |
| 1.2.4 |  |  |  |  |  |  |  |  |  |  |
| 1.2.5 | -0.17% | -0.13% | -0.14% | 102% | 99% |  |  |  |  |  |

### CE3.1: Additional test results

### CE3.1: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 1.1.1 | Elie Mora (Ateme) |  |  |
| 1.1.2 | Elie Mora (Ateme) |  |  |
| 1.1.3 | Alexey Filippov (Huawei) |  |  |
| 1.1.4 | Kiho Choi (Samsung) |  |  |
| 1.2.1 | Mohammed G. Sarwer (MediaTek) |  |  |
| 1.2.2 | Mohammed G. Sarwer (MediaTek) |  |  |
| 1.2.3 | Fabien Racapé (Technicolor) |  |  |
| 1.2.4 | Fabien Racapé (Technicolor) |  |  |
| 1.2.5 | Jinho Lee (ETRI) |  |  |

## CE3.1: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# CE3.2 on ‘Intra prediction modes’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 2.1.1 | Line-based intra coding mode with a restricted number of partitions per block size (with at least 16 samples per partition) |  |
| 2.1.2 | Test 2.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples |  |
| 2.2.1 | Non-linear weighted intra prediction with inverse DCT after prediction (residual added in spatial domain) |  |
| 2.2.2 | Non-linear weighted intra prediction with modified structure of the predictors |  |
| 2.3.1 | Only use DM and LM modes for 2xN or Nx2 chroma blocks |  |
| 2.3.2 | Only use DM and LM modes for all chroma blocks |  |
| 2.4.1 | Enable MDMS |  |
| 2.4.2 | MDMS + fast encoder search |  |
| 2.5.1 | Proposed right-column and bottom-row prediction method |  |
| 2.6.1 | Withdrawn |  |

## CE3.2: Test results

### CE3.2: VTM-2.0.1 anchor

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1.1 | Line-based intra coding mode with a restricted number of partitions per block size (with at least 16 samples per partition) | -1.01% | -0.71% | -0.69% | 148% | 103% | -0.49% | -0.49% | -0.35% | 113% | 100% |
| 2.1.2 | Test 2.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples | -0.82% | -0.58% | -0.56% | 143% | 103% | -0.46% | -0.42% | -0.29% | 112% | 101% |
| 2.2.1 | Non-linear weighted intra prediction with inverse DCT after prediction (residual added in spatial domain) | -2.46% | -1.96% | -1.94% | 264% | 127% | -1.25% | -1.16% | -1.21% | 130% | 106% |
| 2.2.2 | Non-linear weighted intra prediction with modified structure of the predictors | -1.63% | -0.96% | -0.95% | 275% | 113% | -0.90% | -0.80% | -0.85% | 137% | 105% |
| 2.3.1 | Only use DM and LM modes for 2xN or Nx2 chroma blocks | 0.00% | 0.23% | 0.32% | 95% | 95% | 0.02% | 0.18% | 0.32% | 94% | 94% |
| 2.3.2 | Only use DM and LM modes for all chroma blocks | 0.08% | 1.25% | 1.46% | 91% | 95% | 0.03% | 0.81% | 0.95% | 92% | 94% |
| 2.4.1 | Enable MDMS | -0.20% | -1.09% | -1.08% | 99% | 100% | -0.02% | -0.77% | -0.84% | 100% | 100% |
| 2.4.2 | MDMS + fast encoder search | -0.20% | -0.84% | -0.84% | 97% | 100% | 0.06% | -0.69% | -0.76% | 98% | 100% |
| 2.5.1 | Proposed right-column and bottom-row prediction method | -0.07% | -0.04% | 0.02% | 100% | 98% | -0.01% | -0.01% | 0.08% | 100% | 98% |

### CE3.2: BMS-2.1 anchor (optional)

|  |  | All Intra Main10 - Over BMS-2.1 |  |  |  |  | Random Access Main10 - Over BMS-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1.1 | Line-based intra coding mode with a restricted number of partitions per block size (with at least 16 samples per partition) |  |  |  |  |  |  |  |  |  |  |
| 2.1.2 | Test 2.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples |  |  |  |  |  |  |  |  |  |  |
| 2.2.1 | Non-linear weighted intra prediction with inverse DCT after prediction (residual added in spatial domain) |  |  |  |  |  |  |  |  |  |  |
| 2.2.2 | Non-linear weighted intra prediction with modified structure of the predictors |  |  |  |  |  |  |  |  |  |  |
| 2.3.1 | Only use DM and LM modes for 2xN or Nx2 chroma blocks |  |  |  |  |  |  |  |  |  |  |
| 2.3.2 | Only use DM and LM modes for all chroma blocks |  |  |  |  |  |  |  |  |  |  |
| 2.4.1 | Enable MDMS |  |  |  |  |  |  |  |  |  |  |
| 2.4.2 | MDMS + fast encoder search |  |  |  |  |  |  |  |  |  |  |
| 2.5.1 | Proposed right-column and bottom-row prediction method |  |  |  |  |  |  |  |  |  |  |

### CE3.2: Additional test results

### CE3.2: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 2.1.1 | Liang Zhao (Tencent) |  |  |
| 2.1.2 | Liang Zhao (Tencent) |  |  |
| 2.2.1 | Fabien Racape (Technicolor) |  |  |
|  | Sergey Ikonin (Huawei) |  |  |
| 2.2.2 | Fabien Racape (Technicolor) |  |  |
|  | Sergey Ikonin (Huawei) |  |  |
| 2.3.1 | Santiago de Luxán Hernández (HHI) |  |  |
| 2.3.2 | Santiago de Luxán Hernández (HHI) |  |  |
| 2.4.1 | Xiang Ma (Huawei) |  |  |
| 2.4.2 | Xiang Ma (Huawei) |  |  |
| 2.5.1 | Steve Keating (Sony) |  |  |

## CE3.2: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# CE3.3 on ‘Intra reference sample interpolation’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 3.1.1 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size (JVET-J0017) | JVET-L0130 (LGE) |
| 3.1.2 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter with MDIS conditions (JVET-K0064) | JVET-L0324 (Qualcomm) |
| 3.1.3 | Interpolation filter selection between 4-tap cubic and 6-tap Gaussian (convolution of [1 2 1]/4 smoothing filter and 4-tap Gaussian) with MDIS conditions (JVET-K0165) | JVET-L0151 (ETRI) |
| 3.1.4 | Harmonization of shape-, size- and mode-dependent selection of 4-tap interpolation filters (JVET-K0518) with simplified PDPC and wide-angle intra-prediction | JVET-L0275 (Huawei) |
| 3.2.1 | Bilateral reference sample filter + 4-tap cubic interpolation filter | JVET-L0179 (HHI) |
| 3.2.2 | Bilateral reference sample filter + 4-tap cubic interpolation filter + 4-tap Gaussian interpolation filter |  |
| 3.3.1 | Multiple 4-tap filter | JVET-L0052 (Samsung) |

## CE3.3: Test results

### CE3.3: VTM-2.0.1 anchor

| Test # | Description | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.1 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size (JVET-J0017) | -0.41% | -0.44% | -0.42% | 101% | 101% | -0.18% | -0.11% | -0.07% | 100% | 100% |
| 3.1.2 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter with MDIS conditions (JVET-K0064) | -0.46% | -0.58% | -0.61% | 103% | 102% | -0.19% | -0.14% | -0.13% | 103% | 103% |
| 3.1.3 | Interpolation filter selection between 4-tap cubic and 6-tap Gaussian (convolution of [1 2 1]/4 smoothing filter and 4-tap Gaussian) with MDIS conditions (JVET-K0165) | -0.44% | -0.61% | -0.70% | 103% | 101% | -0.18% | -0.47% | -0.47% | 101% | 100% |
| 3.1.4 | Harmonization of shape-, size- and mode-dependent selection of 4-tap interpolation filters (JVET-K0518) with simplified PDPC and wide-angle intra-prediction | -0.45% | -0.57% | -0.57% | 102% | 103% | -0.17% | -0.13% | -0.07% | 100% | 101% |
| 3.2.1 | Bilateral reference sample filter + 4-tap cubic interpolation filter | -0.59% | -0.68% | -0.69% | 102% | 101% | -0.26% | -0.43% | -0.45% | 100% | 100% |
| 3.2.2 | Bilateral reference sample filter + 4-tap cubic interpolation filter + 4-tap Gaussian interpolation filter | -0.60% | -0.58% | -0.61% | 104% | 102% | -0.31% | -0.51% | -0.45% | 101% | 100% |
| 3.3.1 | Multiple 4-tap filter | -0.39% | -0.59% | -0.59% | 105% | 101% | -0.16% | -0.39% | -0.28% | 101% | 100% |

### CE3.3: BMS-2.1 anchor (optional)

| Test # | Description | All Intra Main10 - Over BMS-2.1 |  |  |  |  | Random Access Main10 - Over BMS-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.1 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size (JVET-J0017) |  |  |  |  |  |  |  |  |  |  |
| 3.1.2 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter with MDIS conditions (JVET-K0064) |  |  |  |  |  |  |  |  |  |  |
| 3.1.3 | Interpolation filter selection between 4-tap cubic and 6-tap Gaussian (convolution of [1 2 1]/4 smoothing filter and 4-tap Gaussian) with MDIS conditions (JVET-K0165) |  |  |  |  |  |  |  |  |  |  |
| 3.1.4 | Harmonization of shape-, size- and mode-dependent selection of 4-tap interpolation filters (JVET-K0518) with simplified PDPC and wide-angle intra-prediction |  |  |  |  |  |  |  |  |  |  |
| 3.2.1 | Bilateral reference sample filter + 4-tap cubic interpolation filter |  |  |  |  |  |  |  |  |  |  |
| 3.2.2 | Bilateral reference sample filter + 4-tap cubic interpolation filter + 4-tap Gaussian interpolation filter |  |  |  |  |  |  |  |  |  |  |
| 3.3.1 | Multiple 4-tap filter |  |  |  |  |  |  |  |  |  |  |

### CE3.3: Additional test results

### CE3.3: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3.1.1 | Geert Van der Auwera (Qualcomm) |  |  |
|  | Vasily Rufitskiy (Huawei) |  |  |
| 3.1.2 | Virginie Drugeon (Panasonic) |  |  |
|  | Philipp Merkle (HHI) |  |  |
|  | Alexey Filippov (Huawei) |  |  |
| 3.1.3 | Geert Van der Auwera (Qualcomm) |  |  |
|  | Vasily Rufitskiy (Huawei) |  |  |
| 3.1.4 | Geert Van der Auwera (Qualcomm) |  |  |
|  | Sunmi Yoo (LGE) |  |  |
| 3.2.1 | Vasily Rufitskiy (Huawei) |  |  |
| 3.2.2 | Vasily Rufitskiy (Huawei) |  |  |
| 3.3.1 | Luong Pham Van (Qualcomm) |  |  |

## CE3.3: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# CE3.4 on ‘Bidirectional prediction’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 4.1.1 | Linear interpolation intra prediction (LIP) | JVET-L0131 (LGE) |
| 4.2.1 | Harmonization of distance-weighted directional intra prediction (DWDIP) with simplified PDPC | JVET-L0284 (Huawei) |
| 4.2.2 | Harmonization of DWDIP with simplified PDPC and wide-angle intra-prediction |  |
| 4.3.1 | Withdrawn |  |

## CE3.4: Test results

### CE3.4: VTM-2.0.1 anchor

| Test # | Description | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1 | Linear interpolation intra prediction (LIP) | -0.21% | -0.16% | -0.13% | 105% | 101% | -0.14% | -0.27% | -0.24% | 102% | 101% |
| 4.2.1 | Harmonization of distance-weighted directional intra prediction (DWDIP) with simplified PDPC |  |  |  |  |  |  |  |  |  |  |
| 4.2.2 | Harmonization of DWDIP with simplified PDPC and wide-angle intra-prediction |  |  |  |  |  |  |  |  |  |  |

### CE3.4: BMS-2.1 anchor (optional)

| Test # | Description | All Intra Main10 - Over BMS-2.1 |  |  |  |  | Random Access Main10 - Over BMS-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1 | Linear interpolation intra prediction (LIP) |  |  |  |  |  |  |  |  |  |  |
| 4.2.1 | Harmonization of distance-weighted directional intra prediction (DWDIP) with simplified PDPC |  |  |  |  |  |  |  |  |  |  |
| 4.2.2 | Harmonization of DWDIP with simplified PDPC and wide-angle intra-prediction |  |  |  |  |  |  |  |  |  |  |

### CE3.4: Additional test results

### CE3.4: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 4.1.1 | Vasily Rufitskiy (Huawei) |  |  |
| 4.2.1 | Jonathan Pfaff (HHI) |  |  |
| 4.2.2 | Jonathan Pfaff (HHI) |  |  |

## CE3.4: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# CE3.5 on ‘Cross-component prediction and separate chroma tree’

## Defined tests [1]

| Test# | Short description | Doc. # |
| --- | --- | --- |
| 5.1.1 | Replace the LMS algorithm by a straight-line equation for CCLM mode Luma to Chroma |  |
| 5.2.1 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM + LM-Angular |  |
| 5.2.2 | CCLM + CCLM Cb-to-Cr |  |
| 5.2.3 | CCLM + CCLM Cb-to-Cr + MMLM |  |
| 5.2.4 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM |  |
| 5.2.7 | LM + line buffer restriction |  |
| 5.3.1 | Multiple neighbor-based LM (MNLM) |  |
| 5.4.1 | CCLM + MDLM |  |
| 5.4.2 | CCLM + MDLM with line buffer constraint |  |
| 5.5.1 | CCLM, using 1 luma line buffer |  |
| 5.5.2 | CCLM + MDLM, both using 1 luma line buffer |  |
| 5.6.1 | CCLM + MDLM; using simplified method. |  |
| 5.6.2 | CCLM; using 1 luma line buffer; using simplified method. |  |
| 5.6.3 | CCLM + MDLM; both using 1 luma line buffer; both using simplified method |  |
| 5.7.2 | Adaptive inter-residual prediction with fast RDO |  |
| 5.8.1 | If above side of the current CU cross CTU boundary, then only one line of above neighboring luma reconstructed samples is used in LM parameters derivation. |  |
| 5.8.2 | If above side of the current CU cross CTU boundary, then above neighboring luma reconstructed samples are not used in LM parameters derivation. |  |
| 5.9.1 | Adaptive Grouping LM |  |

## CE3.5: Test results

### CE3.5: VTM-2.0.1 anchor

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 5.1.1 | Replace the LMS algorithm by a straight-line equation for CCLM mode Luma to Chroma | 0.11% | 0.48% | -0.02% | 100% | 100% | 0.08% | 0.78% | 0.19% | 101% | 97% |
| 5.2.1 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM + LM-Angular | -0.59% | -6.66% | -6.02% | 111% | 106% | -0.25% | -5.49% | -5.01% | 107% | 103% |
| 5.2.2 | CCLM + CCLM Cb-to-Cr | -0.17% | -2.26% | -1.32% | 100% | 102% | -0.04% | -1.75% | -1.16% | 101% | 102% |
| 5.2.3 | CCLM + CCLM Cb-to-Cr + MMLM | -0.49% | -4.52% | -4.29% | 102% | 102% | -0.22% | -3.74% | -3.74% | 103% | 103% |
| 5.2.4 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM | -0.52% | -5.50% | -4.98% | 105% | 106% | -0.23% | -4.69% | -4.29% | 104% | 103% |
| 5.2.7 | LM + line buffer restriction |  |  |  |  |  |  |  |  |  |  |
| 5.3.1 | Multiple neighbor-based LM (MNLM) | -0.60% | -6.27% | -6.47% | 103% | 99% |  |  |  |  |  |
| 5.4.1 | CCLM + MDLM | -0.06% | -2.71% | -3.13% | 101% | 99% | -0.03% | -2.65% | -2.84% | 100% | 99% |
| 5.4.2 | CCLM + MDLM with line buffer constraint | 0.07% | -1.52% | -1.81% | 101% | 99% | 0.04% | -1.55% | -1.77% | 100% | 99% |
| 5.5.1 | CCLM, using 1 luma line buffer | 0.02% | 0.24% | 0.23% | 100% | 98% | 0.02% | 0.32% | 0.35% | 100% | 100% |
| 5.5.2 | CCLM + MDLM, both using 1 luma line buffer | -0.04% | -2.62% | -2.98% | 101% | 99% | -0.02% | -2.45% | -2.69% | 101% | 101% |
| 5.6.1 | CCLM + MDLM; using simplified method. | 0.03% | -2.12% | -2.87% | 101% | 99% | 0.02% | -1.88% | -2.66% | 100% | 98% |
| 5.6.2 | CCLM; using 1 luma line buffer; using simplified method. | 0.13% | 0.77% | 0.25% | 100% | 98% | 0.08% | 1.13% | 0.44% | 99% | 99% |
| 5.6.3 | CCLM + MDLM; both using 1 luma line buffer; both using simplified method | 0.05% | -1.88% | -2.67% | 101% | 99% | 0.03% | -1.59% | -2.38% | 101% | 100% |
| 5.7.2 | Adaptive inter-residual prediction with fast RDO | -0.14% | -2.41% | -2.67% | 101% | 100% | -0.02% | -2.25% | -2.11% | 101% | 100% |
| 5.8.1 | If above side of the current CU cross CTU boundary, then only one line of above neighboring luma reconstructed samples is used in LM parameters derivation. |  |  |  |  |  |  |  |  |  |  |
| 5.8.2 | If above side of the current CU cross CTU boundary, then above neighboring luma reconstructed samples are not used in LM parameters derivation. |  |  |  |  |  |  |  |  |  |  |
| 5.9.1 | Adaptive Grouping LM |  |  |  |  |  |  |  |  |  |  |

### CE3.5: BMS-2.1 anchor (optional)

|  |  | All Intra Main10 - Over BMS-2.1 |  |  |  |  | Random Access Main10 - Over BMS-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 5.1.1 | Replace the LMS algorithm by a straight-line equation for CCLM mode Luma to Chroma |  |  |  |  |  |  |  |  |  |  |
| 5.2.1 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM + LM-Angular |  |  |  |  |  |  |  |  |  |  |
| 5.2.2 | CCLM + CCLM Cb-to-Cr |  |  |  |  |  |  |  |  |  |  |
| 5.2.3 | CCLM + CCLM Cb-to-Cr + MMLM |  |  |  |  |  |  |  |  |  |  |
| 5.2.4 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM |  |  |  |  |  |  |  |  |  |  |
| 5.2.7 | LM + line buffer restriction |  |  |  |  |  |  |  |  |  |  |
| 5.3.1 | Multiple neighbor-based LM (MNLM) |  |  |  |  |  |  |  |  |  |  |
| 5.4.1 | CCLM + MDLM |  |  |  |  |  |  |  |  |  |  |
| 5.4.2 | CCLM + MDLM with line buffer constraint |  |  |  |  |  |  |  |  |  |  |
| 5.5.1 | CCLM, using 1 luma line buffer |  |  |  |  |  |  |  |  |  |  |
| 5.5.2 | CCLM + MDLM, both using 1 luma line buffer |  |  |  |  |  |  |  |  |  |  |
| 5.6.1 | CCLM + MDLM; using simplified method. |  |  |  |  |  |  |  |  |  |  |
| 5.6.2 | CCLM; using 1 luma line buffer; using simplified method. |  |  |  |  |  |  |  |  |  |  |
| 5.6.3 | CCLM + MDLM; both using 1 luma line buffer; both using simplified method |  |  |  |  |  |  |  |  |  |  |
| 5.7.2 | Adaptive inter-residual prediction with fast RDO |  |  |  |  |  |  |  |  |  |  |
| 5.8.1 | If above side of the current CU cross CTU boundary, then only one line of above neighboring luma reconstructed samples is used in LM parameters derivation. |  |  |  |  |  |  |  |  |  |  |
| 5.8.2 | If above side of the current CU cross CTU boundary, then above neighboring luma reconstructed samples are not used in LM parameters derivation. |  |  |  |  |  |  |  |  |  |  |
| 5.9.1 | Adaptive Grouping LM |  |  |  |  |  |  |  |  |  |  |

### CE3.5: Additional test results

### CE3.5: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 5.1.1 | Adarsh Ramasubramonian (Qualcomm) |  |  |
| 5.2.1 | Jangwon Choi (LGE) |  |  |
| 5.2.2 | D. Y. Kim (Chips&Media) |  |  |
| 5.2.3 | Guillaume Laroche (Canon) |  |  |
| 5.2.4 | Guillaume Laroche (Canon) |  |  |
| 5.2.7 | Xiang Ma (Huawei) |  |  |
| 5.3.1 | Adarsh Ramasubramonian (Qualcomm) |  |  |
| 5.4.1 | Kei Kawamura (KDDI) |  |  |
|  | Jonathan Pfaff (HHI) |  |  |
| 5.4.2 | Kei Kawamura (KDDI) |  |  |
| 5.5.1 | Po-Han Lin (ITRI) |  |  |
| 5.5.2 | Po-Han Lin (ITRI) |  |  |
| 5.6.1 | Po-Han Lin (ITRI) |  |  |
| 5.6.2 | Chia-Ming Tsai (MediaTek) |  |  |
| 5.6.3 | Jangwon Choi (LGE) |  |  |
| 5.7.2 | Xiang Ma (Huawei) |  |  |
| 5.8.1 | Jangwon Choi (LGE) |  |  |
| 5.8.2 | Jangwon Choi (LGE) |  |  |
| 5.9.1 | Xiang Ma (Huawei) |  |  |

## CE3.5: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# CE3.6 on ‘Intra mode coding’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 6.1.1 | 6 MPM with truncated binary code for non-MPM, CTU-row constraint | JVET-L0250 (Qualcomm) |
| 6.2.1 | Extended number of MPM rather than 3 | JVET-L0165 (LGE) |
| 6.3.1 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes (“reduced computational complexity” version) | JVET-L0219 (Huawei) |
| 6.3.2 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes. (“Improved BD-Rate gain” version) | JVET-L0220 (Huawei) |
| 6.4.1 | More than 3 MPMs with bypass coded bin, non-MPM FLC | JVET-L0086 (MediaTek) |
| 6.4.2 | More than 3 MPMs with bypass coded bin, CTU-row constraint, non-MPM FLC coding |  |
| 6.5.1 | 6 MPM (order of insertion is the same as in BMS 1.0); Truncated binarization to code the non-MPM modes; Intra mode independent CABAC contexts for signaling of the intra prediction mode; CTU-row constraint | JVET-L0221 (Huawei, MediaTek, LGE, Qualcomm) |

## CE3.6: Test results

### CE3.6: VTM-2.0.1 anchor

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.1.1 | 6 MPM with truncated binary code for non-MPM, CTU-row constraint | -0.42% | -0.40% | -0.35% | 102% | 102% | -0.17% | -0.13% | -0.16% | 100% | 99% |
| 6.2.1 | Extended number of MPM rather than 3 | -0.29% | -0.24% | -0.21% | 100% | 100% | -0.11% | -0.05% | 0.01% | 100% | 100% |
| 6.3.1 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes (“reduced computational complexity” version) |  |  |  |  |  |  |  |  |  |  |
| 6.3.2 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes. (“Improved BD-Rate gain” version) |  |  |  |  |  |  |  |  |  |  |
| 6.4.1 | More than 3 MPMs with bypass coded bin, non-MPM FLC | -0.33% | -0.30% | -0.27% | 100% | 98% | -0.12% | -0.05% | 0.01% | 100% | 98% |
| 6.4.2 | More than 3 MPMs with bypass coded bin, CTU-row constraint, non-MPM FLC coding | -0.29% | -0.24% | -0.21% | 100% | 99% | -0.10% | -0.04% | -0.06% | 100% | 98% |
| 6.5.1 | 6 MPM (order of insertion is the same as in BMS 1.0); Truncated binarization to code the non-MPM modes; Intra mode independent CABAC contexts for signaling of the intra prediction mode; CTU-row constraint |  |  |  |  |  |  |  |  |  |  |

### CE3.6: BMS-2.1 anchor (optional)

|  |  | All Intra Main10 - Over BMS-2.1 |  |  |  |  | Random Access Main10 - Over BMS-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.1.1 | 6 MPM with truncated binary code for non-MPM, CTU-row constraint |  |  |  |  |  |  |  |  |  |  |
| 6.2.1 | Extended number of MPM rather than 3 |  |  |  |  |  |  |  |  |  |  |
| 6.3.1 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes (“reduced computational complexity” version) |  |  |  |  |  |  |  |  |  |  |
| 6.3.2 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes. (“Improved BD-Rate gain” version) |  |  |  |  |  |  |  |  |  |  |
| 6.4.1 | More than 3 MPMs with bypass coded bin, non-MPM FLC |  |  |  |  |  |  |  |  |  |  |
| 6.4.2 | More than 3 MPMs with bypass coded bin, CTU-row constraint, non-MPM FLC coding |  |  |  |  |  |  |  |  |  |  |
| 6.5.1 | 6 MPM (order of insertion is the same as in BMS 1.0); Truncated binarization to code the non-MPM modes; Intra mode independent CABAC contexts for signaling of the intra prediction mode; CTU-row constraint |  |  |  |  |  |  |  |  |  |  |

### CE3.6: Additional test results

### CE3.6: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 6.1.1 | Chris Rosewarne (Canon) |  |  |
| 6.2.1 | Adarsh Ramasubramonian (Qualcomm) |  |  |
| 6.3.1 | Ling Li (LGE) |  |  |
| 6.3.2 | Ling Li (LGE) |  |  |
| 6.4.1 | Steve Keating (Sony) |  |  |
| 6.4.2 | Ikeda Masaru (Sony) |  |  |
| 6.5.1 | Chris Rosewarne (Canon) |  |  |

## CE3.6: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K1023