| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0024-v1 |
| --- | --- |

| Title: | CE4: Summary report on inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Shan Liu Kai Zhang | Email: | haitao.yang@huawei.com shanl@tencent.com zhangkai.video@bytedance.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 4 on inter prediction and motion vector coding. CE4 comprises 8 categories,

- Affine motion compensation

- Affine merge mode

- Planar motion vector prediction

- Merge mode enhancement

- Motion vector coding

- Reference picture boundary padding

- Local illumination compensation

- Motion data storage

All techniques are implemented on top of and test against VTM2.0.1. For each test, a comparative study along with related tests is conducted, results and complexity analysis are provided. Crosschecking reports of all tests are integrated in this document as well.

CE4.1: Affine motion compensation

Categorization & comparative study

Simplification of VTM2.0 Affine AMVP

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.1.1 | Cross-model inheritance for affine candidate derivation | JVET-L0363 |
| 4.1.3.a | Simplified derivation of inherited affine candidates | JVET-L0364 |
| 4.1.3.b | Simplified derivation of constructed affine candidates |  |
| 4.1.3.c | 4.1.3.a + 4.1.3.b |  |
| 4.1.4.a | Simplified inherited affine candidate with scanning order | JVET-L0141 |
| 4.1.4.b | Simplified inherited affine candidate with the reduced number of candidate |  |
| 4.1.4.c | Simplified constructed affine candidate |  |
| 4.1.4.d | 4.1.4.a + 4.1.4.c |  |
| 4.1.4.e | 4.1.4.b + 4.1.4.c |  |
| 4.1.4.g* | 4.1.4.d + simplified AMVP padding |  |
| 4.1.4.h* | 4.1.4.e + simplified AMVP padding |  |
| 4.1.6.a | Simplification on AMVP candidate list construction | JVET-L0271 |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1 | 0.01% | 0.08% | -0.05% | 100% | 100% | 0.00% | -0.08% | -0.23% | 101% | 100% |
| 4.1.3.a | 0.02% | 0.06% | -0.03% | 101% | 100% | 0.02% | -0.06% | -0.16% | 101% | 100% |
| 4.1.3.b | 0.01% | 0.03% | -0.02% | 101% | 100% | 0.01% | -0.09% | -0.16% | 101% | 100% |
| 4.1.3.c | 0.01% | 0.05% | -0.03% | 101% | 100% | 0.01% | -0.14% | -0.29% | 101% | 100% |
| 4.1.4.a | 0.02% | 0.06% | 0.02% | 100% | 100% | -0.01% | 0.03% | -0.12% | 101% | 104% |
| 4.1.4.b | 0.03% | 0.04% | -0.04% | 100% | 100% | -0.01% | 0.06% | -0.08% | 101% | 103% |
| 4.1.4.c | 0.01% | 0.03% | -0.03% | 100% | 100% | -0.01% | -0.08% | -0.29% | 101% | 104% |
| 4.1.4.d |  |  |  |  |  |  |  |  |  |  |
| 4.1.4.e |  |  |  |  |  |  |  |  |  |  |
| 4.1.4.g* | 0.02% | 0.08% | 0.01% | 100% | 100% | 0.01% | 0.01% | -0.07% | 100% | 104% |
| 4.1.4.h* | 0.03% | 0.08% | 0.01% | 100% | 99% | 0.02% | -0.07% | -0.08% | 100% | 103% |
| 4.1.6.a | 0.02% | 0.06% | -0.04% | 100% | 101% | 0.01% | -0.06% | -0.08% | 102% | 108% |

Complexity analysis

| Test# | AMVP list size | Max number of |  |  | MV scaling | Additional buffer |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | inherited affine candidate | constructed affine candidate | candidate comparison |  |  |
| Anchor | 2 | 10 | 1 | 10 | 3 | 2x |
| 4.1.1 | 2 | 10 | 1 | 10 | 3 | 2x |
| 4.1.3.a | 2 | 1 | 1 | 0 | 3 | 2x |
| 4.1.3.b | 2 | 10 | 1 | 10 | 3 | 2x |
| 4.1.3.c | 2 | 1 | 1 | 0 | 3 | 2x |
| 4.1.4.a | 2 | 1 | 1 | 1 | 3 | 2x |
| 4.1.4.b | 2 | 1 | 1 | 1 | 3 | 2x |
| 4.1.4.c | 2 | 10 | 1 | 9 | 3 | 2x |
| 4.1.4.d | 2 | 1 | 1 | 0 | 3 | 2x |
| 4.1.4.e | 2 | 1 | 1 | 0 | 3 | 2x |
| 4.1.4.g* | 2 | 1 | 1 | 0 | 1 | 2x |
| 4.1.4.h* | 2 | 1 | 1 | 0 | 1 | 2x |
| 4.1.6.a | 2 | 1 | 1 | 0 | 1 | 2x |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.1.1 | Y. He (InterDigital) |  |  |  |  |
| 4.1.3.a | J. Lee (LG) | 2 | F | Y | M |
| 4.1.3.b | J. Lee (LG) | 2 | F | Y | M |
| 4.1.3.c | J. Lee (LG) | 2 | F | Y | M |
| 4.1.4.a | H. Chen (Huawei) | 2 | P | Y | M |
| 4.1.4.b | H. Chen (Huawei) | 2 | P | Y | M |
| 4.1.4.c | H. Chen (Huawei) | 2 | P | Y | M |
| 4.1.4.d | H. Chen (Huawei) |  |  |  |  |
| 4.1.4.e | H. Chen (Huawei) |  |  |  |  |
| 4.1.4.g* | H. Chen (Huawei) | 2 | P | Y | M |
| 4.1.4.h* | H. Chen (Huawei) | 2 | P | Y | M |
| 4.1.6.a | J. An (Alibaba) |  |  |  |  |

Line buffer reduction

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.1.4.f* | 4.1.4.h (Simplified affine MVP list construction) + line buffer reduction | JVET-L0141 |
| 4.1.11.a | Line buffer reduction for affine inherited candidates, location 1 | JVET-L0045 |
| 4.1.11.b | Line buffer reduction for affine inherited candidates, location 2 |  |
| 4.1.12 | Simplification of affine AMVP list construction combined with line buffer reduction | JVET-L0364 |
| 4.1.13 | CTU restriction on affine inherited candidates for line buffer reduction | JVET-L0273 |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.4.f* | 0.06% | 0.12% | 0.00% | 100% | 100% | 0.03% | -0.07% | -0.10% | 101% | 104% |
| 4.1.11.a | 0.09% | 0.11% | 0.06% | 101% | 99% | 0.00% | -0.09% | -0.34% | 95% | 96% |
| 4.1.11.b | 0.12% | 0.19% | 0.07% | 95% | 93% | 0.01% | 0.11% | -0.08% | 88% | 91% |
| 4.1.12 | 0.11% | 0.14% | 0.07% | 101% | 101% | 0.04% | 0.03% | -0.02% | 101% | 100% |
| 4.1.13* | 0.05% | 0.12% | 0.01% | 97% | 96% | -0.01% | -0.14% | -0.30% | 102% | 108% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.1.4.f* | H. Chen (Huawei) | 2 | P | Y | M |
| 4.1.11.a | G. Li (Tencent) | 2 | F | Y | M |
| 4.1.11.b | G. Li (Tencent) | 2 | F | Y | M |
| 4.1.12 | K. Zhang (Bytedance) | 2 | F | Y | M |
| 4.1.13* | J. An (Alibaba) |  |  |  |  |

Others

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.1.7.a | Shape dependent control points selection for affine MVP, combined with shape dependent merge candidate selection and shape dependent AMVP list ordering | JVET-L0258 |
| 4.1.7.b | Shape dependent control points selection for affine AMVP |  |
| 4.1.8 | Slice-level 4/6 parameters affine model switching | JVET-L0143 |
| 4.1.10 | Slice level 4/6 parameters affine model switching | JVET-L0273 |
| 4.1.14 | Bypass coding of 4/6 parameter indication flag | JVET-L0143 |
| 4.1.16 | The sub-block size for chroma components is expanded from 2×2 to 4×4. | JVET-L0265 |
| 4.1.17.a | 3(scaling)/3(rotation)/4 model switching | JVET-L0343 |
| 4.1.17.b | 3(scaling)/3(rotation)/6 model switching |  |
| 4.1.17.c | 3(scaling)/3(rotation)/4/6 model switching |  |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.7.a | -0.05% | 0.04% | -0.02% | 100% | 99% | -0.06% | -0.09% | -0.23% | 97% | 100% |
| 4.1.7.b | -0.02% | 0.07% | -0.04% | 100% | 100% | -0.06% | 0.02% | -0.33% | 100% | 100% |
| 4.1.8 | 0.08% | 0.01% | 0.01% | 94% | 96% | 0.06% | 0.05% | 0.01% | 96% | 94% |
| 4.1.10 | 0.08% | 0.01% | 0.01% | 95% | 101% | 0.06% | 0.05% | 0.01% | 100% | 108% |
| 4.1.14 | 0.01% | 0.04% | -0.06% | 100% | 100% | 0.04% | -0.11% | -0.30% | 101% | 103% |
| 4.1.16 | 0.02% | 0.27% | 0.26% | 99% | 97% | -0.02% | 0.19% | -0.10% | 99% | 93% |
| 4.1.17.a | 0.57% | 0.43% | 0.32% | 111% | 100% | 0.20% | 0.01% | -0.23% | 127% | 101% |
| 4.1.17.b | 0.10% | 0.09% | 0.02% | 115% | 100% | 0.11% | -0.10% | -0.14% | 133% | 101% |
| 4.1.17.c | -0.05% | -0.02% | -0.10% | 115% | 100% | 0.00% | -0.07% | -0.08% | 131% | 100% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.1.7.a | H. Chen (Huawei) | 2 | F | Y | M |
| 4.1.7.b | H. Chen (Huawei) | 2 | F | Y | M |
| 4.1.8 |  |  |  |  |  |
| 4.1.10 | J. An (Alibaba) |  |  |  |  |
| 4.1.14 | H. Huang (Qualcomm) | 2 | F | Y | N/A |
| 4.1.16 | H. Chen (Huawei) | 2 | F | Y | M |
| 4.1.17.a | H. Chen (Huawei) | 2 | F | Y | M |
| 4.1.17.b | H. Chen (Huawei) | 2 | F | Y | M |
| 4.1.17.c | H. Chen (Huawei) | 2 | F | Y | M |

Detailed description of tests

CE4.1.1 in JVET-L0363 (Huawei)

In VTM2.0.1, for affine inter mode, the 4-parameter and 6-parameter motion models are selected at the CU level by using an additional affine motion type flag. For affine merge mode, affine model type for a CU is inherited from its neighboring affine CU. Since the derivation of inherited candidate is performed for both AMVP and merge list construction, there exists four kinds of inheritance: “4 to 4”, “6 to 4”, “6 to 6”, and “4 to 6”.

For the purpose of reducing the number of kinds of inheritance, in this test, for affine inter mode, 4-parameter motion model is converted to 6-parameter affine model by using 4-parameter model formula to generate the motion vector of bottom-left control point after MV reconstruction. For affine merge mode, affine model type for a CU is always set to 6-parameter motion model. Therefore, only 2 kinds of inheritance are used: “6 to 4”, and “6 to 6”. In addition, unified 6-parameter affine model is used to generate sub-block motion vector array for affine mode.

CE4.1.3 & CE4.1.12 in JVET-L0364 (Huawei)

In this contribution, a CU based pruning approach for inherited affine candidates is proposed. Two modifications,

- Simplified candidate pruning: 1) the comparison is conducted by comparing the coding units covering the neighboring positions, instead of comparing the derived affine candidates, 2) furthermore, the pruning process for constructed affine candidate is totally removed.

- For constructed affine candidate, the conversion from mv0 and mv2 to mv1 is removed and the conversion from mv0 and mv1 to mv2 for 6-parameter is removed

In addition, the abovementioned affine AMVP simplification is combined with the line buffer reduction mechanism in test4.1.11. The results are provided in test 4.1.12.

CE4.1.4 in JVET-L0141 (LGE)

Figure 1: Spatial positions for inherited candidate

Test4.1.4.a Simplified inherited candidate with the modified scanning order

In this proposal, the maximum two inherited affine candidates are derived from affine motion model of the neighboring blocks and inserted into the candidate list. For the left predictor, the scan order is: {A, D, E} and for the above predictor, the scan order is: {B, C} as shown in Figure 1. Each predictor selects the only one inherited candidate. No pruning check in performed between two inherited candidates.

Test4.1.4.b Simplified inherited candidate with the reduced number of candidate

In this proposal, the maximum number of inherited candidate is set to 1, and scan order is A, B, C, D, and E as shown in Figure 1.

Test4.1.4.c Simplified constructed candidate

In this method, constructed candidate is simplified as follows;

- For 6-parameter model, the constructed candidate is considered only when three motion vectors of CP0, CP1, and CP2 are available.

- For 4-parameter model, the constructed candidate is considered only when two motion vectors of CP0 and CP1 are available.

- No pruning check is performed between inherited candidate and constructed candidate.

Simplified affine MVP padding

In this proposal, HEVC AMVP padding is replaced by simplified affine MVP padding. In simplified affine MVP padding, if the candidate number of inherited and constructed candidate is less than 2, each MVs of CP0, CP1, and CP2 is considered for each addition affine MVP candidate until the candidate number is 2.

Line buffer restriction

In this method, to avoid the line buffer storage increase, inherited candidate is restricted to derive from the above block with CTU boundary.

CE4.1.6 in JVET-L0271 (Qualcomm)

Simplification of inherited affine motion predictors

The checking order of inherited affine motion predictors is simplified such that it’s similar to the checking order of spatial MVPs in HEVC AMVP list construction.  First, a left inherited affine motion predictor is derived from the first block in {A1, A0} that is affine coded and has the same reference picture as in current block. Second, an above inherited affine motion predictor is derived from the first block in {B1, B0, B2} that is affine coded and has the same reference picture as in current block. No pruning process is applied when inserting an inherited affine motion predictor into the candidate list.

Simplification of constructed affine motion predictors

If the current affine motion model is 4-parameter affine, then a constructed affine motion predictor is inserted into the candidate list only if  both mv0 and  mv1 are founded.

If the current affine motion model is 6-parameter affine, then a constructed affine motion predictor is inserted into the candidate list only if mv0, mv1and  mv2 are all founded.

No pruning process is applied when inserting a constructed affine motion predictor into the candidate list.

Simplification of translational motion predictors

In current VTM-2.0.1, the HEVC AMVP list construction process is performed if the number of affine motion predictors is less than 2. Note that the HEVC AMVP list construction process may require certain number of MV scaling and pruning operations.

To further reduce the complexity of affine AMVP list construction, the following applies until the number of affine motion predictors reaches the maximum.

- Derive an affine motion predictor by setting all CPMVs equal to mv2 if available.

- Derive an affine motion predictor by setting all CPMVs equal to mv1 if available.

- Derive an affine motion predictor by setting all CPMVs equal to mv0 if available.

- Derive an affine motion predictor by setting all CPMVs equal to HEVC TMVP if available.

- Derive an affine motion predictor by setting all CPMVs to zero MV.

Note that mvi is already derived in constructed affine motion predictor.

CE4.1.11 in JVET-L0045 (Broadcom)

The basic idea is to use the 4-parameter affine motion model, the bottom-left and bottom-right control point motion vectors (CPMVs) to do affine motion data inheritance for the current PU, if the (source) neighboring PU is located at the top CTU boundary regardless whether it uses the 4- or 6-parameter model. Compared to the current VTM2.0.1 design, no other change is introduced if the source neighboring PU is not located along the top CTU boundary.

The bottom-left and bottom-right CPMVs are stored in the regular motion data line buffer and are shared for the merge/skip and AMVP list derivations, to avoid the need of a separate line buffer to store the control point motion vectors. The stored the control point motion vectors may or may not be same motion vectors used for motion compensation and/or stored for temporal motion vectors (TMVPs).

CE4.1.10 & CE4.1.13 in JVET-L0273 (Qualcomm)

CE4.1.10: slice level 4/6 parameters affine model switching

In CE4.1.10, the affine models are adaptively selected at both slice level and block level. Slice header flag affine4_enable_flag specifies whether the 4-paramter affine is enabled, and affine6_ enable _flag specifies whether the 6-parameter affine model is enabled. If one of flags is equal to 1, affine_flag signaled at block level to indicate whether the allowed affine model is used for this block. If both are equal to 1, another affine_type flag is signaled at block level to specify the affine motion model for the current block. If affine_type is equal to 1, the 6-parameter affine model is used, otherwise the 4-parameter affine model is applied. The statistics from the previous slice are used to determine the affine4_flag and affine6_flag for the current slice. An exemplary syntax table for the CU level affine signaling is illustrated in Table 1.

| if (affine4_enable_flag ¦¦ affine6_enable_flag) |  |
| --- | --- |
| affine_flag | ae(v) |
| if (affine_flag && affine4_enable_flag && affine6_enable_flag) |  |
| affine_type | ae(v) |

CE 4.1.13: CTU restriction on affine inherited candidates for line buffer reduction

In the proposed method, deriving a 6-parameter affine inherited candidate from a neighboring block in the above CTU is illustrated in Fig. 1. The top-left CPMV v0 and top-right CPMVs v1 are derived by the the bottom-left MV vLB and bottom-right MV vRB of the neighboring candidate block:

v0=vRB-vLBneiW*posCurX-posNeiX+vLB

v1=vRB-vLBneiW*posCurX+curW-posNeiX+vLB

Wherein neiW is the width of the neighboring block, curW is the width of current block, posNeiX is the x coordinate of the top-left pixel of neighboring block, and posCurX is the x coordinate of the top-left pixel of current block.

The bottom-left CPMV v2 is derived as follows:

If the MV from left neighbor vleft is available and has the same reference picture as  vLB

v2 =vleft;

Else if the MV from bellow-left neighbor vbellowleft is available and has the same reference picture as  vLB

v2 =vbellowleft;

Else

deltaHorX,deltaHorY=vRB-vLBneiW

∇vVer=(-deltaHorY,deltaHorX)

v2 =vRB-vLBneiW*posCurX-posNeiX+∇vVer*curH+vLB

Wherein curH is the height of current block.

CE4.1.7 in JVET-L0258 (InterDigital)

Adaptive control point positions

If the block is coded with 4-parameter affine model and its block width is no smaller than the block height, the control points are defined at top-left and top-right; If the block width is smaller than the block height, the control points are defined at top-left and bottom-left.

Distance based list ordering for affine merge and AMVP

The control point distance is used to select the affine merge candidate and order the inherited affine MV predictors. The control point distance is calculated as:

| d=max⁡(block_width, block_height);if 4-parameter affine modelblock_width+block_height;if 6-parameter affine model | (1) |
| --- | --- |

The affine merge candidate with maximum control point distance from five spatial neighbors is selected for affine merge mode. The inherited affine MV predictors are also ordered in a descending order using control point distance.

CE4.1.8 & CE4.1.14 JVET-L0143 (LGE)

Slice-level 4/6 parameters affine model switching (Test 4.1.8)

The affine types are adaptively selected at slice level as follows;

Slice header flags affine_type_4_flag and affine_type_6_flag indicate whether the 4-parameter affine and 6-parameter affine model are applied or not. If one of them is equal to 1, affine_flag signaled at block level indicates whether the allowed affine model is used for this block. If both are equal to 1, there is an affine_type flag signaled at block level. If affine_type is equal to 1, the 6-parameter affine model is used, otherwise the 4-parameter affine model is applied. To determine the affine_type_4_flag and affine_type_6_flag for the current slice the statistics from the previous slice are used.

Bypass coding of affine type flag (Test 4.1.14)

Affine type flag is bypass coded to reduce the CABAC throughput.

CE4.1.16 JVET-L0265 (ByteDance)

In this contribution, the minimum sub-block size of chroma-components is also set to be 4×4. The MV of a 4×4 chroma sub-block is calculated as the average of the MVs of the four corresponding 4×4 luma sub-blocks.

CE4.1.17 JVET-L0343 (Sony)

A framework that switching between 3(scaling)/3(rotation)/4(simplified Affine)/6(Affine) is tested.

The 3-parameter models predicts scaling or rotation. It makes a chance to reduce an overhead of motion information.

An encoder can choose a mode in five modes which are a conventional translate, scaling, rotation and Affine. A number of parameters are 2 (one motion vector), 3, 3, 4 (two MVs) and 6 (three MVs) for translate mode, scaling mode, rotation mode and two Affine modes. The motion model index is defined and it is sent to the decoder to indicate a motion model.

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-L0046 | CE4-related: On line buffer reduction for affine mode | M. Zhou (Broadcom) |
| JVET-L0047 | CE4-related: A clean up for affine mode | M. Zhou, B. Heng (Broadcom) |
| JVET-L0048 | CE4-related: Combined tests of JVET-L0046 and JVET-L0047 | M. Zhou (Broadcom) |
| JVET-L0104 | AHG5: Reducing VVC worst-case memory bandwidth by restricting bi-directional 4x4 inter CUs/Sub-blocks | Y.-W. Chen, X. Wang (Kwai Inc.) |
| JVET-L0122 | AHG5: Reduction of worst case memory bandwidth | J. Li, R.-L. Liao, C. S. Lim (Panasonic) |
| JVET-L0193 | CE4-related: On Affine mode restriction | G. Laroche, J. Taquet, C. Gisquet, P. Onno (Canon) |
| JVET-L0259 | CE4-related: Adaptive precision for affine MVD coding | Y. He, X. Xiu, Y. Ye(InterDigital) |
| JVET-L0260 | CE4-related: Affine motion estimation improvements | Y. He, X. Xiu, Y. Ye(InterDigital) |
| JVET-L0281 | CE4-related: Size constrain for inherited affine motion prediction | H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm) |
| JVET-L0317 | CE4-related: Sub-block MV clipping in affine prediction | M. Gao, X. Li, M. Xu, S. Liu (Tencent) |
| JVET-L0330 | CE4-related: Affine model inheritance from single-line motion vectors | K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance) |
| JVET-L0332 | CE4-related: Adaptive Motion Vector Resolution for Affine Inter Mode | H. Liu, L. Zhang, K. Zhang, Y. Wang, P. Zhao, D. Hong (Bytedance) |
| JVET-L0371 | CE4-related: Reducing worst case memory bandwidth in inter prediction | H. Chen, H. Yang, J. Chen (Huawei) |
| JVET-L0373 | CE4-related: Unification for affine motion buffer | H. Chen, H. Yang, J. Chen (Huawei) |
| JVET-L0396 | CE4-related: Affine restrictions for the worst-case bandwidth reduction | L. Pham Van, W.-J. Chien, H. Huang, V. Seregin, M. Karczewicz (Qualcomm) |

CE4.2: Affine merge mode

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.2.1 | Common base for affine merge mode | JVET-L0366 |
| 4.2.2.a | Simplify affine merge list construction by reducing pruning operation | JVET-L0368 |
| 4.2.2.b | Simplify affine merge list construction by reducing MV scaling operation |  |
| 4.2.2.c | Simplify affine merge list construction by reducing maximum number of candidates |  |
| 4.2.2.d | Reduce line buffer in affine merge mode |  |
| 4.2.2.e | 4.2.2.a + 4.2.2.b + 4.2.2.c |  |
| 4.2.2.f | 4.2.2.a + 4.2.2.b + 4.2.2.c + 4.2.2.d |  |
| 4.2.2.g | 4.2.2.f + 4.1.3.c |  |
| 4.2.2.h | 4.2.2.e + 4.4.7.a (HMVP) |  |
| 4.2.3.a | Adding spatial inherit affine merge candidate on VTM affine merge | JVET-L0088 |
| 4.2.3.b | Adding corner derived affine merge candidate on VTM affine merge |  |
| 4.2.3.c | 4.2.3.a + 4.2.3.b |  |
| 4.2.3.d | 4.2.3.c + reducing line buffer |  |
| 4.2.4.b | Additional virtual temporal candidate | JVET-L0156 |
| 4.2.5.a | Simplification of affine merge list construction | JVET-L0278 |
| 4.2.5.b | 4.2.5.a + move ATMVP to affine merge list |  |
| 4.2.6.a | Simplified inherited affine candidate | JVET-L0142 |
| 4.2.6.b | Simplified constructed affine candidate |  |
| 4.2.6.c | 4.2.6.a + 4.2.6.b |  |
| 4.2.6.d | 4.2.6.c + reduced candidate number for affine candidate |  |
| 4.2.6.e | 4.2.6.d + line buffer reduction |  |
| 4.2.6.f | 4.2.6.d + 4.1.4.d |  |
| 4.2.6.g | 4.2.6.e + 4.1.4.f |  |
| 4.2.8.a | 4.2.1 + move ATMVP to affine merge list | JVET-L0369 |
| 4.2.8.b | 4.2.2.e + move ATMVP to affine merge list |  |
| 4.2.8.c | 4.2.2.e + 4.1.6.a (Simplification on AMVP candidate list construction) |  |
| 4.2.8.d | 4.2.8.b + 4.1.6.a (Simplification on AMVP candidate list construction) |  |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.2.1 | -0.75% | -0.62% | -0.78% | 107% | 107% | -0.44% | -0.40% | -0.40% | 108% | 105% |
| 4.2.2.a | -0.73% | -0.66% | -0.75% | 107% | 107% | -0.45% | -0.51% | -0.44% | 108% | 105% |
| 4.2.2.b | -0.73% | -0.59% | -0.78% | 106% | 107% | -0.36% | -0.25% | -0.41% | 107% | 104% |
| 4.2.2.c | -0.70% | -0.54% | -0.76% | 106% | 107% | -0.42% | -0.39% | -0.39% | 108% | 105% |
| 4.2.2.d | -0.65% | -0.50% | -0.69% | 107% | 106% | -0.45% | -0.29% | -0.50% | 109% | 105% |
| 4.2.2.e | -0.66% | -0.52% | -0.69% | 107% | 106% | -0.27% | -0.30% | -0.22% | 107% | 103% |
| 4.2.2.f | -0.57% | -0.46% | -0.60% | 106% | 106% | -0.25% | -0.36% | -0.33% | 106% | 103% |
| 4.2.2.g | -0.56% | -0.38% | -0.58% | 106% | 106% | -0.24% | -0.19% | -0.44% | 107% | 103% |
| 4.2.2.h | -1.10% | -1.04% | -1.24% | 105% | 106% | -0.46% | -0.48% | -0.64% | 106% | 104% |
| 4.2.2.h | -1.26% | -1.31% | -1.47% | 105% | 106% | -0.57% | -0.50% | -0.77% | 106% | 104% |
| 4.2.2.h | -1.31% | -1.35% | -1.56% | 106% | 106% | -0.62% | -0.67% | -0.93% | 107% | 104% |
| 4.2.3.a | -0.06% | -0.06% | -0.15% | 100% | 101% | -0.13% | -0.21% | -0.31% | 100% | 101% |
| 4.2.3.b | 0.25% | 0.22% | 0.09% | 101% | 102% | -0.04% | -0.03% | -0.08% | 100% | 101% |
| 4.2.3.c | -0.28% | -0.22% | -0.33% | 102% | 107% | -0.29% | -0.23% | -0.39% | 101% | 104% |
| 4.2.3.d | -0.17% | -0.14% | -0.28% | 101% | 107% | -0.26% | -0.26% | -0.39% | 101% | 105% |
| 4.2.4.b | -0.85% | -0.69% | -0.83% | 111% | 110% | -0.50% | -0.50% | -0.42% | 113% | 113% |
| 4.2.5.a | -0.72% | -0.55% | -0.73% | 106% | 108% | -0.33% | -0.20% | -0.37% | 109% | 111% |
| 4.2.5.b | -0.76% | -0.64% | -0.76% | 104% | 109% | -0.53% | -0.53% | -0.69% | 108% | 116% |
| 4.2.6.a | -0.71% | -0.57% | -0.72% | 107% | 107% | -0.48% | -0.48% | -0.31% | 109% | 109% |
| 4.2.6.b | -0.73% | -0.59% | -0.78% | 107% | 106% | -0.36% | -0.25% | -0.41% | 108% | 108% |
| 4.2.6.c | -0.70% | -0.56% | -0.73% | 107% | 106% | -0.35% | -0.44% | -0.27% | 108% | 108% |
| 4.2.6.d | -0.64% | -0.51% | -0.61% | 106% | 106% | -0.28% | -0.30% | -0.58% | 107% | 108% |
| 4.2.6.e | -0.56% | -0.46% | -0.58% | 106% | 106% | -0.21% | -0.15% | -0.17% | 107% | 107% |
| 4.2.6.f | -0.62% | -0.49% | -0.68% | 106% | 106% | -0.26% | -0.17% | -0.35% | 107% | 108% |
| 4.2.6.g | -0.49% | -0.39% | -0.52% | 106% | 106% | -0.19% | -0.10% | -0.25% | 106% | 108% |
| 4.2.8.a | -0.76% | -0.61% | -0.80% | 106% | 109% | -0.59% | -0.34% | -0.58% | 108% | 108% |
| 4.2.8.b | -0.69% | -0.57% | -0.73% | 104% | 107% | -0.40% | -0.33% | -0.35% | 105% | 106% |
| 4.2.8.c | -0.64% | -0.53% | -0.67% | 104% | 102% | -0.25% | -0.21% | -0.49% | 105% | 101% |
| 4.2.8.d | -0.69% | -0.57% | -0.73% | 104% | 107% | -0.40% | -0.33% | -0.35% | 105% | 106% |

Complexity analysis

| Test# | AMVP list size | Max number of |  |  | MV scaling | Additional buffer |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | inherited affine candidate | constructed affine candidate | candidate comparison |  |  |
| 4.2.1 | 5 | 5 | 10 | 50 | 30 | 2x |
| 4.2.2.a | 5 | 1 | 10 | 0 | 20 | 2x |
| 4.2.2.b | 5 | 5 | 10 | 50 | 2 | 2x |
| 4.2.2.c | 5 | 5 | 6 | 34 | 22 | 2x |
| 4.2.2.d | 5 | 5 | 10 | 50 | 28 | 1x |
| 4.2.2.e | 5 | 1 | 6 | 0 | 2 | 2x |
| 4.2.2.f | 5 | 1 | 6 | 0 | 0 | 1x |
| 4.2.2.g | / | / | / | / | / | / |
| 4.2.2.h | / | / | / | / | / | / |
| 4.2.2.h | / | / | / | / | / | / |
| 4.2.2.h | / | / | / | / | / | / |
| 4.2.3.a | 8 | 1 | 0 | 0 | 0 | 2x |
| 4.2.3.b | 8 | 0 | 2 | 0 | 2 | 2x |
| 4.2.3.c | 8 | 1 | 2 | 0 | 2 | 2x |
| 4.2.3.d | 8 | 1 | 2 | 0 | 2 | 1x |
| 4.2.4.b | 8 | 5 | 16 | 119 | 64 | 2x |
| 4.2.5.a | 5 | 1 | 10 | 0 | 2 | 2x |
| 4.2.5.b | 5 | 1 | 10 | 0 | 2 | 2x |
| 4.2.6.a | 5 | 1 | 10 | 0 | 20 | 2x |
| 4.2.6.b | 5 | 5 | 10 | 50 | 2 | 2x |
| 4.2.6.c | 5 | 1 | 10 | 0 | 2 | 2x |
| 4.2.6.d | 5 | 1 | 6 | 0 | 2 | 2x |
| 4.2.6.e | 5 | 1 | 6 | 0 | 0 | 1x |
| 4.2.6.f | / | / | / | / | / | / |
| 4.2.6.g | / | / | / | / | / | / |
| 4.2.8.a | 5 | 5 | 10 | 50 | 30 | 2x |
| 4.2.8.b | 5 | 1 | 6 | 0 | 2 | 2x |
| 4.2.8.c | / | / | / | / | / | / |
| 4.2.8.d | / | / | / | / | / | / |
| 4.2.1 | 5 | 5 | 10 | 50 | 30 | 2x |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.2.1 | F. Galpin (Technicolor) | 2 | F | Y | M |
| 4.2.2.a | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.b | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.c | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.d | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.e | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.f | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.g | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.h | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.h | J. Lee (LG) | 2 | F | Y | M |
| 4.2.2.h | J. Lee (LG) | 2 | F | Y | M |
| 4.2.3.a | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.3.b | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.3.c | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.3.d | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.4.b | H. Huang(Qualcomm) | 2 | F | Y |  |
| 4.2.5.a | F. Galpin (Technicolor) | 2 | F | Y | M |
| 4.2.5.b | F. Galpin (Technicolor) | 2 | F | Y | M |
| 4.2.6.a | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.6.b | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.6.c | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.6.d | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.6.e | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.6.f | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.6.g | H. Chen (Huawei) | 2 | P | Y | M |
| 4.2.8.a | Z.-Y. Lin (MediaTek) |  |  |  |  |
| 4.2.8.b | Z.-Y. Lin (MediaTek) |  |  |  |  |
| 4.2.8.c | Z.-Y. Lin (MediaTek) |  |  |  |  |
| 4.2.8.d | Z.-Y. Lin (MediaTek) |  |  |  |  |
| 4.2.1 | F. Galpin (Technicolor) | 2 | F | Y | M |

Detailed description of tests

CE4.2.1 in JVET-L0366 (Huawei)

The affine merge candidate list is constructed as following steps:

- Insert inherited affine candidates

Inherited affine candidate means that the candidate is derived from the affine motion model of its valid neighbor affine coded block. In the common base, as shown in Figure 1, the scan order for the candidate positions is: A1, B1, B0, A0 and B2.

After a candidate is derived, full pruning process is performed to check whether same candidate has been inserted into the list. If a same candidate exists, the derived candidate is discarded.

- Insert constructed affine candidates

If the number of candidates in affine merge candidate list is less than MaxNumAffineCand (set to 5 in this contribution), constructed affine candidates are inserted into the candidate list. Constructed affine candidate means the candidate is constructed by combining the neighbor motion information of each control point.

The motion information for the control points is derived firstly from the specified spatial neighbors and temporal neighbor shown in Figure 1. CPk (k=1, 2, 3, 4) represents the k-th control point. A0, A1, A2, B0, B1, B2 and B3 are spatial positions for predicting CPk (k=1, 2, 3); T is temporal position for predicting CP4.

The coordinates of CP1, CP2, CP3 and CP4 is (0, 0), (W, 0), (H, 0) and (W, H), respectively, where W and H are the width and height of current block.

Figure 1 Candidates position for affine merge mode

The motion information of each control point is obtained according to the following priority order:

For CP1, the checking priority is B2->B3->A2. B2 is used if it is available. Otherwise, if B2 is available, B3 is used. If both B2 and B3 are unavailable, A2 is used. If all the three candidates are unavailable, the motion information of CP1 cannot be obtained.

For CP2, the checking priority is B1->B0.

For CP3, the checking priority is A1->A0.

For CP4, T is used.

Secondly, the combinations of controls points are used to construct an affine merge candidate.

Motion information of three control points are needed to construct a 6-parameter affine candidate. The three control points can be selected from one of the following four combinations ({CP1, CP2, CP4}, {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4}). Combinations {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4} will be converted to a 6-parameter motion model represented by top-left, top-right and bottom-left control points.

Motion information of two control points are needed to construct a 4-parameter affine candidate. The two control points can be selected from one of the following six combinations ({CP1, CP4}, {CP2, CP3}, {CP1, CP2}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4}). Combinations {CP1, CP4}, {CP2, CP3}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4} will be converted to a 4-parameter motion model represented by top-left and top-right control points.

The combinations of constructed affine candidates are inserted into to candidate list as following order:

{CP1, CP2, CP3}, {CP1, CP2, CP4}, {CP1, CP3, CP4}, {CP2, CP3, CP4}, {CP1, CP2}, {CP1, CP3}, {CP2, CP3}, {CP1, CP4}, {CP2, CP4}, {CP3, CP4}

For reference list X (X being 0 or 1) of a combination, the reference index with highest usage ratio in the control points is selected as the reference index of list X, and motion vectors point to difference reference picture will be scaled.

After a candidate is derived, full pruning process is performed to check whether same candidate has been inserted into the list. If a same candidate exists, the derived candidate is discarded.

- Padding with zero motion vectors

If the number of candidates in affine merge candidate list is less than 5, zero motion vectors with zero reference indices are insert into the candidate list, until the list is full.

CE4.2.2 in JVET-L0368 (Huawei)

In this contribution, following simplifications for the common based of affine merge mode are applied:

- The pruning process for inherited affine candidates is simplified by comparing the coding units covering the neighboring positions, instead of comparing the derived affine candidates in VTM-2.0.1. Up to 2 inherited affine candidates are inserted into affine merge list. The pruning process for constructed affine candidates is totally removed.

- The MV scaling operation in constructed affine candidate is removed. If the reference indices of control points are different, the constructed motion model is discarded.

- The number of constructed affine candidates is reduced from 10 to 6. The combinations of constructed affine candidates are inserted into to candidate list as following order:

{CP1, CP2, CP3}, {CP1, CP2, CP4}, {CP1, CP3, CP4}, {CP2, CP3, CP4}, {CP1, CP2}, {CP1, CP3}

CE4.2.3 in JVET-L0088 (MediaTek)

In all the tests, the affine merge candidate list in the common base is removed, and the affine candidates are added to the merge candidate list. There are two kinds of affine candidates. The first one is spatial inherited affine candidate. The second one is constructed affine candidates.

Spatial inherited affine candidates and constructed affine candidates

Spatial inherited affine candidates use affine model from neighboring CUs located at A1, B1, B0, A0, and B2 to derive current CU’s affine model as shown in Figure 1. The maximum number of spatial inherited affine candidates is equal to 5. For the same neighboring block position, spatial inherited affine candidate is always checked first. If the affine candidate is added to the merge list, normal spatial candidate will not be checked. If the affine candidate is available but is pruned by other affine candidates, the normal spatial candidate will be inserted to the merge list without pruning if the MV is available.

To obtain constructed affine candidates, it is proposed to use neighboring MVs as control point MVs. In this contribution, two different constructed affine candidates are evaluated. The first candidate’s control point MVs of (CP1, CP2, CP3) are from MV set of blocks (B2, B1, A1) as shown in Figure 1. The second constructed affine candidate contains temporal MV in the collocated block RB, in which the control point MVs of (CP1, CP2, CP4) are from MV set of blocks (B2, B1, RB) as shown in Figure 1. Available motion triplets of spatial neighbors with the same reference picture are inserted. The second constructed affine candidate will be evaluated only if the first constructed affine candidate is not added to the merge list. The maximum number of constructed affine candidates is equal to 1.

The pruning process for spatial inherited affine candidates and constructed affine candidates are separated. Spatial inherited affine candidates will only be pruned by existing spatial inherited affine candidates. The pruning process for constructed affine candidates is unnecessary since there is at most one constructed affine candidate in the merge list.

The pruning process of spatial inherited affine candidates is done by checking the neighboring affine block’s top-left position. For affine candidate in position A1 and B1, they will not be checked for pruning. For affine candidate in B0, its top-left position will be checked; if the position is the same as the affine block in position B1, the candidate will be pruned. For affine candidate in A0, its top-left position will be checked; if the position is the same as the affine block in position A1, the candidate will be pruned. For affine candidate in B2, its top-left position will be checked, if the position is the same as the affine block in position B1 or position A1, the candidate will be pruned.

In this proposal, spatial inherited affine candidates and constructed affine candidates only consider 6-parameter affine model. Constructed affine candidates are inserted after spatial candidate at location B2.

Figure 1. Position for spatial inherited affine and constructed affine candidates

Line buffer reduction

Instead of allowing to use all MVs in the upper CTU row, it is proposed to reduce the line buffer requirement by using only one MV row above the current CTU row, as shown in Figure 2.

When deriving the affine candidates from the neighboring blocks, two control point MVs or two sub-block MVs of the neighboring blocks are used. For example, in Figure 2, for block A, the VA2 and VA3 are used to derive the 4-parameter affine model for current block. For block B, the VB2 and VB3 are used to derive the 4-parameter affine model for the current block.

Figure 2. MV buffer used in affine candidate derivation

CE4.2.4 in JVET-L0156 (Technicolor)

The new constructed temporal affine merge candidates are interlaced with the constructed spatial candidates, with a total of 16 candidates. The final number of candidates in the list is increased to 8.

| Final number of candidates | Max Number of inherited candidates | Max Number of constructed candidates (4 prms) | Max Number of constructed candidates (6 prms) |
| --- | --- | --- | --- |
| 8 | 5 | 12 | 16 |

The constructed spatial candidates for a 6 parameters model are modified as follows:

| Candidate number | Type | Corner number | Corner name |
| --- | --- | --- | --- |
| 1 | spatial | { 0, 1, 2 } | LT, RT, LB |
| 2 | temporal | { 0, 1, 2 } | LT, RT, LB |
| 3 | spatial | { 0, 1 } | LT, RT |
| 4 | temporal | { 0, 1, 3 } | LT, RT, RB |
| 5 | spatial | { 0, 2 } | LT, LB |
| 6 | temporal | { 0, 2, 3 } | LT, LB, RB |
| 7 | spatial | { 1, 2 } | RT, LB |
| 8 | temporal | { 1, 2, 3 } | RT, LB, RB |
| 9 | spatial | { 0, 1, 3 } | LT, RT, RB |
| 10 | temporal | { 0, 1 } | LT, RT |
| 11 | spatial | { 0, 2, 3 } | LT, LB, RB |
| 12 | temporal | { 0, 2 } | LT, LB |
| 13 | spatial | { 1, 2, 3 } | RT, LB, RB |
| 14 | temporal | { 1, 2 } | RT, LB |
| 15 | spatial | { 0, 3 } | LT, RB |
| 16 | temporal | { 0, 3 } | LT, RB |

While for a 4 parameters model, they are modified as follows:

| Candidate number | Type | Corner number | Corner name |
| --- | --- | --- | --- |
| 1 | spatial | { 0, 1 } | LT, RT |
| 2 | temporal | { 0, 1 } | LT, RT |
| 3 | spatial | { 0, 2 } | LT, LB |
| 4 | temporal | { 0, 2 } | LT, LB |
| 5 | spatial | { 1, 2 } | RT, LB |
| 6 | temporal | { 1, 2 } | RT, LB |
| 7 | spatial | { 0, 3 } | LT, RB |
| 8 | temporal | { 0, 3 } | LT, RB |
| 9 | spatial | { 1, 3 } | RT, RB |
| 10 | temporal | { 1, 3 } | RT, RB |
| 11 | spatial | { 2, 3 } | LB, RB |
| 12 | temporal | { 2, 3 } | LB, RB |

In addition to the test to reject pure translational candidates present in the common base, a test to reject spurious affine models is added, by checking the relative motion of the corners.

The complexity of the test consists in:

Comparing vector differences to a threshold. For 4 parameters model, it consists in 1 component comparisons by model, totaling 12 comparisons. For 6 parameters model, it consists in 2 comparisons by model, totaling 32 comparisons. As in complexity analysis table, 1 comparison consists in 2 vector component comparisons.

CE4.2.5 in JVET-L0278 (Qualcomm)

Test a: simplification of affine common base

In test a, when checking a neighboring affine coded block for inherited affine merge candidate, the PU index of the neighboring block is compared with previous checked neighboring block. If it’s not the same PU as in any of the previously checked neighboring block, the derived inherited affine merge candidate is directly added to the candidate list without any comparison with the existing candidates. When deriving the constructed affine merge candidate, the reference picture of all the control-point MVs should be the same, otherwise the constructed affine merge candidate is considered as unavailable. The constructed affine merge candidate is also directly inserted into the list without pruning.

Test b: move ATMVP candidate to affine merge list

Test b is based on test a. The ATMVP merge candidate in the normal merge list is moved to the first position of the affine merge list.

CE4.2.6 in JVET-L0142 (LGE)

Test a: Simplified inherited candidate

In this method, the maximum two inherited affine candidates are derived from affine motion model of the neighboring blocks and inserted into the candidate list. For the left predictor, the scan order is: {A, D, E} and for the above predictor, the scan order is: {B, C} as shown in Figure 1. Each predictor selects the only one inherited candidate. No pruning check in performed between two inherited candidates.

Figure 1: Spatial positions for inherited candidate

Test b: Simplified constructed candidate

In the common base for affine merge, the available combination of motion information of CPs is only considered when the CPs have the same reference index.

Test d: Reduced candidate number of candidate

In the available constructed candidates, the following candidates are not considered:

{CP2, CP3}, {CP1, CP4}, {CP2, CP4}, {CP3, CP4}

Test e: Line buffer restriction

In this method, to avoid the line buffer storage increase, inherited candidate is restricted to derive from the above block with CTU boundary.

CE4.2.8 in JVET-L0369 (Huawei)

In this contribution, a sub-block based merge candidate list is generated by moving ATMVP from regular merge list to the first location of enhanced affine merge list in JVET-L0368 [3]. The max number of the sub-block based merge list is kept as 5, and for regular merge list, the list length is reduced from 6 to 5.

The sub-block merge candidate list is constructed as following steps:

- Insert ATMVP candidate

- Insert inherited affine candidates

- Insert constructed affine candidates

- Padding with zero motion vectors

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-L0305 | CE4-related: History Based Affine Merge Candidate | J. Zhao, S. Paluri, S. Kim (LGE) |
| JVET-L0320 | CE4-related: affine merge mode with prediction offsets | G. Li, X. Xu, X. Li, S. Liu (Tencent) |
| JVET-L0322 | CE4-related: constrained model-based affine merge | G. Li, X. Xu, X. Li, S. Liu (Tencent) |
| JVET-L0389 | CE4-related: Control point MV offsets for Affine merge mode | Y.-C. Yang, Y.-J. Chang (Foxconn) |
| JVET-L0390 | CE4-related: Simplification of Affine merge common codebase | Y.-C. Yang, Y.-J. Chang (Foxconn) |
| JVET-L0522 | CE4-related : simplified constructed temporal affine merge candidates | F. Galpin, F. Leleannec, A. Robert (technicolor) |

CE4.3: Planar motion vector prediction

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.3.1.a | PMVP with 4x4 sub-block (for block width>=16 or height>=16) | JVET-L0070 |
| 4.3.1.b* | PMVP with 8x8 sub-block (for block width>=16 or height>=16) |  |
| 4.3.1.c* | PMVP with 8x8 sub-block (for block width>=16 and height>=16) |  |
| 4.3.1.d* | PMVP with adaptive 4x4/8x8 sub-block (for block width>=16 or height>=16) |  |
| 4.3.1.e* | PMVP with adaptive 4x4/8x8 sub-block (for block width>=16 and height>=16) |  |
| 4.3.2.a | 4.3.1.a + 4.2.8.b (used as anchor) |  |
| 4.3.2.b | Put 4.3.1.a candidate in 4.2.8.b affine merge list |  |
| 4.3.3.a | MV Planar prediction | JVET-L0413 |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.3.1.a | -0.63% | -0.57% | -0.77% | 104% | 114% | -0.48% | -0.47% | -0.61% | 105% | 108% |
| 4.3.1.b* | -0.53% | -0.48% | -0.62% | 103% | 103% | -0.28% | -0.28% | -0.22% | 104% | 100% |
| 4.3.1.c* | -0.47% | -0.42% | -0.54% | 102% | 103% | -0.27% | -0.26% | -0.25% | 102% | 101% |
| 4.3.1.d* | -0.53% | -0.48% | -0.66% | 104% | 102% | -0.43% | -0.33% | -0.50% | 105% | 100% |
| 4.3.1.e* | -0.48% | -0.40% | -0.56% | 102% | 103% | -0.33% | -0.27% | -0.26% | 103% | 104% |
| 4.3.2.a | -0.99% | -0.88% | -1.05% | 108% | 108% | -0.53% | -0.50% | -0.42% | 111% | 106% |
| 4.3.2.b | -0.92% | -0.79% | -0.94% | 105% | 108% | -0.56% | -0.50% | -0.49% | 106% | 106% |
| 4.3.3.a | -0.18% | -0.08% | -0.15% | 106% | 118% | -0.04% | 0.07% | 0.09% | 109% | 120% |

Complexity analysis

For test 4.3.1, the complexity comprises two part,

- MV scaling for deriving MV for each sub-block in the L-shaped area: (W/uSize+H/uSize+4)*2

- Calculation for deriving MV for each sub-block in a PU, in the same manner as intra planar mode

  - Additions: H/uSize * W/uSize *16 +( W/uSize+H/uSize+1)*8

  - Shifts: H/uSize * W/uSize *12+( W/uSize+ H/uSize+1)*8+1

Bi-directional prediction is assumed for the worst case. uSize denotes the sub-block size.

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.3.1.a | Yi-Wen Chen (Kwai) | 2 | F | Y | M |
| 4.3.1.b* | Yi-Wen Chen (Kwai) | 2 | F | Y | M |
| 4.3.1.c* | Yi-Wen Chen (Kwai) | 2 | F | Y | M |
| 4.3.1.d* | M. Gao(Tencent) | 2 | F | Y | M |
| 4.3.1.e* | M. Gao(Tencent) | 2 | F | Y | M |
| 4.3.2.a | A. Tamse (Samsung) | 2 | P | Y | M |
| 4.3.2.b | A. Tamse (Samsung) | 2 | P | Y | M |
| 4.3.3.a | K.Kazui (Fujitsu) | 2 | P | Y | M |

Detailed description of tests

CE4.3.1 & CE4.3.2 in JVET-L0070 (Hisilicon & Huawei)

Abstract of PMVP

- Separately signaled for each coding unit.

- Once there is two different MV in the L-shaped area, the mode is activated.

- Ref_idx in L0 and L1 is set to 0, all the MV in the L-shaped area is scaled if needed.

- In the case a neighboring block in the L-shaped area is intra coded, it is padded in the same manner as intra reference pixel padding, separately for L0 and L1.

- BR temporal motion is used for planar interpolation. If BR temporal motion is not available, exactly the same planar prediction for intra mode is applied.

- Adaptive sub-block size of 8x8 and 4x4 is signalled for each coding picture

CE4.3.3 in JVET-L0413 (NHK)

Abstract of MV planar prediction

- The interpolation is carried out in a similar way as intra planar prediction.

- In the case that neighboring CU is intra coded, the closest neighboring MV is substituted in a similar way as intra reference sample substitution.

- If the mv_palanr_flag is equal to 1, inter prediction index “inter_pred_idc” syntax and reference index “ref_idx” syntaxes for both L0 and L1 lists are additionally signaled.

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-L0319 | CE4-related: Sub-block MV clipping in planar motion vector prediction | M. Gao, X. Li, M. Xu, S. Liu (Tencent) |

CE4.4: Merge mode enhancement

Categorization & comparative study

Non-adjacent merge candidate

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.4.1.a | Spatial-temporal merge mode (non sub-block STMVP) | JVET-L0354 |
| 4.4.1.b* | 4.4.1.a + 4.4.7.b (HMVP for merge & AMVP) |  |
| 4.4.2.b | Adding non-adjacent candidate by searching round 1 and 2 with virtual block size (width + i * grid_width * 2, height + i * grid_height * 2) | JVET-L0323 |
| 4.4.2.c | 4.4.2.b + 4.4.7.a (HMVP for merge) | JVET-L0321 |
| 4.4.2.d | 4.4.2.b + 4.4.7.b (HMVP for merge&AMVP) |  |
| 4.4.3 | Non-adjacent spatial candidates with reduced line buffer and reduced search points | JVET-L0175 |
| 4.4.4.a | 4.4.2.a + line buffer reduction | JVET-L0089 |
| 4.4.4.b | 4.4.4.a + reference MV position rounding |  |
| 4.4.5.a | Merge mode modification, 15 checked spatial positions | JVET-L0430 |
| 4.4.5.c | Merge mode modification, 10 checked spatial positions |  |
| 4.4.6.a | Non-adjacent merge with no additional line buffer, check 18 potential candidates | JVET-L0399 |
| 4.4.6.b | Non-adjacent merge with no additional line buffer, check 8 potential candidates |  |
| 4.4.6.c | 4.4.6.a + non-sub-block STMVP |  |
| 4.4.6.d | 4.4.6.b + non-sub-block STMVP |  |
| 4.4.7.a | History-based motion vector prediction for merge | JVET-L0266 |
| 4.4.7.c | History-based motion vector prediction for merge & AMVP |  |

Simulation results

| Test# | Merge list size | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.4.1.a | 6 | -0.44% | -0.35% | -0.53% | 100% | 101% | -0.04% | 0.21% | 0.16% | 101% | 101% |
| 4.4.1.a | 7 | -0.52% | -0.43% | -0.57% | 100% | 101% | -0.07% | 0.14% | -0.09% | 102% | 101% |
| 4.4.1.a | 8 | -0.55% | -0.48% | -0.60% | 101% | 101% | -0.13% | 0.16% | 0.18% | 102% | 102% |
| 4.4.1.b |  |  |  |  |  |  |  |  |  |  |  |
| 4.4.2.b | 6 | -0.41% | -0.41% | -0.51% | 99% | 97% | -0.26% | -0.24% | -0.23% | 97% | 96% |
| 4.4.2.b | 8 | -0.56% | -0.65% | -0.77% | 94% | 90% | -0.41% | -0.56% | -0.55% | 93% | 90% |
| 4.4.2.b | 10 | -0.62% | -0.70% | -0.84% | 95% | 90% | -0.49% | -0.56% | -0.58% | 94% | 89% |
| 4.4.2.c | 6 | -0.58% | -0.67% | -0.79% | 92% | 89% | -0.31% | -0.35% | -0.40% | 91% | 87% |
| 4.4.2.c | 8 | -0.80% | -0.94% | -1.07% | 97% | 94% | -0.53% | -0.61% | -0.50% | 95% | 93% |
| 4.4.2.c | 10 | -0.88% | -0.99% | -1.18% | 94% | 89% | -0.59% | -0.58% | -0.49% | 92% | 87% |
| 4.4.2.d | 6 | -0.72% | -0.86% | -0.97% | 99% | 103% | -0.35% | -0.30% | -0.40% | 99% | 99% |
| 4.4.2.d | 8 | -0.94% | -1.14% | -1.26% | 102% | 104% | -0.54% | -0.64% | -0.70% | 101% | 104% |
| 4.4.2.d | 10 | -1.02% | -1.21% | -1.35% | 101% | 102% | -0.66% | -0.67% | -0.62% | 101% | 99% |
| 4.4.3 | 6 | -0.19% | -0.18% | -0.31% | 99% | 99% | -0.14% | -0.08% | -0.14% | 100% | 101% |
| 4.4.3 | 10 | -0.37% | -0.41% | -0.53% | 101% | 100% | -0.30% | -0.24% | -0.43% | 102% | 100% |
| 4.4.4.a | 8 | -0.56% | -0.65% | -0.77% | 100% | 101% | -0.41% | -0.56% | -0.55% | 100% | 102% |
| 4.4.4.b | 8 | -0.47% | -0.53% | -0.62% | 100% | 101% | -0.36% | -0.39% | -0.43% | 100% | 101% |
| 4.4.5.a | 6 |  |  |  |  |  |  |  |  |  |  |
| 4.4.5.a | 8 |  |  |  |  |  |  |  |  |  |  |
| 4.4.5.a | 10 |  |  |  |  |  |  |  |  |  |  |
| 4.4.5.c | 6 |  |  |  |  |  |  |  |  |  |  |
| 4.4.5.c | 8 |  |  |  |  |  |  |  |  |  |  |
| 4.4.5.c | 10 |  |  |  |  |  |  |  |  |  |  |
| 4.4.6.a | 6 | -0.48% | -0.51% | -0.62% | 99% | 100% | -0.28% | -0.29% | -0.23% | 85% | 91% |
| 4.4.6.a | 8 |  |  |  |  |  | -0.42% | -0.35% | -0.36% | 85% | 91% |
| 4.4.6.a | 10 | -0.76% | -0.81% | -0.99% | 100% | 100% | -0.52% | -0.41% | -0.57% | 86% | 90% |
| 4.4.6.b | 6 | -0.40% | -0.41% | -0.49% | 99% | 100% | -0.23% | -0.22% | -0.28% | 85% | 91% |
| 4.4.6.b | 8 |  |  |  |  |  |  |  |  |  |  |
| 4.4.6.b | 10 | -0.61% | -0.65% | -0.80% | 100% | 100% | -0.50% | -0.43% | -0.56% | 87% | 90% |
| 4.4.6.c | 6 | -0.82% | -0.77% | -0.90% | 99% | 101% | -0.19% | 0.08% | 0.06% | 100% | 100% |
| 4.4.6.c | 8 | -1.07% | -1.00% | -1.17% | 100% | 101% | -0.41% | 0.04% | 0.02% | 100% | 99% |
| 4.4.6.c | 10 | -1.19% | -1.20% | -1.34% | 101% | 101% | -0.53% | -0.06% | -0.12% | 101% | 100% |
| 4.4.6.d | 6 | -0.76% | -0.71% | -0.85% | 99% | 101% | -0.14% | 0.11% | 0.15% | 100% | 100% |
| 4.4.6.d | 8 | -0.97% | -0.96% | -1.08% | 100% | 101% | -0.36% | 0.08% | 0.09% | 101% | 100% |
| 4.4.6.d | 10 | -1.10% | -1.08% | -1.25% | 101% | 101% | -0.46% | -0.10% | -0.10% | 101% | 100% |
| 4.4.7.a | 6 | -0.43% | -0.51% | -0.63% | 102% | 102% | -0.18% | -0.27% | -0.35% | 100% | 102% |
| 4.4.7.a | 8 | -0.59% | -0.78% | -0.88% | 102% | 102% | -0.32% | -0.32% | -0.60% | 101% | 102% |
| 4.4.7.a | 10 | -0.65% | -0.77% | -0.91% | 102% | 102% | -0.35% | -0.52% | -0.71% | 102% | 102% |
| 4.4.7.c | 6 | -0.58% | -0.72% | -0.82% | 100% | 103% | -0.23% | -0.27% | -0.21% | 100% | 98% |
| 4.4.7.c | 8 | -0.72% | -0.89% | -1.02% | 100% | 103% | -0.38% | -0.42% | -0.57% | 101% | 103% |
| 4.4.7.c | 10 | -0.78% | -0.98% | -1.12% | 101% | 103% | -0.39% | -0.53% | -0.64% | 101% | 102% |

Complexity analysis

| Test# | Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.4.1.a | 6 | 7 | 20 | 8 | 1 | 256 | 8 |
| 4.4.1.a | 7 | 7 | 21 | 8 | 1 | 256 | 8 |
| 4.4.1.a | 8 | 7 | 21 | 8 | 1 | 256 | 8 |
| 4.4.1.b |  |  |  |  |  |  |  |
| 4.4.2.b | 6 |  |  |  |  |  |  |
| 4.4.2.b | 8 |  |  |  |  |  |  |
| 4.4.2.b | 10 |  |  |  |  |  |  |
| 4.4.2.c | 6 | <21 | <=30 |  |  |  |  |
| 4.4.2.c | 8 | <21 | <=30 |  |  |  |  |
| 4.4.2.c | 10 | <21 | <=30 |  |  |  |  |
| 4.4.2.d | 6 | <21 | <=30 |  |  |  |  |
| 4.4.2.d | 8 | <21 | <=30 |  |  |  |  |
| 4.4.2.d | 10 | <21 | <=30 |  |  |  |  |
| 4.4.3 | 6 | 9 |  |  |  |  |  |
| 4.4.3 | 10 | 9 |  |  |  |  |  |
| 4.4.4.a | 8 |  |  |  |  |  |  |
| 4.4.4.b | 8 |  |  |  |  |  |  |
| 4.4.5.a | 6 |  |  |  |  |  |  |
| 4.4.5.a | 8 |  |  |  |  |  |  |
| 4.4.5.a | 10 |  |  |  |  |  |  |
| 4.4.5.c | 6 |  |  |  |  |  |  |
| 4.4.5.c | 8 |  |  |  |  |  |  |
| 4.4.5.c | 10 |  |  |  |  |  |  |
| 4.4.6.a | 6 | 18 | 107 | 0 | 1 | 0 | 18 |
| 4.4.6.a | 8 | 18 | 143 | 0 | 1 | 0 | 18 |
| 4.4.6.a | 10 | 18 | 175 | 0 | 1 | 0 | 18 |
| 4.4.6.b | 6 | 8 | 57 | 0 | 1 | 0 | 8 |
| 4.4.6.b | 8 | 8 | 73 | 0 | 1 | 0 | 8 |
| 4.4.6.b | 10 | 8 | 85 | 0 | 1 | 0 | 8 |
| 4.4.6.c | 6 | 18 | 122 | 10 | 1 | 0 | 18 |
| 4.4.6.c | 8 | 18 | 164 | 10 | 1 | 0 | 18 |
| 4.4.6.c | 10 | 18 | 202 | 10 | 1 | 0 | 18 |
| 4.4.6.d | 6 | 8 | 72 | 10 | 1 | 0 | 8 |
| 4.4.6.d | 8 | 8 | 94 | 10 | 1 | 0 | 8 |
| 4.4.6.d | 10 | 8 | 112 | 10 | 1 | 0 | 8 |
| 4.4.7.a | 6 | 11 | 21 | 2 | 1 | 0 | 0 |
| 4.4.7.a | 8 | 11 | 29 | 2 | 1 | 0 | 0 |
| 4.4.7.a | 10 | 11 | 34 | 2 | 1 | 0 | 0 |
| 4.4.7.c | 6 | 11 | 21 | 2 | 1 | 0 | 0 |
| 4.4.7.c | 8 | 11 | 29 | 2 | 1 | 0 | 0 |
| 4.4.7.c | 10 | 11 | 34 | 2 | 1 | 0 | 0 |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.4.1.a | Yi-Wen Chen (Kwai) | 2 | F | Y | M |
| 4.4.1.a | Yi-Wen Chen (Kwai) | 2 | F | Y | M |
| 4.4.1.a |  |  |  |  |  |
| 4.4.1.b |  |  |  |  |  |
| 4.4.2.b |  |  |  |  |  |
| 4.4.2.b |  |  |  |  |  |
| 4.4.2.b |  |  |  |  |  |
| 4.4.2.c | Xiaozhen Zheng (DJI) | 2 | F | Y | G |
| 4.4.2.c | Xiaozhen Zheng (DJI) | 2 | F | Y | G |
| 4.4.2.c | Xiaozhen Zheng (DJI) | 2 | F | Y | G |
| 4.4.2.d | Xiaozhen Zheng (DJI) | 2 | F | Y | M |
| 4.4.2.d | Xiaozhen Zheng (DJI) | 2 | F | Y | M |
| 4.4.2.d | Xiaozhen Zheng (DJI) | 2 | F | Y | M |
| 4.4.3 |  |  |  |  |  |
| 4.4.3 |  |  |  |  |  |
| 4.4.4.a | G. Li (Tencent) | 2 | F | Y | M |
| 4.4.4.b | G. Li (Tencent) | 2 | F | Y | M |
| 4.4.5.a |  |  |  |  |  |
| 4.4.5.a |  |  |  |  |  |
| 4.4.5.a |  |  |  |  |  |
| 4.4.5.c |  |  |  |  |  |
| 4.4.5.c |  |  |  |  |  |
| 4.4.5.c |  |  |  |  |  |
| 4.4.6.a |  |  |  |  |  |
| 4.4.6.a |  |  |  |  |  |
| 4.4.6.a |  |  |  |  |  |
| 4.4.6.b |  |  |  |  |  |
| 4.4.6.b |  |  |  |  |  |
| 4.4.6.b |  |  |  |  |  |
| 4.4.6.c |  |  |  |  |  |
| 4.4.6.c |  |  |  |  |  |
| 4.4.6.c |  |  |  |  |  |
| 4.4.6.d |  |  |  |  |  |
| 4.4.6.d |  |  |  |  |  |
| 4.4.6.d |  |  |  |  |  |
| 4.4.7.a |  |  |  |  |  |
| 4.4.7.a |  |  |  |  |  |
| 4.4.7.a |  |  |  |  |  |
| 4.4.7.c | M. Gao (Tencent) | 2 | F | Y | M |
| 4.4.7.c | M. Gao (Tencent) | 2 | F | Y | M |
| 4.4.7.c | M. Gao (Tencent) | 2 | F | Y | M |

Other merge candidates

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.4.8.a | Merge offset extension with 4 candidates | JVET-L0176 |
| 4.4.8.b | Merge offset extension with 8 candidates |  |
| 4.4.8.c* | Merge offset extension with 16 candidates |  |
| 4.4.8.d* | Merge offset extension with 4 candidates, encoder speed up |  |
| 4.4.8.e* | Merge offset extension with 8 candidates, encoder speed up |  |
| 4.4.10.a | Add merge candidates at middle position for inter merge | JVET-L0315 |
| 4.4.10.b | Add new AMVP predictor candidates from middle position |  |
| 4.4.10.c | 4.4.10.a + 4.4.10.b |  |
| 4.4.10.e | 4.4.10.c + 4.4.2.b (non-adjacent merge) |  |
| 4.4.11.a | Combined average merge candidates | JVET-L0071 |
| 4.4.11.b* | 4.4.11.a + duplication checking |  |
| 4.4.11.c* | 4.4.11.a + no MV scaling |  |
| 4.4.11.d* | 4.4.11.c + duplication checking |  |
| 4.4.12.a | Pairwise average candidates without HEVC combined candidate (list size 6/8/10) | JVET-L0090 |
| 4.4.12.b | Pairwise average candidates with HEVC combined candidate (list size 6/8/10) |  |
| 4.4.12.c* | 4.4.12.b + 4.4.7.a (list size 6/8/10) |  |
| 4.4.12.d* | 4.4.12.b with 3 pairs + 4.4.7.a (list size 6/8/10) |  |

Simulation results

| Test# | Merge list size | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.4.8.a | 6 | -0.49% | -0.55% | -0.68% | 108% | 99% | -0.19% | -0.45% | -0.52% | 108% | 100% |
| 4.4.8.b | 6 | -0.62% | -0.68% | -0.85% | 115% | 100% |  |  |  |  |  |
| 4.4.8.c* | 6 | -0.78% | -0.88% | -1.06% | 106% | 98% | -0.17% | -0.37% | -0.51% | 107% | 99% |
| 4.4.8.e* | 6 |  |  |  |  |  |  |  |  |  |  |
| 4.4.10.a | 6 | -0.03% | -0.01% | -0.05% | 102% | 101% | -0.06% | -0.04% | -0.29% | 99% | 96% |
| 4.4.10.b | 6 | -0.02% | -0.01% | -0.06% | 101% | 101% | -0.07% | -0.17% | -0.31% | 99% | 98% |
| 4.4.10.c | 6 | -0.05% | -0.02% | -0.09% | 101% | 101% | -0.10% | -0.09% | -0.26% | 100% | 98% |
| 4.4.10.e | 8 | -0.73% | -0.83% | -1.00% | 102% | 102% | -0.51% | -0.41% | -0.57% | 103% | 101% |
| 4.4.11.a | 6 | -0.37% | -0.20% | -0.31% | 100% | 100% | 0.07% | 0.26% | 0.28% | 100% | 101% |
| 4.4.11.a | 8 | -0.56% | -0.37% | -0.49% | 101% | 100% | -0.11% | 0.15% | 0.00% | 100% | 94% |
| 4.4.11.b* | 6 | -0.40% | -0.25% | -0.36% | 100% | 99% | -0.03% | 0.21% | 0.19% | 100% | 100% |
| 4.4.11.c* | 6 | -0.40% | -0.22% | -0.30% | 100% | 99% | 0.00% | 0.25% | 0.06% | 100% | 99% |
| 4.4.11.c* | 8 | -0.56% | -0.37% | -0.46% | 100% | 100% |  |  |  |  |  |
| 4.4.11.c* | 10 | -0.62% | -0.42% | -0.54% | 100% | 100% |  |  |  |  |  |
| 4.4.11.d* | 6 | -0.45% | -0.30% | -0.38% | 99% | 99% | -0.17% | -0.02% | -0.09% | 99% | 100% |
| 4.4.12.a | 6 | -0.38% | -0.21% | -0.31% | 100% | 101% | 0.01% | 0.14% | 0.13% | 101% | 102% |
| 4.4.12.a | 8 | -0.54% | -0.37% | -0.42% | 101% | 100% | -0.07% | 0.08% | 0.01% | 101% | 102% |
| 4.4.12.a | 10 | -0.61% | -0.38% | -0.54% | 101% | 101% | -0.16% | -0.12% | -0.15% | 102% | 102% |
| 4.4.12.b | 6 | -0.40% | -0.23% | -0.34% | 100% | 101% | 0.05% | 0.21% | 0.29% | 101% | 102% |
| 4.4.12.b | 8 | -0.61% | -0.43% | -0.56% | 101% | 101% | -0.11% | 0.11% | 0.01% | 102% | 102% |
| 4.4.12.b | 10 | -0.69% | -0.50% | -0.64% | 101% | 102% | -0.14% | 0.04% | -0.10% | 102% | 101% |
| 4.4.12.c* | 6 | -0.76% | -0.72% | -0.81% | 99% | 104% | -0.17% | -0.01% | -0.01% | 100% | 102% |
| 4.4.12.c* | 8 | -0.93% | -0.89% | -1.11% | 100% | 102% | -0.32% | -0.16% | -0.24% | 100% | 101% |
| 4.4.12.c* | 10 | -1.05% | -1.03% | -1.22% | 100% | 102% | -0.39% | -0.35% | -0.35% | 100% | 99% |
| 4.4.12.d* | 6 | -0.76% | -0.72% | -0.81% | 99% | 100% | -0.17% | -0.01% | -0.01% | 99% | 98% |
| 4.4.12.d* | 8 | -0.92% | -0.92% | -1.12% | 99% | 99% | -0.30% | -0.23% | -0.37% | 100% | 99% |
| 4.4.12.d* | 10 | -1.07% | -1.17% | -1.22% | 100% | 100% | -0.40% | -0.31% | -0.40% | 100% | 98% |

Complexity analysis

| Test# | Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.4.8.a | 6 | 10 | 15 | 2 | 1 | 0 | 0 | 10 |
| 4.4.8.b | 6 | 14 | 15 | 2 | 1 | 0 | 0 | 14 |
| 4.4.8.c* | 6 | 22 | 15 | 2 | 1 | 0 | 0 | 22 |
| 4.4.8.d* | 6 | 10 | 15 | 2 | 1 | 0 | 0 | 10 |
| 4.4.8.e* | 6 | 14 | 15 | 2 | 1 | 0 | 0 | 14 |
| 4.4.10.a | 6 | 8 | 25 | 0 | 1 | 0 | 8 | 8 |
| 4.4.10.b | 6 | 8 | 25 | 0 | 1 | 0 | 8 | 8 |
| 4.4.10.c | 6 | 8 | 25 | 0 | 1 | 0 | 8 | 8 |
| 4.4.10.e | 8 | 22 | 30 | 0 | 1 | 0 | 22 | 22 |
| 4.4.11.a | 6 | 12 | 15 | 6 | 1 | 0 | 6 | 12 |
| 4.4.11.a | 8 | 12 | 15 | 6 | 1 | 0 | 6 | 12 |
| 4.4.11.b* | 6 | 12 | 27 | 6 | 1 | 0 | 6 | 12 |
| 4.4.11.c* | 6 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.11.c* | 8 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.11.c* | 10 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.11.d* | 6 | 12 | 27 | 2 | 1 | 0 | 6 | 12 |
| 4.4.12.a | 6 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.12.a | 8 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.12.a | 10 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.12.b | 6 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.12.b | 8 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.12.b | 10 | 12 | 15 | 2 | 1 | 0 | 6 | 12 |
| 4.4.12.c* | 6 | / | / | / | / | / | / | / |
| 4.4.12.c* | 8 | / | / | / | / | / | / | / |
| 4.4.12.c* | 10 | / | / | / | / | / | / | / |
| 4.4.12.d* | 6 | / | / | / | / | / | / | / |
| 4.4.12.d* | 8 | / | / | / | / | / | / | / |
| 4.4.12.d* | 10 | / | / | / | / | / | / | / |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.4.8.a |  |  |  |  |  |
| 4.4.8.b |  |  |  |  |  |
| 4.4.8.c* |  |  |  |  |  |
| 4.4.8.d* |  |  |  |  |  |
| 4.4.8.e* |  |  |  |  |  |
| 4.4.10.a | T. Zhou (Sharp) | 2 | F | Y | M |
| 4.4.10.b | T. Zhou (Sharp) | 2 | F | Y | M |
| 4.4.10.c | T. Zhou (Sharp) | 2 | F | Y | M |
| 4.4.10.e | T. Zhou (Sharp) | 2 | F | Y | M |
| 4.4.11.a | F. Chen(Hikvision) |  |  |  |  |
| 4.4.11.a | F. Chen(Hikvision) |  |  |  |  |
| 4.4.11.b* | F. Chen(Hikvision) |  |  |  |  |
| 4.4.11.c* | J. An(Alibaba) | 2 | F | Y | M |
| 4.4.11.c* |  |  |  |  |  |
| 4.4.11.c* |  |  |  |  |  |
| 4.4.11.d* | J. An(Alibaba) | 2 | F | Y | M |
| 4.4.12.a | F. Le Léannec (Technicolor) | 2 | F | Y | M |
| 4.4.12.a | F. Le Léannec (Technicolor) | 2 | F | Y | M/M |
| 4.4.12.a | F. Le Léannec (Technicolor) | 2 | F | Y | M/M |
| 4.4.12.b | F. Le Léannec (Technicolor) | 2 | F | Y | M/M |
| 4.4.12.b | F. Le Léannec (Technicolor) | 2 | F | Y | M/M |
| 4.4.12.b | F. Le Léannec (Technicolor) | 2 | F | Y | M/M |
| 4.4.12.c* |  |  |  |  |  |
| 4.4.12.c* |  |  |  |  |  |
| 4.4.12.c* |  |  |  |  |  |
| 4.4.12.d* |  |  |  |  |  |
| 4.4.12.d* |  |  |  |  |  |
| 4.4.12.d* |  |  |  |  |  |

Merge list reordering

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.4.13.a | Block shape based candidate reordering | JVET-L0186 |
| 4.4.13.b | 4.4.13.a + 4.5.4.b (UMVE with two base merge candidates) |  |
| 4.4.13.c | 4.4.13.a + 4.4.7.a (HMVP for merge) |  |
| 4.4.13.d | 4.4.13.a + 4.4.7.c (HMVP for merge & AMVP) |  |
| 4.4.13.e* | Simplified block shaped base candidate reordering |  |
| 4.4.13.f* | 4.4.13.e + 4.5.4.b (UMVE with two base merge candidates) |  |
| 4.4.13.g* | 4.4.13.e + 4.4.7.a (HMVP for merge) |  |
| 4.4.13.h* | 4.4.13.e + 4.4.7.c (HMVP for merge & AMVP) |  |
| 4.4.14.a | Ranking based spatial merge candidate list for inter prediction | JVET-L0318 |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.4.13.a | -0.07% | -0.04% | -0.14% | 99% | 97% | -0.15% | -0.16% | -0.20% | 99% | 96% |
| 4.4.13.b | -1.37% | -1.22% | -1.37% | 106% | 95% | -0.49% | -0.50% | -0.51% | 104% | 95% |
| 4.4.13.c | -0.50% | -0.65% | -0.69% | 99% | 107% | -0.32% | -0.22% | -0.29% | 98% | 97% |
| 4.4.13.d | -0.64% | -0.78% | -0.88% | 97% | 97% | -0.40% | -0.43% | -0.51% | 98% | 97% |
| 4.4.13.e* | -0.06% | -0.05% | -0.15% | 99% | 97% | -0.17% | -0.24% | -0.21% | 99% | 96% |
| 4.4.13.f* | -1.37% | -1.19% | -1.36% | 107% | 104% | -0.49% | -0.43% | -0.56% | 104% | 99% |
| 4.4.13.g* | -0.50% | -0.62% | -0.70% | 99% | 107% | -0.34% | -0.30% | -0.40% | 98% | 97% |
| 4.4.13.h* | -0.65% | -0.81% | -0.89% | 98% | 97% | -0.38% | -0.43% | -0.39% | 98% | 97% |
| 4.4.14.a | -0.24% | -0.29% | -0.35% | 101% | 104% | -0.36% | -0.15% | -0.14% | 101% | 102% |

Complexity analysis

| Test# | Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.4.13.a | 6 | 6 | 15 | 2 | 1 | 0 | 6 |  |
| 4.4.13.b |  |  |  |  |  |  |  |  |
| 4.4.13.c |  |  |  |  |  |  |  |  |
| 4.4.13.d |  |  |  |  |  |  |  |  |
| 4.4.13.e* | 6 | 6 | 15 | 2 | 1 | 0 | 6 |  |
| 4.4.13.f* |  |  |  |  |  |  |  |  |
| 4.4.13.g* |  |  |  |  |  |  |  |  |
| 4.4.13.h* |  |  |  |  |  |  |  |  |
| 4.4.14.a | 6 | <130 | >129 | 0 | 1 | 0 | 129+1 | sorting |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.4.13.a |  |  |  |  |  |
| 4.4.13.b |  |  |  |  |  |
| 4.4.13.c |  |  |  |  |  |
| 4.4.13.d |  |  |  |  |  |
| 4.4.13.e* |  |  |  |  |  |
| 4.4.13.f* |  |  |  |  |  |
| 4.4.13.g* |  |  |  |  |  |
| 4.4.13.h* |  |  |  |  |  |
| 4.4.14.a | Y. Hsiao (MediaTek) | 2 | F | Y | M |

Detailed description of tests

CE4.4.1 in JVET-L0354 (Sharp)

The proposed method derives two spatial neighbouring motion information and one temporal neighbouring motion information and generates a spatial-temporal candidate. The generated STMVP candidate is placed after ATMVP candidate.

For the spatial neighbouring positions, at most two positions is checked for each above and left.

Afar: (nPbW * 2, -1), A1 (nPbW-1, -1)

Lfar: (-1, nPbH * 2), B1 (-1, nPbH-1)

For the temporal neighbouring position, the same position as VTM / HEVC collocated position is used.

The motion vector of neighbouring blocks are scaled into the first reference picture of each reference picture lists.

The generation of spatial temporal merge candidate is an average of the derived motion vectors. If three motion information is available, the following apply.

mvLX[0] = ((mvLX_A[0] + mvLX_L[0] + mvLX_C[0]) * 43) / 128

mvLX[1] = ((mvLX_A[1] + mvLX_L[1] + mvLX_C[1]) * 43) / 128

Note: If non of spatial inter-prediction blocks are available, the STMVP mode is off.

CE4.4.2 in JVET-L0323 (Tencent)

This contribution proposes to derive the additional merge candidates from the positions non-adjacent to the current block using the same pattern as that in HEVC. To achieve this, for each search round i, a virtual block is generated based on the current block as follows:

First, the relative position of the virtual block to the current is calculated by:

Offsetx =-i*gridX, Offsety = -i*gridY

where the Offsetx and Offsety denote the offset of the top-left corner of the virtual block relative to the top-left corner of the current block, gridX and gridY are the width and height of the search grid.

Second, the width and height of the virtual block are calculated by:

newWidth = i*2*gridX+ currWidth    newHeight = i*2*gridY + currHeight.

where the currWidth and currHeight are the current block width and height. The newWidth and newHeight are the new block width and height.

gridX and gridY are currently set to currWidth and currHeight, respectively.

Figure. 2 illustrates the relationship between the virtual block and the current block.

Figure 2. Illustration of virtual block in the ith search round.

After generating the virtual block, the blocks Ai, Bi, Ci, Di and Ei can be regarded as the HEVC spatial neighboring blocks of the virtual block and their positions are obtained with the same pattern as that in HEVC. Obviously, the virtual block is the current block if the search round i is 0. In this case, the blocks Ai, Bi, Ci, Di and Ei are the spatially neighboring blocks that are used in HEVC merge mode.

When constructing the merge candidate list, the pruning is performed to guarantee each element in merge candidate list to be unique. As more and more blocks will be checked to derive additional merge candidates, the pruning number is correspondingly increased. To limit the pruning number in the worst case, the maximum number of pruning that is allowed in merge list construction is constrained to a pre-defined value MaxPruningNum.

CE4.4.3 in JVET-L0175 (Hisilicon)

In this proposed method, more spatial positions are checked as shown in Figure 2. The extended spatial positions from 6 to 27 are checked according to their numerical order after the temporal candidate. In order to save the MV line buffer, all non-adjacent spatial positions above the current CU is removed.

Figure 2. Positions of extended spatial candidates in the proposed method.

The merge candidate list is constructed as follows:

- Add the spatial positions from position 1 to 5, ATMVP candidates, and temporal candidate, which is the same as in current JEM; (include redundancy check)

- Add the extended spatial positions from position 6 to 14; (include redundancy check)

CE4.4.4 in JVET-L0089 (MediaTek)

CTU boundary constraint

A CTU boundary constraint is applied to non-adjacent spatial merge candidates. MVs belonging to the above/left/above-left CTU and non-adjacent to the current CTU are replaced with MVs belonging to the above/left/above-left CTU and adjacent to the current CTU. As shown in Figure 1, the MV at block A is fetched as an alternative to MV at block B. This CTU boundary constraint also used in CE4.4.2.

Figure 1. Example of CTU boundary constraint; current CU size = 16x16

Reference MV position rounding and 2-CTU constraint

Similar to the temporal MV compression in HEVC that only stores MV data of the top left 4x4 block in each 16x16 block of a reference picture, only the MV data of the top left 4x4 block in each 8x8 block of the current picture are stored for future use of MV coding. In addition, non-adjacent spatial MVs outside the current CTU and left CTU are not used. Figure 2 shows an example.

Figure 2. Example of reference MV position rounding and 2-CTU constraint; current CU size = 16x16

CE4.4.5 in JVET-L0430 (Huawei)

In this proposed method, more spatial positions are checked as shown in Figure The candidates are checked from inside to the outside. And the order of the candidates is Ai, Bi, Ci, Di, Ei. These candidates are added after TMVP candidates and before the combined Bi-predictive candidates.

CE4.4.6 in JVET-L0399 (Qualcomm)

Figure: Spatial neighboring blocks used to derive the spatial merge candidates.

The pattern of spatial merge candidates is shown in Figure. The distances between non-adjacent spatial candidates and current coding block are based on the width and height of current coding block as shown in the function below.

offsetX=2*CUwidth,  coffsetY = 2*CUheight

The line buffer restriction is described as below:

offsetYUpperCTURow=((posLT.y≫log2MaxCodingBlockSize)<<                                                    log2MaxCodingBlockSize)-minPUSize-posLT.y

offsetXLeftCTUColumn=((posLT.x>>log2MaxCodingBlockSize)<<                                                       log2MaxCodingBlockSize)-minPUSize-posLT.x

offsetY = offsetY<offsetUpperCTURow ? offsetUpperCTURow : offsetY

offsetX = offsetX<offsetLeftCTUColumn ? offsetLeftCTUColumn : offsetX

Test4.4.6.a Non-adjacent spatial merge candidates checking 18 potential position

Merge candidates list is as below (new candidates are highlight with underline):

- Spatial candidates for blocks 1-4.

- ATMVP.

- Spatial candidate (block 5).

- TMVP candidate.

- Non-adjacent spatial candidates (blocks 6 to 20).

- Combined candidates.

- Zero candidates.

Test4.4.6.b Non-adjacent spatial merge candidates checking 8 potential position

Merge candidates list is as below (new candidates are highlight with underline):

- Spatial candidates for blocks 1-4.

- ATMVP.

- Spatial candidate (block 5).

- TMVP candidate.

- Non-adjacent spatial candidates (blocks 6 to 13).

- Combined candidates.

- Zero candidates.

Test4.4.6.c a + non-sub-PU STMVP

Merge candidates list is as below (new candidates are highlight with underline):

- Spatial candidates for blocks 1-4.

- ATMVP.

- Non-sub PU STMVP.

- Spatial candidate (block 5).

- TMVP candidate.

- Non-sub PU STMVP and non-adjacent spatial candidates (blocks 6 to 20).

- Combined candidates.

- Zero candidates.

Test4.4.6.d c + non-sub-PU STMVP

Merge candidates list is as below (new candidates are highlight with underline):

- Spatial candidates for blocks 1-4.

- ATMVP.

- Non-sub PU STMVP.

- Spatial candidate (block 5).

- TMVP candidate.

- Non-sub PU STMVP and non-adjacent spatial candidates (blocks 6 to 13).

- Combined candidates.

- Zero candidates.

CE4.4.7 in JVET-L0266 (ByteDance)

Workflow of HMVP

Construction of history FIFO table

A table with multiple HMVP candidates is maintained during the encoding/decoding process. The table is emptied when a new slice is encountered. Whenever there is an inter-coded non-affine block, the associated motion information is added to the last entry of the table as a new HMVP candidate.

In this contribution, the table size S is set to be 6, which indicates up to 6 HMVP candidates may be added to the table. When inserting a new motion candidate to the table, a constrained FIFO rule is utilized wherein redundancy check is firstly applied to find whether there is an identical HMVP in the table. If found, the identical HMVP is removed from the table and all the HMVP candidates afterwards are moved forward, i.e., with indices reduced by 1.

Modification of merge

HMVP candidates could be used in the merge candidate list construction process. The latest several HMVP candidates in the table are checked in order and inserted to the candidate list after the TMVP candidate. Pruning is applied on the HMVP candidates to the spatial or temporal merge candidate excluding sub-block motion candidate (i.e., ATMVP).

To reduce the number of pruning operations, three simplifications are introduced:

- Number of HMPV candidates to be check denoted by L is set as follows:

| L = (N <=4 ) ? M: (8 - N) | (1) |
| --- | --- |

wherein N indicates number of available non-sub block merge candidate and M indicates number of available HMVP candidates in the table.

- In addition, once the total number of available merge candidates reaches the signaled maximally allowed merge candidates minus 1, the merge candidate list construction process is terminated.

- Moreover, the number of pairs for combined bi-predictive merge candidate derivation is reduced from 12 to 6.

Modification of AMVP

HMVP candidates could also be used in the AMVP candidate list construction process. The motion vectors of the last K HMVP candidates in the table are inserted after the TMVP candidate. Only HMVP candidates with the same reference picture as the AMVP target reference picture are used to construct the AMVP candidate list. Pruning is applied on the HMVP candidates. In this contribution, K is set to 4 while the AMVP list size is kept unchanged, i.e., equal to 2.

CE4.4.8 in JVET-L0176 (Hisilicon)

In Merge offset extension, more candidates are checked based on existed candidate. New candidates has mv offset to the mv of existed candidate. New extended mv offset candidates are constructed only based on the first candidate of merge candidate list.

- Additional 4 (cross shape offset) or 8 (cross plus X shape offsets) merge candidates are added.

- Construction of VTM merge list is not touched.

- New candidates are appended to the end of the current merge list.

- No additional list construction efforts.

- No additional candidate pruning.

CE4.4.10 in JVET-L0315 (Tencent)

Two new spatial candidate positions are included in the proposed method. As depicted in Figure, the new candidates are A2, left-middle candidate, and B3, above-middle candidate. These two candidates are added into both merge candidate list and AMVP candidate list. For both list, the number of allowed candidates, as well as the number of full RD evaluations, is kept unchanged.

Test CE4.4.10.a: Candidate extension in Inter Merge Mode

In inter merge mode, the order of spatial merge candidate derivation in the proposed method is {A2, B3, A1, B1, B0, A0, B2}. Pruning is applied to remove redundant merge candidates. And the derivation process stops when maximum number of merge candidates is met.

Test CE4.4.10.b: Candidate extension in AMVP mode

In AMVP mode, the order of derivation of spatial motion vector predictor candidates on left side is

{A2, A0, A1, Scaled A2, Scaled A0, Scaled A1};

and the order of derivation on above side is

{B3, B0, B1, B2, Scaled B3, Scaled B0, Scaled B1, Scaled B2}.

Test CE4.4.10.c: Candidate extension for both Merge and AMVP mode

Test CE4.4.10.c is a combination of CE4.4.10.a and CE4.4.10.b, which applies extended candidates in middle positions to both merge and AMVP candidate derivation processes.

Combined Test with CE4.4.2.b

Non-adjacent spatial candidates are proposed in CE4.4.2, which doesn’t include candidates from middle positions. In this combined test, middle-left and above-middle candidates are added to each round of search based on CE4.4.2.b. And similar to CE4.4.10, all the extended spatial candidates are used for both merge candidates and AMVP candidates.

Let R denote the R-th round of search, candidate positions are as depicted in Figure 3, the order of derivation in the R-th round of search on the left side is:

{AR, FR, CR, Scaled AR, Scaled FR, Scaled CR};

and the order of derivation in the R-th round of search on above side is:

{BR, ER, DR, GR, Scaled BR, Scaled ER, Scaled DR, Scaled GR}.

CE4.4.11 in JVET-L0071 (Hisilicon)

Combined average merge candidates are generated as follows.

- Up to 4 candidates in the merge list is used for deriving new candidates

- The reference picture in L0 and L1 is decided by voting, separately

- Combination of the 4 candidates is performed to derive 6 pair candidates

- For each pair of candidates, L0 and L1 motion vectors are averaged separately

- The existing combined candidates are removed

CE4.4.12 in JVET-L0090 (MediaTek)

Pairwise average candidates are generated by averaging predefined pairs of candidates in the current merge candidate list, and the predefined pairs are defined as {(0, 1), (0, 2), (1, 2), (0, 3), (1, 3), (2, 3)}, where the numbers denote the merge indices to the merge candidate list. The averaged motion vectors are calculated separately for each reference list. If both motion vectors are available in one list, these two motion vectors are averaged even when they point to different reference pictures; if only one motion vector is available, use the one directly; if no motion vector is available, keep this list invalid. The pairwise average candidates replaces the combined candidates in HEVC standard.

CE4.4.13 in JVET-L0186 (HikVision)

The proposed candidate order for Merge mode

| Width> height | B1A1A0B0 |
| --- | --- |
| Width=height | B1A1B0A0 |
| Width<height | A1B1B0A0 |

A simplified candidate order for merge mode is reported in this contribution as well.

| Width>= height | B1A1A0B0 |
| --- | --- |
| Width<height | A1B1B0A0 |

CE4.4.14 in JVET-L0318 (Hisilicon)

It is proposed to replace the original spatial merge candidate list construction method with a ranking based method and the positions of merge candidates are extended to include more potential candidates. It is proposed to sort spatial candidates in descending order of occurrence. The spatial candidates can be fetched from an L-shaped area around the current block.  The proposed spatial candidates are derived and put at the beginning of the merge candidate list. The number of maximum merge candidates is kept unchanged. Other non-spatial merge candidates are added afterwards until the merge list is full.

In proposed method, each scanned spatial candidate block is in minimum block size (4x4 luma samples in VTM), such block will be referred to as min-block in this document. As depicted in Figure 3, the scanning range of spatial candidates includes min-blocks which are immediately adjacent to the current block (including top-left position), M min-blocks from the bottom-left neighboring block downwards, and N min-blocks from the above-right neighboring block to the right.

The value of M is related to the current block height H (in luma pixels). Where M = 2*H/4.

The value of N is determined by the current block width W (in luma pixels). Where N = 2*W/4.

Since ATMVP was adopted and was included in VTM-2.0.1, a harmonization with ATMVP is applied in this test. The harmonization method used is to add ATMVP to the 4th position on the final candidate list, if there’s more than 4 candidate available on the sorted candidate list.

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-L0055 | CE4-related: Redundant Removal for ATMVP | A. Tamse, M. W. Park, S. Jeong, K. Choi (Samsung) |
| JVET-L0091 | CE4-related: Shared merge list | C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-L0092 | CE4-related: A simplification algorithm for ATMVP | C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-L0093 | CE4-related: Simplified pruning in merge mode | C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-L0105 | CE4-related: A second ATMVP candidate | Y.-W. Chen, X. Wang (Kwai Inc.) |
| JVET-L0106 | CE4-related: Modified History-based MVP to support parallel processing | Y.-W. Chen, X. Wang (Kwai Inc.) |
| JVET-L0119 | CE4-related: Non-sub-block ATMVP | K. Abe, T. Toma (Panasonic) |
| JVET-L0144 | CE4-related: Composit merge candidate | J. Lee, J. Nam, N. Park, H. Jang, J. Lim, S. Kim (LGE) |
| JVET-L0158 | CE4-related: History-Based Motion Vector Prediction considering parallel processing | N. Park, H. Jang, J. Nam, J. Lee, J. Lim, S. Kim(LGE) |
| JVET-L0171 | CE4-related: Merge mode with Regression based Motion Vector Field (RMVF) | R. Ghaznavi-Youvalari, A. Aminlou, J. Lainema (Nokia) |
| JVET-L0187 | CE4-related: Combined P List for Merge Candidate List | L. Xu, F. Chen, L. Wang (Hikvision) |
| JVET-L0194 | CE4-related: On Merge Index coding | G. Laroche, J. Taquet, C. Gisquet, P. Onno (Canon) |
| JVET-L0198 | CE4-related: Simplification of ATMVP candidate derivation | S.H. Wang (Peking University), X. Zheng (DJI), S.S Wang, S. Ma |
| JVET-L0207 | CE4 related: simplified non-sub-block STMV | F. Le Léannec, T. Poirier, F. Galpin (Technicolor) |
| JVET-L0216 | Non-CE4: Parallel Merge Estimation for VVC | S. Esenlik, H. Gao, B. Wang, A.M. Kotra, J. Chen (Huawei) |
| JVET-L0257 | CE4-related: Mismatch between text specification and reference software on clipping the positions of collocated blocks for alternative temporal motion vector prediction (ATMVP) | X. Xiu, Y. He, Y. Ye(InterDigital) |
| JVET-L0282 | CE4-related: Merge List Simplification | S. Paluri, J. Zhao, S. Kim (LGE) |
| JVET-L0302 | CE4-related: History based spatial-temporal MV prediction | X. Xu, X. Li, S. Liu (Tencent) |
| JVET-L0303 | CE4-related: Improvement of merge mode | B. Choi (Sharp) |
| JVET-L0309 | CE4-related: Simplification to History Based Motion Vector Prediction | J. Zhao, S. Paluri, S. Kim (LGE) |
| JVET-L0375 | CE4-related: Inter prediction sample filtering | W. Xu, H. Yang, Y. Zhao, J. Chen (Huawei) |
| JVET-L0400 | CE4-related: Simplification on Non-Adjacent Merge/Skip mode | Y. Han, W.-J. Chien, H. Huang, M. Karczewicz(Qualcomm) |
| JVET-L0401 | CE4-related: Modification on History-based Mode Vector Prediction | Y. Han, W.-J. Chien, H. Huang, M. Karczewicz(Qualcomm) |
| JVET-L0411 | CE4-related: Angular merge prediction | S. Iwamura, S. Nemoto, A. Ichigaya |
| JVET-L0425 | CE4-related: History-based MVP without using the last lookup table entry | T. Solovyev, J. Chen, A. Karabutov, S. Ikonin (Huawei) |
| JVET-L0448 | CE4-related: constraint of pruning in history-based motion vector prediction | W. Xu, H. Yang, Y. Zhao, J. Chen (Huawei) |
| JVET-L0468 | CE4-related: Fixed sub-block size and restriction for ATMVP | H. Lee, J. Kang, S.-C. Lim, J. Lee, H. Y. Kim (ETRI) |
| JVET-L0470 | CE4-related: Hash-based pruning for merge list construction | T. Solovyev, J. Chen, S. Ikonin (Huawei) |

CE4.5: Motion vector coding

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.5.1.a | Symmetrical MVD mode | JVET-L0370 |
| 4.5.1.b | Symmetrical MVD combined with MVD representation in K0115 |  |
| 4.5.2.a | Asymmetrical BiMVP mode | JVET-L0298 |
| 4.5.2.c | Asymmetrical BiMVP mode + Symmetrical BiMVP mode |  |
| 4.5.3.b | 4.5.2.a + MVD compression + reconstruction | JVET-L0298 |
| 4.5.3.c | 4.5.2.c + MVD compression + reconstruction |  |
| 4.5.4.a | UMVE ( 1 Base candidate from VVC merge list ) | JVET-L0054 |
| 4.5.4.b | UMVE ( 2 Base candidates from VVC merge list ) |  |
| 4.5.4.c | UMVE ( 4 Base candidates from VVC merge list ) |  |
| 4.5.4.d | UMVE ( 1 Base candidates from new UMVE candidate list ) |  |
| 4.5.4.e | UMVE ( 2 Base candidates from new UMVE candidate list ) |  |
| 4.5.4.f | UMVE ( 4 Base candidates from new UMVE candidate list ) |  |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.5.1.a | -0.58% | -0.42% | -0.44% | 104% | 99% | / | / | / | / | / |
| 4.5.1.b | -0.46% | -0.30% | -0.33% | 102% | 99% | / | / | / | / | / |
| 4.5.2.a | -0.41% | -0.49% | -0.59% | 106% | 99% | / | / | / | / | / |
| 4.5.2.c | -0.46% | -0.55% | -0.68% | 112% | 99% | / | / | / | / | / |
| 4.5.3.b | -0.47% | -0.58% | -0.75% | 114% | 99% | / | / | / | / | / |
| 4.5.3.c | -0.48% | -0.56% | -0.74% | 122% | 99% | / | / | / | / | / |
| 4.5.4.a | -0.92% | -0.89% | -0.99% | 103% | 99% | -0.21% | -0.36% | -0.14% | 103% | 98% |
| 4.5.4.b | -1.29% | -1.17% | -1.33% | 108% | 98% | -0.37% | -0.38% | -0.31% | 106% | 98% |
| 4.5.4.c | -1.35% | -1.19% | -1.34% | 116% | 98% | -0.37% | -0.31% | -0.48% | 113% | 97% |
| 4.5.4.d |  |  |  |  |  |  |  |  |  |  |
| 4.5.4.e |  |  |  |  |  |  |  |  |  |  |
| 4.5.4.f |  |  |  |  |  |  |  |  |  |  |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.5.1.a | S. Jeong (Samsung) |  |  |  |  |
| 4.5.1.b | S. Jeong (Samsung) |  |  |  |  |
| 4.5.2.a | N. Park(LGE) | 2 | F | Y | M |
| 4.5.2.c | N. Park(LGE) | 2 | F | Y | M |
| 4.5.3.b | J. Ye (Tencent) | 2 | F | Y | M |
| 4.5.3.c | J. Ye (Tencent) | 2 | F | Y | M |
| 4.5.4.a | J. Li (Panasonic) | 2 | F | Y | M |
| 4.5.4.b | J. Li (Panasonic) | 2 | F | Y | M |
| 4.5.4.c | J. Li (Panasonic) | 2 | F | Y | M |
| 4.5.4.d |  |  |  |  |  |
| 4.5.4.e |  |  |  |  |  |
| 4.5.4.f |  |  |  |  |  |

Detailed description of tests

CE4.5.1 in JVET-L0370 (Huawei)

The proposed symmetrical MVD is a special case of bi-directional inter AMVP mode, utilizing the symmetry in the forward and the backward motion vector based on linear motion assumption.

In case of bi-directional inter prediction, an SMVD mode flag is signaled to indicate the usage. In SMVD mode, signalling of the reference picture in both direction and the MVD in the backward direction is skipped. The nearest forward reference picture and the nearest backward reference picture is used. The backward MVD is symmetrical to the forward MVD.

mvx0, mvy0=(mvpx0+mvdx0,mvpy0+mvdy0) mvx1, mvy1=(mvpx1-mvdx0,mvpy1-mvdy0)

CE4.5.2 & CE4.5.3 in JVET-L0298 (Sharp)

BiMVP mode does not signal ref_idx and ref_pred_idc. Instead, the first reference pictures whose ref_idx values are equal to 0 for both list0 and list are used for bi-prediction. When bilateral_mode_flag is equal to 1, an additional flag (symmetric_flag) is signalled to indicate whether the BiMVP for the current CU is the Symmetric BiMVP mode or the Asymmetric BiMVP mode.

Symmetric BiMVP mode

In the symmetric BiMVP mode, only mvp_flag and coded MVD for list0 are signaled. Then, both MV predictor (V1x, V1y) and MV difference (offset_x, offset_y) for list1 are derived from MVP = (V0x, V1y) and MVD = (offset_x, offset_y) for list0 by mirroring values. For example, when the forward MV is equal to (Vx + offset_x, Vy + offset_y), its paired backward MV is derived as equal to (-Vx - offset_x, -Vy - offset_y). Since the derived MV for list1 is exactly symmetric with MV for list1, it is called Symmetric BiMVP mode.

Asymmetric BiMVP mode

In the asymmetric BiMVP mode, mvp_flag, coded MVD for list0 and mvp_flag for list1 are signaled. This means that the MVP for list1 is not derived from the MVP for list0; instead, it is explicitly indicated by signaling. For example, when the MV for list0 is equal to (V0x + offset_x, V0y + offset_y), its paired MV for list1 is set equal to (V1x - offset_x, V1y - offset_y). Note that both (V0x, V0y) and (V1x, V1y) are determined by the values of mvp_flag. Even though it is called Asymmetric BiMVP mode in this contribution, its MVD values are symmetric.

Implementation with MV compression/reconstruction process

BiMVP can have only a MVD value among a set of predefined MVD values in integer-pel resolution in the current implementation. This modification employs motion vector compression and reconstruction processes, which are intended to reduce the MVD bits and the encoding complexity.

CE4.5.4 in JVET-L0054 (Samsung)

UMVE provides a new motion vector expression with simplified signaling. The expression method includes starting point, motion magnitude, and motion direction.

Base candidate index defines the starting point. Base candidate index indicates the best candidate among candidates in the list as follows.

| Base candidate IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP |

If the number of base candidate is equal to 1, Base candidate IDX is not signaled.

Distance index is motion magnitude information. Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Direction index represents the direction of the MVD relative to the starting point. The direction index can represent of the four directions as shown below.

| Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-axis | + | – | N/A | N/A |
| y-axis | N/A | N/A | + | – |

UMVE flag is singnaled right after sending a skip flag and merge flag. If skip and merge flag is true, UMVE flag is parsed. If UMVE flage is equal to 1, UMVE syntaxes are parsed. But, if not 1, AFFINE flag is parsed. If AFFINE flag is equal to 1, that is AFFINE mode, But, if not 1, skip/merge index is parsed for VTM’s skip/merge mode.

Related contributions

| JVET # | Title | Source |
| --- | --- | --- |
| JVET-L0168 | Motion vector representing bit reduction | H.Jang, J.Nam, S.Kim, J.Lim(LGE) |
| JVET-L0300 | CE4-related: Generic Vector Coding of Motion Vector Difference | S. Paluri, M. Salehifar, S. Kim (LGE) |
| JVET-L0301 | CE4-related: Improvement of BIMVP | B. Choi (Sharp) |
| JVET-L0355 | Non-CE4: Enhanced ultimate motion vector expression | T. Hashimoto, E. Sasaki, T. Ikai (Sharp) |
| JVET-L0377 | Rounding Align of Adaptive Motion Vector Resolution | Y. Zhang, C.-C. Chen, H. Huang, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm) |
| JVET-L0408 | CE4-related: Improvement on ultimate motion vector expression | J. Li, R.-L. Liao, C. S. Lim (Panasonic) |
| JVET-L0214 | CE4-related: Motion predictor pruning | A. Robert, F. Le Leleannec, F. Galpin, T. Poirier (Technicolor) |

CE4.6: Reference picture boundary padding

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.6.2 | Intra boundary padding | JVET-L0223 |
| 4.6.3 | Combined inter/intra boundary padding |  |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.6.2 | -0.05% | -0.06% | -0.09% | 103% | 105% | -0.07% | -0.06% | -0.02% | 105% | 106% |
| 4.6.3 | -0.24% | -0.15% | -0.24% | 102% | 110% | -0.11% | -0.23% | -0.26% | 104% | 108% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |

Detailed description of tests

CE4.6.1 & CE4.6.2 in JVET-L0223 (HHI)

Inter-prediction based boundary padding in CE4.6.1 uses motion compensated prediction to extend the area of the reference picture. The boundary extension area is divided into blocks of 4xM or Mx4 samples. Each block is filled by motion compensation using the motion information of the adjacent reference picture block. For boundary extensions blocks without associated motion information and for boundary extension areas for which motion information points outside of the reference picture, fallback perpendicular padding is carried out.

Intra-prediction based boundary padding in CE4.6.2 uses angular intra-prediction to fill the area of a referenced block outside the reference picture. The applied angular intra-prediction mode is chosen in encoder and decoder using a probing approach of decoded picture samples.

The harmonized inter/intra-prediction based boundary padding in CE4.6.3 combines per-picture inter-prediction based boundary padding (the particular flavor introduced in JVET-K0363) with per-reference intra-prediction based boundary padding as in CE4.6.2. After generation of the inter-prediction based boundary padding, for each reference block entailing boundary padding samples, the number of boundary padding samples originating from perpendicular boundary padding is evaluated. If this number exceeds a threshold (50% of boundary padding samples), intra-prediction based boundary padding is used for the reference block instead.

CE4.7: Local illumination compensation

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.7.1 | Non-temporal IC using above neighbour CU | JVET-L0056 |
| 4.7.2 | Non-temporal IC using left and above neighbour CU |  |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.7.1 | -0.21% | -0.09% | -0.16% | 125% | 104% | 0.04% | 0.09% | 0.10% | 133% | 103% |
| 4.7.2 | -0.36% | -0.23% | -0.30% | 125% | 107% | -0.14% | -0.21% | -0.17% | 129% | 108% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.7.1 | H. Chen (Huawei) | 2 | F | Y | M |
| 4.7.2 | H. Chen (Huawei) | 2 | F | Y | M |

Detailed description of tests

CE4.7.1 & CE4.7.2 in JVET-L0056 (Samsung)

Computing and storing the parameters

At the end of processing of each CU, linear models (Ax + B = y) are computed by comparing the prediction pixels (x) and the reconstructed pixels (y). The models are computed using linear regression. The lower edge and right edge pixels are used for model computation both the models are stored. Each model contains parameters for each colour component and for each of the two bi-prediction directions (if available). These parameters (sets of A and B) are then stored in the PU for later access.

Furthermore the parameters are only computed for CUs whose width and height are longer than or equal to 8. This reduces the hardware implementation cost considerably as compared to when it is computed for all the CUs.

Tool application

The conditions under which this tool is applicable depends on whether the neighboring blocks are skip coded and/or IC coded and the size of the current block. If applicable, a flag is signaled to indicate whether IPR is to be applied to a given inter coded CU or not. If the tool is to be applied, the motion compensation pixels (x) are refined using the application of the linear model (Ax + B) to obtain the new set of prediction pixels. The model used is derived from the current CU’s neighbors. Top, top left, left and left bottom neighbours are considered for borrowing the parameter. Since multiple neighbors may be available, pruning is performed to obtain a single model. Intra coded neighbours are discarded in the pruning process. Next, the neighbours which have trivial model [x = y] are discarded in the pruning process. Models not having the same reference indices are discarded next. If no model is remaining, a trivial model is used. If model(s) still remain, a predefined priority order is used for checking the neighbors.

Additionally, when performing RDO check for determining whether IPR will be used or not, mean-removed SAD (Sum of Absolute Difference) is used instead of the regular SAD for the ME (Motion Estimation) process.

The linear regression method used is same as that in JEM. Also, using two sides for parameter derivation improves the gain.

Key points

- LIC model pre-computation and storage after processing a CU

- LIC model pruning to get the only one model applied to a CU

Related contributions

| JVET# | Title | Source |
| --- | --- | --- |
| JVET-L0068 | CE4-related: Modified LIC | KWU |
| JVET-L0069 | CE4-related: Modified LIC for Bi-prediction | H. Choi, J. Lee, S. Park, D. Sim (KWU) |
| JVET-L0120 | CE4-related: Low pipeline latency LIC | K. Abe, T. Toma, J. Li (Panasonic) |
| JVET-L0197 | CE4-related: Generalized bi-prediction improvements | Y.-C. Su, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-L0201 | AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences | P.Bordes, E.Francois (Technicolor) |
| JVET-L0203 | CE4-related: LIC with reduced memory buffer | P.Bordes, F.Le Leleannec, F.Galpin, E.Francois (Technicolor) |
| JVET-L0296 | CE4-related: encoder speed up and bug fix for generalized bi-prediction in BMS-2.1 | Y. He, J. Luo, X. Xiu, Y. Ye (InterDigital) |

CE4.8: Motion data storage

Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 4.8.1 | Motion information storage with alternative spatial candidates for each 8x8 region | JVET-L0169 |
| 4.8.2 | Motion information storage for each 16x16 region |  |
| 4.8.3 | Motion information storage with alternative spatial candidates for each 16x16 region |  |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.8.1 | 0.00% | 0.04% | -0.03% | 98% | 82% | 0.02% | -0.01% | 0.05% | 99% | 99% |
| 4.8.2 | 0.29% | 0.32% | 0.27% | 100% | 99% | 0.61% | 0.74% | 0.63% | 100% | 97% |
| 4.8.3 | 0.17% | 0.18% | 0.14% | 100% | 99% | 0.45% | 0.43% | 0.42% | 99% | 98% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |

Detailed description of tests

CE4.8.1 & CE4.8.2 & CE4.8.3 in JVET-L0169 (LGE)

The propose method checks whether there is a valid motion vector sequentially in five positions of an 8x8 or a 16x16 region, C -> TL -> TR -> BL -> BR, and uses the first valid motion vector for temporal motion vector storage.

Recommendations

Review CE4 document if needed. Review all CE4-related documents.