| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0025-v1 |
| --- | --- |

| Title: | CE5: Summary report on the Arithmetic Coding Engine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer Fraunhofer HHI Amir Said Qualcomm Inc. | Email: | heiner.kirchhoffer@ hhi.fraunhofer.de asaid@qti.qualcomm.com |
| Source: | CE 5 coordinators |  |  |

_____________________________

# Abstract

This is the summary report of Core Experiment 5 (CE5) on the Arithmetic Coding Engine. Twelve experiments have been conducted. Seven experiments in Subtest 1 (experiments CE5.1.1 – 5.1.7) addresses modifications to the probability estimation only. Four experiments in Subtest 2 (experiments CE5.2.1 – 5.2.4) addresses modifications to the coding interval subdivision only. One experiment in Subtest 3 (experiment 5.3.1) addresses the combination of Subtest 1 with changes to the coding interval subdivision. Probability estimators that employ custom window sizes achieve a coding gain relative to the CEM 1 reference configuration as specified in the CE5 description. However, it is unclear to what extent these effects are caused by suboptimal context model initialization values as discussed in JVET-L0552. Furthermore, the results show that a final rLPS design that has a maximum size equal to or less than 2048 bit is sufficient to achieve the compression efficiency.

# Test conditions and guidelines

It was mandatory to provide results without changes to the context initialization values when a proposal introduces new initialization values. All tests were implemented in the BMS-2.0.1 software. For simulations only the VTM configuration according to the common test conditions (JVET-L1010) was mandatory. Results are compared against the so-called ‘CEM1’ software which is described in the CE5 description JVET-K1025 (counter-based probability estimator using two 15 bit counter variables and window size parameters 16 and 128) as well as against the BMS-2.0.1 software.

The experiments were implemented in separate branches in the following git repository:

https://vcgit.hhi.fraunhofer.de/JVET-K-CE5/VVCSoftware_BMS.git

# Experiment list

The following table lists all tests that were performed in this CE and gives a summary of the tool by describing the differences to the CEM1 software (JVET-K1025).

Note on the naming convention of CE tests: ‘CE5.x.y.z’ means ‘CE5.x.y’ as specified in the CE5 description with configuration ‘z’ according to the following tables.

| CE5.1 – Probability estimation |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool (differences to CEM1) | Crosscheckers |
| 5.1.1 | Sharp Labs | JVET-L0335 | Modifies the constant 2b to 2b-1 (e.g., 32768 to 32767) in the probability estimate update function. | Qualcomm |
| 5.1.2 | Samsung | JVET-L0057 | Only the short window size model is updated for the first 31 bins of a context model. | HHI |
| 5.1.3 | HHI | JVET-L0461 | Configuration 1: Counter variable size per context model is reduced from 15 +15 bit to 10 + 14 bit. Configuration 2: Like configuration 1 but with custom window sizes. One 4 bit constant per context model specifies a pair of custom window sizes. | Samsung |
| 5.1.4 | HHI | JVET-L0462 | Configuration 1: State-based probability estimator using a transition table for the probability update and two state variables with 8 and 12 bit, respectively. Configuration 2: Like configuration 1 but with custom window sizes. One 5 bit constant per context model specifies a pair of custom window sizes. | Qualcomm |
| 5.1.5 | HHI | JVET-L0462 | Like CE5.1.4, config. 2, but only one state variable with 12 bit is used per context model. A 3 bit constant is used for the custom window size. | Sharp Labs |
| 5.1.6 | Qualcomm | JVET-L0115 | Custom window sizes are used. One 4 bit constant per context model specifies a pair of custom window sizes. Fixed window parameters 4 and 7 are used for contexts of SIG, parity, and Gtx (x = 1, 2), so may not be stored. | HHI |
| 5.1.7 | Qualcomm | JVET-L0116 | One counter variable is used together with custom window sizes. One 3 bit variable per context model specifies the custom window sizes. Fixed window parameter 4 is used for the contexts of SIG, parity, and Gtx (x = 1, 2), so may not be stored. | HHI |
| HEVC |  |  | Uses one 7 bit state variable and a 63x6 bit transition table. |  |

Contributions related to CE5.1:

JVET-L0429: CE5-related: lookup table-free implementation of the probability update for tests CE5.1.4 and CE5.1.5

JVET-L0552: Training initial CABAC states

| CE5.2 – Coding interval subdivision |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool (differences to CEM1) | Crosscheckers |
| 5.2.1 | Sharp Labs | JVET-L0335 | Modify the subinterval range computation for the LPS symbol to ((r >> 5) * (qLPS >> (b − 5)) >> 1) + 4. Table-based implementation possible (32x8x8bit). | MediaTek |
| 5.2.2 | MediaTek | JVET-L0094 | Multiplier-based interval subdivision. Configuration 1: 6-bit by 5-bit multiplier. Configuration 2: 6-bit by 4-bit multiplier. Configuration 3: 5-bit by 5-bit multiplier. Configuration 4: 5-bit by 4-bit multiplier. Config. 1 and 2 can implemented as 32x8x8 bit LUT. Config. 3 and 4 can implemented as 16x8x8 bit LUT. | Qualcomm |
| 5.2.3 | HHI | JVET-L0461 | Configuration 1: A LUT of size 32x8x8 bit is used. Configuration 2: A LUT of size 16x8x8 bit is used. | Qualcomm |
| 5.2.4 | Qualcomm | JVET-L0117 | Combination of a LUT and some computation operations (including ‘bit-scan-reverse’) is used. Configuration 1: A LUT of size 16x16x8 bit is used, and can be implemented by a 6-bit by 6-bit multiplier. Configuration 2: A LUT of size 8x16x8 bit is used, and can be implemented by a 5-bit by 6-bit multiplier. Configuration 3: A LUT of size 8x8x8 bit is used, and can be implemented by a 5-bit by 5-bit multiplier. | HHI |
| HEVC |  |  | Uses a 64x4x8 bit LUT. |  |

| CE5.3 – Combinations of CE5.1 with coding interval subdivision techniques |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool (differences to CEM1) | Crosscheckers |
| 5.3.1 | HHI | JVET-L0462 | The probability estimator of CE5.1.4.2 is combined with a LUT-based coding interval subdivision. Configuration 1: A LUT of size 32x8x8 bit is used. Configuration 2: A LUT of size 16x8x8 bit is used. | Sharp Labs |

Contributions related to CE5.3:

JVET-L0426: CE5-related: Alternative implementation of CABAC range sub-interval derivation for test CE 5.1.4

# Summary of the results

## Subtest 1: Changes to the probability estimation stage only

Average test results for subtest 5.1 (relative to CEM-1 and VTM-2.0.1).

| Averages for Subtest 5.1 |  | Over CEM1, VTM configuration |  |  |  |  | Over BMS-2.0.1, VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | CE5.1.1 | 0,01% | 0,01% | -0,02% | 100% | 101% |  |  |  |  |  |
|  | CE5.1.2 | -0,03% | -0,10% | -0,01% | 103% | 100% | -0.94% | -1.35% | -1.33% | 104% | 102% |
|  | CE5.1.3.1 | 0,03% | 0,32% | 0,31% | 104% | 101% | -0,89% | -0,94% | -1,00% | 106% | 103% |
|  | CE5.1.4.1 | 0,02% | -0,06% | -0,02% | 101% | 100% | -0,90% | -1,31% | -1,33% | 102% | 99% |
|  | CE5.1.3.2 (C*) | -0,09% | 0,07% | 0,12% | 105% | 103% | -1,01% | -1,18% | -1,20% | 107% | 105% |
|  | CE5.1.4.2 (C*) | -0,05% | -0,36% | -0,30% | 109% | 102% | -0,96% | -1,61% | -1,62% | 109% | 101% |
|  | CE5.1.6 (C*) | -0,21% | 0,18% | -0,08% | 97% | 98% | -1.12% | -1.08% | -1.39% | 93% | 97% |
|  | CE5.1.5 (CS*) | 0,11% | -0,13% | 0,04% | 107% | 101% | -0,81% | -1,38% | -1,28% | 107% | 101% |
|  | CE5.1.7 (CS*) | 0,07% | 0,15% | 0,11% | 95% | 96% | -0.85% | -1.10% | -1.20% | 91% | 95% |
| RA | CE5.1.1 | 0,00% | 0,01% | -0,05% | 104% | 103% |  |  |  |  |  |
|  | CE5.1.2 | -0.19% | -0.15% | -0.30% | 101% | 101% | -0.70% | -0.83% | -0.78% | 102% | 101% |
|  | CE5.1.3.1 | -0,01% | -0,25% | -0,29% | 108% | 104% | -0,52% | -0,94% | -0,77% | 102% | 105% |
|  | CE5.1.4.1 | -0,07% | -0,14% | -0,11% | 101% | 100% | -0,58% | -0,82% | -0,59% | 94% | 96% |
|  | CE5.1.3.2 (C*) | -0,43% | -0,37% | -0,37% | 109% | 106% | -0,94% | -1,05% | -0,85% | 103% | 106% |
|  | CE5.1.4.2 (C*) | -0,35% | -0,27% | -0,47% | 114% | 109% | -0,86% | -0,95% | -0,95% | 106% | 104% |
|  | CE5.1.6 (C*) | -0,42% | 0,32% | -0,26% | 107% | 98% | -0.93% | -0.37% | -0.74% | 97% | 99% |
|  | CE5.1.5 (CS*) | -0,15% | -0,55% | -0,64% | 113% | 108% | -0,66% | -1,23% | -1,12% | 106% | 103% |
|  | CE5.1.7 (CS*) | -0,03% | 0,73% | 0,45% | 105% | 97% | -0.54% | 0.04% | -0.04% | 95% | 99% |
| LB | CE5.1.1 | 0,02% | 0,12% | 0,08% | 115% | 110% |  |  |  |  |  |
|  | CE5.1.2 | -0.13% | -0.21% | -0.17% | 102% | 100% | -0.72% | -0.62% | 0.35% | 101% | 98% |
|  | CE5.1.3.1 | 0,13% | -0,47% | -1,27% | 110% | 104% | -0,48% | -0,84% | -0,65% | 101% | 105% |
|  | CE5.1.4.1 | -0,04% | 0,02% | -0,43% | 100% | 98% | -0,65% | -0,34% | 0,20% | 92% | 96% |
|  | CE5.1.3.2 (C*) | -0,31% | -0,77% | -1,19% | 112% | 107% | -0,91% | -1,13% | -0,56% | 103% | 107% |
|  | CE5.1.4.2 (C*) | -0,36% | -0,28% | -1,35% | 116% | 102% | -0,96% | -0,65% | -0,73% | 106% | 99% |
|  | CE5.1.6 (C*) | -0,32% | 0,31% | -0,99% | 101% | 97% | -0.93% | -0.04% | -0.35% | 98% | 106% |
|  | CE5.1.5 (CS*) | -0,09% | -0,83% | -1,62% | 116% | 106% | -0,70% | -1,20% | -1,00% | 106% | 103% |
|  | CE5.1.7 (CS*) | -0,02% | 0,58% | 0,43% | 100% | 97% | -0.63% | 0.21% | 1.10% | 96% | 106% |

(C*): Two states and two custom window sizes are used per context model

(CS*): One state and one custom window size is used per context model

Memory requirements of tools in subtest 5.1

| CE # | Document | RAM per Ctx | ROM per Ctx | ROM for LUTs | Comments |
| --- | --- | --- | --- | --- | --- |
| HEVC |  | 7 bit |  | 378 bit |  |
| CE5.1.1 | JVET-L0335 | 30 bit | - | - | Does not affect RAM per ctx. Can be combined with 5.1.2, 5.1.3.x, 5.1.6 and 5.1.7. |
| CE5.1.2 | JVET-L0057 | 35 bit | - | - | Only short window for the first 31 bins |
| CE5.1.3.1 | JVET-L0461 | 24 bit | - | - |  |
| CE5.1.3.2 |  | 24 bit | 4 bit | - | Custom WS |
| CE5.1.4.1 | JVET-L0462 | 20 bit | - | 256 bit |  |
| CE5.1.4.2 |  | 20 bit | 5 bit | 256 bit | Custom WS |
| CE5.1.5 | JVET-L0462 | 12 bit | 3 bit | 256 bit | Custom WS, One State |
| CE5.1.6 | JVET-L0115 | 30 bit | 4 bit | - | Custom WS |
| CE5.1.7 | JVET-L0116 | 15 bit | 3 bit | - | Custom WS, One State |

## Subtest 2 and 3: Changes to the coding interval subdivision

The following table compares the overall compression efficiency of subtests 2 and 3. All tests in CE5.2 use the probability estimator of CEM1. Only the coding interval subdivision is different for each test.

Average test results for subtest 5.2 (relative to CEM-1).

| Averages for Subtest 5.2 |  | Over CEM1, VTM configuration |  |  |  |  |  | Multiplier |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | LUT size | size | Comments |
| AI | CE5.2.1 (JVET-L0335) | 0,07% | 0,08% | 0,08% | 106% | 103% | 32x8x8 bit | 5x4 bit |  |
|  | CE5.2.2 (JVET-L0094) | -0,01% | -0,01% | -0,01% | 100% | 98% | 32x8x8 bit | 6x5 bit |  |
|  |  | 0,04% | 0,05% | 0,05% | 100% | 99% | 32x8x8 bit | 6x4 bit |  |
|  |  | 0,17% | 0,17% | 0,17% | 100% | 98% | 16x8x8 bit | 5x5 bit |  |
|  |  | 0,22% | 0,22% | 0,22% | 100% | 98% | 16x8x8 bit | 5x4 bit |  |
|  | CE5.2.3 (JVET-L0461) | 0,05% | 0,05% | 0,05% | 104% | 101% | 32x8x8 bit | n/a |  |
|  |  | 0,19% | 0,19% | 0,19% | 104% | 102% | 16x8x8 bit | n/a |  |
|  | CE5.2.4 (JVET-L0117) | -0,05% | -0,06% | -0,04% | 101% | 103% | 16x16x8 bit | 6x6 bit | Involves the ‘bit-scan-reverse’ instruction |
|  |  | -0,04% | -0,05% | -0,03% | 99% | 98% | 8x16x8 bit | 5x6 bit |  |
|  |  | -0,02% | -0,03% | -0,02% | 102% | 102% | 8x8x8 bit | 5x5 bit |  |
| RA | CE5.2.1 (JVET-L0335) | 0,08% | 0,08% | 0,08% | 100% | 98% | 32x8x8 bit | 5x4 bit |  |
|  | CE5.2.2 (JVET-L0094) | 0,00% | 0,00% | 0,00% | 100% | 100% | 32x8x8 bit | 6x5 bit |  |
|  |  | 0,06% | 0,06% | 0,06% | 100% | 100% | 32x8x8 bit | 6x4 bit |  |
|  |  | 0,16% | 0,16% | 0,16% | 100% | 100% | 16x8x8 bit | 5x5 bit |  |
|  |  | 0,21% | 0,21% | 0,21% | 100% | 100% | 16x8x8 bit | 5x4 bit |  |
|  | CE5.2.3 (JVET-L0461) | 0,05% | 0,05% | 0,05% | 107% | 105% | 32x8x8 bit | n/a |  |
|  |  | 0,17% | 0,17% | 0,17% | 107% | 104% | 16x8x8 bit | n/a |  |
|  | CE5.2.4 (JVET-L0117) | -0,03% | -0,09% | -0,08% | 111% | 98% | 16x16x8 bit | 6x6 bit | Involves the ‘bit-scan-reverse’ instruction |
|  |  | -0,02% | -0,08% | -0,07% | 113% | 100% | 8x16x8 bit | 5x6 bit |  |
|  |  | 0,00% | -0,06% | -0,06% | 115% | 102% | 8x8x8 bit | 5x5 bit |  |
| LB | CE5.2.1 (JVET-L0335) | 0,07% | 0,07% | 0,07% | 103% | 102% | 32x8x8 bit | 5x4 bit |  |
|  | CE5.2.2 (JVET-L0094) | -0,05% | -0,05% | -0,05% | 100% | 99% | 32x8x8 bit | 6x5 bit |  |
|  |  | 0,03% | 0,02% | 0,02% | 100% | 100% | 32x8x8 bit | 6x4 bit |  |
|  |  | 0,14% | 0,15% | 0,15% | 100% | 100% | 16x8x8 bit | 5x5 bit |  |
|  |  | 0,22% | 0,22% | 0,22% | 100% | 99% | 16x8x8 bit | 5x4 bit |  |
|  | CE5.2.3 (JVET-L0461) | 0,03% | 0,03% | 0,03% | 110% | 103% | 32x8x8 bit | n/a |  |
|  |  | 0,17% | 0,17% | 0,17% | 111% | 103% | 16x8x8 bit | n/a |  |
|  | CE5.2.4 (JVET-L0117) | -0,04% | -0,05% | -0,12% | 104% | 101% | 16x16x8 bit | 6x6 bit | Involves the ‘bit-scan-reverse’ instruction |
|  |  | -0,03% | -0,04% | -0,11% | 105% | 103% | 8x16x8 bit | 5x6 bit |  |
|  |  | -0,02% | -0,02% | -0,09% | 105% | 102% | 8x8x8 bit | 5x5 bit |  |

Results of CE5.3.1 are shown relative to CE5.1.4.2 because all tests use the same probability estimator as CE5.1.4.2. Only the coding interval subdivision is different for each test.

Average test results for subtest 5.3.1 (relative to CE5.1.4.2).

| Averages for Subtest 5.1 |  | Over CE5.1.4.2, VTM configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | LUT size |
| AI | CE5.3.1.1 | 0,01% | 0,01% | 0,01% | 100% | 101% | 32x8x8 bit |
|  | CE5.3.1.2 | 0,11% | 0,11% | 0,11% | 100% | 100% | 16x8x8 bit |
| RA | CE5.3.1.1 | 0,01% | 0,01% | 0,01% | 99% | 100% | 32x8x8 bit |
|  | CE5.2.1.2 | 0,13% | 0,13% | 0,13% | 100% | 99% | 16x8x8 bit |
| LB | CE5.3.1.1 | 0,02% | 0,02% | 0,02% | 99% | 100% | 32x8x8 bit |
|  | CE5.3.1.2 | 0,15% | 0,14% | 0,14% | 100% | 102% | 16x8x8 bit |

# Crosscheck reports

The following table lists the reported crosschecks.

| CE # | Crosschecker | Simulation results match | Implementation matches the description | Issues raised by crosscheckers |
| --- | --- | --- | --- | --- |
| CE5.1.1 | Qualcomm | Yes | Yes | None |
| CE5.1.2 | HHI | Yes | Yes | None |
| CE5.1.3 | Samsung | Yes | Yes | None |
| CE51.4 | Qualcomm | Yes | Yes | None |
| CE5.1.5 | Sharp Labs |  |  |  |
| CE5.1.6 | HHI | Yes | Yes | None |
| CE5.1.7 | HHI | Yes | Yes | None |
| CE5.2.1 | MediaTek | Yes | Yes | None |
| CE5.2.2 | Qualcomm | Yes | Yes | None |
| CE5.2.3 | Qualcomm | Yes | Yes | None |
| CE5.2.4 | HHI | Yes | Yes | None |
| CE5.3.1 | Sharp Labs | Yes | Yes, but see note | CE description lacks details about the combination with custom window sizes. Some tables updated late (Sep 20). |