| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0054 |
| --- | --- |

| Title: | CE4 Ultimate motion vector expression (Test 4.5.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsoo Jeong Min Woo Park Yinji Piao Minsoo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | ss00.jeong@samsung.com m.w.park@samsung.com yj1003.piao@samsung.com mss.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents ultimate motion vector expression (UMVE) proposed in JVET-K0115. UMVE  is a motion vector expression method for skip and merge modes. Based on the UMVE proposed in JVET-K0115, in this contribution three tests are conducted; (Test 4.5.4 (a)) UMVE mode uses one of VVC merge candidates as a base motion vector, (Test 4.5.4 (b)) UMVE mode uses two of VVC merge candidates as a base motion vector, (Test 4.5.4 (c)) UMVE mode uses four of VVC merge candidates as a base motion vector.When the simulation for three tests are conducted based on VTM, results reportedly show –0.92% BD-rate with 103% and 99% encoding and decoding time under RA condition, and –0.21% BD-rate with 103% and 98% encoding and decoding time under LDB condition for Test 4.5.4 (a), –1.29% BD-rate with 108% and 98% encoding and decoding time under RA condition, and –0.37% BD-rate with 106% and 98% encoding and decoding time under LDB condition for Test 4.5.4 (b), –1.35% BD-rate with 116% and 98% encoding and decoding time under RA condition, and –0.37% BD-rate with 113% and 97% encoding and decoding time under LDB condition for Test 4.5.4 (c).

Proposed Methods

In this contribution, ultimate motion vector expression (UMVE) is presented. UMVE is used for either skip or merge modes with a proposed motion vector expression method.

UMVE re-uses merge candidate as same as using in VVC. Among the merge candidates, a candidate can be selected, and is further expanded by the proposed motion vector expression method.

UMVE provides a new motion vector expression with simplified signaling. The expression method includes starting point, motion magnitude, and motion direction.

Figure 1-1. UMVE Search Process

Figure 1-2 UMVE Search Point

This proposed technique uses a merge candidate list as it is. But only candidates which are default merge type (MRG_TYPE_DEFAULT_N) are considered for UMVE’s expansion.

Base candidate index defines the starting point. Base candidate index indicates the best candidate among candidates in the list as follows.

Table 1. Base candidate IDX

| Base candidate IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP |

If the number of base candidate is equal to 1, Base candidate IDX is not signaled.

Distance index is motion magnitude information. Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

Table 2. Distance IDX

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Direction index represents the direction of the MVD relative to the starting point. The direction index can represent of the four directions as shown below.

Table 3. Direction IDX

| Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-axis | + | – | N/A | N/A |
| y-axis | N/A | N/A | + | – |

UMVE flag is singnaled right after sending a skip flag and merge flag. If skip and merge flag is true, UMVE flag is parsed. If UMVE flage is equal to 1, UMVE syntaxes are parsed. But, if not 1, AFFINE flag is parsed. If AFFINE flag is equal to 1, that is AFFINE mode, But, if not 1, skip/merge index is parsed for VTM’s skip/merge mode.

Additional line buffer due to UMVE candidates is not needed. Because a skip/merge candidate of software is directly used as a base candidate. Using input UMVE index, the supplement of MV is decided right before motion compensation. There is no need to hold long line buffer for this.

Experimental Results

The simulation was carried out under the JVET common test condition [1]. The software was compiled with GCC version 7.3 and the AVX SIMD instructions. Table 1 shows summary of all results.

Table 1. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| RA | test a | -0.92% | -0.89% | -0.99% | 103% | 99% |
|  | test b | -1.29% | -1.17% | -1.33% | 108% | 98% |
|  | test c | -1.35% | -1.19% | -1.34% | 116% | 98% |
| LDB | test a | -0.21% | -0.36% | -0.14% | 103% | 98% |
|  | test b | -0.37% | -0.38% | -0.31% | 106% | 98% |
|  | test c | -0.37% | -0.31% | -0.48% | 113% | 97% |

Table 2. Results for CE4.5.4 test a

Table 3. Results for CE4.5.4 test b

Table 4. Results for CE4.5.4 test c

Conclusion

In this contribution, the merge mode with new MV expression is proposed. This mode can be applied to any merge candidates in list. There is no decoder complexity increase.

References

[1] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).