| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0054_WD |
| --- | --- |

| Title: | Proposed WD for CE4 Ultimate motion vector expression (Test 4.5.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsoo Jeong Min Woo Park Yinji Piao Minsoo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | ss00.jeong@samsung.com m.w.park@samsung.com yj1003.piao@samsung.com mss.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Proposed Working Draft Changes

This document provides the proposed text of VVC Working Draft. The changes are highlighted in yellow and strikethrough.

      - Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] |  |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| umve_flag[ x0 ][ y0 ] | ae(v) |
| if( umve_flag[ x0 ][ y0 ] = = 1 ) |  |
| umve_idx_coding ( x0, y0) |  |
| else if( merge_affine_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| umve_flag[ x0 ][ y0 ] | ae(v) |
| if( umve_flag[ x0 ][ y0 ] = = 1 ) |  |
| umve_idx_coding ( x0, y0 ) |  |
| else if(merge_affine_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

### UMVE index syntax ( new )

| umve_idx_coding ( x0, y0 ) { | Descriptor |
| --- | --- |
| base_mv_idx[ x0 ][ y0 ] | ae(v) |
| distance_idx[ x0 ][ y0 ] | ae(v) |
| direction_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

#### 7.4.5.5 Coding unit semantics

cu_skip_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B slice, no more syntax elements except the merging candidate index merge_idx[ x0 ][ y0 ], the umve flag umve_flag[ x0 ][ y0 ] and the merge affine flag merge_affine_flag[ x0 ][ y0 ] are parsed after cu_skip_flag[ x0 ][ y0 ]. cu_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not skipped. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When cu_skip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

umve_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B slice, ultimate motion vector expression is used to generate the prediction MV of the current coding unit. umve_flag[ x0 ][ y0 ] equal to 1 specifies that no more syntax elements except umve_idx_coding() are parsed after umve_flag[ x0 ][ y0 ].umve_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not predicted by ultimate motion vector expression.

When umve_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

base_mv_idx[ x0 ][ y0 ] specifies the base MV candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When base_mv_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

distance_idx[ x0 ][ y0 ] specifies the distance index, which indicates a distance offset from an umve base MV, for the current coding unit where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When distance_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

DistFromBaseMV[ x0 ][ y0 ] represents a distance offset from a umve base MV

Table 7-X – distance_idx[ x0 ][ y0 ]

| distance_idx[ x ][ y ] | binarization | DistFromBaseMV[ x0 ][ y0 ] |
| --- | --- | --- |
| 0 | 0 | 1 |
| 1 | 10 | 2 |
| 2 | 110 | 4 |
| 3 | 1110 | 8 |
| 4 | 11110 | 16 |
| 5 | 111110 | 32 |
| 6 | 1111110 | 64 |
| 7 | 1111111 | 128 |

direction_idx[ x0 ][ y0 ] specifies the direction index of DistFromBaseMV[ x0 ][ y0 ] for the current coding unit where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-Y – direction_idx[ x0 ][ y0 ]

| direction_idx[ x ][ y ] | binarization | sign[x][y][0] | sign[x][y][1] |
| --- | --- | --- | --- |
| 0 | 00 | +1 | 0 |
| 1 | 01 | -1 | 0 |
| 2 | 10 | 0 | +1 |
| 3 | 11 | 0 | -1 |

Proposed motion derivation process

- 8.3 Decoding process for coding units coded in inter prediction mode

- 8.3.2 Derivation process for motion vector components and reference indices

- 8.3.2.2 Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.3.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the luma coding block width as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1.

- The derivation process for subblock-based temporal merging candidates as specified in clause 8.3.2.4 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight , the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 as inputs and the output being the availability flag availableFlagSbCol, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the reference indices refIdxLXSbCol, the luma motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and the prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

- The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-113)

predFlagL0Col = availableFlagL0Col		(8-114)

predFlagL1Col = 0		(8-115)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-116)

predFlagL1Col = availableFlagL1Col		(8-117)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-118)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagSbCol )
	mergeCandList[ i++ ] = SbCol
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in clause 8.3.2.6 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added into mergeCandList. The number of candidates being added, numCombMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in clause 8.3.2.7 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- If umve_flag[ x0 ][ y0 ] is equal to 1, merge_idx[ xCb ][ yCb ] is set to base_mv_idx[ xCb ][ yCb ]

- The variables refineMvL0 and refineMvL1 are derived as follows:

- If umve_flag[ x0 ][ y0 ] is equal to 0, the following applies:

  - refineMvL0[0] = 0

  - refineMvL0[1] = 0

  - refineMvL1[0] = 0

  - refineMvL1[1] = 0

- Otherwise (umve_flag[ x0 ][ y0 ] is equal to 1), the following applies with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ):

  - If both predFlagL0N and predFlagL1N are equal to 1, the following applies:

    - If DiffPicOrderCnt( RefPicList0[ refIdxLN0 ], currPic) * DiffPicOrderCnt( currPic, RefPicList1[ refIdxLN1 ] ) is greater than 0, the following applies:

      - refineMvL0[0] = (DistFromBaseMV<<2) * sign[x0][y0][0]

      - refineMvL0[1] = (DistFromBaseMV<<2) * sign[x0][y0][1]

      - refineMvL1[0] = -1 * (DistFromBaseMV<<2) * sign[x0][y0][0]

      - refineMvL1[1] = -1 * (DistFromBaseMV<<2) * sign[x0][y0][1]

    - Otherwise if both predFlagL0N and predFlagL1N are equal to 1 and DiffPicOrderCnt( RefPicList0[ refIdxLN0 ], currPic) * DiffPicOrderCnt( currPic, RefPicList1[ refIdxLN1 ] ) is less than 0, the following applies:

      - refineMvL0[0] = (DistFromBaseMV<<2) * sign[x0][y0][0]

      - refineMvL0[1] = (DistFromBaseMV<<2) * sign[x0][y0][1]

      - refineMvL1[0] = (DistFromBaseMV<<2) * sign[x0][y0][0]

      - refineMvL1[1] = (DistFromBaseMV<<2) * sign[x0][y0][1]

    - if Abs(DiffPicOrderCnt( RefPicList0[ refIdxLN0 ], currPic)) is greater than Abs(DiffPicOrderCnt( currPic, RefPicList1[ refIdxLN1 ] ), the following applies:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )

refineMvL1 = Clip3( −32768, 32767, Sign( distScaleFactor * refineMvL1) *

( ( Abs( distScaleFactor * refineMvL1) + 127 )  >>  8 ) )

where td and tb are derived as follows:

td = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList0[ refIdxLN0 ]) )

tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList1[ refIdxLN1 ]) )

    - Otherwise if Abs(DiffPicOrderCnt( RefPicList0[ refIdxLN0 ], currPic)) is less than Abs(DiffPicOrderCnt( currPic, RefPicList1[ refIdxLN1 ] ) ), the following applies:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )

refineMvL0 = Clip3( −32768, 32767, Sign( distScaleFactor * refineMvL0) *

( ( Abs( distScaleFactor * refineMvL0) + 127 )  >>  8 ) )

where td and tb are derived as follows:

td = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList1[ refIdxLN1 ]) )

tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList0[ refIdxLN0 ]) )

  - Otherwise if predFlagL0N is equal to 1, the following applies:

    - refineMvL0[0] = (DistFromBaseMV<<2) * sign[x0][y0][0]

    - refineMvL0[1] = (DistFromBaseMV<<2) * sign[x0][y0][1]

    - refineMvL1[0] = 0

    - refineMvL1[1] = 0

  - Otherwise (if predFloagL1N is equal to 1), the following applies:

    - refineMvL0[0] = 0

    - refineMvL0[1] = 0

    - refineMvL1[0] = (DistFromBaseMV<<2) * sign[x0][y0][0]

    - refineMvL1[1] = (DistFromBaseMV<<2) * sign[x0][y0][1]

- The following applies:

- If umve_flag[ x0 ][ y0 ] equal to 0 and mergeCandList[ merge_idx[ xCb ][ yCb ] ] is equal to SbCol, the following assignments are made for xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1

refIdxLX = refIdxLXSbCol		(8-119)

predFlagLX[ xSbIdx ][ ySbIdx ] = predFlagLXSbCol[ xSbIdx ][ ySbIdx ]	(8-120)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 0 ]	(8-121)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 1 ]	(8-122)

- Otherwise, the following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

numSbX = 1		(8-123)

numSbY = 1		(8-124)

refIdxLX = refIdxLXN		(8-125)

predFlagLX[ 0 ][ 0 ] = predFlagLXN

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ] + refineMvLX[0]

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ] + refineMvLX[1]

- 8.3.2.4 Derivation process for subblock-based temporal merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, and availableFlagB1 of the neighbouring coding units,

- the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, and refIdxLXB1 of the neighbouring coding units,

- the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, and predFlagLXB1 of the neighbouring coding units,

- the motion vectors in 1/16 fractional-sample accuracy mvLXA0, mvLXA1, mvLXB0, and mvLXB1 of the neighbouring coding units.

Outputs of this process are:

- the availability flag availableFlagSbCol,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the reference indices refIdxL0SbCol and refIdxL1SbCol,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0SbCol[ xSbIdx ][ ySbIdx ] and mvL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the prediction list utilization flags predFlagL0SbCol[ xSbIdx ][ ySbIdx ] and predFlagL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

The availability flag availableFlagSbCol is derived as follows.

- If either slice_temporal_mvp_enable_flag or sps_sbtmvp_flag is equal to 0, availableFlagSbCol is set equal to 0.

- Otherwise, if umve_flag[x0][y0] is equal to 0, availableFlagSbCol is set equal to 0.

- Otherwise, the following ordered steps apply.

- The location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block and the location ( xCtr, yCtr ) of the below-right center sample of the current luma coding block are derived as follows:

- 8.3.2.6 Derivation process for combined bi-predictive merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0combCandk and mvL1combCandk of every new candidate combCandk added into mergeCandList during the invocation of this process.

The variable MaxNumMergeCandTemp is derived as follows:

- If umve_flag[x0][y0] is equal to 0, MaxNumMergeCandTemp is set to equal to MaxNumMergeCand.

- Otherwise (if umve_flag[x0][y0] is equal to 1, MaxNumMergeCandTemp is set to equal to 2.

When numOrigMergeCand is greater than 1 and less than MaxNumMergeCandTemp, the variable numInputMergeCand is set equal to numCurrMergeCand, the variable combIdx is set equal to 0, the variable combStop is set equal to FALSE and the following ordered steps are repeated until combStop is equal to TRUE:

- The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Table 8-6.

- The following assignments are made, with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList:

  - l0Cand = mergeCandList[ l0CandIdx ]

  - l1Cand = mergeCandList[ l1CandIdx ]

- When all of the following conditions are true:

  - predFlagL0l0Cand  = =  1

  - predFlagL1l1Cand  = =  1

  - ( DiffPicOrderCnt( RefPicList0[ refIdxL0l0Cand ], RefPicList1[ refIdxL1l1Cand ] )  !=  0 )  | |  
( mvL0l0Cand  !=  mvL1l1Cand )

the candidate combCandk with k equal to ( numCurrMergeCand − numInputMergeCand ) is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to combCandk, and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are derived as follows and numCurrMergeCand is incremented by 1:

refIdxL0combCandk = refIdxL0l0Cand	(8-167)

refIdxL1combCandk = refIdxL1l1Cand	(8-168)

predFlagL0combCandk = 1	(8-169)

predFlagL1combCandk = 1	(8-170)

mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]	(8-171)

mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]	(8-172)

mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]	(8-173)

mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]	(8-174)

numCurrMergeCand = numCurrMergeCand + 1	(8-175)

- The variable combIdx is incremented by 1.

- When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numCurrMergeCand is equal to MaxNumMergeCandTemp, combStop is set equal to TRUE.

Table 8-6 – Specification of l0CandIdx and l1CandIdx

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 |
| l1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 |

- 8.3.2.7  Derivation process for zero motion vector merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process,

- the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process.

The variable MaxNumMergeCandTemp is derived as follows:

- If umve_flag[x0][y0] is equal to 0, MaxNumMergeCandTemp is set to equal to MaxNumMergeCand.

- Otherwise (if umve_flag[x0][y0] is equal to 1, MaxNumMergeCandTemp is set to equal to 2.

The variable numRefIdx is derived as follows:

- If slice_type is equal to P, numRefIdx is set equal to num_ref_idx_l0_active_minus1 + 1.

- Otherwise (slice_type is equal to B), numRefIdx is set equal to Min( num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1 ).

When numCurrMergeCand is less than MaxNumMergeCandTemp, the variable numInputMergeCand is set equal to numCurrMergeCand, the variable zeroIdx is set equal to 0 and the following ordered steps are repeated until numCurrMergeCand is equal to MaxNumMergeCandTemp:

- 9.5.3.1  General

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element.

Table 9-4 specifies the type of binarization process associated with each syntax element and corresponding inputs.

The specification of the truncated Rice (TR) binarization process, the k-th order Exp-Golomb (EGk) binarization process and the fixed-length (FL) binarization process are given in clauses 9.5.3.3 through 9.5.3.5, respectively.

| Table 9-4 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_quadtree( ) | qt_split_cu_flag[ ][ ] | FL | cMax = 1 |
| multi_type_tree( ) | mtt_split_cu_flag | FL | cMax = 1 |
|  | mtt_split_cu_vertical_flag | FL | cMax = 1 |
|  | mtt_split_cu_binary_flag | FL | cMax = 1 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | intra_luma_mpm_remainder[ ][ ] | FL | cMax = 63 |
|  | intra_chroma_pred_mode[ ][ ] | 9.5.3.6 | - |
|  | merge_affine_flag[ ][ ] | FL | cMax = 1 |
|  | merge_flag[ ][ ] | FL | cMax = 1 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
|  | umve_flag[ ][ ] | FL | cMax = 1 |
|  | base_mv_idx[ ][ ] | FL | cMax = 1 |
|  | distance_idx[ ][ ] | TR | cMax = 7, cRiceParam = 0 |
|  | direction_idx[ ][ ] | FL | cMax = 3, cRiceParam = 0 |
|  | inter_pred_idc[ x0 ][ y0 ] | 9.5.3.7 | - |
|  | inter_affine_flag[ ][ ] | FL | cMax = 1 |
|  | cu_affine_type_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
|  | amvr_mode[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | cu_cbf | FL | cMax = 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| transform_unit( ) | tu_cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cr[ ][ ][ ] | FL | cMax = 1 |
|  | cu_mts_flag[ ][ ] | FL | cMax = 1 |
| residual_coding( ) | transform_skip_flag[ ][ ][ ] | FL | cMax = 1 |
|  | last_sig_coeff_x_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | par_level_flag[ ] | FL | cMax = 1 |
|  | rem_abs_gt1_flag[ ] | FL | cMax = 1 |
|  | rem_abs_gt2_flag[ ] | FL | cMax = 1 |
|  | abs_remainder[ ] | 9.5.3.8 | cIdx, x0, y0, xC, yC, log2TbWidth, log2TbHeight |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |
|  | mts_idx[ ][ ] | FL | cMax = 3 |

- Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).