| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-K0056-v1 |
| --- | --- |

| Title: | [CE4.7.1 and CE4.7.2 – Non Temporal Illumination Compensation in JVET-K1024] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea |  | tamse.anish@samsung.com m.w.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution proposes alternative method to compensate for illumination difference between the reference frame and the current frame. The tool is called Non-Temporal Local Illumination Compensation. The illumination compensation method which was used in JEM makes access to the neighbouring reconstruction pixels and the corresponding collocated reference pixels to compute the parameters for the illumination compensation model. This pixel access causes increase in memory bandwidth. The proposed method tries to overcome this by deriving the illumination compensation parameters by using only the prediction pixels and the reconstruction pixels of the current CU.

This tool shows BD-Rate improvement of -0.36% compared to VTM 2.0.1 anchor under RA conditions.

# Introduction

Illumination compensation is an inter coding tool. It tries to compensate for illumination difference when directly using pixels from reference frame for current frame as prediction pixels is not ideal. The illumination compensation method which was used in JEM makes access to the neighbouring reconstruction pixels and the corresponding collocated reference pixels to compute the parameters for the illumination compensation model. This model is then applied to the current motion compensated pixels to compensate for illumination difference. The access to reference frame pixels and neighbouring reconstructed pixels is not desirable as it causes increase in memory bandwidth. The proposed method tries to overcome this by deriving the illumination compensation parameters in an alternative fashion. The parameters are derived using the motion compensation pixels of a given block and the reconstructed pixels of the same block. The derived parameters are then stored at CU level. When a future block needs to use illumination compensation, it borrows parameters from a neighbouring CU which stores previously computed parameter.

# Tool Description

- Computing and storing the parameters

At the end of processing of each CU, linear models (Ax + B = y) are computed by comparing the prediction pixels (x) and the reconstructed pixels (y). The models are computed using linear regression. The lower edge and right edge pixels are used for model computation both the models are stored. Each model contains parameters for each colour component and for each of the two bi-prediction directions (if available). These parameters (sets of A and B) are then stored in the PU for later access.

Furthermore the parameters are only computed for CUs whose width and height are longer than or equal to 8. This reduces the hardware implementation cost considerably as compared to when it is computed for all the CUs.

Figure 1 – Parameter computation process

- Tool application

The conditions under which this tool is applicable depends on whether the neighboring blocks are skip coded and/or IC coded and the size of the current block. If applicable, a flag is signaled to indicate whether IPR is to be applied to a given inter coded CU or not. If the tool is to be applied, the motion compensation pixels (x) are refined using the application of the linear model (Ax + B) to obtain the new set of prediction pixels. The model used is derived from the current CU’s neighbors. Top, top left, left and left bottom neighbours are considered for borrowing the parameter. Since multiple neighbors may be available, pruning is performed to obtain a single model. Intra coded neighbours are discarded in the pruning process. Next, the neighbours which have trivial model [x = y] are discarded in the pruning process. Models not having the same reference indices are discarded next. If no model is remaining, a trivial model is used. If model(s) still remain, a predefined priority order is used for checking the neighbors.

Figure 1 – Model parameter derivation

Additionally, when performing RDO check for determining whether IPR will be used or not, mean-removed SAD (Sum of Absolute Difference) is used instead of the regular SAD for the ME (Motion Estimation) process.

The linear regression method used is same as that in JEM. Also, using two sides for parameter derivation improves the gain.

# Tested variations

- Test 4.7.1 – Compute and store only bottom edge parameters. During model derivation, use only top neighbors.

- Test 4.7.2 – Compute and store both right and bottom edge parameters. During model derivation use both top and left neighbors.

# Experimental Results

The experiments were performed on JVET CTC. The software was compiled using GCC version 7.3. SIMD optimization level used was up to AVX.

Test 4.7.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.21% | -0.54% | 134% | 100% |
| Class A2 | -0.07% | -0.05% | -0.20% | 115% | 104% |
| Class B | -0.31% | -0.20% | -0.09% | 123% | 105% |
| Class C | 0.02% | 0.10% | 0.07% | 130% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.09% | -0.16% | 125% | 104% |
| Class D | 0.20% | 0.13% | 0.21% | 127% | 108% |
| Class F (optional) | -0.31% | -0.15% | -0.18% | 121% | 105% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.29% | 0.20% | 0.21% | 153% | 103% |
| Class E | 0.01% | 0.17% | 0.27% | 107% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.40% | 0.47% | 0.30% | 138% | 108% |
| Class F (optional) | -1.11% | -0.83% | -0.87% | 145% | 106% |

Test 4.7.2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -0.42% | -0.71% | 134% | 103% |
| Class A2 | -0.12% | -0.17% | -0.20% | 114% | 106% |
| Class B | -0.49% | -0.32% | -0.23% | 123% | 108% |
| Class C | -0.12% | -0.03% | -0.15% | 128% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.23% | -0.30% | 125% | 107% |
| Class D | 0.13% | -0.02% | 0.15% | 126% | 111% |
| Class F (optional) | -0.56% | -0.39% | -0.40% | 121% | 108% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.08% | 0.10% | -0.05% | 143% | 108% |
| Class E | 0.00% | -0.04% | 0.26% | 107% | 109% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.33% | 0.25% | -0.07% | 135% | 111% |
| Class F (optional) | -1.82% | -1.02% | -1.33% | 142% | 109% |

# Patent rights declaration(s)

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).