| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0058 |
| --- | --- |

| Title: | CE6: NSST with modified NSST sets and signaling (Test2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiho Choi, Minsoo Park, Kwang Pyo Choi Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | kiho14.choi@samsung.com mss.park@samsung.com kp5.choi@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

- Abstract

This contribution presents a technology described in CE 6.2.3 that is NSST with modified sets and signaling. The proposed method uses the following four parts: 1) reduced angle sets in total as half of that in NSST, 2) new angles sets considering four modes, 3) additional on/off flag to indicate NSST usage, and 4) enabling NSST 8x8 in BMS. Testing results on the proposed method without NSST 8x8 (CE 6.2.3-a) show 0.6%, 0.4%, 0.2%, and 0.2% BD rate savings on average for All Intra (AI), Random Access (RA), Low Delayed B (LDB), and Low Delayed P (LDP) configurations, respectively, compared to BMS2.0.1 with VTM configure setting; and 0.3%, 0.1%, 0.0%, and 0.0% BD rate losses on average for AI, RA, LDB and LDP configurations, respectively, compared to BMS configure setting. Testing results on the proposed method with NSST 8x8 (CE 6.2.3-b) show 1.8%, 1.0%, 0.4%, and 0.4% BD rate savings on average for AI, RA, LDB, and LDP configurations, respectively, compared to BMS2.0.1 with VTM configure setting; and 0.9%, 0.6%, 0.3%, and 0.2% BD rate savings on average for AI, RA, LDB and LDP configurations, respectively, compared to BMS configure setting.

Proposed method

The proposed method in this contribution was introduced in JVET-K0174 [1]. Main difference between NSST in BMS and the proposed method is the following four parts: 1) reduced angle sets in total as half of that in NSST, 2) new angles sets considering four modes, 3) additional on/off flag to indicate NSST usage, and 4) enabling NSST 8x8 in BMS. In this contribution, two sets are evaluated with and without NSST 8x8 on top of proposed method.

NSST in BMS uses totally 35×3 non-separable secondary transforms for 4×4block size, where 35 is the number of transform sets specified by the intra prediction mode, denoted as set, and 3 is the number of NSST candidate for each intra prediction mode.

In the proposed method, NSST is using totally 12x4 non-separable secondary transforms. The total number of non-separable secondary transforms and modes to be evaluated in a block are different from those in BMS. Candidates to be tested in a block are four where 12 is the number of transform sets specified by the intra prediction mode, denoted as set, and 4 is the number of NSST candidate for each intra prediction mode. Table 1 shows grouping of intra prediction for candidates of NSST where intra65 is enabled.

Table 1: Mapping from intra prediction mode to transform set index in the proposed method

| Grouped id | Intra prediction index |
| --- | --- |
| 0 | 0, 1 |
| 1 | 2, 4, 6, 62, 64, 66 |
| 2 | 3, 5, 7, 61, 63, 65 |
| 3 | 8, 10, 12, 56, 58, 60 |
| 4 | 9, 11, 13, 55, 57, 59 |
| 5 | 14, 15, 16, 52, 53, 54 |
| 6 | 17, 18, 19, 49, 50, 51 |
| 7 | 20, 21, 22, 46, 47, 48 |
| 8 | 23, 25, 27, 41, 43, 45 |
| 9 | 24, 26, 28, 40, 42, 44 |
| 10 | 29, 31, 33, 35, 37, 39 |
| 11 | 30, 32, 34, 36, 38 |

In order to support four modes in the proposed NSST, a flag for on/off is added and mode flag are used to indicate which mode should be applied. The mode flag is represented with two bits, then those bits are encoded with CABAC coding.

Compared to BMS in NSST, the proposed method uses reduced look-up table for NSST sets about half of NSST storage. But, to evaluate additional mode in the given sets, one RDO loop is additionally needed.

Test result

Test conditions follows the description in CE6 document [2]. BMS SW was used for these experiments, as specified in JVET-K1010 [3].

Test result compared to anchor

Table 2 shows test results on CE 6.2.3-a compared to anchors of VTM and BMS where the proposed method disables NSST 8x8.

Table 2: Test results on CE 6.2.3-a (Proposed method without NSST8x8)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.4% | -1.6% | -1.6% | 349% | 97% | 0.3% | 0.6% | 0.4% | 125% | 100% |
| Class A2 | -0.4% | -1.7% | -1.4% | 365% | 101% | 0.2% | 0.6% | 0.3% | 126% | 101% |
| Class B | -0.5% | -1.5% | -2.8% | 388% | 101% | 0.2% | 0.5% | 0.3% | 127% | 101% |
| Class C | -0.8% | -1.6% | -2.5% | 365% | 100% | 0.4% | 0.6% | 0.4% | 127% | 99% |
| Class E | -1.0% | -2.0% | -3.7% | 380% | 100% | 0.4% | 0.6% | 0.5% | 126% | 99% |
| Overall | -0.6% | -1.7% | -2.4% | 371% | 100% | 0.3% | 0.6% | 0.4% | 126% | 100% |
| Class D | -1.0% | -1.5% | -2.5% | 391% | 101% | 0.4% | 0.6% | 0.4% | 128% | 100% |
| Class F (optional) | -1.0% | -1.5% | -1.9% | 384% | 102% | 0.3% | 0.4% | 0.3% | 132% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.4% | -1.6% | -1.8% | 175% | 102% | 0.1% | 0.4% | 0.3% | 111% | 100% |
| Class A2 | -0.3% | -1.5% | -1.3% | 165% | 100% | 0.0% | 0.3% | 0.1% | 111% | 101% |
| Class B | -0.3% | -2.0% | -3.2% | 177% | 101% | 0.1% | 0.4% | 0.2% | 111% | 100% |
| Class C | -0.4% | -1.3% | -1.9% | 187% | 100% | 0.1% | 0.2% | 0.3% | 113% | 100% |
| Overall | -0.4% | -1.6% | -2.2% | 177% | 101% | 0.1% | 0.3% | 0.2% | 112% | 100% |
| Class D | -0.4% | -1.4% | -2.1% | 181% | 100% | 0.1% | 0.6% | 0.0% | 112% | 100% |
| Class F (optional) | -0.6% | -1.1% | -1.5% | 200% | 100% | 0.1% | 0.2% | 0.2% | 115% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.1% | -0.6% | -1.1% | 146% | 99% | 0.0% | 0.2% | 0.1% | 108% | 101% |
| Class C | -0.1% | -0.6% | -1.0% | 147% | 100% | 0.0% | -0.2% | 0.2% | 108% | 100% |
| Class E | -0.4% | -0.9% | -2.2% | 114% | 99% | -0.1% | 0.1% | -0.3% | 103% | 100% |
| Overall | -0.2% | -0.7% | -1.3% | 137% | 100% | 0.0% | 0.0% | 0.0% | 107% | 101% |
| Class D | 0.0% | -0.2% | -0.5% | 136% | 101% | 0.0% | 0.2% | 0.8% | 102% | 99% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.1% | -0.5% | -0.8% | 169% | 102% | 0.0% | 0.1% | 0.1% | 114% | 102% |
| Class C | -0.1% | -0.7% | -1.0% | 172% | 101% | 0.0% | -0.1% | 0.2% | 111% | 101% |
| Class E | -0.4% | -1.2% | -2.7% | 123% | 99% | 0.1% | 0.1% | -0.4% | 103% | 99% |
| Overall | -0.2% | -0.8% | -1.3% | 157% | 101% | 0.0% | 0.1% | 0.0% | 110% | 101% |
| Class D | -0.1% | 0.1% | -0.8% | 160% | 101% | 0.0% | 1.0% | 0.7% | 108% | 98% |
| Class F (optional) | -0.1% | -0.8% | -1.1% | 174% | 101% | 0.0% | 0.3% | 0.3% | 111% | 101% |
| Class F (optional) | -0.1% | -0.5% | -0.8% | 169% | 102% | 0.0% | 0.1% | 0.1% | 114% | 102% |

Table 3 shows test results on CE 6.2.3-b compared to anchors of VTM and BMS where the proposed method enables NSST 8x8.

Table 3: Test results on CE 6.2.3-b (Proposed method with NSST8x8)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.1% | -2.8% | -2.6% | 353% | 96% | -1.4% | -0.6% | -0.6% | 127% | 99% |
| Class A2 | -1.3% | -2.5% | -2.1% | 369% | 103% | -0.7% | -0.3% | -0.4% | 127% | 103% |
| Class B | -1.5% | -2.6% | -3.5% | 397% | 106% | -0.8% | -0.5% | -0.4% | 134% | 103% |
| Class C | -2.1% | -2.5% | -3.2% | 397% | 104% | -0.9% | -0.3% | -0.3% | 138% | 103% |
| Class E | -2.2% | -2.9% | -4.3% | 387% | 102% | -0.8% | -0.3% | -0.1% | 135% | 102% |
| Overall | -1.8% | -2.6% | -3.2% | 383% | 103% | -0.9% | -0.4% | -0.4% | 133% | 102% |
| Class D | -2.0% | -2.2% | -3.1% | 413% | 104% | -0.7% | -0.1% | -0.2% | 135% | 103% |
| Class F (optional) | -2.0% | -2.1% | -2.5% | 398% | 104% | -0.6% | -0.2% | -0.4% | 131% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.3% | -2.3% | -2.5% | 183% | 102% | -0.8% | -0.5% | -0.4% | 115% | 101% |
| Class A2 | -0.7% | -1.7% | -1.5% | 174% | 101% | -0.4% | 0.1% | -0.2% | 114% | 101% |
| Class B | -0.9% | -2.8% | -3.7% | 185% | 101% | -0.5% | -0.4% | -0.4% | 116% | 101% |
| Class C | -1.1% | -1.8% | -2.2% | 195% | 101% | -0.6% | -0.4% | -0.2% | 118% | 101% |
| Overall | -1.0% | -2.2% | -2.6% | 185% | 101% | -0.6% | -0.3% | -0.3% | 116% | 101% |
| Class D | -0.9% | -1.7% | -1.9% | 191% | 101% | -0.4% | 0.2% | 0.1% | 116% | 102% |
| Class F (optional) | -1.2% | -1.5% | -1.8% | 213% | 101% | -0.5% | -0.3% | -0.3% | 121% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.3% | -1.2% | -1.4% | 155% | 100% | -0.2% | -0.2% | -0.3% | 110% | 101% |
| Class C | -0.4% | -0.8% | -1.0% | 152% | 101% | -0.3% | -0.3% | -0.1% | 110% | 101% |
| Class E | -0.8% | -2.1% | -3.0% | 115% | 99% | -0.3% | -0.4% | -0.6% | 103% | 101% |
| Overall | -0.4% | -1.3% | -1.7% | 143% | 100% | -0.3% | -0.3% | -0.3% | 109% | 101% |
| Class D | -0.2% | 0.2% | -0.9% | 139% | 101% | -0.1% | -0.2% | 0.4% | 104% | 100% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.4% | -0.9% | -1.4% | 176% | 100% | -0.2% | -0.3% | -0.5% | 114% | 104% |
| Class C | -0.4% | -1.0% | -1.0% | 180% | 102% | -0.3% | -0.1% | -0.2% | 114% | 103% |
| Class E | -0.5% | -1.7% | -2.9% | 126% | 100% | -0.2% | -0.8% | -1.2% | 104% | 100% |
| Overall | -0.4% | -1.1% | -1.7% | 163% | 101% | -0.2% | -0.4% | -0.6% | 112% | 103% |
| Class D | -0.2% | -0.3% | -0.5% | 167% | 102% | -0.2% | 0.6% | 0.0% | 111% | 99% |
| Class F (optional) | -0.4% | -0.8% | -1.1% | 181% | 102% | -0.3% | 0.0% | -0.1% | 115% | 101% |
| Class F (optional) | -0.4% | -0.9% | -1.4% | 176% | 100% | -0.2% | -0.3% | -0.5% | 114% | 104% |

VTM test result

For testing proposed method (i.e., CE 6.2.3-a) on VTM, VTM configuration files were used with BMS 2.0.1 software, and results are shown in Table 2. The average BD-rate gains for AI, RA, LDB and LDP are 0.6%, 0.4%, 0.2% and 0.2% respectively.

For testing proposed method (i.e., CE 6.2.3-b) on VTM, VTM configuration files were used with BMS 2.0.1 software, and results are shown in Table 3. The average BD-rate gains for AI, RA, LDB and LDP are 1.8%, 1.0%, 0.4% and 0.4% respectively.

BMS test result

For testing proposed method (i.e., CE 6.2.3-a) on BMS, BMS configuration files were used with BMS 2.0.1 software, and results are shown in Table 2. The average BD-rate losses for AI, RA, LDB and LDP are 0.3%, 0.1%, 0.0% and 0.0% respectively.

For testing proposed method (i.e., CE 6.2.3-b) on BMS, BMS configuration files were used with BMS 2.0.1 software, and results are shown in Table 3. The average BD-rate gains for AI, RA, LDB and LDP are 0.9%, 0.6%, 0.3% and 0.2% respectively.

Conclusion

This contribution presents a technology described in CE 6.2.3 that is NSST with modified sets and signaling. Main difference between NSST in BMS and the proposed method is the following four parts: 1) reduced angle sets in total as half of that in NSST, 2) new angles sets considering four modes, 3) additional on/off flag to indicate NSST usage, and 4) enabling NSST 8x8 in BMS. Testing results described in the CE documents are provided in this contribution. The benefit of using the proposed method comparing to NSST in BMS is that the proposed method uses only half of NSST tables while preserving coding performance. It is recommend to adopt the proposed method into the next version of test model of VVC.

Reference

[1] K. Choi, M. Park, C. Kim, “CE6: NSST with modified NSST sets and signaling (Test2.3)”, JVET-K0174, Ljubljana, SI, July 2018

[2] A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-K1026, Ljubljana, SI, July 2018

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard.