| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0060 |
| --- | --- |

| Title: | CE6-related: Unified matrix for transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiho Choi, Kwang Pyo Choi Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | kiho14.choi@samsung.com kp5.choi@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

- Abstract

This contribution presents a unified matrix for all kernels of transform. Currently, VTM2.0 uses three types of transform as DCT2, DCT8, and DST7 depending on supported block size. The proposed a matrix in this contribution can provide all three types of transform with all block sizes from 4x4 to 64x64 by using a 64x64 matrix designed with 8-bit representation. Testing results on the proposed method show 0.0%, 0.1%, 0.1% and 0.2% BD rate losses on average for All Intra (AI), Random Access (RA), Low Delayed B (LDB), and Low Delayed P (LDP) configurations, respectively, compared to VTM configure setting, and 0.0%, 0.1%, and 0.0% BD rate losses on average for AI, RA, and LDP respectively, and 0.1% gain for LDB configuration, compared to BMS configure setting.

Introduction

Currently, VTM2.0 uses three types of transform as DCT2, DCT8, and DST7 depending on supported block size. Table 1 shows supported transforms according to block size with MTS usage. If MTS is disabled, DCT2 is only supported in VTM2.0 with block size from 4x4 to 64x64.

Table 1. Transform with supported block size

|  | MTS usage | Supported block size for Luma | Coefficient representation |
| --- | --- | --- | --- |
| DCT2 | X | 4x4 - 64x64 | 10-bit |
| DCT8 | O | 4x4 - 32x32 | 10-bit |
| DST7 | O | 4x4 - 32x32 | 10-bit |

The intention of this contribution is to improve the existing transforms in two aspects.

- For the use of MTS, current SW and specification contain all matrixes for all types of transforms with supported block sizes.

- The current coefficients of basis vectors are designed with 10-bit representation.

In HEVC transform design, DCT2 was supported from 4x4 to 32x32 block size, and all kernels can be derived from one matrix by sampling a subset of coefficients and basis vectors of larger transforms. So, there is no individual matrix for type or block size, and it would be allowed for re-use of logic for multiply/adds in hardware design.

And, coefficients and basis vectors are designed with 8-bit representation, so it allows to limit the bit width to 16 at all intermediate stages. The design of coefficients provides more additional flexibility in implementations, such as direct matrix multiplications with SIMD instructions for the innermost loop, or multi-stage butterfly implementations utilizing symmetry properties of the basis vectors.

In order to improve the two aspects mentioned above, this contribution presents a unified matrix for all kernels of transform designed with 8-bit representation.

Proposed method

During the development of transform for VVC standard, the following facts have been observed:

- A partial of the odd bases in DCT-2 is very similar to the scaled DCT-8 bases [1]

- DST-7 is identical with DCT-8 by flipping matrix of DCT-8 horizontally [2]

- Coefficients with 8-bit representation does not affect coding loss in all scenarios [3]

From those observations, this contribution proposed a unified matrix designed with 8-bit representation. The proposed unified matrix is based on 64x64 that can derive all kernel type with all block sizes used in VTM. The proposed matrix is shown in Table 2. The basis vectors shown in Table 2 have the properties supported in transform of HEVC:

- Even rows with index 0,2,4,… are symmetric with a symmetry point before coefficient N/2.

- Odd rows with index 1,3,5,7,…are anti-symmetric with a symmetry point before coefficient N/2.

- Even rows with index 0,4,8,… have additional symmetry point before elements N/4 and 3N/4.

- Even rows with index 2,6,10,…have additional anti-symmetry points before elements N/4 and 3N/4.

And, similar to bit width in HEVC, the bit width of all intermediate transform stages is 16.

DCT2 can be derived by sampling a subset of coefficients in the matrix as follows:

    - DCT2_NxN[l, k] = matrix_64x64[l, 2(6- log2N) *k]

    - where DCT2_NxN[l, k] represents the l-th element of the k-th basis vector of DCT-2 for NxN matrix and N is a size of matrix from 64 to 2.

DCT8 can be derived by sampling a subset of coefficients in the matrix as follows:

    - DCT8_NxN[l, k] = matrix_64x64[l, 2(6- log2N) *k +2(5 - log2N)]

    - where DCT8_NxN[l, k] represents the l-th element of the k-th basis vector of DCT-8 for NxN matrix and N is a size of matrix from 32 to 4.

DCT7 can be derived by sampling a subset of coefficients in the matrix as follows:

    - DCT7_NxN[l, k] = (-1)^k * DCT8_NxN[N – l - 1, k]

    - DCT7_NxN[l, k] = (-1)^k * matrix_64x64[N – l - 1, 2(6- log2N) *k +2(5 - log2N)]

    - where DCT7_NxN[l, k] represents the l-th element of the k-th basis vector of DCT-7 for NxN matrix and N is a size of matrix from 32 to 4.

Table 2. Proposed unified 64x64 matrix for transform

64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 90, 90, 89, 89, 88, 87, 86, 84, 82, 81, 79, 76, 74, 71, 69, 66, 63, 60, 56, 53, 49, 46, 42, 38, 34, 30, 26, 22, 17, 13, 9, 4, -4, -9,-13,-17,-22,-26,-30,-34,-38,-42,-46,-49,-53,-56,-60,-63,-66,-69,-71,-74,-76,-79,-81,-82,-84,-86,-87,-88,-89,-89,-90,-90, 89, 88, 87, 84, 81, 77, 73, 67, 62, 55, 48, 41, 33, 25, 17, 9, -9,-17,-25,-33,-41,-48,-55,-62,-67,-73,-77,-81,-84,-87,-88,-89,-89,-88,-87,-84,-81,-77,-73,-67,-62,-55,-48,-41,-33,-25,-17, -9, 9, 17, 25, 33, 41, 48, 55, 62, 67, 73, 77, 81, 84, 87, 88, 89, 90, 88, 84, 79, 71, 63, 53, 42, 30, 17, 4, -9,-22,-34,-46,-56,-66,-74,-81,-86,-89,-90,-89,-87,-82,-76,-69,-60,-49,-38,-26,-13, 13, 26, 38, 49, 60, 69, 76, 82, 87, 89, 90, 89, 86, 81, 74, 66, 56, 46, 34, 22, 9, -4,-17,-30,-42,-53,-63,-71,-79,-84,-88,-90, 90, 87, 80, 70, 58, 43, 26, 9, -9,-26,-43,-58,-70,-80,-87,-90,-90,-87,-80,-70,-58,-43,-26, -9, 9, 26, 43, 58, 70, 80, 87, 90, 90, 87, 80, 70, 58, 43, 26, 9, -9,-26,-43,-58,-70,-80,-87,-90,-90,-87,-80,-70,-58,-43,-26, -9, 9, 26, 43, 58, 70, 80, 87, 90, 89, 84, 74, 60, 42, 22, 0,-22,-42,-60,-74,-84,-89,-89,-84,-74,-60,-42,-22, 0, 22, 42, 60, 74, 84, 89, 89, 84, 74, 60, 42, 22,-22,-42,-60,-74,-84,-89,-89,-84,-74,-60,-42,-22, 0, 22, 42, 60, 74, 84, 89, 89, 84, 74, 60, 42, 22, 0,-22,-42,-60,-74,-84,-89, 88, 81, 67, 48, 25, 0,-25,-48,-67,-81,-88,-88,-81,-67,-48,-25, 25, 48, 67, 81, 88, 88, 81, 67, 48, 25, 0,-25,-48,-67,-81,-88,-88,-81,-67,-48,-25, 0, 25, 48, 67, 81, 88, 88, 81, 67, 48, 25,-25,-48,-67,-81,-88,-88,-81,-67,-48,-25, 0, 25, 48, 67, 81, 88, 89, 79, 60, 34, 4,-26,-53,-74,-87,-90,-82,-66,-42,-13, 17, 46, 69, 84, 90, 86, 71, 49, 22, -9,-38,-63,-81,-89,-88,-76,-56,-30, 30, 56, 76, 88, 89, 81, 63, 38, 9,-22,-49,-71,-86,-90,-84,-69,-46,-17, 13, 42, 66, 82, 90, 87, 74, 53, 26, -4,-34,-60,-79,-89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 88, 71, 42, 4,-34,-66,-86,-89,-76,-49,-13, 26, 60, 82, 90, 81, 56, 22,-17,-53,-79,-90,-84,-63,-30, 9, 46, 74, 89, 87, 69, 38,-38,-69,-87,-89,-74,-46, -9, 30, 63, 84, 90, 79, 53, 17,-22,-56,-81,-90,-82,-60,-26, 13, 49, 76, 89, 86, 66, 34, -4,-42,-71,-88, 87, 67, 33, -9,-48,-77,-89,-81,-55,-17, 25, 62, 84, 88, 73, 41,-41,-73,-88,-84,-62,-25, 17, 55, 81, 89, 77, 48, 9,-33,-67,-87,-87,-67,-33, 9, 48, 77, 89, 81, 55, 17,-25,-62,-84,-88,-73,-41, 41, 73, 88, 84, 62, 25,-17,-55,-81,-89,-77,-48, -9, 33, 67, 87, 87, 63, 22,-26,-66,-88,-86,-60,-17, 30, 69, 89, 84, 56, 13,-34,-71,-89,-82,-53, -9, 38, 74, 90, 81, 49, 4,-42,-76,-90,-79,-46, 46, 79, 90, 76, 42, -4,-49,-81,-90,-74,-38, 9, 53, 82, 89, 71, 34,-13,-56,-84,-89,-69,-30, 17, 60, 86, 88, 66, 26,-22,-63,-87, 87, 58, 9,-43,-80,-90,-70,-26, 26, 70, 90, 80, 43, -9,-58,-87,-87,-58, -9, 43, 80, 90, 70, 26,-26,-70,-90,-80,-43, 9, 58, 87, 87, 58, 9,-43,-80,-90,-70,-26, 26, 70, 90, 80, 43, -9,-58,-87,-87,-58, -9, 43, 80, 90, 70, 26,-26,-70,-90,-80,-43, 9, 58, 87, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86, 84, 48, -9,-62,-88,-77,-33, 25, 73, 89, 67, 17,-41,-81,-87,-55, 55, 87, 81, 41,-17,-67,-89,-73,-25, 33, 77, 88, 62, 9,-48,-84,-84,-48, 9, 62, 88, 77, 33,-25,-73,-89,-67,-17, 41, 81, 87, 55,-55,-87,-81,-41, 17, 67, 89, 73, 25,-33,-77,-88,-62, -9, 48, 84, 84, 42,-22,-74,-89,-60, 0, 60, 89, 74, 22,-42,-84,-84,-42, 22, 74, 89, 60, 0,-60,-89,-74,-22, 42, 84, 84, 42,-22,-74,-89,-60, 60, 89, 74, 22,-42,-84,-84,-42, 22, 74, 89, 60, 0,-60,-89,-74,-22, 42, 84, 84, 42,-22,-74,-89,-60, 0, 60, 89, 74, 22,-42,-84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 82, 30,-42,-87,-76,-17, 53, 89, 69, 4,-63,-90,-60, 9, 71, 89, 49,-22,-79,-86,-38, 34, 84, 81, 26,-46,-88,-74,-13, 56, 90, 66,-66,-90,-56, 13, 74, 88, 46,-26,-81,-84,-34, 38, 86, 79, 22,-49,-89,-71, -9, 60, 90, 63, -4,-69,-89,-53, 17, 76, 87, 42,-30,-82, 81, 25,-48,-88,-67, 0, 67, 88, 48,-25,-81,-81,-25, 48, 88, 67,-67,-88,-48, 25, 81, 81, 25,-48,-88,-67, 0, 67, 88, 48,-25,-81,-81,-25, 48, 88, 67, 0,-67,-88,-48, 25, 81, 81, 25,-48,-88,-67, 67, 88, 48,-25,-81,-81,-25, 48, 88, 67, 0,-67,-88,-48, 25, 81, 81, 17,-60,-90,-49, 30, 86, 74, 4,-69,-88,-38, 42, 89, 66, -9,-76,-84,-26, 53, 90, 56,-22,-82,-79,-13, 63, 89, 46,-34,-87,-71, 71, 87, 34,-46,-89,-63, 13, 79, 82, 22,-56,-90,-53, 26, 84, 76, 9,-66,-89,-42, 38, 88, 69, -4,-74,-86,-30, 49, 90, 60,-17,-81, 80, 9,-70,-87,-26, 58, 90, 43,-43,-90,-58, 26, 87, 70, -9,-80,-80, -9, 70, 87, 26,-58,-90,-43, 43, 90, 58,-26,-87,-70, 9, 80, 80, 9,-70,-87,-26, 58, 90, 43,-43,-90,-58, 26, 87, 70, -9,-80,-80, -9, 70, 87, 26,-58,-90,-43, 43, 90, 58,-26,-87,-70, 9, 80, 79, 4,-74,-82,-13, 69, 86, 22,-63,-88,-30, 56, 89, 38,-49,-90,-46, 42, 90, 53,-34,-89,-60, 26, 87, 66,-17,-84,-71, 9, 81, 76,-76,-81, -9, 71, 84, 17,-66,-87,-26, 60, 89, 34,-53,-90,-42, 46, 90, 49,-38,-89,-56, 30, 88, 63,-22,-86,-69, 13, 82, 74, -4,-79, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 76, -9,-84,-66, 26, 89, 53,-42,-90,-38, 56, 88, 22,-69,-82, -4, 79, 74,-13,-86,-63, 30, 89, 49,-46,-90,-34, 60, 87, 17,-71,-81, 81, 71,-17,-87,-60, 34, 90, 46,-49,-89,-30, 63, 86, 13,-74,-79, 4, 82, 69,-22,-88,-56, 38, 90, 42,-53,-89,-26, 66, 84, 9,-76, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 74,-22,-89,-42, 60, 84, 0,-84,-60, 42, 89, 22,-74,-74, 22, 89, 42,-60,-84, 0, 84, 60,-42,-89,-22, 74, 74,-22,-89,-42, 60, 84,-84,-60, 42, 89, 22,-74,-74, 22, 89, 42,-60,-84, 0, 84, 60,-42,-89,-22, 74, 74,-22,-89,-42, 60, 84, 0,-84,-60, 42, 89, 22,-74, 73,-25,-89,-33, 67, 77,-17,-88,-41, 62, 81, -9,-87,-48, 55, 84,-84,-55, 48, 87, 9,-81,-62, 41, 88, 17,-77,-67, 33, 89, 25,-73,-73, 25, 89, 33,-67,-77, 17, 88, 41,-62,-81, 9, 87, 48,-55,-84, 84, 55,-48,-87, -9, 81, 62,-41,-88,-17, 77, 67,-33,-89,-25, 73, 71,-34,-89,-13, 82, 56,-53,-84, 9, 89, 38,-69,-74, 30, 90, 17,-81,-60, 49, 86, -4,-88,-42, 66, 76,-26,-90,-22, 79, 63,-46,-87, 87, 46,-63,-79, 22, 90, 26,-76,-66, 42, 88, 4,-86,-49, 60, 81,-17,-90,-30, 74, 69,-38,-89, -9, 84, 53,-56,-82, 13, 89, 34,-71, 70,-43,-87, 9, 90, 26,-80,-58, 58, 80,-26,-90, -9, 87, 43,-70,-70, 43, 87, -9,-90,-26, 80, 58,-58,-80, 26, 90, 9,-87,-43, 70, 70,-43,-87, 9, 90, 26,-80,-58, 58, 80,-26,-90, -9, 87, 43,-70,-70, 43, 87, -9,-90,-26, 80, 58,-58,-80, 26, 90, 9,-87,-43, 70, 69,-46,-84, 17, 90, 13,-86,-42, 71, 66,-49,-82, 22, 90, 9,-87,-38, 74, 63,-53,-81, 26, 89, 4,-88,-34, 76, 60,-56,-79, 30, 89,-89,-30, 79, 56,-60,-76, 34, 88, -4,-89,-26, 81, 53,-63,-74, 38, 87, -9,-90,-22, 82, 49,-66,-71, 42, 86,-13,-90,-17, 84, 46,-69, 67,-48,-81, 25, 88, 0,-88,-25, 81, 48,-67,-67, 48, 81,-25,-88, 88, 25,-81,-48, 67, 67,-48,-81, 25, 88, 0,-88,-25, 81, 48,-67,-67, 48, 81,-25,-88, 0, 88, 25,-81,-48, 67, 67,-48,-81, 25, 88,-88,-25, 81, 48,-67,-67, 48, 81,-25,-88, 0, 88, 25,-81,-48, 67, 66,-56,-74, 46, 81,-34,-86, 22, 89, -9,-90, -4, 89, 17,-87,-30, 82, 42,-76,-53, 69, 63,-60,-71, 49, 79,-38,-84, 26, 88,-13,-90, 90, 13,-88,-26, 84, 38,-79,-49, 71, 60,-63,-69, 53, 76,-42,-82, 30, 87,-17,-89, 4, 90, 9,-89,-22, 86, 34,-81,-46, 74, 56,-66, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 63,-66,-60, 69, 56,-71,-53, 74, 49,-76,-46, 79, 42,-81,-38, 82, 34,-84,-30, 86, 26,-87,-22, 88, 17,-89,-13, 89, 9,-90, -4, 90,-90, 4, 90, -9,-89, 13, 89,-17,-88, 22, 87,-26,-86, 30, 84,-34,-82, 38, 81,-42,-79, 46, 76,-49,-74, 53, 71,-56,-69, 60, 66,-63, 62,-67,-55, 73, 48,-77,-41, 81, 33,-84,-25, 87, 17,-88, -9, 89,-89, 9, 88,-17,-87, 25, 84,-33,-81, 41, 77,-48,-73, 55, 67,-62,-62, 67, 55,-73,-48, 77, 41,-81,-33, 84, 25,-87,-17, 88, 9,-89, 89, -9,-88, 17, 87,-25,-84, 33, 81,-41,-77, 48, 73,-55,-67, 62, 60,-74,-42, 84, 22,-89, 0, 89,-22,-84, 42, 74,-60,-60, 74, 42,-84,-22, 89, 0,-89, 22, 84,-42,-74, 60, 60,-74,-42, 84, 22,-89, 89,-22,-84, 42, 74,-60,-60, 74, 42,-84,-22, 89, 0,-89, 22, 84,-42,-74, 60, 60,-74,-42, 84, 22,-89, 0, 89,-22,-84, 42, 74,-60, 58,-80,-26, 90, -9,-87, 43, 70,-70,-43, 87, 9,-90, 26, 80,-58,-58, 80, 26,-90, 9, 87,-43,-70, 70, 43,-87, -9, 90,-26,-80, 58, 58,-80,-26, 90, -9,-87, 43, 70,-70,-43, 87, 9,-90, 26, 80,-58,-58, 80, 26,-90, 9, 87,-43,-70, 70, 43,-87, -9, 90,-26,-80, 58, 56,-81,-22, 90,-17,-82, 53, 60,-79,-26, 90,-13,-84, 49, 63,-76,-30, 89, -9,-86, 46, 66,-74,-34, 89, -4,-87, 42, 69,-71,-38, 88,-88, 38, 71,-69,-42, 87, 4,-89, 34, 74,-66,-46, 86, 9,-89, 30, 76,-63,-49, 84, 13,-90, 26, 79,-60,-53, 82, 17,-90, 22, 81,-56, 55,-81,-17, 89,-25,-77, 62, 48,-84, -9, 88,-33,-73, 67, 41,-87, 87,-41,-67, 73, 33,-88, 9, 84,-48,-62, 77, 25,-89, 17, 81,-55,-55, 81, 17,-89, 25, 77,-62,-48, 84, 9,-88, 33, 73,-67,-41, 87,-87, 41, 67,-73,-33, 88, -9,-84, 48, 62,-77,-25, 89,-17,-81, 55, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 49,-89, 22, 71,-79, -9, 86,-60,-38, 90,-34,-63, 84, -4,-81, 69, 26,-89, 46, 53,-88, 17, 74,-76,-13, 87,-56,-42, 90,-30,-66, 82,-82, 66, 30,-90, 42, 56,-87, 13, 76,-74,-17, 88,-53,-46, 89,-26,-69, 81, 4,-84, 63, 34,-90, 38, 60,-86, 9, 79,-71,-22, 89,-49, 48,-88, 25, 67,-81, 0, 81,-67,-25, 88,-48,-48, 88,-25,-67, 81,-81, 67, 25,-88, 48, 48,-88, 25, 67,-81, 0, 81,-67,-25, 88,-48,-48, 88,-25,-67, 81, 0,-81, 67, 25,-88, 48, 48,-88, 25, 67,-81, 81,-67,-25, 88,-48,-48, 88,-25,-67, 81, 0,-81, 67, 25,-88, 48, 46,-90, 42, 49,-90, 38, 53,-89, 34, 56,-89, 30, 60,-88, 26, 63,-87, 22, 66,-86, 17, 69,-84, 13, 71,-82, 9, 74,-81, 4, 76,-79, 79,-76, -4, 81,-74, -9, 82,-71,-13, 84,-69,-17, 86,-66,-22, 87,-63,-26, 88,-60,-30, 89,-56,-34, 89,-53,-38, 90,-49,-42, 90,-46, 43,-90, 58, 26,-87, 70, 9,-80, 80, -9,-70, 87,-26,-58, 90,-43,-43, 90,-58,-26, 87,-70, -9, 80,-80, 9, 70,-87, 26, 58,-90, 43, 43,-90, 58, 26,-87, 70, 9,-80, 80, -9,-70, 87,-26,-58, 90,-43,-43, 90,-58,-26, 87,-70, -9, 80,-80, 9, 70,-87, 26, 58,-90, 43, 42,-89, 60, 22,-84, 74, 0,-74, 84,-22,-60, 89,-42,-42, 89,-60,-22, 84,-74, 0, 74,-84, 22, 60,-89, 42, 42,-89, 60, 22,-84, 74,-74, 84,-22,-60, 89,-42,-42, 89,-60,-22, 84,-74, 0, 74,-84, 22, 60,-89, 42, 42,-89, 60, 22,-84, 74, 0,-74, 84,-22,-60, 89,-42, 41,-88, 62, 17,-81, 77, -9,-67, 87,-33,-48, 89,-55,-25, 84,-73, 73,-84, 25, 55,-89, 48, 33,-87, 67, 9,-77, 81,-17,-62, 88,-41,-41, 88,-62,-17, 81,-77, 9, 67,-87, 33, 48,-89, 55, 25,-84, 73,-73, 84,-25,-55, 89,-48,-33, 87,-67, -9, 77,-81, 17, 62,-88, 41, 38,-87, 74, -9,-63, 90,-53,-22, 81,-82, 26, 49,-89, 66, 4,-71, 88,-42,-34, 86,-76, 13, 60,-90, 56, 17,-79, 84,-30,-46, 89,-69, 69,-89, 46, 30,-84, 79,-17,-56, 90,-60,-13, 76,-86, 34, 42,-88, 71, -4,-66, 89,-49,-26, 82,-81, 22, 53,-90, 63, 9,-74, 87,-38, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 34,-82, 84,-38,-30, 81,-86, 42, 26,-79, 87,-46,-22, 76,-88, 49, 17,-74, 89,-53,-13, 71,-89, 56, 9,-69, 90,-60, -4, 66,-90, 63,-63, 90,-66, 4, 60,-90, 69, -9,-56, 89,-71, 13, 53,-89, 74,-17,-49, 88,-76, 22, 46,-87, 79,-26,-42, 86,-81, 30, 38,-84, 82,-34, 33,-81, 84,-41,-25, 77,-87, 48, 17,-73, 88,-55, -9, 67,-89, 62,-62, 89,-67, 9, 55,-88, 73,-17,-48, 87,-77, 25, 41,-84, 81,-33,-33, 81,-84, 41, 25,-77, 87,-48,-17, 73,-88, 55, 9,-67, 89,-62, 62,-89, 67, -9,-55, 88,-73, 17, 48,-87, 77,-25,-41, 84,-81, 33, 30,-76, 89,-63, 9, 49,-86, 84,-46,-13, 66,-90, 74,-26,-34, 79,-89, 60, -4,-53, 87,-82, 42, 17,-69, 90,-71, 22, 38,-81, 88,-56, 56,-88, 81,-38,-22, 71,-90, 69,-17,-42, 82,-87, 53, 4,-60, 89,-79, 34, 26,-74, 90,-66, 13, 46,-84, 86,-49, -9, 63,-89, 76,-30, 26,-70, 90,-80, 43, 9,-58, 87,-87, 58, -9,-43, 80,-90, 70,-26,-26, 70,-90, 80,-43, -9, 58,-87, 87,-58, 9, 43,-80, 90,-70, 26, 26,-70, 90,-80, 43, 9,-58, 87,-87, 58, -9,-43, 80,-90, 70,-26,-26, 70,-90, 80,-43, -9, 58,-87, 87,-58, 9, 43,-80, 90,-70, 26, 26,-69, 89,-81, 46, 4,-53, 84,-88, 63,-17,-34, 74,-90, 76,-38,-13, 60,-87, 86,-56, 9, 42,-79, 90,-71, 30, 22,-66, 89,-82, 49,-49, 82,-89, 66,-22,-30, 71,-90, 79,-42, -9, 56,-86, 87,-60, 13, 38,-76, 90,-74, 34, 17,-63, 88,-84, 53, -4,-46, 81,-89, 69,-26, 25,-67, 88,-81, 48, 0,-48, 81,-88, 67,-25,-25, 67,-88, 81,-48, 48,-81, 88,-67, 25, 25,-67, 88,-81, 48, 0,-48, 81,-88, 67,-25,-25, 67,-88, 81,-48, 0, 48,-81, 88,-67, 25, 25,-67, 88,-81, 48,-48, 81,-88, 67,-25,-25, 67,-88, 81,-48, 0, 48,-81, 88,-67, 25, 22,-60, 84,-89, 74,-42, 0, 42,-74, 89,-84, 60,-22,-22, 60,-84, 89,-74, 42, 0,-42, 74,-89, 84,-60, 22, 22,-60, 84,-89, 74,-42, 42,-74, 89,-84, 60,-22,-22, 60,-84, 89,-74, 42, 0,-42, 74,-89, 84,-60, 22, 22,-60, 84,-89, 74,-42, 0, 42,-74, 89,-84, 60,-22, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 17,-49, 74,-88, 89,-76, 53,-22,-13, 46,-71, 87,-89, 79,-56, 26, 9,-42, 69,-86, 90,-81, 60,-30, -4, 38,-66, 84,-90, 82,-63, 34,-34, 63,-82, 90,-84, 66,-38, 4, 30,-60, 81,-90, 86,-69, 42, -9,-26, 56,-79, 89,-87, 71,-46, 13, 22,-53, 76,-89, 88,-74, 49,-17, 17,-48, 73,-87, 88,-77, 55,-25, -9, 41,-67, 84,-89, 81,-62, 33,-33, 62,-81, 89,-84, 67,-41, 9, 25,-55, 77,-88, 87,-73, 48,-17,-17, 48,-73, 87,-88, 77,-55, 25, 9,-41, 67,-84, 89,-81, 62,-33, 33,-62, 81,-89, 84,-67, 41, -9,-25, 55,-77, 88,-87, 73,-48, 17, 13,-38, 60,-76, 87,-90, 86,-74, 56,-34, 9, 17,-42, 63,-79, 88,-90, 84,-71, 53,-30, 4, 22,-46, 66,-81, 89,-89, 82,-69, 49,-26, 26,-49, 69,-82, 89,-89, 81,-66, 46,-22, -4, 30,-53, 71,-84, 90,-88, 79,-63, 42,-17, -9, 34,-56, 74,-86, 90,-87, 76,-60, 38,-13, 9,-26, 43,-58, 70,-80, 87,-90, 90,-87, 80,-70, 58,-43, 26, -9, -9, 26,-43, 58,-70, 80,-87, 90,-90, 87,-80, 70,-58, 43,-26, 9, 9,-26, 43,-58, 70,-80, 87,-90, 90,-87, 80,-70, 58,-43, 26, -9, -9, 26,-43, 58,-70, 80,-87, 90,-90, 87,-80, 70,-58, 43,-26, 9, 9,-26, 42,-56, 69,-79, 86,-89, 90,-87, 81,-71, 60,-46, 30,-13, -4, 22,-38, 53,-66, 76,-84, 89,-90, 88,-82, 74,-63, 49,-34, 17,-17, 34,-49, 63,-74, 82,-88, 90,-89, 84,-76, 66,-53, 38,-22, 4, 13,-30, 46,-60, 71,-81, 87,-90, 89,-86, 79,-69, 56,-42, 26, -9, 9,-25, 41,-55, 67,-77, 84,-88, 89,-87, 81,-73, 62,-48, 33,-17, 17,-33, 48,-62, 73,-81, 87,-89, 88,-84, 77,-67, 55,-41, 25, -9, -9, 25,-41, 55,-67, 77,-84, 88,-89, 87,-81, 73,-62, 48,-33, 17,-17, 33,-48, 62,-73, 81,-87, 89,-88, 84,-77, 67,-55, 41,-25, 9, 4,-13, 22,-30, 38,-46, 53,-60, 66,-71, 76,-81, 84,-87, 89,-90, 90,-89, 88,-86, 82,-79, 74,-69, 63,-56, 49,-42, 34,-26, 17, -9, 9,-17, 26,-34, 42,-49, 56,-63, 69,-74, 79,-82, 86,-88, 89,-90, 90,-89, 87,-84, 81,-76, 71,-66, 60,-53, 46,-38, 30,-22, 13, -4,

Memory usage

Table 3 shows the memory usage for VTM and proposed method to store core transforms. It can be seen that memory usage of proposed method is reduced to 69% of VTM’s memory usage for transform

Table 3. Memory Usage for Proposed method

| Transform size | # of elements | VTM |  |  | Proposal |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (size^2) | # of kernels | Bits per elements | Bits amounts | # of kernels | Bits per elements | Bits amounts |
| 2 | 4 | 1 | 10 | 40 | 0 | 8 | 0 |
| 4 | 16 | 2 | 10 | 320 | 0 | 8 | 0 |
| 8 | 64 | 2 | 10 | 1280 | 0 | 8 | 0 |
| 16 | 256 | 2 | 10 | 5120 | 0 | 8 | 0 |
| 32 | 1024 | 2 | 10 | 20480 | 0 | 8 | 0 |
| 64 | 16384 | 1 | 10 | 163840 | 1 | 8 | 131072 |
| Total [bits] |  | 191080 |  |  | 131072 |  |  |
|  |  | (~23 KBytes) |  |  | (~ 16 Kbytes ) |  |  |
| Over VTM [%] |  | 100 |  |  | 69 |  |  |

Test result

Test conditions follows the description in CE6 document [4]. BMS SW was used for these experiments, as specified in JVET-K1010 [5].

Test result compared to anchor

Table 4 shows test results on the proposed method compared to anchors of VTM and BMS.

Table 4: Test results compared to VTM and BMS

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.2% | 0.2% | 0.2% | 95% | 99% | 0.2% | 0.2% | 0.4% | 100% | 100% |
| Class A2 | 0.1% | 0.1% | 0.1% | 101% | 101% | 0.2% | 0.1% | 0.2% | 101% | 102% |
| Class B | 0.0% | -0.1% | -0.1% | 101% | 100% | 0.1% | 0.0% | 0.0% | 101% | 100% |
| Class C | -0.3% | -0.2% | -0.2% | 97% | 100% | -0.2% | -0.2% | -0.2% | 101% | 99% |
| Class E | -0.2% | -0.4% | -0.2% | 101% | 100% | -0.1% | 0.0% | 0.0% | 101% | 100% |
| Overall | 0.0% | -0.1% | -0.1% | 99% | 100% | 0.0% | 0.0% | 0.0% | 101% | 100% |
| Class D | -0.4% | -0.2% | -0.5% | 102% | 101% | -0.2% | -0.3% | -0.4% | 101% | 100% |
| Class F (optional) | -0.2% | -0.2% | -0.1% | 101% | 101% | -0.2% | -0.3% | -0.4% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.2% | 0.2% | 0.2% | 104% | 101% | 0.2% | 0.2% | 0.2% | 103% | 100% |
| Class A2 | 0.2% | 0.1% | 0.1% | 102% | 99% | 0.2% | 0.1% | 0.1% | 102% | 101% |
| Class B | 0.1% | 0.0% | 0.1% | 100% | 100% | 0.1% | 0.2% | -0.1% | 101% | 100% |
| Class C | 0.0% | 0.1% | 0.1% | 101% | 100% | -0.2% | -0.1% | 0.0% | 101% | 99% |
| Overall | 0.1% | 0.1% | 0.1% | 101% | 100% | 0.1% | 0.1% | 0.0% | 102% | 100% |
| Class D | -0.1% | -0.3% | -0.1% | 99% | 99% | -0.2% | 0.2% | -0.1% | 101% | 100% |
| Class F (optional) | 0.0% | 0.0% | -0.2% | 97% | 99% | -0.1% | -0.1% | -0.2% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 0.2% | 0.3% | 0.0% | 103% | 100% | 0.0% | 0.1% | -0.2% | 102% | 100% |
| Class C | 0.1% | 0.0% | 0.0% | 100% | 100% | -0.3% | -0.3% | -0.2% | 105% | 100% |
| Class E | 0.1% | -0.3% | -0.4% | 100% | 99% | 0.0% | 0.0% | -0.9% | 100% | 100% |
| Overall | 0.1% | 0.0% | -0.1% | 102% | 100% | -0.1% | 0.0% | -0.4% | 103% | 100% |
| Class D | 0.0% | 1.0% | -0.6% | 100% | 100% | -0.2% | -0.2% | 0.0% | 96% | 100% |
| Class F (optional) | 0.0% | 0.4% | 0.8% | 100% | 99% | -0.2% | -0.5% | -0.5% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-2.0.1 with VTM cfgs |  |  |  |  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 0.2% | 0.4% | 0.4% | 100% | 101% | 0.0% | 0.2% | -0.1% | 101% | 100% |
| Class C | 0.1% | 0.0% | -0.1% | 100% | 101% | -0.2% | -0.2% | -0.5% | 101% | 100% |
| Class E | 0.1% | 0.0% | 0.0% | 100% | 101% | 0.2% | -0.5% | -0.3% | 99% | 98% |
| Overall | 0.2% | 0.2% | 0.1% | 100% | 101% | 0.0% | -0.1% | -0.2% | 100% | 99% |
| Class D | 0.1% | 0.0% | -0.4% | 101% | 101% | -0.4% | 0.5% | 0.3% | 100% | 99% |
| Class F (optional) | 0.1% | -0.2% | 0.2% | 100% | 100% | 0.0% | 0.5% | 0.2% | 100% | 98% |

VTM test result

For testing proposed method for VTM testing, VTM configuration files were used with BMS 2.0.1 software, and results are shown in Table 4. Testing results on the proposed method show 0.0%, 0.1%, 0.1% and 0.2% BD rate losses on average for All Intra (AI), Random Access (RA), Low Delayed B (LDB), and Low Delayed P (LDP) configurations, respectively, compared to VTM configure setting.

BMS test result

For testing proposed method for BMS testing, BMS configuration files were used with BMS 2.0.1 software, and results are shown in Table 4. Testing results on the proposed method show 0.0%, 0.1%, and 0.0%BD rate losses on average for All Intra (AI), Random Access (RA), and Low Delayed P (LDP) configurations, respectively, and 0.1% gain for LDB configuration, compared to BMS configure setting.

Conclusion

This contribution presents a unified matrix for all kernels of transform. The benefit of using the proposed method is that all kernels of transforms can be derived from one matrix by sampling a subset of coefficients and basis vectors of larger transforms. So, it takes only 69% of VTM’s memory usage for transform. Additionally, coefficients and basis vectors are designed with 8-bit representation, so it allows to limit the bit width to 16 at all intermediate stages. The design of coefficients provides more additional flexibility in implementations, such as direct matrix multiplications with SIMD instructions for the innermost loop, or multi-stage butterfly implementations utilizing symmetry properties of the basis vectors. The testing results show that the proposed method does not affect noticeable coding loss in all scenarios. Therefore, it is recommend to adopt the proposed method into the next version of test model of VVC.

Proposed text for WD

This section provides texts related to proposed method on top of VVC WD2 clauses 8.3.4. The modification is highlighted with yellow mark.

8.3.4	Transformation process for scaled transform coefficients

8.3.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-6 depending on mts_idx[ x ][ y ] and CuPredMode[ x ][ y ].

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- Each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 is transformed to e[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.3.4.2 for each column x = 0..nTbW − 1 with the height of the transform block nTbH, the list d[ x ][ y ] with y = 0..nTbH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- The intermediate sample values g[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-112)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.3.4.2 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the list g[ x ][ y ] with x = 0..nTbW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

Table 8-6 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ] and CuPredMode[ x ][ y ]

| mts_idx[ x ][ y ] | CuPredMode[ x ][ y ] = = MODE_INTRA |  | CuPredMode[ x ][ y ] = = MODE_INTER |  |
| --- | --- | --- | --- | --- |
|  | trTypeHor | trTypeVer | trTypeHor | trTypeVer |
| −1 (inferred) | 0 | 0 | 0 | 0 |
| 0 (00) | 1 | 1 | 2 | 2 |
| 1 (01) | 2 | 1 | 1 | 2 |
| 2 (10) | 1 | 2 | 2 | 1 |
| 3 (11) | 2 | 2 | 1 | 1 |

8.3.4.2	Transformation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- a list of scaled transform coefficients x[ j ] with j = 0..nTbS − 1,

- a transform kernel type variable trType.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTbS − 1.

The transformation matrix derivation process as specified in clause 8.3.4.3 in invoced with the transform size nTbS and the transform kernel Type trType as input, and the transformation maxtrix transMatrix as output.

Depending on the value of trType, the following applies:, the list of transformed samples y[ i ] with i = 0..nTbS − 1 is derived as follows:

- If trType is equal to 0, the following transform matrix multiplication applies:

y[i]= j=0nTbs-1transMatrix[i][j*26-Log2(nTbS) ]*x[j]  with i = 0..nTbS − 1	(8-113)

- If trType is equal to 1, the following transform matrix multiplication applies:

y[i]= j=0nTbs-1transMatrix[i][j*26-Log2nTbS+25-Log2nTbS]*x[j]  with i = 0..nTbS − 1	(8-114)

- If trType is equal to 2, the following transform matrix multiplication applies:

y[i]= j=0nTbs-1transMatrix[nTbS - i - 1][j*26-Log2nTbS+25-Log2nTbS]*x[j]  with i = 0..nTbS − 1	 	 (8-115)

`

8.3.4.3	Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..63	(8-116)

transMatrixCol0to15 =			 	 	 	 (8-117)

{

{ 64   64   64   64  64  64  64  64  64  64  64  64  64  64  64  64 }

{ 90   90   89   89  88  87  86  84  82  81  79  76  74  71  69  66 }

{ 89   88   87   84  81  77  73  67  62  55  48  41  33  25  17  9 }

{ 90   88   84   79  71  63  53  42  30  17  4  -9  -22  -34  -46  -56 }

{ 90   87   80   70  58  43  26  9  -9  -26  -43  -58  -70  -80  -87  -90 }

{ 89   84   74   60  42  22  0  -22  -42  -60  -74  -84  -89  -89  -84  -74 }

{ 88   81   67   48  25  0  -25  -48  -67  -81  -88  -88  -81  -67  -48  -25 }

{ 89   79   60   34  4  -26  -53  -74  -87  -90  -82  -66  -42  -13  17  46 }

{ 89   75   50   18  -18  -50  -75  -89  -89  -75  -50  -18  18  50  75  89 }

{ 88   71   42    4  -34  -66  -86  -89  -76  -49  -13  26  60  82  90  81 }

{ 87   67   33   -9  -48  -77  -89  -81  -55  -17  25  62  84  88  73  41 }

{ 87   63   22  -26  -66  -88  -86  -60  -17  30  69  89  84  56  13  -34 }

{ 87   58    9  -43  -80  -90  -70  -26  26  70  90  80  43  -9  -58  -87 }

{ 86   53    0  -53  -86  -86  -53  0  53  86  86  53  0  -53  -86  -86 }

{ 84   48   -9  -62  -88  -77  -33  25  73  89  67  17  -41  -81  -87  -55 }

{ 84   42  -22  -74  -89  -60  0  60  89  74  22  -42  -84  -84  -42  22 }

{ 84   35  -35  -84  -84  -35  35  84  84  35  -35  -84  -84  -35  35  84 }

{ 82   30  -42  -87  -76  -17  53  89  69  4  -63  -90  -60  9  71  89 }

{ 81   25  -48  -88  -67  0  67  88  48  -25  -81  -81  -25  48  88  67 }

{ 81   17  -60  -90  -49  30  86  74  4  -69  -88  -38  42  89  66  -9 }

{ 80    9  -70  -87  -26  58  90  43  -43  -90  -58  26  87  70  -9  -80 }

{ 79    4  -74  -82  -13  69  86  22  -63  -88  -30  56  89  38  -49  -90 }

{ 77    0  -77  -77  0  77  77  0  -77  -77  0  77  77  0  -77  -77 }

{ 76   -9  -84  -66  26  89  53  -42  -90  -38  56  88  22  -69  -82  -4 }

{ 75  -18  -89  -50  50  89  18  -75  -75  18  89  50  -50  -89  -18  75 }

{ 74  -22  -89  -42  60  84  0  -84  -60  42  89  22  -74  -74  22  89 }

{ 73  -25  -89  -33  67  77  -17  -88  -41  62  81  -9  -87  -48  55  84 }

{ 71  -34  -89  -13  82  56  -53  -84  9  89  38  -69  -74  30  90  17 }

{ 70  -43  -87    9  90  26  -80  -58  58  80  -26  -90  -9  87  43  -70 }

{ 69  -46  -84   17  90  13  -86  -42  71  66  -49  -82  22  90  9  -87 }

{ 67  -48  -81   25  88  0  -88  -25  81  48  -67  -67  48  81  -25  -88 }

{ 66  -56  -74   46  81  -34  -86  22  89  -9  -90  -4  89  17  -87  -30 }

{ 64  -64  -64   64  64  -64  -64  64  64  -64  -64  64  64  -64  -64  64 }

{ 63  -66  -60   69  56  -71  -53  74  49  -76  -46  79  42  -81  -38  82 }

{ 62  -67  -55   73  48  -77  -41  81  33  -84  -25  87  17  -88  -9  89 }

{ 60  -74  -42   84  22  -89  0  89  -22  -84  42  74  -60  -60  74  42 }

{ 58  -80  -26   90  -9  -87  43  70  -70  -43  87  9  -90  26  80  -58 }

{ 56  -81  -22   90  -17  -82  53  60  -79  -26  90  -13  -84  49  63  -76 }

{ 55  -81  -17   89  -25  -77  62  48  -84  -9  88  -33  -73  67  41  -87 }

{ 53  -86    0   86  -53  -53  86  0  -86  53  53  -86  0  86  -53  -53 }

{ 50  -89   18   75  -75  -18  89  -50  -50  89  -18  -75  75  18  -89  50 }

{ 49  -89   22   71  -79  -9  86  -60  -38  90  -34  -63  84  -4  -81  69 }

{ 48  -88   25   67  -81  0  81  -67  -25  88  -48  -48  88  -25  -67  81 }

{ 46  -90   42   49  -90  38  53  -89  34  56  -89  30  60  -88  26  63 }

{ 43  -90   58   26  -87  70  9  -80  80  -9  -70  87  -26  -58  90  -43 }

{ 42  -89   60   22  -84  74  0  -74  84  -22  -60  89  -42  -42  89  -60 }

{ 41  -88   62   17  -81  77  -9  -67  87  -33  -48  89  -55  -25  84  -73 }

{ 38  -87   74   -9  -63  90  -53  -22  81  -82  26  49  -89  66  4  -71 }

{ 35  -84   84  -35  -35  84  -84  35  35  -84  84  -35  -35  84  -84  35 }

{ 34  -82   84  -38  -30  81  -86  42  26  -79  87  -46  -22  76  -88  49 }

{ 33  -81   84  -41  -25  77  -87  48  17  -73  88  -55  -9  67  -89  62 }

{ 30  -76   89  -63  9  49  -86  84  -46  -13  66  -90  74  -26  -34  79 }

{ 26  -70   90  -80  43  9  -58  87  -87  58  -9  -43  80  -90  70  -26 }

{ 26  -69   89  -81  46  4  -53  84  -88  63  -17  -34  74  -90  76  -38 }

{ 25  -67   88  -81  48  0  -48  81  -88  67  -25  -25  67  -88  81  -48 }

{ 22  -60   84  -89  74  -42  0  42  -74  89  -84  60  -22  -22  60  -84 }

{ 18  -50   75  -89  89  -75  50  -18  -18  50  -75  89  -89  75  -50  18 }

{ 17  -49   74  -88  89  -76  53  -22  -13  46  -71  87  -89  79  -56  26 }

{ 17  -48   73  -87  88  -77  55  -25  -9  41  -67  84  -89  81  -62  33 }

{ 13  -38   60  -76  87  -90  86  -74  56  -34  9  17  -42  63  -79  88 }

{ 9   -26   43  -58  70  -80  87  -90  90  -87  80  -70  58  -43  26  -9 }

{ 9   -26   42  -56  69  -79  86  -89  90  -87  81  -71  60  -46  30  -13 }

{ 9   -25   41  -55  67  -77  84  -88  89  -87  81  -73  62  -48  33  -17 }

{ 4   -13   22  -30  38  -46  53  -60  66  -71  76  -81  84  -87  89  -90 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..63	(8-118)

transMatrixCol16to31 =		 	 	 	 (8-119)

{

{ 64  64  64  64  64  64  64  64  64  64  64  64  64  64  64  64 }

{ 63  60  56  53  49  46  42  38  34  30  26  22  17  13  9  4 }

{ -9  -17  -25  -33  -41  -48  -55  -62  -67  -73  -77  -81  -84  -87  -88  -89 }

{ -66  -74  -81  -86  -89  -90  -89  -87  -82  -76  -69  -60  -49  -38  -26  -13 }

{ -90  -87  -80  -70  -58  -43  -26  -9  9  26  43  58  70  80  87  90 }

{ -60  -42  -22  0  22  42  60  74  84  89  89  84  74  60  42  22 }

{ 25  48  67  81  88  88  81  67  48  25  0  -25  -48  -67  -81  -88 }

{ 69  84  90  86  71  49  22  -9  -38  -63  -81  -89  -88  -76  -56  -30 }

{ 89  75  50  18  -18  -50  -75  -89  -89  -75  -50  -18  18  50  75  89 }

{ 56  22  -17  -53  -79  -90  -84  -63  -30  9  46  74  89  87  69  38 }

{ -41  -73  -88  -84  -62  -25  17  55  81  89  77  48  9  -33  -67  -87 }

{ -71  -89  -82  -53  -9  38  74  90  81  49  4  -42  -76  -90  -79  -46 }

{ -87  -58  -9  43  80  90  70  26  -26  -70  -90  -80  -43  9  58  87 }

{ -53  0  53  86  86  53  0  -53  -86  -86  -53  0  53  86  86  53 }

{ 55  87  81  41  -17  -67  -89  -73  -25  33  77  88  62  9  -48  -84 }

{ 74  89  60  0  -60  -89  -74  -22  42  84  84  42  -22  -74  -89  -60 }

{ 84  35  -35  -84  -84  -35  35  84  84  35  -35  -84  -84  -35  35  84 }

{ 49  -22  -79  -86  -38  34  84  81  26  -46  -88  -74  -13  56  90  66 }

{ -67  -88  -48  25  81  81  25  -48  -88  -67  0  67  88  48  -25  -81 }

{ -76  -84  -26  53  90  56  -22  -82  -79  -13  63  89  46  -34  -87  -71 }

{ -80  -9  70  87  26  -58  -90  -43  43  90  58  -26  -87  -70  9  80 }

{ -46  42  90  53  -34  -89  -60  26  87  66  -17  -84  -71  9  81  76 }

{ 77  77  0  -77  -77  0  77  77  0  -77  -77  0  77  77  0  -77 }

{ 79  74  -13  -86  -63  30  89  49  -46  -90  -34  60  87  17  -71  -81 }

{ 75  -18  -89  -50  50  89  18  -75  -75  18  89  50  -50  -89  -18  75 }

{ 42  -60  -84  0  84  60  -42  -89  -22  74  74  -22  -89  -42  60  84 }

{ -84  -55  48  87  9  -81  -62  41  88  17  -77  -67  33  89  25  -73 }

{ -81  -60  49  86  -4  -88  -42  66  76  -26  -90  -22  79  63  -46  -87 }

{ -70  43  87  -9  -90  -26  80  58  -58  -80  26  90  9  -87  -43  70 }

{ -38  74  63  -53  -81  26  89  4  -88  -34  76  60  -56  -79  30  89 }

{ 88  25  -81  -48  67  67  -48  -81  25  88  0  -88  -25  81  48  -67 }

{ 82  42  -76  -53  69  63  -60  -71  49  79  -38  -84  26  88  -13  -90 }

{ 64  -64  -64  64  64  -64  -64  64  64  -64  -64  64  64  -64  -64  64 }

{ 34  -84  -30  86  26  -87  -22  88  17  -89  -13  89  9  -90  -4  90 }

{ -89  9  88  -17  -87  25  84  -33  -81  41  77  -48  -73  55  67  -62 }

{ -84  -22  89  0  -89  22  84  -42  -74  60  60  -74  -42  84  22  -89 }

{ -58  80  26  -90  9  87  -43  -70  70  43  -87  -9  90  -26  -80  58 }

{ -30  89  -9  -86  46  66  -74  -34  89  -4  -87  42  69  -71  -38  88 }

{ 87  -41  -67  73  33  -88  9  84  -48  -62  77  25  -89  17  81  -55 }

{ 86  0  -86  53  53  -86  0  86  -53  -53  86  0  -86  53  53  -86 }

{ 50  -89  18  75  -75  -18  89  -50  -50  89  -18  -75  75  18  -89  50 }

{ 26  -89  46  53  -88  17  74  -76  -13  87  -56  -42  90  -30  -66  82 }

{ -81  67  25  -88  48  48  -88  25  67  -81  0  81  -67  -25  88  -48 }

{ -87  22  66  -86  17  69  -84  13  71  -82  9  74  -81  4  76  -79 }

{ -43  90  -58  -26  87  -70  -9  80  -80  9  70  -87  26  58  -90  43 }

{ -22  84  -74  0  74  -84  22  60  -89  42  42  -89  60  22  -84  74 }

{ 73  -84  25  55  -89  48  33  -87  67  9  -77  81  -17  -62  88  -41 }

{ 88  -42  -34  86  -76  13  60  -90  56  17  -79  84  -30  -46  89  -69 }

{ 35  -84  84  -35  -35  84  -84  35  35  -84  84  -35  -35  84  -84  35 }

{ 17  -74  89  -53  -13  71  -89  56  9  -69  90  -60  -4  66  -90  63 }

{ -62  89  -67  9  55  -88  73  -17  -48  87  -77  25  41  -84  81  -33 }

{ -89  60  -4  -53  87  -82  42  17  -69  90  -71  22  38  -81  88  -56 }

{ -26  70  -90  80  -43  -9  58  -87  87  -58  9  43  -80  90  -70  26 }

{ -13  60  -87  86  -56  9  42  -79  90  -71  30  22  -66  89  -82  49 }

{ 48  -81  88  -67  25  25  -67  88  -81  48  0  -48  81  -88  67  -25 }

{ 89  -74  42  0  -42  74  -89  84  -60  22  22  -60  84  -89  74  -42 }

{ 18  -50  75  -89  89  -75  50  -18  -18  50  -75  89  -89  75  -50  18 }

{ 9  -42  69  -86  90  -81  60  -30  -4  38  -66  84  -90  82  -63  34 }

{ -33  62  -81  89  -84  67  -41  9  25  -55  77  -88  87  -73  48  -17 }

{ -90  84  -71  53  -30  4  22  -46  66  -81  89  -89  82  -69  49  -26 }

{ -9  26  -43  58  -70  80  -87  90  -90  87  -80  70  -58  43  -26  9 }

{ -4  22  -38  53  -66  76  -84  89  -90  88  -82  74  -63  49  -34  17 }

{ 17  -33  48  -62  73  -81  87  -89  88  -84  77  -67  55  -41  25  -9 }

{ 90  -89  88  -86  82  -79  74  -69  63  -56  49  -42  34  -26  17  -9 }

},

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol16to31[47 −m ][ n ]	(8-120)
		 with m =32..47, n = 0..63

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol0to15[63 −m ][ n ]	(8-121)
		with m =48..63, n = 0..63

Reference

[1] X. Zhao, X. Li, S. Liu, “CE6-related: Compound orthonormal transform”, JVET-K0292, Ljubljana, SI, July 2018

[2] T. Tsukuba, M.I keda, T. Suzuki, “CE6: Transform Matrix Replacement (CE6.1.8.1)”, JVET-K123, Ljubljana, SI, July 2018

[3] X. Zhao, X. Li, S. Liu, “CE6-related: On 8-bit primary transform core”, JVET-K0290, Ljubljana, SI, July 2018

[4] A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-K1026, Ljubljana, SI, July 2018

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard.