| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0061 |
| --- | --- |

| Title: | CE9-related: Bi-directional optical flow for VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiho Choi, Min Woo Park, Minsoo Park Anish Tamse Woong-il Choi Kwang Pyo Choi Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | kiho14.choi@samsung.com m.w.park@samsung.com mss.park@samsung.com tamse.anish@samsung.com woongil.choi@samsung.com kp5.choi@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

- Abstract

This contribution describes Bi-directional optical flow (BIO) which is used in BMS2.0. In JVET11th meeting, a simplification of BIO considering an early termination based on the similarity between two bi-prediction signals was adopted for BMS2.1. In this contribution, testing results of BIO in BMS2.1 is provided. For VTM test in RA case, BIO in BMS2.1 shows 1.4% coding gain on average for luma with 104% encoding time and 120% decoding time compared to BMS2.1 with VTM configuration.

Introduction

Bi-directional optical flow (BIO) was proposed in [1]. BIO can be classified as decoder motion derivation method and can be applied only for Bi prediction. In the construction of prediction block, instead of conventional half-sum more precise combination of two references is used. Overhead for BIO is not needed for this method as the encoder calculates motion vector for each pixel and this procedure is repeated on the decoder side.

In JVET11th meeting, a simplification of BIO considering an early termination based on the similarity between two bi-prediction signals was adopted for BMS2.0 [2].

In this contribution, testing results of BIO in BMS2.0 is provided. For the test, the proposed method in this contribution is identical to the one used BMS.

Test result

Test conditions follows the description in CTC [3]. For testing, BIO = 1 is set to VTM and BIO = 0 to BMS.

VTM test result (Tool on test)

For testing proposed method on VTM, VTM configuration files were used with BMS 2.1 software, and results are shown in Table 1. The average BD-rate gains for RA is 1.4% in the table.

BMS test result (Tool off test)

For testing proposed method on BMS, BMS configuration files were used with BMS 2.1 software, and results are shown in Table 1. The average BD-rate loss for RA is 1.0 in the table.

Table 1: Test results on CE 5.2.4

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1 with VTM cfg |  |  |  |  | Over BMS-2.1 with BMS cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -0.42% | -0.18% | 104% | 118% | 0.64% | 0.23% | 0.03% | 98% | 84% |
| Class A2 | -1.83% | -0.53% | -0.23% | 104% | 114% | 1.29% | 0.22% | -0.07% | 98% | 86% |
| Class B | -1.30% | -0.48% | -0.42% | 105% | 122% | 0.89% | 0.26% | 0.12% | 98% | 83% |
| Class C | -1.52% | -0.87% | -0.66% | 104% | 125% | 1.11% | 0.40% | 0.41% | 98% | 82% |
| Overall | -1.41% | -0.58% | -0.40% | 104% | 120% | 0.98% | 0.28% | 0.14% | 98% | 84% |
| Class D | -2.28% | -1.40% | -1.17% | 104% | 124% | 1.68% | 0.74% | 0.64% | 98% | 81% |
| Class F (optional) | -0.14% | -0.08% | -0.07% | 104% | 115% | 0.02% | -0.06% | -0.10% | 98% | 88% |

Conclusion

In this contribution, BIO in BMS 2.1 is tested. Testing results show a good coding performance in all scenarios. It is recommend to adopt the proposed method into the next version of test model of VVC.

Reference

[1] A. Alshin et al, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, San Diego, US, 10–20 Apr. 2018.

[2] X. Xiu, Y. He, Y. Ye, “CE9.5.3: Bi-directional optical flow (BIO) simplification”, JVET-K0344, San Diego, US, 10–20 Apr. 2018

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard.