| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0062-v1 |
| --- | --- |

| Title: | CE11: Test results of CE11.1.5 long-tap deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woong Il Choi Kiho Choi 2215-1205, 70, Dongtansunhwan-daero 20-gil, Hwaseong-si, Gyeonggi-do, Republic of Korea | Tel: Email: | +82 10 2452 0977 woongil.choi@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

- Abstract

This contribution is corresponding to CE11 test 11.1.5 and it proposes changes on filtering process of deblocking filter on large block boundary. The purpose of this document is to show the performance result of JVET-K0112 algorithm implemented on VTM 2.0.1 software and complexity analysis result. The test results show that the proposed method achieves 0.01%, -0.03%, 0.07% and 0.05% BDBR for AI, RA, LDB, LDP compared to VTM 2.0.1 and 0.00%, -0.06%, 0.09% and -0.03% BDBR for AI, RA, LDB, LDP compared to VTM 2.0.1 with ALF switched off, respectively.

Complexity analysis

Complexity of the proposal is evaluated based on the following criteria described in the CE11 document, JVET-K1031 [1].

- Number of samples from the block boundary that can be modified by deblocking (P+Q): 7+7 for vertical edge, 3+7 for horizontal edge

- Number of samples from the block boundary that are needed for deblocking decisions: two additional samples(p7, q7) are needed to decide the longer-tap strong filter

- Number of filtering operations: 0 Multiplication, 136 additions, 66 shifts for vertical edge, 0 Multiplication, 90 additions, 51 shifts for horizontal edge

- Number of line buffers: 4 lines

- Strong filter is applied luma only

Experimental results

The proposed method is implemented based on VTM-2.0.1 [2] and the simulation is performed under common test conditions of SDR sequences in JVET-K1010 [3]. According to CE description [1], the test is performed by compared to VTM with ALF on and off and the results are shown in table 1 and table 2.

Table 1 Simulation results with VTM2 with ALF switched on

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.00% | 0.00% | 100% | 102% |
| Class A2 | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class B | 0.02% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.02% | -0.01% | -0.01% | 100% | 99% |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.06% | 0.01% | 100% | 101% |
| Class A2 | -0.03% | 0.04% | -0.10% | 100% | 100% |
| Class B | 0.02% | -0.01% | -0.10% | 100% | 99% |
| Class C | 0.01% | 0.00% | -0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.02% | -0.06% | 100% | 100% |
| Class D | 0.04% | -0.04% | 0.10% | 101% | 100% |
| Class F (optional) | 0.03% | -0.01% | -0.02% | 102% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.05% | -0.29% | 101% | 98% |
| Class C | 0.05% | -0.19% | -0.14% | 100% | 103% |
| Class E | 0.07% | 0.24% | -0.28% | 100% | 99% |
| Overall | 0.07% | 0.02% | -0.24% | 100% | 100% |
| Class D | 0.10% | 0.33% | -0.01% | 100% | 100% |
| Class F (optional) | 0.06% | 0.08% | 0.00% | 99% | 98% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.20% | 0.06% | 101% | 100% |
| Class C | 0.00% | -0.10% | -0.03% | 101% | 102% |
| Class E | 0.19% | -0.25% | 0.82% | 99% | 101% |
| Overall | 0.05% | -0.01% | 0.22% | 100% | 101% |
| Class D | 0.07% | 0.72% | -0.43% | 100% | 98% |
| Class F (optional) | 0.14% | 0.06% | -0.36% | 100% | 97% |

Table 2 Simulation results on VTM2 with ALF switched off

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class A2 | 0.01% | -0.01% | 0.00% | 99% | 102% |
| Class B | 0.02% | 0.00% | 0.00% | 101% | 103% |
| Class C | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.03% | -0.01% | -0.01% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.05% | -0.01% | 102% | 101% |
| Class A2 | -0.09% | 0.01% | -0.01% | 100% | 100% |
| Class B | -0.01% | 0.02% | 0.05% | 101% | 100% |
| Class C | 0.01% | 0.03% | -0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.01% | -0.01% | 101% | 100% |
| Class D | 0.02% | -0.07% | 0.18% | 102% | 99% |
| Class F (optional) | 0.08% | 0.08% | 0.08% | 103% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.25% | -0.05% | 100% | 97% |
| Class C | 0.01% | -0.14% | -0.11% | 100% | 102% |
| Class E | 0.26% | -0.39% | 0.20% | 101% | 99% |
| Overall | 0.09% | -0.04% | -0.01% | 100% | 99% |
| Class D | -0.04% | -0.03% | -0.05% | 100% | 99% |
| Class F (optional) | 0.06% | 0.07% | 0.46% | 100% | 101% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.16% | -0.05% | 98% | 100% |
| Class C | 0.01% | 0.19% | -0.20% | 103% | 110% |
| Class E | -0.05% | 0.39% | -0.03% | 99% | 99% |
| Overall | -0.03% | 0.09% | -0.09% | 100% | 103% |
| Class D | -0.04% | -0.08% | 0.07% | 100% | 97% |
| Class F (optional) | 0.04% | -0.02% | 0.03% | 99% | 95% |

Conclusion

This contribution shows test results of the proposed long-tap filter in JVET-K0112 on VTM2. Complexity analysis is also reported under several aspect described in CE description. This result is supposed to be compared with the other CE11.1 tests.

References

- “Description of Core Experiment 11 (CE11): Deblocking”, JVET-K1031, July 2018.

- “Algorithm description for Versatile Video Coding and Test Model 2 (VTM2)”, JVET-K1002.

- “JVET common test conditions and software reference configurations”, JVET-K1010

Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).