| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 October 2018 | Document: JVET-L0063-v1 |
| --- | --- |

| Title: | CE1-related : Split Unit Coding Order |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Jie Chen Anish Tamse Minsoo Park Kiho Choi KwangPyo Choi | Tel: Email: | [+82-10-6296-4261] [yj1003.piao@samsung.com] |
| Source: | Samsung Electronics Co.,Ltd |  |  |

_____________________________

# Abstract

This contribution presents split unit coding order (SUCO) proposed in CfP response JVET-J0024[1]. SUCO allows referring to not only left but also right reconstructed pixels and right coded information at leaf node by supporting more flexible coding order at split unit. A split unit(SU) which is further partitioned into quad-tree or vertically partitioned into Bi- or Ternary- tree can be either coded from left to right(L2R) or right to left (R2L) by a flag indicating the coding order. CU-level intra prediction and inter prediction tools which utilize left information need to be extended accordingly to the right for more efficient coding based on the availability of neighborhood. The simplest SUCO provides 0.xx% BD-rate gain in AI and 0.xx% gain in RA configuration on VTM2.0. Maximum xx% BD gain on average can be achieved in RA with other SUCO configuration.

# Introduction

A CTU is recursively partitioned by quad- tree structure, binary tree structure or triple tree structure. The units in the partitioning process, which are further split into two, three, or four units are named split units (SUs). Usually the coding order of a split unit is from left to right and from above to bottom because of z scanning order of quad tree structure and raster scan of CTUs in a picture. However, normal left to right coding order is more beneficial to left inclined features than right inclined features. Not limited to intra prediction, even in inter prediction blocks with right inclined features cannot find similar motion information from left and above neighborhood.

Figure 1. Allowed coding order in QT / TT / BT structure

SUCO enables more flexible coding order, such as left to right (L2R) and right to left (R2L), to allow intra prediction from right reference pixels and inter prediction with right motion vector predictors. If a SU is partitioned vertically, a flag is signaled to indicate L2R or R2L coding order as shown in Figure 1. If a SU is partitioned by quad tree structure, a flag is shared for above two units and bottom two units as shown in Figure 1. If no flag is signaled for the coding order of a SU, all the following coding orders of that SU implicitly inherit from previous level SU.

Figure 2. Example if SUCO of a CTU

Figure2 shows the example of SUCO in a CTU. Due to more flexible coding order in SU level, the neighboring availability of a leaf CU become more diverse than common left and above neighbors in HEVC. As shown in Figure 3, there are four availability cases if only left and right neighboring blocks are considered, i.e., LR_10, LR_01, LR_11 and LR_00. Above block is always available unless the current CU lies on the top boundary of a slice. Availability of the above left or above right corner block depends on the corresponding left or right neighbor availability.

Figure3. Neighboring availability of a CU

| (a) | (b) |
| --- | --- |

Figure 4. (a) intra reference pixels, (b) Inter MVP position

Intra prediction methods are designed to utilize three neighboring reference pixel sets (Figure 4(a)) for its prediction according to their availability. Inter prediction tools which utilize neighboring MVs are also extended to consider three-side neighboring block information (Figure 4(b)). Most tools are simply extended to the right symmetrically when left is not available but right is available. Some tools such as DC, Planar, Skip/Merge are extended more specifically by considering left and right availability.

Tools extended to utilize right information are as follows,

- Intra prediction

  - Reference pixel

  - Angular prediction, Planar, DC

  - Wide angle prediction

  - MPM

  - DPCM

  - PDPC

  - LM_CHROMA

- Inter prediction

  - Skip/Merge

  - AMVP

  - AFFINE

- Others

  - Dual tree

  - Deblock

  - Context derivation for flag

- Encoder side

  - Harmonization with current encoder optimization

  - Context derivation for flag

SUCO is signaled at the following conditions,

- Min(su_width, su_height) >= MinSUCOSize

- Max(su_width, su_height) <= MaxSUCOSize

- split_mode is vertical split or quardtree split

- su_width >= su_height

where MinSUCOSize and MaxSUCOSize are defined as follows,

- MaxSUCOSize: the maximum allowed SU size for which SUCO is signaled, is defined as MaxSUCOSize = 1<< (Log2CTUsize – MaxSUCODepthFromTop)

- MinSUCOSize: the minimum allowed SU size for which SUCO is signaled, is defined as MinSUCOSize = 1<< (Log2CTUsize – MinSUCODepthFromTop)

# Results

SUCO was implemented on top of VTM 2.0.1. Tests were conducted under VTM common test condition [2]. Several configurations of SUCO have been tested in this contribution.

- C1:

  - Applied condition: All slice

  - Applied size is set to 64x64 ~ 32x32

- C2:

  - Applied condition: All slice

  - Applied size is set to 128x128 ~ 16x16

- C3:

  - Applied condition: Inter slice only

  - Applied size is set to 128x128 ~ 16x16

The summary results in three different configurations are presented in following tables. More details can be found in accompanying excel sheets.

Table I. Test result of C1 configuration

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table II. Test results of C2 configuration

Table III. Test results of C3 configuration

# Conclusions

This document presents test results of SUCO on top of VTM 2.0.1 s/w. Since current implementation of SUCO still needs further optimization of encoder complexity, we suggest further study of SUCO in CE. Evidence of SUCO in other platforms can be found in JVET-K0497[3].

# Reference

- JVET, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- JVET, “JVET common test conditions and software reference configurations”, JVET-K1010, 11th JVET Meeting, Ljubljana, SI, 11-18 July., 2018

- JVET, “CE1.4 related : Evidence of Split Unit Coding Order”, JVET-K0497, 11th JVET Meeting, Ljubljana, SI, 11-18 July., 2018

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).