| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0071_r1 |
| --- | --- |

| Title: | CE4.4.11: Combined Average Merge Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang Xu Chen Yongbing Lin Jianhua Zheng | Email: | zhangna25@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of Core Experiment (CE) 4.4.11 Test (a) – (e) for the combined average merge candidates. In Test (a) (which is also 4.2.15 test C in the 11th JVET meeting), combined bi-predictive merge candidates are replaced by the proposed combined average merge candidates. It provides   0.37% BD bit-rate saving. In Test (b), a + redundancy is removed when adding a proposed combined average merge candidate. It provides 0.40% BD bit-rate saving. In Test (c), b + enlarge merge candidate list size to 8. It provides 0.56% BD bit-rate saving. In Test (d), a + no motion vector scaling. It provides 0.40% BD bit-rate saving. In Test (e), d + redundancy is removed when adding a proposed combined average merge candidate. It provides 0.45% BD bit-rate saving.

Introduction

Combined bi-predictive merge candidates in current VTM-2.0.1 are created by combining original merge candidates. They are added following original merge candidates to decrease coding loss by using fixed list size. However, it is invalid for the P slice. Combined bi-predictive merge candidates are replaced by the proposed combined average merge candidates in [1].

Proposed Method

Combined average merge candidates are generated as follows. The motion information of the original merge candidates for each list is scaled to the derived reference picture for a given list. The scaled motion information is stored in a new array. The derived reference picture for a given list is the reference picture with the highest utilization rate among the reference pictures of the original merge candidates (up to 4) in that list. After scaling the motion information, all available motion vectors (up to 2) of the combined merge candidates are averaged separately for each reference list. The averaged motion vector is assigned as the motion vector of the combined average merge candidate. The derived reference picture for a given list is the reference picture of all the combined average merge candidates in that list.

In Figure 1, three combined average merge candidates are added into the merge candidate list.

| Merge_idx | L0 | L1 |
| --- | --- | --- |
| 0 | mvL0_A, ref0 | mvL1_A, ref1 |
| 1 | mvL0_B, ref0 | mvL1_B, ref1 |
| 2 | - | mvL1_C, ref1 |

(a)Original merge candidate list

| Merge_idx | L0 | L1 |
| --- | --- | --- |
| 0 | mvL0_A, ref0 | mvL1_A, ref1 |
| 1 | mvL0_B, ref0 | mvL1_B, ref1 |
| 2 | - | mvL1_C, ref1 |
| 3 | ( mvL0_A + mvL0_B)/2, ref0 | ( mvL1_A + mvL1_B)/2, ref1 |
| 4 | mvL0_A, ref0 | ( mvL1_A + mvL1_C)/2, ref1 |
| 5 | mvL0_B, ref0 | ( mvL1_B + mvL1_C)/2, ref1 |

(b)Merge Candidate List after adding combined average candidates

Figure 1．An example for combined average merge candidates

The combined average merge candidates have six combinations at most. The six combinations are described as follows:

uiPriority0[6] = {0 , 0,  1,  0,  1,  2};

uiPriority1[6] = {1 , 2,  2,  3,  3,  3};

Four more tests are also tested:

- Redundancy is removed when adding a proposed combined average merge candidate.

- 1) + enlarge merge candidate list size to 8.

- When two MVs are both available in a list, if the two reference indices are equal, the motion vectors are averagely calculated; otherwise, the MV and reference index with the smaller merge index are used. If only one MV is available in a list, use the MV and reference index directly. If no MV is available, keep this list invalid.

- 3)+ redundancy is removed when adding a proposed combined average merge candidate.

Experimental Results

The proposed method has been implemented on VTM-2.0.1[2], and all the tests were conducted under the common test conditions [3]. The experimental results of the Test (a) – (e) compared to VTM-2.0.1 are shown in Table1, Table2, Table3, Table4, and Table5 separately.

Table 1. Experimental results of the Test (a) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.24% | -0.38% | 100% | 100% |
| Class A2 | -0.40% | -0.29% | -0.45% | 100% | 100% |
| Class B | -0.36% | -0.11% | -0.34% | 100% | 100% |
| Class C | -0.28% | -0.20% | -0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | -0.20% | -0.31% | 100% | 100% |
| Class D | -0.39% | -0.40% | -0.39% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.38% | 0.26% | 100% | 102% |
| Class C | 0.16% | 0.18% | 0.42% | 100% | 101% |
| Class E | -0.12% | 0.15% | 0.13% | 100% | 99% |
| Overall | 0.07% | 0.26% | 0.28% | 100% | 101% |
| Class D | 0.15% | 0.16% | -0.11% | 101% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.50% | 0.30% | 0.03% | 101% | 101% |
| Class C | -0.57% | -0.52% | -0.63% | 102% | 100% |
| Class E | -0.24% | 0.17% | 0.74% | 101% | 99% |
| Overall | -0.46% | 0.00% | -0.01% | 101% | 100% |
| Class D | -0.73% | 0.17% | -0.67% | 102% | 102% |

Table 2. Experimental results of the Test (b) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.25% | -0.44% | 100% | 99% |
| Class A2 | -0.42% | -0.30% | -0.45% | 100% | 99% |
| Class B | -0.39% | -0.25% | -0.36% | 100% | 98% |
| Class C | -0.32% | -0.21% | -0.23% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.40% | -0.25% | -0.36% | 100% | 99% |
| Class D | -0.46% | -0.49% | -0.42% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.29% | 0.34% | 99% | 92% |
| Class C | 0.13% | 0.18% | 0.18% | 100% | 100% |
| Class E | -0.37% | 0.10% | -0.05% | 100% | 97% |
| Overall | -0.03% | 0.21% | 0.19% | 100% | 96% |
| Class D | 0.02% | 0.60% | -0.26% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.63% | 0.10% | 0.10% | 100% | 91% |
| Class C | -0.74% | -0.53% | -0.66% | 101% | 99% |
| Class E | -0.38% | 0.14% | 0.42% | 99% | 89% |
| Overall | -0.60% | -0.10% | -0.07% | 100% | 93% |
| Class D | -0.83% | -0.07% | -0.48% | 101% | 101% |

Table 3. Experimental results of the Test (c) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.68% | -0.41% | -0.62% | 100% | 100% |
| Class A2 | -0.61% | -0.54% | -0.70% | 100% | 98% |
| Class B | -0.53% | -0.28% | -0.40% | 100% | 99% |
| Class C | -0.46% | -0.34% | -0.37% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.56% | -0.37% | -0.49% | 101% | 100% |
| Class D | -0.62% | -0.58% | -0.68% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.29% | 0.11% | 100% | 99% |
| Class C | 0.03% | 0.16% | 0.20% | 101% | 92% |
| Class E | -0.47% | -0.10% | -0.44% | 100% | 90% |
| Overall | -0.11% | 0.15% | 0.00% | 100% | 94% |
| Class D | 0.02% | 0.57% | 0.12% | 101% | 99% |

Table 4. Experimental results of the Test (d) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.32% | -0.36% | 100% | 98% |
| Class A2 | -0.41% | -0.23% | -0.44% | 100% | 98% |
| Class B | -0.42% | -0.20% | -0.33% | 100% | 99% |
| Class C | -0.30% | -0.14% | -0.13% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.40% | -0.22% | -0.30% | 100% | 99% |
| Class D | -0.40% | -0.47% | -0.27% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.23% | -0.06% | 100% | 100% |
| Class C | 0.06% | 0.11% | 0.15% | 100% | 97% |
| Class E | -0.09% | 0.46% | 0.15% | 100% | 98% |
| Overall | 0.00% | 0.25% | 0.06% | 100% | 99% |
| Class D | 0.03% | 0.94% | 0.26% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.45% | 0.18% | 0.45% | 101% | 100% |
| Class C | -0.54% | -0.31% | -0.52% | 102% | 97% |
| Class E | -0.11% | 0.67% | 0.90% | 100% | 99% |
| Overall | -0.39% | 0.14% | 0.24% | 101% | 99% |
| Class D | -0.57% | -0.34% | -0.99% | 102% | 102% |

Table 5. Experimental results of the Test (e) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.40% | -0.40% | 100% | 99% |
| Class A2 | -0.44% | -0.38% | -0.50% | 99% | 98% |
| Class B | -0.47% | -0.26% | -0.38% | 99% | 99% |
| Class C | -0.38% | -0.20% | -0.30% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -0.30% | -0.38% | 99% | 99% |
| Class D | -0.50% | -0.47% | -0.38% | 99% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | 0.17% | 0.10% | 99% | 100% |
| Class C | -0.10% | -0.08% | -0.15% | 99% | 99% |
| Class E | -0.34% | -0.26% | -0.32% | 99% | 101% |
| Overall | -0.17% | -0.02% | -0.09% | 99% | 100% |
| Class D | -0.13% | 0.26% | -0.65% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.66% | 0.12% | 0.29% | 100% | 100% |
| Class C | -0.75% | -0.68% | -0.84% | 100% | 99% |
| Class E | -0.68% | -0.22% | 0.12% | 99% | 99% |
| Overall | -0.69% | -0.23% | -0.13% | 100% | 99% |
| Class D | -0.80% | -0.68% | -1.22% | 100% | 101% |

Conclusion

This contribution proposes a combined average merge candidate method. Simulation results show that compared to VTM-2.0.1, in RA condition, Test (a) provides 0.37% BD bit-rate saving; Test (b) provides 0.40% BD bit-rate saving; Test (c) provides 0.56% BD bit-rate saving; Test (d) provides 0.40% BD bit-rate saving; Test (e) provides 0.45% BD bit-rate saving. It is recommended to adopt Test (d) to the next version of VTM/BMS because of its simplicity.

Reference

- X. Chen, N. Zhang, J. Zheng, “CE 4: Enhanced Merge Mode (Test 4.2.15),” JVET-K0198, Ljubljana, SI, 10–18 Jul. 2018.

- VTM-2.0.1, https://vcgit.hhi.fraunhofer.de/JVET-K-CE4/VVCSoftware_VTM/tree/VTM-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, SI, 10–18 Jul. 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).