| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0083-v1 |
| --- | --- |

| Title: | CE2-related: Reduction of bits for ALF coefficient fractional part |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chi Su, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yu-chi.su, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the contribution, it is proposed to reduce bits for adaptive loop filter (ALF) coefficient fractional part. In VTM2.0.1, the fractional part of each ALF coefficient is represented in nine bits. It is reported that, compared with VTM2.0.1, 6-bit fractional part results in 0.10%, 0.09%, and -0.04% luma BD-rates for AI, RA, and LB, respectively, and 7-bit fractional part results in -0.01%, -0.03%, and -0.09% luma BD-rates for AI, RA, and LB, respectively. The reduction of bits for ALF coefficient fractional part is also tested on top of CE2.2.2 from JVET-L0082. The real value range of the center ALF coefficient is within [0.0, 2.0), the real value range of non-center ALF coefficients is within [-1.0, 1.0), and the number of bits for ALF coefficient fractional part is six or seven. It is shown that compared to VTM2.0.1, 6-bit fractional part with the reduced ranges results in 0.10%, 0.08%, and -0.03% luma BD-rates for AI, RA, and LB, respectively, and 7-bit fractional part with the reduced ranges results in -0.01%, -0.04%, and -0.11% luma BD-rates for AI, RA, and LB, respectively. With the 7-bit fractional part with the reduced ranges for ALF coefficients, the numbers of required bits to represent a non-center coefficient and a center coefficient are reduced from 11 to 8 and 15 to 8, respectively, when compared to VTM2.0.1. There are almost no run time changes in all the above tests.

# Proposed methods

In VTM2.0.1, the number of bits for ALF coefficient fractional part is nine. In this contribution, four tests related to ALF coefficient fractional part bit number reduction are presented. In the first and second tests, ALF coefficients with 6-bit and 7-bit fractional parts are evaluated. In the third and fourth tests, based on CE2.2.2 from JVET-L0082 [1], wherein the real value ranges of a non-center coefficient and a center coefficient are [-1,1) and [0,2), respectively, ALF coefficients with 6-bit and 7-bit fractional parts are evaluated. Table 1 shows comparison between different tests.

Table 1. Summary of different tests

|  | Number of bits for fractional part | Number of bits for a non-center coefficient | Number of bits for a center coefficient | Others |
| --- | --- | --- | --- | --- |
| VTM2.0.1 | 9 | 11 | 15 |  |
| CE2.2.2 | 9 | 10 | 10 | Reduced range |
| Test1 | 6 | 8 | 12 |  |
| Test2 | 7 | 9 | 13 |  |
| Test3 | 6 | 7 | 7 | Reduced range |
| Test4 | 7 | 8 | 8 | Reduced range |

# Experimental results

The proposed methods have been implemented on top of VTM2.0.1 [2]. Table 2 and Table 3 show the results of 6-bit and 7-bit fractional parts, respectively. Table 4 and Table 5 present the results of 6-bit and 7-bit fractional parts with reduced ranges, respectively.

# Conclusions

In this contribution, reducing ALF coefficient fractional part to seven bits, constraining center ALF coefficient real value range within [0.0, 2.0), and constraining non-center ALF coefficient real value range within [-1.0, 1.0) together achieve -0.01%, -0.04%, and -0.11% luma BD-rates for AI, RA, and LB, respectively, with very minor runtime changes, when compared to VTM2.0.1. The number of required bits is reduced from 11 to 8 for representing a non-center ALF coefficient, and from 15 to 8 for representing a center ALF coefficient.

# Patent rights declaration

MediaTek, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Y.-C, Su, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE2.2.1 and CE2.2.2: ALF coefficient coding and range constraints,” Document of Joint Video Experts Team, JVET-L0082.

[2] VTM2.0.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

Table 2. Results of Test1: 6-bit fractional part for ALF coefficients

Table 3. Results of Test2: 7-bit fractional part for ALF coefficients

Table 4. Results of Test3: 6-bit fractional part with reduced ranges for ALF coefficients

Table 5. Results of Test4: 7-bit fractional part with reduced ranges for ALF coefficients

# Working Draft

The following text is modified based on JVET-K0564, since there is no related description in JVET-K1001.

The modified part is highlighted by different color.

### 7.4.4 Adaptive loop filter semantics

alf_coeff_chroma[ i ] specifies the i-th coefficient of chroma filter, the order of exp-Golomb binarization is derived as golombOrder[ golombOrderIdx[ i ] ]. It is a requirement of bitstream conformance that the values of alf_coeff_chroma[ i ] shall be in the range of -27 to 27-1.

alf_coeff_luma_delta[ i ][ j ] specifies the j-th coefficient of i-th signaled luma filter coefficient delta, the order of exp-Golomb binarization is derived as golombOrder[ golombOrderIdx[ j ] ].When alf_coeff_luma_delta[ i ][ j ] is not present, it is inferred to be equal 0. It is a requirement of bitstream conformance that the values of alf_coeff_luma_delta[ i ][ j ] shall be in the range of -27 to 27-1.

#### 7.5.1.1 Derivation process for luma filter coefficients

Input of this process is filterType, and deltaFilterIndex.

– filterType specifies whether 7x7 (0) or 5x5 (1) filter is applied.

Outputs of this process is a luma filter coefficient array cL.

The binary array filter_pattern[ i ] with i=0..12 determines which coefficients are available based on 7x7 filter pattern and is defined as follows:

filter_pattern[ ] = { 0, 0, 1, 0, 0, 1, 1, 1, 0, 0, 1, 1, 1 }.

The variable filterCoefficients[ i ][ j ] with i = 0...number_of_filters_minus1, j = 0..11 is initialized as the following

filterCoefficients[ i ][ j ] = alf_coeff_luma_delta[ i ][ j ].

If filterDeltaPredMode is equal 1, filterCoefficients[ i ][ j ], with i = 0..number_of_filters_minus1, j = 0..11 are updated as follows:

filterCoefficients[ i ][ j ] += filterCoefficients[ i – 1 ][ j ]

It is a requirement of bitstream conformance that the values of filterCoefficients[ i ][ j ] with i = 0...number_of_filters_minus1, j = 0..11 shall be in the range of -27 to 27-1.

Variable k is set equal to 0, and the luma filter coefficients cL with elements cL[ i ][ j ], with i = 0..24 and j = 0..11 are derived as follows

  - If filter_pattern[ i ] is equal 1 or filterType is equal 0

cL[ i ][ j ] = filterCoefficients[ deltaFilterIndex[ i ] ][ k ] 				(7-35)

k++

  - Otherwise,

cL[ i ][ j ] = 0

The last filter coefficient for j = 12 is derived as follows

cL[ i ][ 12 ] = 128 –k (cL[ i ][ k ]<<1), with k = 0..11

It is a requirement of bitstream conformance that the values of cL[ i ][ 12 ] with i = 0..24 shall be in the range of 0 to 28-1.

#### 7.5.1.2 Derivation process for chroma filter coefficients

Input to this process is a variable cIdx specifying colour component index.

Outputs of this process are the 7 filter coefficients cc for the colour component cIdx.

The chroma filter coefficients cC with elements cC[ i ], i = 0..5 are derived as follows

cC[ i ] = alf_coeff_chroma[ i ]	(7-36)

The last filter coefficient for i = 6 is derived as follows

cC[ 6 ] = 512 –k (cC[ k ]<<1), with k = 0..5.

It is a requirement of bitstream conformance that the values of cC[ 6 ] shall be in the range of 0 to 28-1.

#### 7.5.1.3 Coding tree block filtering process for luma samples

Inputs of this process are:

–	a reconstructed luma picture sample array recPicture prior to the adaptive loop filtering process,

–	a filtered reconstructed luma picture sample array alfPicture,

–	a location ( xC, yC ) specifying the top-left sample of the granularity block relative to the top left sample of the current picture,

–	a filter type filterType,

–	a 25x13 array of luma filter coefficients cL.

Output of this process is the modified filtered reconstructed luma picture sample array alfPicture.

…

…

A variable sum is set to 0, and the following ordered steps are applied:

  - If filterType is equal 0

sum += filterCoeff[ 0 ] * ( recPicture[ hx, vy + 3 ] + recPicture[ hx, vy − 3 ] )

sum += filterCoeff[ 1 ] * ( recPicture[ hx + 1, vy + 2 ] + recPicture[ hx − 1, vy − 2 ] )

sum += filterCoeff[ 3 ] *( recPicture[ hx − 1, vy + 2 ] + recPicture[ hx + 1, vy − 2 ] )

sum += filterCoeff[ 4 ] * ( recPicture[ hx + 2, vy + 1 ] + recPicture[ hx − 2, vy − 1 ] )

sum += filterCoeff[ 8 ] * ( recPicture[ hx − 2, vy + 1 ] + recPicture[ hx + 2, vy − 1 ] )

sum += filterCoeff[ 9 ] * ( recPicture[ hx + 3, vy ] + recPicture[ hx − 3, vy ] )

sum += filterCoeff[ 2 ] * ( recPicture[ hx, vy + 2 ] + recPicture[ hx, vy − 2 ] )

sum += filterCoeff[ 5 ] * ( recPicture[ hx + 1, vy + 1 ] + recPicture[ hx − 1, vy − 1 ] )

sum += filterCoeff[ 6 ] * ( recPicture[ hx, vy + 1 ] + recPicture[ hx, vy − 1 ] )

sum += filterCoeff[ 7 ] * ( recPicture[ hx − 1, vy + 1 ] + recPicture[ hx + 1, vy − 1 ] )

sum += filterCoeff[ 10 ] * ( recPicture[ hx + 2, vy ] + recPicture[ hx − 2, vy ] )

sum += filterCoeff[ 11 ] *  ( recPicture[ hx + 1, vy ] + recPicture[ hx − 1, vy ] )

sum += filterCoeff[ 12 ] * recPicture[ hx, vy ]

sum = ( sum + 64 ) >> 7

alfPictureL[ x ][ y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, sum )

#### 7.5.1.4 Coding tree block filtering process for chroma samples

Inputs of this process are:

–	a reconstructed chroma picture sample array recPicture prior to the adaptive loop filtering process,

–	a filtered reconstructed chroma picture sample array alfPicture,

–	a location ( xC, yC ) specifying the top-left sample of the granularity block relative to the top left sample of the current picture,

–	an array of chroma filter coefficients cC.

Output of this process is the modified filtered reconstructed chroma picture sample array alfPicture.

…

…

A variable sum is derived as follows:

sum = cC[ 0 ] * ( recPicture[ hx, vy + 2 ] + recPicture[ hx, vy − 2 ] )

sum += cC[ 1 ] * ( recPicture[ hx + 1, vy + 1 ] + recPicture[ hx − 1, vy − 1 ] )

sum += cC[ 2 ] * ( recPicture[ hx, vy + 1 ] + recPicture[ hx, vy − 1 ] )

sum += cC[ 3 ] * ( recPicture[ hx − 1, vy + 1 ] + recPicture[ hx + 1, vy − 1 ] )

sum += cC[ 4 ] * ( recPicture[ hx + 2, vy ] + recPicture[ hx − 2, vy ] )

sum += cC[ 5 ] * ( recPicture[ hx + 1, vy ] + recPicture[ hx − 1, vy ] )

sum += cC[ 6 ] * recPictureC[ hx, vy ]

sum = ( sum + 64 ) >> 7

alfPicture[ x ][ y ] = Clip3( 0, ( 1 << BitDepthC ) − 1, sum )