| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0093-v1 |
| --- | --- |

| Title: | CE4-related: Simplified pruning in merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chia Chen, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

A simplification of the pruning process in merge mode is proposed. The worst case merge candidate comparison count in the pruning process is reduced from 16 to 6. It is reported that when the proposed method is applied to VTM2.0.1, the luma BD-rates are 0.01% and 0.02% for RA and LB, respectively.

# Introduction

Merge mode utilizes spatial neighboring coding units (CUs) to derive motion information of the current CU, so parallel derivation of motion information for multiple neighboring CUs is not possible. To achieve higher coding gain, merge mode is getting more and more complex. Therefore, this contribution aims at the operation simplification of merge mode without losing coding efficiency.

In VTM2.0.1, full pruning of spatial and temporal candidates is used. In HEVC, partial pruning is adopted. In this contribution, a straightforward method to simplify pruning as HEVC is proposed. Only one or two  comparisons for each candidate are allowed in the pruning process.

# Proposed method

Figure 1 shows the definition of A1, B1, B0, A0, B2, TBR, TCTR positions.

Figure 1. Merge candidate positions

Figure 2 shows the pruning process in VTM2.0.1. The spatial candidates A1, B1, B0, A0, and B2 are fully pruned with each other. The temporal candidate (TBR or TCTR) is pruned by all available spatial candidates and by the scaled center motion vector (MV) in ATMVP. The scaled center MV in ATMVP is derived as follows: find the initial vector according to the available spatial merge candidates, constrain and modify the initial vector, locate the corresponding block pointed by the modified initial vector, find the center MV of the corresponding block, and scale the center MV to reference index 0. To apply pruning when adding B1, B0, A0, B2, and TBR or TCTR to the merge candidate list, at most 1, 2, 3, 4, 6 merge candidate comparisons are required, respectively. Therefore, the worst case merge candidate comparison count is 16.

Figure 2. Pruning process in VTM2.0.1

The proposed pruning process is shown in Figure 3. Candidate B1 is pruned only by A1, candidate B0 is pruned only by B1, candidate A0 is pruned only by A1, and candidate B2 is pruned only by A1 and B1. The temporal candidate is pruned only by the scaled center MV in ATMVP. In this way, the worst case merge candidate comparison count is reduced from 16 to 6.

Figure 3. Proposed pruning process

# Simulation results

The proposed method is integrated on VTM2.0.1. All tests were conducted under the common test condition [1]. Table 1 shows the results of proposed method.

Table 1. Results of the proposed simplified pruning on VTM2.0.1

# Conclusions

This contribution proposes a simplification of pruning process in merge mode. The simplification does not impact coding efficiency and reduces the number of merge candidate comparisons from 16 to 6.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-K1010.