| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0094-v1 |
| --- | --- |

| Title: | CE5.2.2: CABAC range sub-interval derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In JEM CABAC engine, the rangeLPS is derived by using a 512×64×9-bit look-up table, where the size of the rangeLPS look-up table is 144 times of that in HEVC. In CE5.2.2, two alternative rangeLPS derivation methods are proposed. Each of the two methods can be implemented with a look-up table or a multiplier. For both methods, the size of the table for deriving the rangeLPS value is the same as that in HEVC, i.e., 32×8×8 bits, but the tables of the two methods are different. In CE5.2.2.a, the table look-up values are calculated by using a 6-bit by 5-bit multiplier. In CE5.2.2.b, the table look-up values are calculated by using a 6-bit by 4-bit multiplier. Simulation results reportedly show that, compared with using the 512×64×9-bit look-up table, the average luma BD-rate increases are less than 0.00% and 0.06% for CE5.2.2.a and CE5.2.2.b, respectively, on CEM1 software. Results of using a 16×8×8-bit look-up table or a 5-bit by 5-bit multiplier in CE5.2.2.a and results of using a 16×8×8-bit look-up table or a 5-bit by 4-bit multiplier in CE5.2.2.b are also reported as follows. Compared with using the 512×64×9-bits look up table, the average luma BD-rate increases are less than 0.17% and 0.22% for CE5.2.2.a and CE5.2.2.b, respectively, on CEM1 software.

# Introduction

In JEM CABAC engine, the range is divided into rangeLPS and rangeMPS for coding each bin. The range sub-interval derivation is realized by using a 512×64×9-bit look-up table (LUT). The table covers probability range of (0, 1). Compared with the 64×4×8-bit rangeLPS table in HEVC, the JEM CABAC LUT size is 144 times. To reduce the table size, two methods are proposed in this contribution. In both CE5.2.2.a and CE5.2.2.b, a 32×8×8-bit table is used for deriving the rangeLPS value. The values in the look-up table can be computed by a 6-bit by 5-bit multiplier in CE5.2.2.a or by a 6-bit by 4-bit multiplier in CE5.2.2.b.

# Proposed methods

## CE5.2.2.a: RangeLPS derivation by using a 6-bit by 5-bit multiplier

The interval subdivision is realized using a 6-bit by 5-bit multiplier or equivalently a 32x8x8 table. The rangeLPS derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 63 – (P>>9) : (P>>9)

rangeLPS  = ((2 × probIdx +1) × (2 × ( range >> 5 ) + 1)) >> 3

or

rangeLPS  = LUT_32x8[ probIdx ][ ( range >> 5 ) & 7 ]

where the P is the probability of a context variable and the range is the current range in binary arithmetic coder (BAC). The supplementary results of using a 5-bit by 5-bit multiplier or equivalently a 16x8x8 table is also provided. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 31 – (P>>10) : (P>>10)

rangeLPS  = ((2 × probIdx +1) × (2 × ( range >> 5 ) + 1)) >> 2

or

rangeLPS  = LUT_16x8[ probIdx ][ ( range >> 5 ) & 7 ]

## CE5.2.2.b: RangeLPS derivation by using a 6-bit by 4-bit multiplier

The interval subdivision is realized using a 6-bit by 4-bit multiplier or equivalently a 32x8x8 table. The rangeLPS derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 63 – (P>>9) : (P>>9)

rangeLPS  = ((2 × probIdx +1) × ( range >> 5 )) >> 3

or

rangeLPS  = LUT_32x8[ probIdx ][ ( range >> 5 ) & 7 ]

where the P is the probability of a context variable and the range is the current range in BAC. Compared with the CE5.2.2.a, in CE5.2.2.b, the ( range >> 5 ) is used instead of (2 × ( range >> 5 ) + 1)). The supplementary results of using a 5-bit by 4-bit multiplier or equivalently a 16x8x8 table is also provided. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 31 – (P>>10) : (P>>10)

rangeLPS  = ((2 × probIdx +1) × ( range >> 5 )) >> 2

or

rangeLPS  = LUT_16x8[ probIdx ][ ( range >> 5 ) & 7 ]

In CE5.2.2.b, when the range is 256, the maximum rangeLPS is 124, which guarantees that during renormalization after processing an MPS, the range register is left shifted by at most one bit position.

# Simulation results

The proposed methods are implemented on CEM1 software [1]. The results of the multiplier version CE5.2.2.a (6-bit by 5-bit multiplier) are shown in Table 1. The look-up table version CE5.2.2.a produces exactly the same BD-rates as the multiplier version CE5.2.2a. The multiplier version CE5.2.2.b (6-bit by 4-bit multiplier) are shown in Table 2. The look-up table version CE5.2.2.b produces exactly the same BD-rates as the multiplier version CE5.2.2b. The simulations were performed following the VTM2.0 CTC. The anchor is CEM1. The supplementary results of using a 5-bit by 5-bit multiplier in CE5.2.2.a are shown in Table 3. The supplementary results of using a 5-bit by 4-bit multiplier in CE5.2.2.b are shown in Table 4.

Table 1.  Results of CE5.2.2.a: RangeLPS derivation by using a 6-bit by 5-bit multiplier

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.03% | 100% | 97% |
| Class A2 | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class C | -0.01% | 0.00% | 0.00% | 100% | 98% |
| Class E | 0.02% | 0.02% | 0.02% | 101% | 98% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F (optional) | 0.01% | 0.01% | 0.01% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.01% | 100% | 101% |
| Class A2 | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class C | 0.02% | 0.02% | 0.02% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.02% | 0.02% | 0.02% | 100% | 99% |
| Class F (optional) | 0.03% | 0.03% | 0.03% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.07% | -0.07% | 100% | 100% |
| Class C | -0.03% | -0.03% | -0.03% | 100% | 99% |
| Class E | -0.06% | -0.07% | -0.07% | 100% | 99% |
| Overall | -0.05% | -0.05% | -0.05% | 100% | 99% |
| Class D | -0.03% | -0.04% | -0.04% | 100% | 99% |
| Class F (optional) | -0.01% | -0.01% | -0.01% | 100% | 100% |

Table 2.  Results of CE5.2.2.b: RangeLPS derivation by using a 6-bit by 4-bit multiplier

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | 0.02% | 100% | 98% |
| Class A2 | 0.05% | 0.05% | 0.05% | 100% | 101% |
| Class B | 0.04% | 0.05% | 0.05% | 100% | 98% |
| Class C | 0.04% | 0.05% | 0.05% | 100% | 98% |
| Class E | 0.07% | 0.07% | 0.07% | 100% | 98% |
| Overall | 0.04% | 0.05% | 0.05% | 100% | 99% |
| Class D | 0.02% | 0.03% | 0.03% | 100% | 99% |
| Class F (optional) | 0.08% | 0.08% | 0.08% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.03% | 0.04% | 100% | 101% |
| Class A2 | 0.05% | 0.04% | 0.04% | 100% | 99% |
| Class B | 0.05% | 0.05% | 0.05% | 100% | 99% |
| Class C | 0.10% | 0.10% | 0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.06% | 0.06% | 100% | 100% |
| Class D | 0.10% | 0.10% | 0.10% | 100% | 99% |
| Class F (optional) | 0.11% | 0.10% | 0.10% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class C | 0.06% | 0.06% | 0.06% | 100% | 99% |
| Class E | 0.04% | 0.02% | 0.02% | 100% | 100% |
| Overall | 0.03% | 0.02% | 0.02% | 100% | 100% |
| Class D | 0.05% | 0.04% | 0.04% | 100% | 98% |
| Class F (optional) | 0.06% | 0.07% | 0.07% | 100% | 99% |

Table 3.  Supplementary results of CE5.2.2.a: RangeLPS derivation by using a 5-bit by 5-bit multiplier

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.21% | 0.21% | 100% | 97% |
| Class A2 | 0.18% | 0.18% | 0.18% | 100% | 99% |
| Class B | 0.18% | 0.18% | 0.18% | 100% | 98% |
| Class C | 0.15% | 0.15% | 0.15% | 100% | 98% |
| Class E | 0.13% | 0.13% | 0.13% | 100% | 98% |
| Overall | 0.17% | 0.17% | 0.17% | 100% | 98% |
| Class D | 0.12% | 0.12% | 0.12% | 100% | 99% |
| Class F (optional) | 0.11% | 0.11% | 0.11% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.17% | 0.17% | 100% | 98% |
| Class A2 | 0.16% | 0.16% | 0.16% | 100% | 101% |
| Class B | 0.16% | 0.16% | 0.16% | 100% | 100% |
| Class C | 0.14% | 0.14% | 0.14% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.16% | 0.16% | 100% | 100% |
| Class D | 0.14% | 0.13% | 0.13% | 100% | 99% |
| Class F (optional) | 0.11% | 0.11% | 0.11% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | 0.18% | 0.18% | 100% | 100% |
| Class C | 0.14% | 0.14% | 0.14% | 100% | 100% |
| Class E | 0.10% | 0.10% | 0.10% | 100% | 100% |
| Overall | 0.14% | 0.15% | 0.15% | 100% | 100% |
| Class D | 0.12% | 0.12% | 0.12% | 100% | 99% |
| Class F (optional) | 0.11% | 0.11% | 0.11% | 100% | 99% |

Table 4.  Supplementary results of CE5.2.2.b: RangeLPS derivation by using a 5-bit by 4-bit multiplier

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | 0.26% | 0.26% | 100% | 96% |
| Class A2 | 0.23% | 0.23% | 0.23% | 100% | 99% |
| Class B | 0.23% | 0.23% | 0.23% | 100% | 99% |
| Class C | 0.19% | 0.19% | 0.19% | 99% | 98% |
| Class E | 0.18% | 0.18% | 0.18% | 100% | 97% |
| Overall | 0.22% | 0.22% | 0.22% | 100% | 98% |
| Class D | 0.14% | 0.15% | 0.15% | 100% | 99% |
| Class F (optional) | 0.18% | 0.18% | 0.18% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.22% | 0.22% | 0.22% | 100% | 100% |
| Class A2 | 0.21% | 0.21% | 0.21% | 100% | 97% |
| Class B | 0.21% | 0.21% | 0.21% | 100% | 101% |
| Class C | 0.22% | 0.22% | 0.22% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.21% | 0.21% | 0.21% | 100% | 100% |
| Class D | 0.21% | 0.21% | 0.21% | 100% | 100% |
| Class F (optional) | 0.18% | 0.18% | 0.18% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.23% | 0.23% | 0.23% | 100% | 100% |
| Class C | 0.22% | 0.22% | 0.22% | 100% | 99% |
| Class E | 0.20% | 0.20% | 0.20% | 100% | 99% |
| Overall | 0.22% | 0.22% | 0.22% | 100% | 99% |
| Class D | 0.20% | 0.19% | 0.19% | 100% | 99% |
| Class F (optional) | 0.17% | 0.18% | 0.18% | 100% | 99% |

# Hardware synthesis results

The methods proposed in CE5.2.1, CE5.2.2.a, CE5.2.2.b, CE5.2.3, and CE5.2.4 are implemented in Verilog and synthesized by using TSMC 12nm technology. Each implementation is synthesized to reach its critical path. The results reportedly show that, when using the same table size (e.g. 16x16x8 LUT or 32x8x8 LUT), CE5.2.2.a (LUT version) has the shortest critical path (highest throughput), and CE5.2.1 (multiplier version) has the smallest area (lowest cost), as shown in Table 5.

Table 5.  Synthesis result of CE5.2.1, CE5.2.2.a, CE5.2.2.b, CE5.2.3, and CE5.2.4

|  | CEM1 | CE5.2.1 | CE5.2.1 | CE5.2.2.a | CE5.2.2.a | CE5.2.2.b | CE5.2.2.b | CE5.2.3 | CE5.2.4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Version | 512x64x9 LUT | 5b x 4b mult. | 32x8x8 LUT | 32x8x8 LUT | 6b x 5b mult. | 32x8x8 LUT | 6b x 4b mult. | 32x8x8 LUT | 16x16x8 LUT |
| Total Area (um2) | 3128.28 | 26.95 | 83.76 | 76.09 | 36.27 | 77.60 | 27.50 | 89.03 | 187.67 |
| Table Area (um2) | 3127.91 | 25.18 | 77.93 | 72.88 | 33.47 | 75.76 | 25.44 | 86.11 | 116.67 |
| Rest Area (um2) | 0.37 | 1.77 | 5.82 | 3.21 | 2.80 | 1.84 | 2.06 | 2.91 | 71.00 |
| Critical Path (ns) | 0.620 | 0.506 | 0.383 | 0.373 | 0.497 | 0.382 | 0.505 | 0.392 | 0.658 |

Table 6 shows the results of the implementations with smaller look-up table size or smaller multiplier. The results reportedly show that, when using the same table size (e.g. 16x8x8 LUT), CE5.2.2.a (LUT version) and CE5.2.2.b (LUT version) have the shortest critical path (highest throughput), and CE5.2.2.b (multiplier version) has the smallest area (lowest cost).

Table 6.  Synthesis result of CE5.2.2.a, CE5.2.2.b, CE5.2.3, and CE5.2.4 with smaller LUT size

|  | CE5.2.2.a | CE5.2.2.a | CE5.2.2.b | CE5.2.2.b | CE5.2.3 | CE5.2.4 | CE5.2.4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16x8x8 LUT | 5b x 5b mult. | 16x8x8 LUT | 5b x 4b mult. | 16x8x8 LUT | 8x16x8 LUT | 8x8x8 LUT |
| Total Area (um2) | 47.22 | 28.94 | 55.81 | 27.10 | 64.92 | 112.32 | 89.87 |
| Table Area (um2) | 45.45 | 26.95 | 53.86 | 25.55 | 61.78 | 67.50 | 41.47 |
| Rest Area (um2) | 1.77 | 1.99 | 1.95 | 1.55 | 3.13 | 44.83 | 48.40 |
| Critical Path (ns) | 0.335 | 0.486 | 0.335 | 0.458 | 0.344 | 0.611 | 0.563 |

# Conclusions

In this contribution, two alternative range subinterval derivation methods are proposed. For each of the two methods, a 32×8×8-bit table is used for deriving the rangeLPS value. The two methods use different tables, but the table sizes of the two methods are the same as that in HEVC. In CE5.2.2.a, the table look-up values are calculated by using a 6-bit by 5-bit multiplier. In CE5.2.2.b, the table look-up values are calculated by using a 6-bit by 4-bit multiplier. Implementation-wise a look-up table or a multiplier can be chosen. Simulation results reportedly show that, compared with using the 512×64×9-bit look up table, the average luma BD-rate increases are less than 0.00% in CE5.2.2.a and 0.06% in CE5.2.2.b compared with CEM1.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] H. Kirchhoffer, A. Said, "Description of Core Experiment 5 (CE5): Arithmetic Coding Engine," Document of Joint Video Experts Team, JVET-K1025.