| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0097-v1 |
| --- | --- |

| Title: | CE7-related: Context modeling using quantization index for dependent quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the dependent quantization, it is proposed to use quantization index for context modeling. Using quantization index for context modeling can remove the state transition process for de-quantization. Therefore, parallel de-quantization can be applied in hardware implementation. Simulation results reportedly show negligible impact on coding efficiency.

# Introduction

In JVET-K-meeting, transform coefficient coding and quantization presented in JVET-K0072 [1] was adopted into the VTM2. In this proposal, a method for parallel de-quantization is proposed.

# Proposed method

It is proposed to use quantization index instead of coefficient level for context modeling. The context modeling design is modified accordingly. The context modelling and binarization depend on the following measures for the local neighbourhood:

•	state: state of the dependent quantizer

•	numSig: the number of non-zero levels in the local neighbourhood;

•	sumAbs1: the sum of partially reconstructed absolute quantization indexes (absQIdx1) after the first pass in the local neighbourhood;

•	sumAbs: the sum of reconstructed absolute quantization indexes in the local neighbourhood

Let x and y represent the coordinates of a current scan position inside the transform block.

The selected probability models further depend on the diagonal position d, where d = x + y

Based on the values of numSig, sumAbs1, and d, the probability models for coding sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag are selected. The Rice parameter for binarizing abs_remainder is selected based on the values of sumAbs and numSig.

Context modelling for the significance flag

The probability model used for coding the sig_coeff_flag is determined as follows.

- If the current colour component represents the luma component, the probability model used is selected among a set of 54 probability models (3×18). The context index ctxIdSig (specifying one of the 54 probability models) is determined according to

ctxIdSig = 18 * max(0, state−1) + min( sumAbs1>>1, 5 ) + ( d < 2 ? 12 : ( d < 5 ? 6 : 0 ) )

- Otherwise (the current colour component represents a chroma component), the probability model used is selected among a set of 36 probability models (3×12). The context index ctxIdSig is determined according to

ctxIdSig = 12 * max(0, state−1) + min(sumAbs1>>1, 5 ) + ( d < 2 ? 6 : 0 ) )

Context modelling for the parity flag

Similar as for the significance flag, different sets of probability models are used for luma and chroma. The same set of probability models is used for both chroma components. For the luma component, one of 21 probability models is selected; for the chroma component, one of 11 probability models is selected. Hence, in total, 32 context models (21 for luma and 11 for chroma) are used for coding the par_level_flag.

The context index ctxIdPar specifying the probability model used is determined as follows:

- If the current scan position is equal to the position of the last non-zero level (as indicted by the transmitted x and y coordinates), ctxIdPar is set equal to 0;

- Otherwise, if the current colour component is the luma component, the context index is set to

ctxIdPar = 1 + min((sumAbs1>>1)– numSig, 4 ) + ( d == 0 ? 15 : ( d < 3 ? 10 : ( d < 10 ? 5 : 0 ) ) )

- Otherwise (the current colour component is the chroma component), the context index is set to

ctxIdPar = 1 + min((sumAbs1>>1)– numSig, 4 ) + ( d == 0 ? 5 : 0 )

Context modelling for the greater flags

Similar as for the parity flag, one of 32 context models (21 for luma and 11 for chroma) is used for coding the rem_abs_gt1_flag and rem_abs_gt2_flag. Different sets of context models are used for the rem_abs_gt1_flag and rem_abs_gt2_flag.

The context indexes ctxIdGt1 and ctxIdGt1 specifying the probability models used are determined in the same way as the context index ctxIdPar (see above):

ctxIdGt1 = ctxIdPar
			ctxIdGt2 = ctxIdPar

It should be noted that the presence of the rem_abs_gt1_flag and its context model do not depend on the value of the directly preceding parity flag.

Rice parameter for binarization of abs_remainder

The non-binary syntax element abs_remainder is binarized using the same class of Rice codes as in HEVC and VTM1. The resulting bins are coded in the bypass mode of the arithmetic coding engine. The Rice parameter ricePar is determined as follows:

- If (sumAbs>>1)– numSig is less than 12, ricePar is set equal to 0;

- Otherwise, if (sumAbs>>1)– numSig is less than 25, ricePar is set equal to 1;

- Otherwise, ricePar is set equal to 2.

# Experiment results

The proposed modifications have been implemented on top of VTM2.0.1 [2]. The common test conditions [3] are used for evaluation. Table 1 shows the results of using quantization index for context modeling.

Table 1. Results of using quantization index for context modeling.

# Conclusions

This contribution proposes a method that uses quantization indexes for context modeling, which can remove the state transition process in quantization stage and also enable parallel de-quantization in hardware implementation. The impact on coding efficiency is negligible.

# References

[1] H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “Non-CE7: Alternative entropy coding for dependent quantization,” JVET-K0072, July 2018.

[2] VTM-2.0.1: https://vcgit.hhi.fraunhofer.de/chenhuanbang/VVCSoftware_VTM/

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, August 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).