The added parts are highlighted in yellow

The deleted parts are highlighted in green

Table 7-4 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( cbWidth + cbHeight ) != 128 | ( cbWidth + cbHeight ) = = 128 |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | n.a. |

      - 8.3.2.1 General

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the chroma motion vectors in 1/32 fractional-sample accuracy mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

Let the variable LX be RefPicListX, with X being 0 or 1, of the current picture.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1, as well as predFlagL0 and predFlagL1, the following applies:

- If merge_flag[ xCb ][ yCb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in clause 8.3.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL0[ xSbIdx ][ ySbIdx ], mvL1[ xSbIdx ][ ySbIdx ], the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX and refIdxLX, in PRED_LX, and in the syntax elements ref_idx_lX and MvdLX, the following ordered steps apply:

- The variables numSbX and numSbY are derived as follows:

numSbX = 1		(8-101)

numSbY = 1		(8-102)

- The variables refIdxLX and predFlagLX are derived as follows:

- If inter_pred_idc[ xCb ][ yCb ] is equal to PRED_LX or PRED_BI,

refIdxLX = ref_idx_lX[ xCb ][ yCb ]	(8-103)

predFlagLX[ 0 ][0 ] = 1		(8-104)

- Otherwise, the variables refIdxLX and predFlagLX are specified by:

refIdxLX = −1		(8-105)

predFlagLX[ 0 ][0 ] = 0		(8-106)

- The variable mvdLX is derived as follows:

mvdLX[ 0 ] = MvdLX[ xCb ][ yCb ][ 0 ]	(8-107)

mvdLX[ 1 ] = MvdLX[ xCb ][ yCb ][ 1 ]	(8-108)

- When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in clause 8.3.2.8 is invoked with the luma coding block location ( xCb, yCb ), the coding block width cbWidth, the coding block height cbHeight and the variable refIdxLX as inputs, and the output being mvpLX.

- When predFlagLX is equal to 1, the luma motion vector mvLX is derived as follows:

uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 218 ) % 218	(8-109)

mvLX[ 0 ][0 ][ 0 ] = ( uLX[ 0 ]  >=  217 ) ? ( uLX[ 0 ] − 218 ) : uLX[ 0 ]	(8-110)

uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 218 ) % 218	(8-111)

mvLX[ 0 ][0 ][ 1 ] = ( uLX[ 1 ]  >=  217 ) ? ( uLX[ 1 ] − 218 ) : uLX[ 1 ]	(8-112)

NOTE 1– The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −217 to 217 − 1, inclusive.

When all of the following conditions are true, refIdxL1 is set equal to −1 and predFlagL1 is set equal to 0.

– predFlagL0 is equal to 1.

– predFlagL1 is equal to 1.

– nPbSw is equal to 4.

– nPbSh is equal to 4.

– TwoVersionsOfCurrDecPicFlag is equal to 1.

– noIntegerMvFlag is equal to 1.

– identicalMvs is equal to 0.

When predFlagLX, with X being 0 or 1, is equal to 1, the derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLX[ xSbIdx ][ ySbIdx ] as input, and the output being mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

///////ATMVP

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| sps_cclm_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_sbtmvp_enabled_flag ) |  |
| log2_sbtmvp_default_size_minus23 | u(1) |

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus23 | u(3) |
| } |  |

sbtmvp_size_override_flag equal to 1 specifies that the syntax element log2_sbtmvp_active_size_minus23 is present for the current slice. sbtmvp_size_override_flag equal to 0 specifies that the syntax element log2_atmvp_active_size_minus23 is not present and log2_sbtmvp_size_active_minus23 is inferred to be equal to log2_sbtmvp_default_size_minus23.

log2_sbtmvp_active_size_minus23 plus 23 specifies the value of the subblock size that is used for deriving the motion parameters for the subblock-based TMVP of the current slice. When log2_sbtmvp_size_active_minus23 is is not present, it is inferred to be equal to log2_sbtmvp_default_size_minus23. The variable is derived as follows:

Log2SbtmvpSize  =  log2_sbtmvp_size_active_minus23 + 23	(7-36)

      - 8.3.2.2 Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.3.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the luma coding block width as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1.

- When both cbWidth  and cbHeight are larger than 4, the derivation process for subblock-based temporal merging candidates as specified in clause 8.3.2.4 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight , the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 as inputs and the output being the availability flag availableFlagSbCol, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the reference indices refIdxLXSbCol, the luma motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and the prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

- The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-113)

predFlagL0Col = availableFlagL0Col		(8-114)

predFlagL1Col = 0		(8-115)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-116)

predFlagL1Col = availableFlagL1Col		(8-117)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-118)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagSbCol )
	mergeCandList[ i++ ] = SbCol
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in clause 8.3.2.6 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added into mergeCandList. The number of candidates being added, numCombMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in clause 8.3.2.7 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following applies:

- If mergeCandList[ merge_idx[ xCb ][ yCb ] ] is equal to SbCol, the following assignments are made for xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1

refIdxLX = refIdxLXSbCol		(8-119)

predFlagLX[ xSbIdx ][ ySbIdx ] = predFlagLXSbCol[ xSbIdx ][ ySbIdx ]	(8-120)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 0 ]	(8-121)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 1 ]	(8-122)

- Otherwise, the following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

numSbX = 1		(8-123)

numSbY = 1		(8-124)

refIdxLX = refIdxLXN		(8-125)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-126)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-127)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-128)

///////Affine

      - 8.3.3.1 General

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the luma subblock motion vector array in 1/16 fractional-sample accuracy mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2 ) − 1, ySbIdx = 0 .. ( cbHeight >> 2 ) − 1, X being 0 and 1,

- the chroma subblock motion vector array in 1/32 fractional-sample accuracy mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2 ) − 1, ySbIdx = 0 .. ( cbHeight >> 2 ) − 1, X being 0 and 1.

For the derivation of the variables mvLX[ xSbIdx ][ ySbIdx ] and mvCLC[ xSbIdx ][ ySbIdx ], the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1, the following applies:

- For the derivation of the number of control point motion vectors numCpMv, the control point motion vectors cpMvL0[ cpIdx ] and cpMvL1[ cpIdx ] with cpIdx ranging from 0 to numCpMv − 1, refIdxL0, refIdxL1, predFlagL0 and predFlagL1, the following applies:

- If merge_affine_flag[ xCb ][ yCb ] is equal to 1, the derivation process for affine control point motion vectors and reference indices in affine merge mode as specified in 8.3.3.2 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight as input, the number of control point motion vectors numCpMv, the control point motion vectors cpMvL0[ cpIdx ], cpMvL1[ cpIdx ], with cpIdx ranging from 0 to numCpMv − 1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as output.

- Otherwise (merge_affine_flag[ xCb ][ yCb ] is equal to 0), for X being replaced by either 0 or 1 in the variables predFlagLX, cpMvLX, MvdCpLX, and refIdxLX, in PRED_LX, and in the syntax element ref_idx_lX, the following ordered steps apply:

- The number of control point motion vectors numCpMv is set equal to MotionModelIdc[ xCb ][ yCb ] + 1.

- The variables refIdxLX and predFlagLX with X being 0 or 1 are derived as follows:

- If inter_pred_idc[ xCb ][ yCb ] is equal to PRED_LX or PRED_BI,

refIdxLX = ref_idx_lX[ xCb ][ yCb ]		(8-242)

predFlagLX = 1		(8-243)

- Otherwise, the variables refIdxLX and predFlagLX are specified by:

refIdxLX = −1		(8-244)

predFlagLX = 0		(8-245)

- The variable mvdCpLX[ cpIdx ] with cpIdx ranging from 0 to numCpMv − 1, is derived as follows:

mvdCpLX[ cpIdx ][ 0 ] = MvdCpLX[ xCb ][ yCb ][ cpIdx ][ 0 ]	(8-246)

mvdCpLX[ cpIdx ][ 1 ] = MvdCpLX[ xCb ][ yCb ][ cpIdx ][ 1 ]	(8-247)

- When predFlagLX is equal to 1, the derivation process for luma affine control point motion vector predictors as specified in clause 8.3.3.4 is invoked with the luma coding block location ( xCb, yCb ), and the variables cbWidth, cbHeight, refIdxLX, and the number of control point motion vectors numCpMv as inputs, and the output being mvpCpLX[ cpIdx ] with cpIdx ranging from 0 to numCpMv − 1.

- When predFlagLX is equal to 1, the luma motion vectors cpMvLX[ cpIdx ] with cpIdx ranging from 0 to NumCpMv − 1, are derived as follows:

uLX[ cpIdx ][ 0 ] = ( mvpCpLX[ cpIdx ][ 0 ] + mvdCpLX[ cpIdx ][ 0 ] + 218 ) % 218	(8-248)

cpMvLX[ cpIdx ][ 0 ] = (uLX[ cpIdx ][ 0 ]  >=  217 ) ? (uLX[ cpIdx ][ 0 ] − 218 ) : 
	uLX[ cpIdx ][ 0 ]	(8-249)

uLX[ cpIdx ][ 1 ] = ( mvpCpLX[ cpIdx ][ 1 ] + mvdCpLX[ cpIdx ][ 1 ] + 218 ) % 218	(8-250)

cpMvLX[ cpIdx ][ 1 ] = (uLX[ cpIdx ][ 1 ]  >=  217 ) ? (uLX[ cpIdx ][ 1 ] − 218 ) : 
	uLX[ cpIdx ][ 1 ]	(8-251)

- The derivation process for motion vector arrays from affine control point motion vectors as specified in subclause 8.3.3.7 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma prediction block height cbHeight, the number of control point motion vectors numCpMv, the control point motion vectors cpMvL1[ cpIdx ], cpMvL0[ cpIdx ] with cpIdx being 0..2, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 and xSbShift and ySbShift as inputs, the luma motion vector array mvLX[ xSbIdx ][ ySbIdx ] and the chroma motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2xSbShift ) − 1, ySbIdx = 0 .. ( cbHeight >> 2ySbShift ) − 1, X being 0 and 1 as output. The variables xSbShift and ySbShift are derived as follows:

if predFlagL0 equal to 0 or predFlagL1 equal to 0

xSbShift =2

ySbShift = 2

Otherwise if cbHeight > cbWidth

xSbShift =2

ySbShift = 3

Otherwise

xSbShift =3

ySbShift = 2

- For xSbIdx = 0..( cbWidth >> 2xSbShift ) − 1 and ySbIdx = 0..( cbHeight >> 2ySbShift ) − 1, the motion vectors MvLX with X being 0 and 1 are derived as follows:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + xSbIdx * (1<< xSbShift), yCb + ySbIdx * (1<< ySbShift) )	(8-252)

- If one or more of the following conditions are true,

- xSbIdx and ySbIdx are both equal to 0 (top-left subblock)

- xSbIdx is equal to ( cbWidth >> 2xSbShift ) − 1 and ySbIdx is equal to 0 (top-right subblock)

- numCpMv is equal to 3 and xSbIdx is equal to 0 and ySbIdx is equal to ( cbHeight >> 2ySbShift ) − 1 (top-right subblock)

the following applies for x = 0..3 and y = 0..3:

MvL0[ xSb + x ][ ySb + y ] = cpMvL0[ xSbIdx ][ ySbIdx ]	(8-253)

MvL1[ xSb + x ][ ySb + y ] = cpMvL1[ xSbIdx ][ ySbIdx ]	(8-254)

- Otherwise, the following applies for x = 0..3 and y = 0..3:

MvL0[ xSb + x ][ ySb + y ] = mvL0[ xSbIdx ][ ySbIdx ]	(8-255)

MvL1[ xSb + x ][ ySb + y ] = mvL1[ xSbIdx ][ ySbIdx ]	(8-256)

- For x = 0..cbWidth − 1 and y = 0..cbHeight − 1, the reference indices refIdxLX and the prediction list utilization flags PredFlagLX with X being 0 and 1 are derived as follows:

RefIdxL0[ xCb + x ][ yCb + y ] = refIdxL0		(8-257)

RefIdxL1[ xCb + x ][ yCb + y ] = refIdxL1		(8-258)

PredFlagL0[ xCb + x ][ yCb + y ] = predFlagL0		(8-259)

PredFlagL1[ xCb + x ][ yCb + y ] = predFlagL1		(8-260)

#### Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvL0[ cpIdx ] and cpMvL1[ cpIdx ], with cpIdx = 0..numCpMv − 1,

- the prediction list utilization flags predFlagL0 and predFlagL1.

- xSbShift and ySbShift

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2xSbShift ) − 1, ySbIdx = 0 .. ( cbHeight >> 2ySbShift ) − 1, X being 0 and 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2xSbShift ) − 1, ySbIdx = 0 .. ( cbHeight >> 2ySbShift ) − 1, X being 0 and 1.

When predFlagLX is equal to 1, the following applies for X being 0 and 1:

- The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(8-328)

log2CbH = Log2( cbHeight )		(8-329)

- The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(8-330)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(8-331)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(8-332)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(8-333)

- If numCpMv is equal to 3, the variable dHorY and dVerY are derived as follow:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(8-334)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(8-335)

- Otherwise ( numCpMv is equal to 2), the variable dHorY and dVerY are derived as follows:

dHorY = − dVerX		(8-336)

dVerY = dHorX		(8-337)

- For xSbIdx = 0..( cbWidth >> 2xSbShift ) − 1 and ySbIdx = 0..( cbHeight >> 2ySbShift ) − 1, the following applies:

  - The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 21<<(xSbShift-1) + ( xSbIdx << 2xSbShift	(8-338)

yPosCb = 21<<(ySbShift-1) + ( ySbIdx << 2ySbShift )                                                  (8-339)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-340)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-341)

  - The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLX[ xSbIdx ][ ySbIdx ] as input, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.