| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0113-v1 |
| --- | --- |

| Title: | On final reference picture lists in the CTC random access simulation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Jianle Chen 10180 Telesis Court San Diego, CA, 92121 USA | Tel: Email: | +1-858-375-8065 fnu.hendry@huawei.com +1-908-803-3888 yekui.wang@huawei.com +1-858-999-4243 jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes a change to the reference picture lists for the CTC random access simulation. It was asserted that having reference pictures with POC values that are both less and greater than the POC of the current picture in a final reference picture list (i.e., either reference picture list L0 or reference picture list L1) does not give compression benefit. It is proposed to simplify the reference picture lists for random access simulations such that a reference picture list contains reference pictures with POC values that all are either less or greater than the POC of the current picture. Simulation result shows performance gain in average −0.19%, −0.26% and −0.26% for Y, U and V, respectively. It was also reported that the proposed change also provides a small reduction to overall average encoding time, which is 2%. It was asserted that the proposed change is a non-normative encoder-only change.

# Introduction

In the current common test condition (CTC) for random access (RA), for some positions within a group of pictures for the RA configuration, the reference picture lists (RPLs) for pictures at those positions contain reference pictures with both POC values less and greater than the POC of the current picture. For example, for the picture with POC 40 the encoder log below shows that L1 contains a reference picture with POC that is greater than the current picture's POC (i.e., reference picture with POC 48) and a reference picture with POC that is less the current picture's POC (i.e., reference picture with POC 32):

POC   40 TId: 1 ( B-SLICE, QP 23 )   61344 bits [Y 41.0202 dB   U 44.1508 dB   V 45.2048 dB] [ET  13 ] [L0 32 24 ] [L1 48 32 ]

It is debatable whether it is beneficial to have mixed reference pictures (i.e., reference pictures with both POC values less and greater than current picture's POC) in one RPL. Our hypotheses on this are as follows:

- In the VTM software, per the CTC, in an RPL that contains mixed reference pictures, the reference pictures with POC values greater than that of the current picture are also available in the other RPL. From the example above, the reference picture with POC 32 in the L1 is also available in L0. This duplication comes from the way RPL is constructed from reference picture set (RPS) in the VTM software that is the same as in HEVC and by simply setting the number of active list entries to be 2. It seems that removing such duplication, i.e., removing the reference picture with POC 32 from L1 for the picture with POC 40, would not hurt coding efficiency.

- Simplifying the final L0 or L1 to contain only one active entry, particularly by removing one for which the reference picture already exists in the other final, is likely to be beneficial for coding efficiency as the reference index to the only entry does not need to be signalled at coding block level.

Based on the above hypotheses, we did simulations to study the impact of such simplification of the final RPLs to the coding performance.

# Test results

We updated the RPL construction function at the encoder side of the reference software VTM-2.0.1. The modification to the function is summarized as follows:

- After the RPL L0 is initially constructed, if it contains reference pictures with both POC values less than and greater than current picture's POC value, remove the reference pictures with POC values greater than current picture's POC value from the list.

- After the RPL L1 is initially constructed, if it contains reference pictures with both POC values less than and greater than current picture's POC value, remove the reference pictures with POC values less than current picture's POC value from the list.

The differences between reference pictures in the RPLs before and after this modification are shown in Annex A.

Table 1 tabulates the simulation results. The results support our hypothesis that having mixed reference pictures in an RPL is not beneficial from coding efficiency point of view. Moreover, having shorter RPLs is also beneficial for reducing encoding complexity as less reference pictures need to be tested, as can be seen from consistent reduction in the encoding time.

Table 1 Performance of the proposed modification for the CTC RA simulation

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Tango2 | -0.56% | -0.96% | -0.93% | 98% | 101% |  |
| FoodMarket4 | -0.31% | -0.21% | -0.24% | 99% | 100% |  |
| CampfireParty2 | 0.00% | -0.14% | -0.04% | 99% | 101% |  |
| CatRobot1 | -0.35% | -0.41% | -0.43% | 98% | 101% |  |
| DaylightRoad2 | -0.40% | -0.65% | -0.49% | 98% | 100% |  |
| ParkRunning3 | -0.31% | -0.28% | -0.30% | 98% | 101% |  |
| MarketPlace | -0.23% | -0.25% | -0.24% | 98% | 100% |  |
| RitualDance | -0.13% | -0.19% | -0.18% | 98% | 101% |  |
| Cactus | -0.09% | -0.13% | -0.24% | 98% | 100% |  |
| BasketballDrive | -0.11% | -0.15% | -0.29% | 98% | 100% |  |
| BQTerrace | 0.15% | -0.04% | 0.05% | 98% | 100% |  |
| BasketballDrill | -0.12% | -0.05% | -0.15% | 98% | 101% |  |
| BQMall | -0.19% | -0.14% | -0.12% | 98% | 99% |  |
| PartyScene | -0.08% | -0.19% | -0.27% | 98% | 100% |  |
| RaceHorses | -0.10% | -0.08% | -0.05% | 98% | 100% |  |
| BasketballPass | -0.16% | -0.32% | -0.25% | 98% | 99% |  |
| BQSquare | 0.06% | -0.06% | 0.08% | 98% | 99% |  |
| BlowingBubbles | -0.09% | -0.02% | -0.01% | 98% | 98% |  |
| RaceHorses | -0.21% | -0.51% | -0.89% | 98% | 100% |  |
| FourPeople |  |  |  |  |  |  |
| Johnny |  |  |  |  |  |  |
| KristenAndSara |  |  |  |  |  |  |
| Class A1 | -0.29% | -0.43% | -0.41% | 99% | 101% |  |
| Class A2 | -0.36% | -0.45% | -0.41% | 98% | 100% |  |
| Class B | -0.08% | -0.15% | -0.18% | 98% | 100% |  |
| Class C | -0.12% | -0.12% | -0.15% | 98% | 100% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.19% | -0.26% | -0.26% | 98% | 100% |  |
| Class D | -0.10% | -0.23% | -0.27% | 98% | 99% |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |

# Proposal

We propose that the JVET discusses the necessity to have mixed reference pictures in an RPL for RA tests. If it is concluded that there is no good reason to have it, we suggest using the above simplification to the reference picture list in the Common Test Conditions as it shows compression benefits as well as encoding time reduction.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the authors).

Annex A

The logs below were taken from encoder log file for the beginning 65 pictures of the BQSquare test sequence in the CTC RA simulation. It is meant to show the difference between the content of the final RPLs of the current implementation and the modified implementation. Impacted final RPLs are highlighted.

As can be seen from the logs, 7 of the 16 pictures of the first full GOP (pictures with POC 1-16) were impacted by this modification, while 5 of the 16 pictures of each of the other full GOPs were impacted by this modification, 1 instance on list 0 and 4 instances on list 1.

Current RPLs              Modified RPLs

POC    0 TId: 0 ( I-SLICE, QP 19 )   [L0 ] [L1 ]  		  [L0 ] [L1 ]

POC   16 TId: 0 ( B-SLICE, QP 23 )  [L0 0 ] [L1 0 ]		  [L0 0 ] [L1 0 ]

POC    8 TId: 1 ( B-SLICE, QP 23 )  [L0 0 16 ] [L1 16 0 ]		  [L0 0 ] [L1 16 ]

POC    4 TId: 2 ( B-SLICE, QP 26 )  [L0 0 8 ] [L1 8 16 ]	 	  [L0 0 ] [L1 8 16 ]

POC    2 TId: 3 ( B-SLICE, QP 27 )  [L0 0 4 ] [L1 4 8 ] 		  [L0 0 ] [L1 4 8 ]

POC    1 TId: 4 ( B-SLICE, QP 29 )  [L0 0 2 ] [L1 2 4 ] 		  [L0 0 ] [L1 2 4 ]

POC    3 TId: 4 ( B-SLICE, QP 29 )  [L0 2 0 ] [L1 4 8 ] 		  [L0 2 0 ] [L1 4 8 ]

POC    6 TId: 3 ( B-SLICE, QP 27 )  [L0 4 0 ] [L1 8 16 ]	 	  [L0 4 0 ] [L1 8 16 ]

POC    5 TId: 4 ( B-SLICE, QP 29 )  [L0 4 0 ] [L1 6 8 ] 		  [L0 4 0 ] [L1 6 8 ]

POC    7 TId: 4 ( B-SLICE, QP 29 )  [L0 6 4 ] [L1 8 16 ]		  [L0 6 4 ] [L1 8 16 ]

POC   12 TId: 2 ( B-SLICE, QP 26 )  [L0 8 0 ] [L1 16 8 ] 		  [L0 8 0 ] [L1 16 ]

POC   10 TId: 3 ( B-SLICE, QP 27 )  [L0 8 0 ] [L1 12 16 ]		  [L0 8 0 ] [L1 12 16 ]

POC    9 TId: 4 ( B-SLICE, QP 29 )  [L0 8 0 ] [L1 10 12 ] 	 	  [L0 8 0 ] [L1 10 12 ]

POC   11 TId: 4 ( B-SLICE, QP 29 )  [L0 10 8 ] [L1 12 16 ] 	  [L0 10 8 ] [L1 12 16 ]

POC   14 TId: 3 ( B-SLICE, QP 27 )  [L0 12 8 ] [L1 16 12 ] 	  [L0 12 8 ] [L1 16 ]

POC   13 TId: 4 ( B-SLICE, QP 29 )  [L0 12 8 ] [L1 14 16 ] 	  [L0 12 8 ] [L1 14 16 ]

POC   15 TId: 4 ( B-SLICE, QP 29 )  [L0 14 12 ] [L1 16 14 ]	  [L0 14 12 ] [L1 16 ]

POC   32 TId: 0 ( B-SLICE, QP 23 )  [L0 16 0 ] [L1 16 0 ]		  [L0 16 0 ] [L1 16 0 ]

POC   24 TId: 1 ( B-SLICE, QP 23 )  [L0 16 8 ] [L1 32 16 ] 	  [L0 16 8 ] [L1 32 ]

POC   20 TId: 2 ( B-SLICE, QP 26 )  [L0 16 8 ] [L1 24 32 ] 	  [L0 16 8 ] [L1 24 32 ]

POC   18 TId: 3 ( B-SLICE, QP 27 )  [L0 16 8 ] [L1 20 24 ] 	  [L0 16 8 ] [L1 20 24 ]

POC   17 TId: 4 ( B-SLICE, QP 29 )  [L0 16 18 ] [L1 18 20 ]	  [L0 16 ] [L1 18 20 ]

POC   19 TId: 4 ( B-SLICE, QP 29 )  [L0 18 16 ] [L1 20 24 ]	  [L0 18 16 ] [L1 20 24 ]

POC   22 TId: 3 ( B-SLICE, QP 27 )  [L0 20 16 ] [L1 24 32 ]	  [L0 20 16 ] [L1 24 32 ]

POC   21 TId: 4 ( B-SLICE, QP 29 )  [L0 20 16 ] [L1 22 24 ]	  [L0 20 16 ] [L1 22 24 ]

POC   23 TId: 4 ( B-SLICE, QP 29 )  [L0 22 20 ] [L1 24 32 ]	  [L0 22 20 ] [L1 24 32 ]

POC   28 TId: 2 ( B-SLICE, QP 26 )  [L0 24 16 ] [L1 32 24 ]	  [L0 24 16 ] [L1 32 ]

POC   26 TId: 3 ( B-SLICE, QP 27 )  [L0 24 16 ] [L1 28 32 ]	  [L0 24 16 ] [L1 28 32 ]

POC   25 TId: 4 ( B-SLICE, QP 29 )  [L0 24 16 ] [L1 26 28 ]	  [L0 24 16 ] [L1 26 28 ]

POC   27 TId: 4 ( B-SLICE, QP 29 )  [L0 26 24 ] [L1 28 32 ]	  [L0 26 24 ] [L1 28 32 ]

POC   30 TId: 3 ( B-SLICE, QP 27 )  [L0 28 24 ] [L1 32 28 ]	  [L0 28 24 ] [L1 32 ]

POC   29 TId: 4 ( B-SLICE, QP 29 )  [L0 28 24 ] [L1 30 32 ]	  [L0 28 24 ] [L1 30 32 ]

POC   31 TId: 4 ( B-SLICE, QP 29 )  [L0 30 28 ] [L1 32 30 ]	  [L0 30 28 ] [L1 32 ]

POC   48 TId: 0 ( B-SLICE, QP 23 )  [L0 32 16 ] [L1 32 16 ]	  [L0 32 16 ] [L1 32 16 ]

POC   40 TId: 1 ( B-SLICE, QP 23 )  [L0 32 24 ] [L1 48 32 ]	  [L0 32 24 ] [L1 48 ]

POC   36 TId: 2 ( B-SLICE, QP 26 )  [L0 32 24 ] [L1 40 48 ]	  [L0 32 24 ] [L1 40 48 ]

POC   34 TId: 3 ( B-SLICE, QP 27 )  [L0 32 24 ] [L1 36 40 ]	  [L0 32 24 ] [L1 36 40 ]

POC   33 TId: 4 ( B-SLICE, QP 29 )  [L0 32 34 ] [L1 34 36 ]	  [L0 32 ] [L1 34 36 ]

POC   35 TId: 4 ( B-SLICE, QP 29 )  [L0 34 32 ] [L1 36 40 ]	  [L0 34 32 ] [L1 36 40 ]

POC   38 TId: 3 ( B-SLICE, QP 27 )  [L0 36 32 ] [L1 40 48 ]	  [L0 36 32 ] [L1 40 48 ]

POC   37 TId: 4 ( B-SLICE, QP 29 )  [L0 36 32 ] [L1 38 40 ]	  [L0 36 32 ] [L1 38 40 ]

POC   39 TId: 4 ( B-SLICE, QP 29 )  [L0 38 36 ] [L1 40 48 ]	  [L0 38 36 ] [L1 40 48 ]

POC   44 TId: 2 ( B-SLICE, QP 26 )  [L0 40 32 ] [L1 48 40 ]	  [L0 40 32 ] [L1 48 ]

POC   42 TId: 3 ( B-SLICE, QP 27 )  [L0 40 32 ] [L1 44 48 ]	  [L0 40 32 ] [L1 44 48 ]

POC   41 TId: 4 ( B-SLICE, QP 29 )  [L0 40 32 ] [L1 42 44 ]	  [L0 40 32 ] [L1 42 44 ]

POC   43 TId: 4 ( B-SLICE, QP 29 )  [L0 42 40 ] [L1 44 48 ]	  [L0 42 40 ] [L1 44 48 ]

POC   46 TId: 3 ( B-SLICE, QP 27 )  [L0 44 40 ] [L1 48 44 ]	  [L0 44 40 ] [L1 48 ]

POC   45 TId: 4 ( B-SLICE, QP 29 )  [L0 44 40 ] [L1 46 48 ]	  [L0 44 40 ] [L1 46 48 ]

POC   47 TId: 4 ( B-SLICE, QP 29 )  [L0 46 44 ] [L1 48 46 ]	  [L0 46 44 ] [L1 48 ]

POC   64 TId: 0 ( I-SLICE, QP 19 )   [L0 ] [L1 ]			  [L0 ] [L1 ]

POC   56 TId: 1 ( B-SLICE, QP 23 )  [L0 48 40 ] [L1 64 48 ]	  [L0 48 40 ] [L1 64 ]

POC   52 TId: 2 ( B-SLICE, QP 26 )  [L0 48 40 ] [L1 56 64 ]	  [L0 48 40 ] [L1 56 64 ]

POC   50 TId: 3 ( B-SLICE, QP 27 )  [L0 48 40 ] [L1 52 56 ]	  [L0 48 40 ] [L1 52 56 ]

POC   49 TId: 4 ( B-SLICE, QP 29 )  [L0 48 50 ] [L1 50 52 ]	  [L0 48 ] [L1 50 52 ]

POC   51 TId: 4 ( B-SLICE, QP 29 )  [L0 50 48 ] [L1 52 56 ]	  [L0 50 48 ] [L1 52 56 ]

POC   54 TId: 3 ( B-SLICE, QP 27 )  [L0 52 48 ] [L1 56 64 ]	  [L0 52 48 ] [L1 56 64 ]

POC   53 TId: 4 ( B-SLICE, QP 29 )  [L0 52 48 ] [L1 54 56 ]	  [L0 52 48 ] [L1 54 56 ]

POC   55 TId: 4 ( B-SLICE, QP 29 )  [L0 54 52 ] [L1 56 64 ]	  [L0 54 52 ] [L1 56 64 ]

POC   60 TId: 2 ( B-SLICE, QP 26 )  [L0 56 48 ] [L1 64 56 ]	  [L0 56 48 ] [L1 64 ]

POC   58 TId: 3 ( B-SLICE, QP 27 )  [L0 56 48 ] [L1 60 64 ]	  [L0 56 48 ] [L1 60 64 ]

POC   57 TId: 4 ( B-SLICE, QP 29 )  [L0 56 48 ] [L1 58 60 ]	  [L0 56 48 ] [L1 58 60 ]

POC   59 TId: 4 ( B-SLICE, QP 29 )  [L0 58 56 ] [L1 60 64 ]	  [L0 58 56 ] [L1 60 64 ]

POC   62 TId: 3 ( B-SLICE, QP 27 )  [L0 60 56 ] [L1 64 60 ]	  [L0 60 56 ] [L1 64 ]

POC   61 TId: 4 ( B-SLICE, QP 29 )  [L0 60 56 ] [L1 62 64 ]	  [L0 60 56 ] [L1 62 64 ]

POC   63 TId: 4 ( B-SLICE, QP 29 )  [L0 62 60 ] [L1 64 62 ]	  [L0 62 60 ] [L1 64 ]