| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0114-v1 |
| --- | --- |

| Title: | On slicing and tiling in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA Jianle Chen 2330 Central Expy Santa Clara, CA 95050, USA Maxim Sychev 23, 143441, MKAD 69 km (vneshn.) Moscow, Russia, 143441 Miska M. Hannuksela Hatanpaan valtatie 30, Tampere, Finland | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com +1-858-999-4243 jianle.chen@huawei.com +7-925-008-1124 sychev.maxim@huawei.com miska.hannuksela@nokia.com |
| Source: | Huawei Technologies. Co., Ltd; Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a design of slicing and tiling for VVC. It is asserted that the proposed slicing and tiling design can be summarized as follows:

- Tiling is basically the same as in HEVC, where a picture may be partitioned into tile columns and rows, and each tile covers a rectangular region of the picture.

- Each slice exclusively contains an integer number of complete tiles covering a rectangular region of a picture.

- Each slice is in its own NAL unit.

- The design enables extracting a set of motion-constrained tile sets (MCTSs) from a bitstream to be a conforming sub-bitstream without the need of rewriting slice headers or any other part of the VCL NAL units.

- Signalling of end_of_slice_flag for each CTU is avoided.

Detailed specification text for the proposal, based on JVET-K1001-v5, is provided as an attachment of this contribution.

# Introduction

## Traditional slicing for MTU size matching

As discussed at a JVET meeting session of the 11th JVET in Ljubljana, and as discussed in more detail in JVET-L0110, traditional slicing for maximum transfer unit (MTU) size matching becomes less to not meaningful nowadays. Thanks to the fast progress in networking technologies, transmission loss become less for different networks, including the best-effort, unmanaged Internet. Consequently, nowadays in video conversational applications like video telephony and video conferencing, as well as non-conversational applications like broadcast, there is no more displaying of error-concealed pictures. An error-concealed picture is a decoded picture from a transmitted coded picture for which there is some data loss (e.g., loss of some slices) of the coded picture or a reference picture for at least some part of the coded picture is not error-free (e.g., that reference picture was an error-concealed picture). This is probably due to the following two facts: 1) typically it is difficult to automatically estimate the quality of an error-concealed picture, and 2) nowadays due to scarcity of packet losses passed to the application layer, if a transmission error occurs to a picture, the system can afford to wait for an error-free picture to be decoded and available for displaying without letting the user see too long or too frequent picture freezing.

Therefore, having a slicing design still with MTU size matching as a purpose for VVC would unnecessarily complicate the design and/or unnecessarily cost more bits.

## Tiles in HEVC

HEVC supports tiles, mainly for parallel processing purposes.

Tiles define horizontal and vertical boundaries that partition a picture into tile columns and rows. The scan order of CTUs is changed to be local within a tile (in the order of a CTU raster scan of a tile), before decoding the top-left CTU of the next tile in the order of tile raster scan of a picture. Similar to regular slices, tiles break in-picture prediction dependencies as well as entropy decoding dependencies. However, they do not need to be included into individual NAL units; hence, tiles cannot be used for MTU size matching. Each tile can be processed by one processor/core, and the inter-processor/inter-core communication required for in-picture predictions (for sample values, motion vectors, coding modes, etc.) between processing units decoding neighboring tiles is limited to conveying the shared slice header in case a slice is spanning more than one tile, and loop filtering related sharing of reconstructed samples and metadata. When more than one tile is included in a slice, the entry point byte offset for each tile other than the first one in the slice is signalled in the slice header.

In HEVC, for each slice and tile, either or both of the following conditions must be fulfilled: 1) all coded treeblocks in a slice belong to the same tile; 2) all coded treeblocks in a tile belong to the same slice.

A set of tiles covering a rectangular picture region may be encoded as a motion-constrained tile set (MCTS). For an MCTS, motion vectors are restricted to point to full-sample locations inside the MCTS and to fractional-sample locations that require only full-sample locations inside the MCTS for interpolation, and the usage of motion vector candidates for temporal motion vector prediction derived from blocks outside the MCTS is disallowed. This way, each MCTS may be independently decoded without the existence of tiles not included in the MCTS. MCTSs can be signalled using the temporal MCTSs SEI message.

The latest HEVC specification includes three recently-introduced MCTS-related SEI messages, namely temporal MCTSs SEI message, MCTSs extraction information set SEI message, and MCTSs extraction information nesting SEI message. The temporal MCTSs SEI message indicates existence of MCTSs in the bitstream and signals the MCTSs. The MCTSs extraction information sets SEI message provides supplemental information that can be used in the MCTS sub-bitstream extraction (specified as part of the semantics of the SEI message) to generate a conforming bitstream for an MCTS set. The information consists of a number of extraction information sets, each defining a number of MCTS sets and containing RBSP bytes of the replacement VPSs, SPSs, and PPSs to be used during the MCTS sub-bitstream extraction process. When extracting a sub-bitstream according to the MCTS sub-bitstream extraction process, parameter sets (VPSs, SPSs, and PPSs) need to be rewritten or replaced, and slice headers need to be slightly updated because one or all of the slice address related syntax elements (including first_slice_segment_in_pic_flag and slice_segment_address) typically would need to have different values.

## What slicing and tiling schemes should be included in VVC?

### Slices

As discussed above, it is suggested to get rid of the MTU-size-matching purpose of slices. That means that CTUs in a slice do not need to follow the raster scan order.

Assuming tiles are needed, at least for parallel processing purposes.

Do the CTUs in one slice have to be one or more tiles? Does it make sense to have more slices in a picture than tiles in the picture? To simplify the questions, we can ask ourselves this question: what’s the purpose of having multiple slices in a picture while there is only one tile in a picture? It seems that the only meaningful purpose of this is MTU-size matching, but that is not a good motivation as discussed above. Therefore, as a result of getting rid of the MTU-size-matching purpose of slices, it no longer makes sense to have more slices in a picture than tiles in the picture, and a slice should exclusively contain an integer number of complete tiles. Consequently, all slice boundaries would be tile boundaries.

Then, does it make sense in any scenario for a slice not to contain a rectangular region within a picture? The only slightly useful scenario we could figure out is something like a "background" shape that is a rectangle but with some rectangular holes inside the rectangle. However, such a background shape can be easily represented with a few slices each covering a solid rectangle. Therefore, each slice should be a set of CTUs covering a rectangular region of a picture.

With the above, then slices may be used for the following purposes:

- Parallel processing

  - Same as tiles, and realized by using tiles.

  - When used for parallel processing, the tiles within a slice do not have to be an MCTS.

- Region of interest (ROI) or sub-picture based picture access

  - This purpose is also realized by using tiles.

  - And for this purpose, the tiles within a slice should be an MCTS.

- Excluding the purposes overlapping with tiles, then the only purpose/functionality of slices would only be to encapsulate tiles within a picture into multiple NAL units.

  - Potentially, and purely editorially, if we call the “slice header” as “tile header” or “tile set header”, then the spec does not need to have the term "slice" at all.

Each slice should be in its own NAL unit. Thus one VCL NAL unit is one slice, and vice versa. Note that in HEVC, one VCL NAL unit may contain one slice or a subset of a slice.

### Tiles

The purposes of tiles are parallel processing as well as rregion of interest (ROI) or sub-picture based picture access, which has recently become an important tool for enabling viewport-dependent omnidirectional video delivery optimizations.

There are two possibilities for encapsulation of tiles into NAL units:

- One tile in one NAL unit

  - This then would be one slice.

  - If the tile is motion constrained (i.e., an MCTS containing just one tile), usually it should be in its own NAL unit, to make full use of it being motion constrained, as otherwise sending of just the tile is not possible.

- A set of tiles (covering a rectangle) in one NAL unit

  - This then would also be one slice.

  - Usually an MCTS should be in its own NAL unit, for the same reason as above.

  - Enabling easy extraction of MCTSs into a conforming sub-bitstream, e.g., without the need for rewriting of slice headers would be desirable (no change of slice data of course, and rewriting of SPSs and PPS is always needed).

### Relationship between slices and tiles

Following the above discussions, the relationship between slices and tiles would be as follows:

- A slice may contain one or multiple complete tiles.

- The case of one tile containing multiple slices is disallowed.

# Proposal

The proposed design of slicing and tiling for VVC is summarized as follows:

- Tiling is the basically the same as in HEVC, where a picture may be partitioned into tile columns and rows, and each tile covers a rectangular region of the picture.

- Each slice exclusively contains an integer number of complete tiles covering a rectangular region of a picture. Each slice is in its own NAL unit.

  - In the PPS, the tile IDs of the top-left and the bottom-right tiles in the slice are signalled or inferred.

  - The tile ID of the top-left slice is included in the slice header.

  - The addresses of the CTUs contained in a slice are derived based on derived tile IDs of the tiles contained in the slice and the tile configuration signalled in the PPS.

- The design enables extracting a set of MCTSs from a bitstream to be a conforming sub-bitstream without the need of rewriting slice headers or any other part of the VCL NAL units.

  - The tile IDs may be either derived to be the tile indices, or explicitly signalled in the PPS wherein the tile configuration is signalled.

  - The length, in bits, for explicitly signalled tile IDs in the PPS as well as the top-left and the bottom-right tile IDs in the header, is explicitly signalled in the PPS.

  - Tile ID values do not have to be continuous within a picture; however, they are constrained to be monotonously increasing in the order of tile raster scan of a picture.

- Signalling of the end_of_slice_flag for each CTU is avoided.

Detailed specification text for the proposal, based on JVET-K1001-v5, is provided as an attachment of this contribution.

# Items for further study

## On support of sub-frame-interval delay

It is suggested to study further whether the proposed design is sufficient for real-time very low latency applications with sub-frame-interval delay (e.g. remote controlling of a drone or wireless screen sharing). Such applications could operate according to the following principles:

- They encode each picture as multiple VCL NAL units.

- Each VCL NAL unit is transmitted immediately after its encoding has been completed.

- At the receiving end, each received VCL NAL unit is decoded as soon as possible.

It would good to understand how much additional delay, if any, the determination of slice boundaries "upfront" in the PPS could cause. If the additional delay is remarkable, an operation mode where the encoder can determine flexibly whether to “close” a slice at the end of each tile (and send the NAL unit immediately) could be reasonable.

## On signalling of entry point offsets

The need for entry point offsets should be carefully considered. Currently, in the proposal, entry point offsets are mandatorily present in multi-tile slices, as in HEVC. However, signalling of entry point offsets would be unnecessary if those tiles are not intended to be decoded in parallel. It is therefore suggested to study whether there are scenarios where multiple tiles are included in one slice but the multiple tiles are not supposed to be decoded in parallel. If so, then signalling of entry point offsets should be made optional.

One way to avoid using entry point offsets is to include only one tile in each slice. Provided that the slice header byte count is small, it might not be justified to keep entry point offsets as a mechanism for decoder parallelization. The byte count overheads of both approaches should be studied further.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd and Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).