| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0117-v1 |
| --- | --- |

| Title: | CE5: Binary arithmetic coding range update with small table or short multiplications (Test 5.2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Jie Dong, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin | Tel: Email: | +1-408-533-9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results for Test CE5.2.4, where small multiplication tables are proposed to efficiently compute the range sub-intervals for arithmetic coding. Specifically, the performances of 16×16, 8×16, and 8×8 tables are tested on CEM1 (i.e., the test model specified by CE5 [1]), which uses a large 512×64 table.  The proposed tables reduce the number of memory and allow efficient computations for range sub-intervals, while achieving minor gains. Under the CE5 test conditions [1], the average coding gains are:

with 16×16 table:  -0.05% AI, -0.03% RA, and -0.04% LDB;

with 8×16 table:  -0.04% AI, -0.02% RA, and -0.03% LDB;

with 8×8 table:  -0.02% AI, -0.00% RA, and -0.02% LDB.

# Introduction

In this experiment, before the computation of the interval sub-division, the probabilities are scaled using the integer function, as shown in (1),

z(P) = floor(log2(P))                                                                (1)

which can be computed efficiently with “bit-scan-reverse” instructions, simple hardware, or table look-up, using the same table employed for renormalization of the arithmetic coding interval.

When the probability estimate is scaled by 2R, and M bits are used for representing probability (after the most-significant 1), and N bits for the range (in the same manner), the operands (or indices for a look-up table) can be obtained as in (3) and (4), respectively.

m(P) = (P >> (z(P) – M)) & (2M – 1)                                                   (3)

n(R) = (R >> (8 – N)) & (2N – 1)                                                     (4)

The range is updated using (5),

RL = ( TN [m(P), n(R)] >> (R – 2 – z(P)) ) | 2                                          (5)

where TN [m(P), n(R)] is obtained by using (6) as below.

TN [m(P), n(R)] = ( ((2M+1 + 1) + 2×m(P)) × ((2N+1 + 1)+2× n(R)) ) >> (M + N – 4)                   (6)

In practice, TN[m, n] can be generated on-the-fly by substituting m(P) and n(R) into (6) or by looking up a pre-defined 2-D table with m(P) and n(R) as the searching indices.

Three different table sizes 16×16, 8×16, and 8×8 are tested.

# Results

The method is integrated into the modified BMS2.0.1 software, called CEM1, where CABACEngine 1 is used, the parameters for double windows are 4 and 7, respectively, and the interval subdivision table size is 512×64. VTM configuration is used. The performances of using interval subdivision tables 16×16, 8×16, and 8×8 are shown in Tables 1, 2, and 3 respectively.

Table 1 Comparison of CEM1 using the proposed 16×16 interval subdivision table against CEM1 anchor

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.12% | -0.08% | 90% | 99% |
| Class A2 | -0.05% | -0.05% | -0.08% | 100% | 103% |
| Class B | -0.05% | -0.05% | -0.05% | 104% | 105% |
| Class C | -0.04% | -0.04% | -0.05% | 108% | 105% |
| Class E | 0.00% | -0.02% | 0.05% | 100% | 101% |
| Overall | -0.05% | -0.06% | -0.04% | 101% | 103% |
| Class D | -0.02% | -0.04% | -0.02% | 103% | 101% |
| Class F (optional) | -0.02% | 0.01% | 0.00% | 105% | 101% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.02% | -0.08% | 105% | 103% |
| Class A2 | -0.01% | 0.04% | -0.02% | 110% | 90% |
| Class B | -0.05% | -0.24% | -0.12% | 114% | 98% |
| Class C | 0.00% | -0.08% | -0.09% | 114% | 100% |
| Overall | -0.03% | -0.09% | -0.08% | 111% | 98% |
| Class D | 0.00% | -0.18% | 0.07% | 116% | 102% |
| Class F (optional) | -0.03% | 0.09% | -0.04% | 124% | 103% |
|  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.11% | -0.22% | -0.29% | 101% | 104% |
| Class C | -0.06% | -0.02% | -0.09% | 107% | 103% |
| Class E | 0.09% | 0.19% | 0.11% | 103% | 94% |
| Overall | -0.04% | -0.05% | -0.12% | 104% | 101% |
| Class D | -0.01% | 0.10% | -0.21% | 101% | 102% |
| Class F (optional) | -0.07% | -0.05% | -0.53% | 105% | 96% |

Table 2 Comparison of CEM1 using the proposed 8×16 interval subdivision table against CEM1 anchor

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.12% | -0.07% | 88% | 85% |
| Class A2 | -0.05% | -0.05% | -0.07% | 114% | 103% |
| Class B | -0.05% | -0.04% | -0.04% | 89% | 97% |
| Class C | -0.04% | -0.03% | -0.04% | 106% | 105% |
| Class E | 0.01% | -0.02% | 0.06% | 101% | 102% |
| Overall | -0.04% | -0.05% | -0.03% | 99% | 98% |
| Class D | -0.01% | -0.04% | -0.01% | 107% | 103% |
| Class F (optional) | -0.01% | 0.03% | 0.01% | 106% | 99% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.03% | -0.07% | 126% | 105% |
| Class A2 | 0.00% | 0.04% | -0.01% | 119% | 98% |
| Class B | -0.04% | -0.23% | -0.11% | 103% | 98% |
| Class C | 0.01% | -0.07% | -0.08% | 113% | 100% |
| Overall | -0.02% | -0.08% | -0.07% | 113% | 100% |
| Class D | 0.02% | -0.17% | 0.08% | 117% | 102% |
| Class F (optional) | -0.02% | 0.10% | -0.02% | 126% | 102% |
|  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.10% | -0.21% | -0.28% | 101% | 106% |
| Class C | -0.05% | -0.01% | -0.08% | 106% | 106% |
| Class E | 0.11% | 0.20% | 0.13% | 112% | 97% |
| Overall | -0.03% | -0.04% | -0.11% | 105% | 103% |
| Class D | 0.00% | 0.12% | -0.20% | 102% | 101% |
| Class F (optional) | -0.06% | -0.04% | -0.52% | 113% | 96% |

Table 3 Comparison of CEM1 using the proposed 8×8 interval subdivision table against CEM1 anchor

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.10% | -0.06% | 99% | 97% |
| Class A2 | -0.03% | -0.03% | -0.05% | 114% | 105% |
| Class B | -0.03% | -0.02% | -0.03% | 90% | 98% |
| Class C | -0.02% | -0.02% | -0.02% | 104% | 109% |
| Class E | 0.02% | 0.00% | 0.07% | 109% | 104% |
| Overall | -0.02% | -0.03% | -0.02% | 102% | 102% |
| Class D | 0.01% | -0.02% | 0.01% | 104% | 104% |
| Class F (optional) | 0.00% | 0.04% | 0.02% | 106% | 98% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.05% | -0.05% | 126% | 106% |
| Class A2 | 0.01% | 0.06% | 0.01% | 128% | 103% |
| Class B | -0.02% | -0.21% | -0.09% | 104% | 100% |
| Class C | 0.02% | -0.06% | -0.07% | 113% | 101% |
| Overall | 0.00% | -0.06% | -0.06% | 115% | 102% |
| Class D | 0.03% | -0.16% | 0.09% | 114% | 103% |
| Class F (optional) | -0.01% | 0.11% | -0.01% | 128% | 101% |
|  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.08% | -0.19% | -0.26% | 101% | 104% |
| Class C | -0.04% | 0.01% | -0.06% | 107% | 106% |
| Class E | 0.12% | 0.21% | 0.14% | 109% | 94% |
| Overall | -0.02% | -0.02% | -0.09% | 105% | 102% |
| Class D | 0.01% | 0.13% | -0.18% | 103% | 102% |
| Class F (optional) | -0.04% | -0.02% | -0.51% | 103% | 97% |

# References

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine,” JVET-K1025, 11th Meeting, Ljubljana, Slovenia, July 2018.

- A. Said, H. Egilmez, Y.-H. Chao, M. Karczewicz, V. Seregin, “CE5: Binary arithmetic coding range update with small table or short multiplications (test B1),” JVET-K0383, 11th Meeting, Ljubljana, Slovenia, July 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).