| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 03–12 Oct. 2018 | Document: JVET-L0125 |
| --- | --- |

| Title: | CE10: Diagonal motion partitions with uni-prediction constraint (Test 10.3.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

This contribution provides the simulation results of CE10.3.3.a and CE10.3.3.b. In this test, the non-rectangular prediction is applied to MERGE and AMVP modes with uni-prediction constraint. In CE10.3.3.a, the non-rectangular prediction with the constraint is applied to MERGE and AMVP modes. The test 10.3.3.a can achieve 0.47% and 0.83% BD-rate gain in random access and low-delay B configuration, respectively. In CE10.3.3.b, the proposed method with the constraint is applied to AMVP mode only. The test 10.3.3.b can achieve 0.27% and 0.49% BD-rate gain in random access and low-delay B configuration, respectively.

# Introduction

The proposed prediction method uses only two non-rectangular partitions, left and right diagonal, for each coding unit (CU), but it can cover the various partitions in geometry motion partition (GEO) [1][2]. The motivation of the proposed prediction method is that binary or ternary split CUs can help represent arbitrary motion partitions. It means that the binary or ternary split CUs can make rectangular shape blocks which have different block width and height compared with only using the quadtree (QT) block structure, and the various angle and distance parameters of GEO can be represented by only two diagonal partitions combined with MTT block structure. Therefore, the proposed non-rectangular prediction can reduce signaling overhead with MTT split syntaxes. The proposed method is applied to MERGE and AMVP modes. In this contribution reports the coding efficiency of CE10.3.3.a and the CE103.3.b [3].

(a) Left diagonal			 (b) Right diagonal

Figure 1. Two diagonal motion partitions in the proposed method

## Test 10.3.3.a: MERGE and AMVP modes with uni-prediction constraint

In worst case, the proposed method needs double memory bandwidth compared to bi-prediction. This memory bandwidth problem occurs because memory cannot be patched with the non-rectangular shape. These are potential problems with mask-based motion compensation methods. It is necessary to find the optimal trade-off between memory bandwidth and performance.

In this test, an additional constraint to reduce increment of memory bandwidth is proposed to solve the memory bandwidth problem. In test 10.3.3.a, the proposed non-rectangular prediction is applied to MERGE and AMVP modes with uni-prediction only. Figure 2 represents an example of uni-prediction constraint for non-rectangular prediction. With this constraint, there is no memory bandwidth increment compared to general bi-prediction of rectangular and square block.

Figure 2. An example of uni-prediction constraint for non-rectangular prediction

## Test 10.3.3.b: AMVP mode with uni-prediction constraint

In test 10.3.3.b, the proposed non-rectangular prediction is only applied to AMVP with uni-prediction constraint. This test reports the coding efficiency of the non-rectangular prediction for AMVP mode only condition.

# Performance evaluation

The proposed methods were evaluated under JVET common test conditions (CTC) [3]. The proposed methods were integrate on BMS-2.0.1 with VTM configuration. Table 1 and Tabe 2 show BD-rate reductions of the test 10.3.3.a and the test 10.3.3.b, respectively.

Table 1. BD-rate reduction of the test 10.3.3.a

|  | Over BMS-2.0.1 with VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.27% | -0.37% | 184% | 104% |  |  |  |  |  |
| Class A2 | -0.41% | -0.65% | -0.72% | 179% | 104% |  |  |  |  |  |
| Class B | -0.34% | -0.52% | -0.61% | 174% | 102% | -0.50% | -0.66% | -0.65% | 173% | 100% |
| Class C | -0.92% | -1.09% | -1.12% | 168% | 102% | -0.95% | -1.17% | -1.05% | 180% | 100% |
| Class E |  |  |  |  |  | -1.21% | 0.22% | 1.58% | 169% | 98% |
| Overall | -0.47% | -0.65% | -0.72% | 175% | 103% | -0.83% | -0.61% | -0.23% | 174% | 99% |
| Class D | -0.62% | -0.73% | -0.86% | 165% | 103% | -0.84% | -0.27% | -0.84% | 171% | 100% |

Table 2. BD-rate reduction of the test 10.3.3.b

|  | Over BMS-2.0.1 with VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | -0.15% | 161% | 102% |  |  |  |  |  |
| Class A2 | -0.26% | -0.49% | -0.53% | 158% | 98% |  |  |  |  |  |
| Class B | -0.18% | -0.35% | -0.35% | 156% | 106% | -0.25% | -0.34% | -0.52% | 156% | 104% |
| Class C | -0.58% | -0.58% | -0.64% | 153% | 101% | -0.55% | -0.75% | -0.79% | 160% | 101% |
| Class E |  |  |  |  |  | -0.83% | 0.28% | 0.75% | 153% | 99% |
| Overall | -0.27% | -0.38% | -0.42% | 157% | 102% | -0.49% | -0.32% | -0.29% | 157% | 102% |
| Class D | -0.45% | -0.53% | -0.47% | 152% | 102% | -0.60% | -0.10% | -0.80% | 159% | 100% |

# References

- Y. Ahn, H. Ryu, and D. Sim, “Diagonal motion partitions on top of QTBT block structure,” JVET-H0087, 8th JVET meeting, Macau, CN, Oct. 2017.

- Y. Ahn and D. Sim, “CE10-related: Diagonal motion partitions on top of MTT block structure,” JVET-K0270, 11th JVET meeting, Ljubljana, SI, July 2018.

- C.-W. Hsu and M. Winken, “Description of core experiment 10: combined and multi-hypothesis prediction,” JVET-K1030, 11th JVET meeting, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).