| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0128-v1 |
| --- | --- |

| Title: | CE1-related: Transform tiling for pipelined processing of large CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, A. Dorrell, Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au andrew.dorrell@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia Pty Ltd. |  |  |

_____________________________

# Abstract

VTM-2.0 introduced a constraint to prohibit performing a ternary split of a coding tree node with a side length exceeding 64. This restriction prevents any TU from spanning a 64x64 grid boundary (in luma, with corresponding constraint in chroma), which may be a unit of granularity for a pipelined architecture to process a frame. The current design performs a TU tiling operation for a CU larger than the available transform sizes. This contribution proposes to extend the tiling operation of TUs within a CU such that smaller TUs are used to prevent the resulting TUs from crossing the above-mentioned 64x64 grid boundary, instead of imposing the ternary split restriction. Coding results in RA are -0.17%, -0.14%, and -0.18% and LDB are -0.15%, -0.23%, and -0.18% in Y, Cb, and Cr channels, respectively.

# Introduction

Implementations of HEVC, with a 64x64 CTU size, generally process data in a pipelined manner with a 32x32 processing region size, reducing on-chip memory requirement to one-quarter the amount that would be required were CTUs stored entirely on-chip. Due to the largest transform size being 32x32 and use of a quadtree split as the only split in the HEVC coding tree, TUs of HEVC are always fully contained within 32x32 regions. For VVC, with a 128x128 CTU size, implementers may wish to perform a similar optimisation, for example with a 64x64 processing region size. However, the presence of ternary splits may result in TUs spanning 64x64 processing region boundaries.

The transform is an operation that needs to be performed within a pipeline stage, due to the nature of internal dependencies within the transform. Other operations, such as intra prediction and inter prediction, may be performed in stages, i.e. a CU spanning multiple pipeline procession regions may be processed in stages, possibly with minor increases in intra-CTU line buffering to hold intermediate results for intra prediction.

VVC Draft 2 [1] introduces a prohibition on ternary splits of blocks with a side length exceeding 64, discussed in review of JVET-K0556 [1]. Although this prohibition is sufficient to enable pipelined architectures, some coding loss is incurred. Table 1 shows coding results from removing the split restriction (using a VTM-2.0rc1 basis, however the coding performance change relative to VTM-2.0.1 is minor).

Table 1. Split restriction coding impact.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0rc1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 107% |
| Class A2 | 0.00% | 0.00% | 0.00% | 104% | 95% |
| Class B | 0.00% | 0.00% | 0.00% | 104% | 113% |
| Class C | 0.00% | 0.00% | 0.00% | 95% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 94% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 105% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0rc1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.51% | -0.46% | 115% | 96% |
| Class A2 | -0.35% | -0.35% | -0.33% | 111% | 96% |
| Class B | -0.22% | -0.24% | -0.16% | 111% | 97% |
| Class C | -0.06% | -0.04% | 0.00% | 111% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.26% | -0.21% | 112% | 95% |
| Class D | -0.04% | 0.02% | 0.02% | 108% | 116% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0rc1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | 0.06% | 0.08% | 114% | 90% |
| Class C | -0.06% | -0.15% | -0.20% | 108% | 102% |
| Class E | -0.47% | -0.12% | -0.25% | 119% | 117% |
| Overall | -0.21% | -0.06% | -0.10% | 113% | 100% |
| Class D | -0.06% | -0.01% | -0.24% | 104% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Description of the proposed method

This contribution proposes to perform a TU tiling operation with a TU sized such that the resulting TUs are each fully contained within the boundaries of the pipeline processing regions. This results in use of smaller TUs that used in VTM-1.0 however ternary splitting is no longer needs restriction. A common case is of a ternary split at the top of the coding tree, resulting in a centre CU of dimensions 64 and 128. In this case, the CU will be tiled with eight 32x32 TUs instead of two 64x64 TUs.

TU tiling is already performed for CUs exceeding available TU sizes, and the approach is extended to select smaller TUs where the larger TU would span the 64x64 pipeline procession region boundary.

# Experimental results

Simulation results under the JVET common test conditions [3] are presented in Table 2.

Table 2. TU tiling for enabling pipeline processing results.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 92% |
| Class E | 0.00% | 0.00% | 0.00% | 93% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 90% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.33% | -0.24% | 118% | 95% |
| Class A2 | -0.27% | -0.31% | -0.31% | 115% | 106% |
| Class B | -0.17% | -0.05% | -0.15% | 114% | 107% |
| Class C | -0.02% | 0.01% | -0.06% | 105% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.14% | -0.18% | 112% | 101% |
| Class D | -0.02% | -0.02% | -0.05% | 107% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.23% | -0.25% | 111% | 100% |
| Class C | -0.03% | -0.04% | -0.06% | 107% | 104% |
| Class E | -0.35% | -0.51% | -0.21% | 114% | 104% |
| Overall | -0.15% | -0.23% | -0.18% | 110% | 102% |
| Class D | -0.04% | 0.19% | -0.45% | 101% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

This input document presents a method to enable pipelined implementations of VVC, while retaining large CU sizes, in particular those spanning pipeline processing boundaries, as may be useful especially at lower bit rates. Coding results in RA are -0.17%, -0.14%, and -0.18% and LDB are -0.15%, -0.23%, and -0.18% in Y, Cb, and Cr channels, respectively.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2)”, JVET-K1001, Ljubljana, SI, July 2018.

- C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1-related: Constraint for binary and ternary partitions”, JVET-K0556, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).