| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0131 |
| --- | --- |

| Title: | CE3-4.1: Harmonization of Linear interpolation intra prediction (LIP) with Simplified position dependent intra prediction combination (PDPC) and wide-angle intra prediction (WAIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Jungah Choi Sunmi Yoo Ling Li Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 jin78.heo@lge.com jangwon84.choi@lge.com jungah.choi@lge.com sunmi.yoo@lge.com l.li@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In JVET-K0090 [1] LGE’s response to the call for proposals (CFP) linear interpolation intra prediction (LIP) was introduced. In this contribution, we harmonized LIP with simplified position dependent intra prediction combination (PDPC) [2] and wide-angle intra prediction (WAIP) [3]. Simulation results for CE3-4.1 reportedly showed average luma BD-rate savings of 0.21% and 0.14% compared to VTM2.0.1-AI and VTM2.0.1-RA, respectively.

Proposed method

Since simplified PDPC and WAIP were adopted in the last Ljubljana meeting, in this contribution we harmonized LIP with simplified PDPC and WAIP.

LIP is applied on top of intra prediction. An LIP flag for an intra coded CU is signalled at the CU level to indicate whether LIP intra prediction mode is used or not. Moreover, LIP prediction was only applied to angular intra prediction modes except for Planar and DC modes.

The LIP intra prediction method first computes the intensity value of the bottom right (BR) neighboring sample. Then, it generates the intensity values of the bottom row and the right column of the neighboring samples. These samples serve as references on the opposite side of the CU and are derived using the intensity values of the BR neighboring sample and the intensity values of other corner neighboring samples, i.e., the top right neighboring sample, the bottom left neighboring sample.

The first stage of the intra angular mode prediction involves extrapolating samples from a projected reference sample location based on a given direction. The projected reference sample location is computed with bilinear interpolation which is used to obtain the value of the projected reference sample using two closest reference samples located at integer position.

The second stage in the prediction process requires an identical approach to generate another predicted sample value at the same sample location. The second predicted sample value is obtained by projecting the location of the sample onto the opposite side of the reference sample array by applying the selected prediction direction and interpolating the value to 1/32 sample position accuracy. The bilinear interpolation is performed linearly using the two closest opposite side reference samples in the direction of prediction.

The LIP takes advantage of the difference in accuracy between the input intensities in the final interpolation process. Specifically, larger weights are applied to the positions that are closer to more reliable neighboring positions. Final predicted sample value is calculated.

Figure 1 shows intra coding process including LIP, simplified PDPC and WAIP.

Figure 1. Harmonization of LIP, Simplified PDPC and WAIP

Experimental results

The proposed method has been implemented based on BMS2.0.1 reference software and experimentally evaluated for VTM test according to common test conditions defined in JVET-J1010 [4] and Core Experiment description in JVET-J1023 [5].

Table 1 Test 4.1 VTM-AI results

Table 2 Test 4.1 VTM-RA results

Reference

- J. Heo, et al., “CE3: Linear interpolation intra prediction (Tests 7.3.1, 7.3.2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0090, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- G. Van der Auwera, et al., “CE3: Simplified PDPC (Test 2.4.1),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0063, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- F. Racape, et al., “CE3-related: Wide-angle intra prediction for non-square blocks,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0500, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Document of Joint Video Experts Team (JVET), JVET-J1023, 10th Meeting: San Diego, CA, 10–20 April 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).