| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0133 |
| --- | --- |

| Title: | CE 6-2.1: Reduced Secondary Transform (RST) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution, reports test results of CE 6-2.1 on reduced secondary transform (RST). The proposed RST is designed to reduce complexity compared to BMS 4x4 NSST as follows:

- The number of transform kernel is reduced from 103 (BMS NSST) to 4 or 8 (4 transform set and one or two kernels per each transform set).

- The number of worst case multiplications is reduced from 16 (BMS) to 8 by adjusting RST according to block size

The above mentioned RST is tested on top of CE 6.1.1 where MTS candidate selection simplification methods, Simplified_1 and Simplified_2 [2], are considered. The four subtests in CE 6-2.1 can be described as follows:

- CE 6-2.1a: (A) + (B) + (C) + (D) + Simplified_1,

- CE 6-2.1b: (A) + (B) + (C) + Simplified_2,

- CE 6-2.1c: (A) + (D) + Simplified_1,

- CE 6-2.1d: (A) + Simplified_2,  where

| Feature | Description |
| --- | --- |
| (A) | The number of RST candidates to be tried is reduced from 3 to 1. |
| (B) | The number of transform sets is reduced from 35 to 4. |
| (C) | Worst case computational complexity is halved. |
| (D) | Each own transform in a transform set is applied for both cases with MTS flag on and off. |

The experimental results of all CE 6-2.1 subtests are summarized as follows:

As first test (CE 6-2.1a) results, with 94% (AI), 101% (RA), and 100% (LD) encoding time compared to VTM anchor, BD-rate degradation is -1.02% (AI), -0.58% (RA), and -0.28% (LD).

As second test results (CE 6-1.1b), with 124% (AI), 109% (RA), and 105% (LD) encoding time compared to VTM anchor, BD-rate difference is -1.16% (AI), -0.61% (RA), and -0.30% (LD).

As third test (CE 6-1.1c) results, with 92% (AI), 101% (RA), and 101% (LD) encoding time compared to VTM anchor, BD-rate degradation is -1.60% (AI), -0.91% (RA), and -0.40% (LD).

As fourth test results (CE 6-1.1d), with 121% (AI), 109% (RA), and 104% (LD) encoding time compared to VTM anchor, BD-rate difference is -1.75% (AI), -0.93% (RA), and -0.43% (LD).

The number of transforms per transform set is 2 for CE 6-2.1a and CE 6-2.1c, and 1 for CE 6-2.1b and CE 6-2.1d, from which memory usage of CE 6-2.1a accounts to 10KB and that of CE 6-2.1b to 5KB.

Proposed Method

1.1) Reduced Secondary Transform (RST):

The RST is already explained in JVET-K0099 [1]. In this contribution, 16x64 and 16x16 matrices are employed for 8x8 and 4x4 blocks, respectively. For notational convenience, the 16x64 transform is denoted as RST8x8 and the 16x16 one as RST4x4.

1.2) MTS Candidates Selection Simplification:

The Simplified_1 and Simplified_2 firstly shown in the Abstract section are explained briefly as follows:

A) Simplified_1: Using 1 MTS candidate for all modes, DST7 for both horizontal and vertical transform

B) Simplified_2: Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode

1.3) Reduction method of worst case complexity:

If we use RST8x8 and RST4x4, worst case in terms of multiplication count occurs when all TUs consist of 4x4 TU or 8x8 TU. Therefore, top 8x64 and 8x16 matrices (in other words, first 8 transform basis vectors from the top in each matrix) are applied to 8x8 TU and 4x4 TU, respectively.

In the case of blocks larger than 8x8 TU, worst case does not occur because RST8x8 is applied to top-left 8x8 region. For 8x4 TU or 4x8 TU, RST4x4 is applied to only top-left 4x4 region to avoid worst case happening.

With the above worst case handling, the worst case number of multiplications becomes 8 per pixel. Note that BMS 4x4 NSST requires 16 multiplications per pixel.

1.4) Memory usage reduction:

The number of transforms per transform set is reduced from 35 to 4. The memory map with the four transform sets is shown in the following table:

Each transform set consists of 1 or 2 transforms. In the case of CE 6-2.1a and CE 6-2.1c, two transforms are required because transforms when MTS flag is 0 and 1 are different, and in the case of CE 6-2.1b and CE 6-2.1d, only one transform exists in each transform set. Certainly, only one transform is tried during encoding for both cases, which help reduce encoding time dramatically.

In the following table, memory usage requirement for both cases is summarized.

| Config. | RST4x4 | RST8x8 | Total memory usage |
| --- | --- | --- | --- |
| CE 6-2.1a | 2KB (8x16x16) | 8KB (8x16x64) | 10KB |
| CE 6-2.1b | 1KB (4x16x16) | 4KB (4x16x64) | 5KB |

Experimental results

The proposed method has been implemented based on BMS 2.0.1 reference software and experimentally evaluated under VTM configuration according to common test conditions defined in JVET-K1010 [3] and Core Experiment description in JVET-K1026 [4].

2.1) CE 6-2.1a

Anchor is BTM 2.0.1 with VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.46% | -2.15% | -2.09% | 94% | 92% |
| Class A2 | -0.62% | -1.88% | -1.24% | 98% | 96% |
| Class B | -0.77% | -1.87% | -2.62% | 94% | 95% |
| Class C | -1.08% | -1.98% | -2.38% | 91% | 99% |
| Class E | -1.33% | -2.08% | -3.17% | 95% | 96% |
| Overall | -1.02% | -1.98% | -2.34% | 94% | 96% |
| Class D | -0.79% | -1.58% | -2.16% | 92% | 126% |
| Class F (optional) | -1.04% | -1.61% | -1.88% | 94% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.43% | -1.83% | 103% | 100% |
| Class A2 | -0.50% | -1.30% | -1.12% | 100% | 99% |
| Class B | -0.48% | -1.50% | -2.34% | 100% | 100% |
| Class C | -0.46% | -1.19% | -1.36% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.58% | -1.37% | -1.73% | 101% | 99% |
| Class D | -0.32% | -1.10% | -1.47% | 99% | 92% |
| Class F (optional) | -0.39% | -0.95% | -1.35% | 98% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.72% | -1.04% | 101% | 99% |
| Class C | -0.19% | -0.87% | -0.99% | 100% | 98% |
| Class E | -0.53% | -0.99% | -1.47% | 100% | 100% |
| Overall | -0.28% | -0.84% | -1.13% | 100% | 99% |
| Class D | -0.12% | 0.10% | -0.10% | 100% | 96% |
| Class F (optional) | -0.32% | -0.29% | -1.08% | 99% | 98% |

2.2) CE 6-2.1b

Anchor is BTM 2.0.1 with VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.67% | -2.22% | -2.04% | 125% | 95% |
| Class A2 | -0.86% | -1.76% | -1.15% | 127% | 98% |
| Class B | -0.90% | -1.68% | -2.41% | 126% | 98% |
| Class C | -1.10% | -1.67% | -2.20% | 122% | 100% |
| Class E | -1.49% | -2.13% | -3.11% | 123% | 98% |
| Overall | -1.16% | -1.86% | -2.21% | 124% | 98% |
| Class D | -0.87% | -1.41% | -1.88% | 123% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.04% | -1.48% | -1.95% | 112% | 100% |
| Class A2 | -0.54% | -1.38% | -1.08% | 107% | 99% |
| Class B | -0.50% | -1.68% | -2.33% | 109% | 100% |
| Class C | -0.47% | -0.93% | -1.24% | 110% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.61% | -1.38% | -1.71% | 109% | 100% |
| Class D | -0.36% | -1.24% | -1.23% | 108% | 93% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | -0.68% | -1.26% | 106% | 100% |
| Class C | -0.19% | -0.63% | -0.82% | 106% | 100% |
| Class E | -0.55% | -0.76% | -1.81% | 102% | 101% |
| Overall | -0.30% | -0.69% | -1.25% | 105% | 100% |
| Class D | -0.06% | -0.25% | -0.71% | 104% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.3) CE 6-2.1c

Anchor is BTM 2.0.1 with VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.69% | -2.35% | -2.20% | 93% | 92% |
| Class A2 | -0.96% | -2.25% | -1.66% | 97% | 96% |
| Class B | -1.19% | -2.34% | -3.10% | 93% | 94% |
| Class C | -2.24% | -2.69% | -3.17% | 89% | 96% |
| Class E | -1.97% | -2.59% | -3.51% | 92% | 94% |
| Overall | -1.60% | -2.44% | -2.79% | 92% | 95% |
| Class D | -1.76% | -2.17% | -2.97% | 90% | 93% |
| Class F (optional) | -1.98% | -2.06% | -2.30% | 91% | 96% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.15% | -1.50% | -1.88% | 104% | 100% |
| Class A2 | -0.72% | -1.60% | -1.45% | 101% | 99% |
| Class B | -0.74% | -1.87% | -2.88% | 101% | 101% |
| Class C | -1.09% | -1.32% | -1.75% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.91% | -1.60% | -2.09% | 101% | 100% |
| Class D | -0.78% | -1.42% | -1.60% | 100% | 94% |
| Class F (optional) | -1.02% | -1.22% | -1.48% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -0.84% | -0.89% | 101% | 99% |
| Class C | -0.46% | -0.68% | -0.96% | 100% | 99% |
| Class E | -0.49% | -0.87% | -2.10% | 100% | 100% |
| Overall | -0.40% | -0.79% | -1.22% | 101% | 99% |
| Class D | -0.21% | -0.47% | -0.77% | 100% | 96% |
| Class F (optional) | -0.36% | -0.44% | -0.53% | 100% | 98% |

2.4) CE 6-2.1d

Anchor is BTM 2.0.1 with VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.94% | -2.46% | -2.33% | 123% | 94% |
| Class A2 | -1.25% | -2.24% | -1.67% | 125% | 98% |
| Class B | -1.37% | -2.30% | -3.05% | 123% | 96% |
| Class C | -2.15% | -2.33% | -2.95% | 117% | 97% |
| Class E | -2.15% | -2.77% | -3.60% | 119% | 95% |
| Overall | -1.75% | -2.40% | -2.77% | 121% | 96% |
| Class D | -1.80% | -2.03% | -2.82% | 119% | 93% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | -1.67% | -2.07% | 113% | 102% |
| Class A2 | -0.76% | -1.49% | -1.40% | 108% | 100% |
| Class B | -0.79% | -1.99% | -2.85% | 109% | 100% |
| Class C | -1.04% | -1.31% | -1.76% | 108% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.93% | -1.65% | -2.11% | 109% | 99% |
| Class D | -0.76% | -1.22% | -1.53% | 107% | 91% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.31% | -0.80% | -1.13% | 105% | 100% |
| Class C | -0.38% | -0.58% | -1.03% | 105% | 99% |
| Class E | -0.70% | -1.44% | -1.88% | 102% | 103% |
| Overall | -0.43% | -0.89% | -1.28% | 104% | 100% |
| Class D | -0.28% | -0.34% | -1.02% | 104% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.4) Requested tests

Tests without normative change of MTS signaling, i.e. only “a+b+c” under CTC to assess the runtime versus performance benefit of NSST standalone, are requested during 12th JVET meeting, where

- Use one transform kernel per transform set

- Use only 4 transform set

- Apply worst case handling method to reduce the number of multiplications

Anchor is BTM 2.0.1 with VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.63% | -2.25% | -2.14% | 137% | 93% |
| Class A2 | -0.84% | -1.73% | -1.13% | 139% | 97% |
| Class B | -0.87% | -1.69% | -2.45% | 140% | 95% |
| Class C | -1.07% | -1.61% | -2.07% | 138% | 101% |
| Class E | -1.40% | -2.17% | -3.09% | 139% | 96% |
| Overall | -1.13% | -1.85% | -2.20% | 139% | 96% |
| Class D | -0.84% | -1.37% | -1.79% | 139% | 96% |
| Class F (optional) | -0.93% | -1.47% | -1.63% | 137% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.01% | -1.50% | -1.97% | 114% | 97% |
| Class A2 | -0.51% | -1.40% | -0.95% | 109% | 97% |
| Class B | -0.49% | -1.68% | -2.19% | 112% | 98% |
| Class C | -0.48% | -0.87% | -1.23% | 114% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.59% | -1.37% | -1.64% | 113% | 98% |
| Class D | -0.33% | -0.80% | -1.11% | 112% | 93% |
| Class F (optional) | -0.46% | -0.77% | -1.03% | 116% | 98% |

2.4.1) Two kernels per set (CE 6-2.1a without normative change of MTS signaling)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.40% | -2.23% | -2.09% | 95% | 92% |
| Class A2 | -0.54% | -1.73% | -1.17% | 98% | 96% |
| Class B | -0.72% | -1.81% | -2.51% | 96% | 96% |
| Class C | -1.02% | -1.91% | -2.39% | 93% | 97% |
| Class E | -1.26% | -2.01% | -3.08% | 95% | 97% |
| Overall | -0.96% | -1.92% | -2.28% | 95% | 96% |
| Class D | -0.70% | -1.57% | -2.03% | 92% | 94% |
| Class F (optional) | -0.99% | -1.64% | -1.85% | 94% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.95% | -1.46% | -1.78% | 103% | 99% |
| Class A2 | -0.45% | -1.35% | -1.12% | 100% | 98% |
| Class B | -0.43% | -1.52% | -2.05% | 101% | 99% |
| Class C | -0.35% | -1.09% | -1.22% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -1.36% | -1.59% | 101% | 99% |
| Class D | -0.22% | -1.36% | -1.19% | 100% | 93% |
| Class F (optional) | -0.36% | -1.00% | -1.13% | 98% | 99% |

Conclusion

In the proposed RST, it is reportedly observed that BD-rate/encoding time/decoding time was -1.02%/94%/96% (AI), -0.58%/101%/99% (RA), and -0.28%/100%/99% (LD) for the configuration of one RST candidate per transform set and Simplified_1 along with MTS flag dependent RST selection and worst case complexity handling. Also, -1.16%/124%/98% (AI), -0.61%/109%/100% (RA), and -0.30%/105%/100% (LD) were achieved for the configuration of one RST candidate per transform set and Simplified_2 along with worst case complexity handling. The memory usage of the former configuration is 10KB, and that of the latter 5KB. As reported, substantial BD-rate is achieved with reasonable memory cost and no increase or small increase of encoding time. It is strongly recommended that the proposed RST should be adopted in the next VTM.

Reference

- M. Salehifar et al., “CE 6.2.6: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0099, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- M. Salehifar et al., “CE 6-1.1 (a,b): Selection of MTS Candidates”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-LXXXX, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- J. Boyce et al., “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1026, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).