| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0135-v1 |
| --- | --- |

| Title: | CE6: Further simplification of AMT with adjustment stages (Test CE6.1.6b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Victorien Lorcy Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | b<>com, Orange |  |  |

_____________________________

# Abstract

This contribution reports a method for complexity reduction of the DST7/DCT8 kernels for AMT. The kernels are approximated using a low complexity adjustment stage, implemented as a sparse matrix, used as a preprocessor to the HEVC partial butterfly DCT2 algorithm.

The results show approximately 3% and 12% reduction for encoding and decoding time with 0.06% Y BD-rate loss against VTM 2.0.1 anchor in AI configuration.

# Outline of the proposal

This proposal is a continuation of the proposals JVET-J0066, JVET-K0272 and JVET-K0299 [1,2,3]. It is proposed to approximate DCTs and DSTs with an adjustment stage followed by a DCT2 (or inverse DCT3).

A forward transform is approximated by the combination of a block band matrix and a transform from the DCT2 family (DCT3, DST2 and DST3) which is implemented with a partial butterfly algorithm (as for HEVC).

The key-features of the proposals are:

- The HEVC DCT2 partial butterfly algorithm is reused

- The adjustment matrix is orthogonal and can be symmetric, in this proposal the size-16 and size-32 adjustment matrices require overall the storage of 167 coefficients using 9 bits. The DST7, DCT8 and their inverse transforms use the same coefficients.

- The adjustment method can approximate any kernel. Implementations were provided in K0299 and K0375 for DST1, DST4, DST7, DCT1, DCT4, DCT5, DCT8.

- The method is applicable to any transform sizes (sizes 16, 32 and 64 were demonstrated).

- The adjustment stage can be efficiently implemented using SIMD

- As such this method can further apply for subsequent evolution of the test model.

The coding results are presented and can be summarized as follows:

All Intra configuration

|  | Y | U | V | Enc | Dec |
| --- | --- | --- | --- | --- | --- |
| Compared to VTM | 0.06% | 0.04% | 0.07% | 97% | 88% |
| Compared to BMS | 0.06% | 0.03% | 0.03% | 96% | 90% |

Random Access configuration

| Compared to VTM | 0.05% | 0.05% | 0.07% | 99% | 98% |
| --- | --- | --- | --- | --- | --- |
| with inter transforms enabled | 0.03% | 0.03% | 0.06% | 96% | 96% |
| Compared to BMS | 0.03% | 0.00% | 0.04% | 96% | 97% |

Low Delay Configuration

| Compared to VTM | -0.02% | -0.07% | -0.08% | 99% | 100% |
| --- | --- | --- | --- | --- | --- |
| with inter transforms enabled | 0.00% | 0.00% | 0.05% | 96% | 96% |
| Compared to BMS | -0.03% | -0.22% | 0.14% | 96% | 96% |

# Implementation

The code base is shared with CE6.1.6a but this proposal provides further simplifications:

- the adjustment matrices used for the forward and inverse transform contain a maximum of 5 taps per row.

- symmetries are imposed for the size-32 adjustment matrix: instead of storing 5 diagonals (see below), 3 diagonals are actually stored, the storage requirements for the coefficients is consequently 93 coefficients (on 9 bits). For size-16, 74 coefficients are required.

Figure 1: The size-32 adjustment matrix, a maximum of 5 significative values per row/column is required, the upper diagonals deduce from the lower ones (with sign inversion for the first upper diagonal). As such, 32+31+30=93 coefficients are stored.

- the DST7 and DCT8 use the same adjustment matrices with reverse row order (DCT8 and DST7 being the dual of each other, i.e. reversed rows and sign changes)

Applying the 32-sized adjustment matrix can be efficiently done with SIMD instructions:

for(k=0;k<M    ;k++)  yk =d0,k*xk;

for(k=0;k<M-1  ;k++) {yk+=d1,k*xk+1; yk+1-=d1,k*xk;}

for(k=0;k<M-2  ;k++) {yk+=d2,k*xk+2; yk+2+=d2,k*xk;}

yk denotes the adjusted signal, xk is the input. d0,k, d1,k and d2,k are the diagonals of the adjustment matrix, respectively, the main, the first upper diagonal and second upper diagonal.

The complexity of the adjustment stage is as follows.

| Transform dimension | N = 16 | N = 32 |
| --- | --- | --- |
| Memory | 666 bits | 837 bits |
| Multiplications | 74 | 128 |
| Additions | 90 | 186 |
| Shift | 16 | 32 |

DCT2 and DCT3 with partial butterfly are also used.

# Coding results

The results are presented according to the common test conditions with specific requirements established for the Core Experiments on Transforms and Transform Signaling.

## Configuration VTM in BMS2.0.1

### All Intra

|  | Over BMS 2.0.1 in vtm cfg |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.05% | 0.04% | 96% | 84% |
| Class A2 | 0.12% | 0.05% | 0.04% | 97% | 87% |
| Class B | 0.07% | 0.08% | 0.07% | 97% | 88% |
| Class C | 0.01% | 0.05% | 0.21% | 98% | 93% |
| Class E | 0.05% | 0.05% | -0.08% | 97% | 90% |
| Overall | 0.06% | 0.04% | 0.07% | 97% | 88% |
| Class D | 0.00% | 0.22% | -0.14% | 98% | 95% |
| Class F (optional) | 0.00% | -0.06% | -0.04% | 98% | 95% |

### Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in vtm cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.00% | 98% | 97% |
| Class A2 | 0.07% | 0.06% | 0.06% | 99% | 99% |
| Class B | 0.08% | 0.02% | 0.09% | 98% | 98% |
| Class C | 0.02% | 0.12% | 0.11% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.05% | 0.07% | 99% | 98% |
| Class D | 0.02% | -0.35% | 0.07% | 99% | 100% |
| Class F (optional) | -0.01% | 0.16% | -0.10% | 99% | 100% |

### Low Delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in vtm cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.10% | -0.07% | 99% | 99% |
| Class C | 0.04% | -0.19% | -0.19% | 99% | 100% |
| Class E | -0.16% | -0.19% | 0.03% | 99% | 100% |
| Overall | -0.02% | -0.07% | -0.08% | 99% | 100% |
| Class D | -0.02% | 0.45% | 0.40% | 99% | 100% |
| Class F (optional) | -0.10% | 0.11% | -0.44% | 101% | 100% |

## Configuration VTM in BMS2.0.1 with EMT=3

In the vtm configuration and when EMT is allowed for inter coding mode (EMT=3).

### Random Access

Presented with VTM2 as anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in vtm cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.29% | 0.42% | 116% | 97% |
| Class A2 | -0.50% | 0.33% | 0.43% | 117% | 99% |
| Class B | -0.37% | 0.41% | 0.39% | 120% | 99% |
| Class C | -0.24% | 0.76% | 0.50% | 126% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | 0.46% | 0.43% | 120% | 99% |
| Class D | -0.38% | 0.08% | 0.47% | 126% | 100% |
| Class F (optional) | -0.10% | 0.37% | 0.10% | 115% | 99% |

And with VTM2 / EMT=3 as anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in vtm cfg EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | 0.04% | 96% | 96% |
| Class A2 | 0.06% | 0.03% | 0.14% | 97% | 97% |
| Class B | 0.03% | 0.01% | 0.03% | 96% | 96% |
| Class C | 0.03% | 0.17% | 0.06% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.03% | 0.06% | 96% | 96% |
| Class D | -0.03% | -0.40% | 0.07% | 97% | 98% |
| Class F (optional) | 0.01% | 0.13% | -0.10% | 97% | 98% |

### Low Delay B

Presented with VTM2 as anchor:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in vtm cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.76% | 0.66% | 0.74% | 129% | 100% |
| Class C | -0.52% | 0.88% | 0.82% | 134% | 101% |
| Class E | -0.39% | 0.76% | 0.57% | 118% | 101% |
| Overall | -0.59% | 0.76% | 0.72% | 128% | 101% |
| Class D | -0.62% | 1.28% | 1.10% | 134% | 100% |
| Class F (optional) | -0.58% | 1.08% | 0.27% | 127% | 101% |

And with VTM2 / EMT=3 as anchor:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in vtm cfg EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.11% | 0.18% | 96% | 95% |
| Class C | -0.01% | 0.20% | 0.06% | 97% | 96% |
| Class E | -0.01% | -0.07% | -0.19% | 97% | 98% |
| Overall | 0.00% | 0.00% | 0.05% | 96% | 96% |
| Class D | -0.11% | 0.24% | 0.60% | 97% | 97% |
| Class F (optional) | -0.24% | 0.19% | -0.40% | 97% | 98% |

## BMS

Running the BMS_2.0.1 software in BMS configuration leads to the following numbers:

### All Intra

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in bms cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.05% | 0.10% | 94% | 85% |
| Class A2 | 0.12% | 0.00% | 0.03% | 96% | 89% |
| Class B | 0.06% | 0.02% | 0.03% | 96% | 89% |
| Class C | 0.04% | 0.10% | 0.03% | 97% | 94% |
| Class E | 0.04% | -0.01% | -0.02% | 96% | 91% |
| Overall | 0.06% | 0.03% | 0.03% | 96% | 90% |
| Class D | 0.02% | -0.20% | 0.02% | 97% | 96% |
| Class F (optional) | 0.01% | -0.03% | -0.01% | 96% | 96% |

### Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in bms cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.13% | 95% | 96% |
| Class A2 | 0.03% | 0.06% | -0.09% | 96% | 97% |
| Class B | 0.04% | 0.10% | 0.10% | 96% | 97% |
| Class C | 0.01% | -0.07% | 0.00% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | 0.04% | 96% | 97% |
| Class D | -0.02% | 0.53% | 0.25% | 98% | 98% |
| Class F (optional) | -0.01% | -0.11% | -0.15% | 97% | 98% |

### Low Delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 in bms cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.16% | 0.17% | 95% | 95% |
| Class C | -0.04% | -0.23% | -0.07% | 96% | 96% |
| Class E | -0.09% | -0.33% | 0.36% | 97% | 97% |
| Overall | -0.03% | -0.22% | 0.14% | 96% | 96% |
| Class D | 0.00% | 0.10% | 1.01% | 98% | 97% |
| Class F (optional) | -0.04% | -0.10% | -0.29% | 96% | 98% |

# Conclusion

This contribution is proposed for adoption in the next release of VVC draft specification, as it reduces the complexity of the EMT transform stage.

The VTM2 EMTs are computed through an adjustment stage combined with a DCT2 transform, the latter being implemented using a partial butterfly implementation. Consequently, existing hardware architecture can be reused for the DCT2 stage.

# References

- JVET-J0066, Complexity Reduction for Adaptive Multiple Transforms (AMT) using Adjustment Stages, A. Said et al.

- JVET-K0272, CE6.1.2: Efficient Implementations of AMT with Transform Adjustment Stages, A. Said et al.

- JVET-K0299, CE6-related: Further simplification for AMT complexity reduction (CE6.1.2), P. Philippe et al.

- JVET-K0375, CE6.1.1: Extended AMT, A. Said et al.

# Patent rights declaration(s)

b<>com, Orange may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: adjustment matrices

The arrays below represent the required storage to represent the adjustment matrices (stored in Rom.cpp):

For the forward, inverse DCT8/DST7 of size 16,

const TMatrixCoeff m_Adj_dst7_16[74] = {

256, 256, 255, 254, 253, 252, 251, 250, 248, 246, 244, 241, 239, 237, 232, 242
     7,  12, 19,  25,  29,  34,  37,  41,  48,  52,  57,  64,  64,  71,  84,
    -7, -12,-19, -25, -29, -34, -37, -41, -48, -52, -57, -63, -64, -69, -81,
     0,  0,  -1,   0,   1,   1,   3,   3,  5,   5,   7,   5,  13,   3,
     0,  1,   3,   3,   3,   4,   3,   5,  5,   7,   8,  12,   6,  21

};

For the forward, inverse DCT8/DST7 of size 32,

const TMatrixCoeff m_Adj_dst7_32[93] = {

256, 256, 256, 256, 255, 255, 255, 254, 254, 253, 252, 252, 252, 251, 250, 250,
	249, 248, 247, 246, 245, 244, 244, 242, 240, 240, 238, 236, 235, 233, 230, 242,
 	  4,  6,  8, 11, 13, 17, 20, 22, 24, 29, 32, 31, 34, 38, 39, 
	 39, 43, 47, 49, 51, 54, 54, 55, 62, 62, 63, 68, 70, 73, 75, 83,
	  0, 0, 0, 0, 0, 1, 1, 1, 1, 2, 2, 2, 3, 3, 3, 3,
     4, 5, 5, 6, 6, 6, 7, 8, 8, 9, 10, 11, 12, 13

};