| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0150 |
| --- | --- |

| Title: | CE3: Multiple reference line prediction (Test 1.2.3 and Test 1.2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents test results of CE3-1.2.3 and CE3-1.2.4 described in JVET-K1023. CE3-1.2 presents multiple reference line based intra prediction which is averaging two reference lines with weights. In CE3-1.2.3 and CE3-1.2.4, weight values of 3 and 1 are applied to nearest and extended reference sample line, respectively. In addition, reference sample smoothing is not applied to extended reference line in CE3-1.2.4. It is reported that the average luma BD-rate gain for AI case of CE3-1.2.3 and CE3-1.2.4 are 0.18% and 0.19% compared to VTM 2.0, respectively

- Algorithm description

In JVET-K0166[1], a multiple reference line based intra prediction is proposed and included in CE3[2]. In CE3-1.2.3 and CE3-1.2.4, two reference sample lines are used for prediction by averaging the two reference sample lines with weights. The weight values are 3 and 1 for the nearest and extended reference sample line, respectively as depicted in Fig. 1. The extended reference line is enabled for angular intra prediction modes except vertical and horizontal modes. On the other hand, the extended reference line is disabled for small block sizes and when the current block is located at the top in a CTU. In addition, reference sample smoothing process is not applied to the extended reference line in CE3-1.2.4.

Figure 1. An example of multiple reference line based intra prediction

Experimental results

The experiments were performed based on BMS 2.0.1 software [3] with VTM 2.0.1 configuration file under JVET common test condition [4].

Table 1 and Table 2 show experimental results of the CE3-1.2.3 and CE3-1.2.4 compared to VTM 2.0.1. The luma BD-rate gain of 0.18% and 0.19 for AI case are observed for the CE3-1.2.3 and CE3-1.2.4, respectively.

Table 1. Results of CE3-1.2.3 over VTM 2.0.1

Table 2. Results of CE3-1.2.4 over VTM 2.0.1

Conclusions

This contribution reports the experimental result of CE3-1.2.3 and CE3-1.2.4. In the tests, multiple reference lines are used for prediction by averaging two reference lines. The experimental results reportedly show that the average luma BD-rates gains of CE3-1.2.3 and CE3-1.2.4 are 0.18% and 0.19% for AI case compared to VTM 2.0.1, respectively.

Reference

- J. Lee, H. Lee, S.-C. Lim, J. Kang, and H. Y. Kim, “CE3: Multi-line based intra prediction (Test 5.3.1, 5.3.2, 5.3.3),” JVET-K0166, July. 2018

- G. Van der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-K1023, July 2018.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, July 2018.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).