| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0151 |
| --- | --- |

| Title: | CE3: Intra reference sample interpolation (Test 3.1.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents test results of CE3-3.1.3 described in JVET-K1023. CE3-3.1.3 presents interpolation filter selection between 4-tap cubic filter and 6-tap Gaussian filter based on intra prediction mode and block size. The 6-tap Gaussian filter is generated by convolving 4-tap Gaussian filter and 3-tap [1/4, 2/4, 1/4] filter. Reference sample smoothing process which is MDIS is removed, but MDIS on/off condition is used for the filter selection. It is reported that the average luma BD-rate gain for AI and RA cases are 0.44% and 0.18% compared to VTM 2.0.1.

- Algorithm description

In JVET-K0165[1], a 6-tap combined filter (CF) for intra prediction is proposed and included in CE3[2]. The 6-tap filter combines two filters which are 3-tap [1/4, 2/4, 1/4] reference sample smoothing filter and 4-tap Gaussian filter. Therefore, the 6-tap interpolation filter is used for intra angular prediction, and the reference sample smoothing process is removed. For the prediction, a 4-tap cubic filter and the 6-tap Gaussian filter is selected according to a condition according to intra prediction mode and block size. The condition is same as the MDIS on/off condition in VTM 2.0 with modified threshold values considering wide angle prediction modes as described in Table 1. When the minimum distance between an intra prediction mode of current block and horizontal or vertical mode is greater than the threshold value, the 6-tap filter is applied. Otherwise, the 4-tap filter is used for intra prediction. The threshold value ‘30’ means the 4-tap filter is always used, and ‘0’ means the 6-tap filter is always used regardless of intra prediction mode. Filter coefficients of the 4-tap and 6-tap filter are represented in Table 2.

Table 1. Threshold values for distance between an intra prediction mode and horizontal/vertical mode

|  | nTbS = 1 | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 30 | 30 | 14 | 2 | 0 | 0 |

Table 2. 4-tap cubic and 6-tap Gaussian filters

| Pel(x/32) | 4-tap cubic | 6-tap Gaussian |
| --- | --- | --- |
| 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 | { 0, 256, 0, 0 } { -3, 252, 8, -1 } { -5, 247, 17, -3 } { -7, 242, 25, -4 } { -9, 236, 34, -5 } { -10, 230, 43, -7 } { -12, 224, 52, -8 } { -13, 217, 61, -9 } { -14, 210, 70, -10 } { -15, 203, 79, -11 } { -16, 195, 89, -12 } { -16, 187, 98, -13 } { -16, 179, 107, -14 } { -16, 170, 116, -14 } { -17, 162, 126, -15 } { -16, 153, 135, -16 } { -16, 144, 144, -16 } | { 0, 256, 512, 256, 0, 0 } { 43, 247, 416, 264, 53, 1 } { 40, 240, 414, 270, 58, 2 } { 37, 233, 413, 277, 62, 2 } { 34, 226, 412, 284, 66, 2 } { 31, 218, 410, 292, 71, 2 } { 28, 210, 407, 299, 77, 3 } { 26, 203, 404, 306, 82, 3 } { 23, 195, 401, 313, 88, 4 } { 21, 188, 398, 320, 93, 4 } { 19, 180, 393, 327, 100, 5 } { 17, 173, 389, 333, 106, 6 } { 16, 167, 385, 339, 111, 6 } { 14, 159, 380, 346, 118, 7 } { 13, 153, 375, 351, 124, 8 } { 11, 145, 370, 358, 131, 9 } { 10, 138, 364, 364, 138, 10 } |

Experimental results

The experiments were performed based on BMS 2.0.1 software [3] with VTM 2.0.1 configuration file under JVET common test condition [4].

Table 3 shows experimental results of the CE3-3.1.3 compared to VTM 2.0.1. The luma BD-rate gain of 0.44% for AI case and 0.18% for RA case are observed.

Table 3. Results of CE3-3.1.3 over VTM 2.0.1

Conclusions

This contribution reports the experimental results of CE3-3.1.3. In the test, 4-tap filter and 6-tap filter is selected according to intra prediction mode and block size. In addition, the reference sample construction is simplified by removing the reference sample smoothing process. The experimental results reportedly show that the average luma BD-rates gain of 0.44% and 0.18% for AI and RA cases compared to VTM 2.0.1, respectively.

Reference

- J. Lee, H. Lee, S.-C. Lim, J. Kang, and H. Y. Kim, “CE3: Combined filter (Test 2.6.1),” JVET-K0165, July. 2018

- G. Van der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-K1023, July 2018.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, July 2018.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).