| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0152 |
| --- | --- |

| Title: | CE3-related: Simplification of PDPC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents modification of position dependent intra prediction combination (PDPC). It includes three modifications which are block size restriction, removal of interpolation, and restriction on wide angle intra prediction modes. It is reported that the average luma BD-rate gain of all the modifications for AI case is x.xx% compared to VTM 2.0.1.

- Algorithm description

The position dependent intra prediction combination (PDPC)[1] is applied to intra predicted samples for all block sizes when an intra prediction mode is one of DC, Planar, horizontal/vertical, angular mode less than mode 11, and angular mode greater than mode 57.

In this contribution, three simplifications of PDPC such as block size restriction, removal of interpolation, and restriction on wide angle intra prediction modes are proposed to reduce complexity.

  - Block size restriction

The PDPC applies filters to intra predicted samples using reference samples with weights. Therefore, each predicted sample is refined by applying higher weight to closer reference sample. However, the PDPC may not perform efficiently for large blocks since the predicted samples are similar to the reference samples. It could reduce complexity without loss by restricting PDPC on large blocks, so the proposed method restricts PDPC when block width or height is 64. As a result, the maximum block size for PDPC is 32x32.

  - Removal of interpolation

For angular prediction, bilinear interpolation filter is applied to reference samples to generated prediction samples. For example, above two reference samples are interpolated for prediction when a vertical angular prediction mode is used as depicted in Fig. 1. In addition, left two reference samples are also interpolated in PDPC. To reduce computational complexity, an integer reference sample is used for PDPC rather than applying the interpolation in this simplification.

Figure 1. Interpolation for intra prediction and PDPC in VTM 2.0

  - Restriction on wide angle intra prediction modes

In VTM 2.0, wide angle intra prediction (WAIP) modes are used when a ratio of non-square block and an intra prediction mode are satisfied with a certain condition. By harmonizing PDPC with WAIP, the PDPC is also applied to the WAIP modes. However, the PDPC reflecting higher weight to closer reference sample may not be efficient since the references samples for prediction are already replaced with closer reference sample by the WAIP. In this modification, the PDPC is simply disabled for the WAIP modes.

Experimental results

The experiments were performed based on BMS 2.0.1 software [2] with VTM 2.0.1 configuration file under JVET common test condition [3].

Below Tables show experimental results of the each proposed method compared to VTM 2.0.1. The average luma BD-rate gain for AI case of all the proposed methods is x.xx% as represented in Table 4.

Table 1: T1. Block size restriction

Table 2: T2. Removal of interpolation

Table 3: T3. Restriction on WAIP modes

Table 4: T4. T1 + T2 + T3

Table 1. Results of T1 over VTM 2.0.1

Table 2. Results of T2 over VTM 2.0.1

Table 3. Results of T3 over VTM 2.0.1

Table 4. Results of T4 over VTM 2.0.1

Conclusions

This contribution reports three simplifications of PDPC to reduce complexity. The average luma BD-rates is almost same when all the proposed methods are applied, and encoding/decoding time is slightly decreased. We suggest the proposed methods to be adopted into VTM 3.0.

Reference

- G. Van der Auwera, V. Seregin, A. Said, A. K. Ramasubramonian, and M. Karczewicz, “CE3: Simplified PDPC (Test 2.4.1),” JVET-K0063, July. 2018

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, July 2018.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex - Decoding process (Based on JVET-K1001-v5)

Block size restriction

8.2.4.2.1	General intra sample prediction

…

When nTbW is less than MaxTbSizeY, nTbH is less than MaxTbSizeY, and one of the following conditions is true, the position-dependent prediction sample filtering process specified in clause 8.2.4.2.9 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is equal to INTRA_ANGULAR18

- predModeIntra is equal to INTRA_ANGULAR50

- predModeIntra is less than or equal to INTRA_ANGULAR10

- predModeIntra is greater than or equal to INTRA_ANGULAR58

Removal of interpolation

8.2.4.2.9	Position-dependent intra prediction sample filtering process

Inputs to this process are:

…

- Otherwise, if predModeIntra is less than or equal to INTRA_ANGULAR10, the following ordered steps apply:

- The variables dXPos[ y ], dXFrac[ y ], dXInt[ y ] and dX[ x ][ y ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dXPos[ y ] =	( ( y + 1 ) * invAngle + 2 )  >>  2
dXFrac[ y ] =	dXPos[ y ] & 63		(8-86)
dXInt[ y ] =	dXPos [ y ]  >>  6
dX[ x ][ y ] =	x + dXInt[ y ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = 0		(8-87)

refT[ x ][ y ] = ( dX[ x ][ y ] < refW − 1 )  ? mainRef[ dX[ x ][ y ] ] : 0                        (8-88)

( ( 64 − dXFrac[ y ] ) * mainRef[ dX[ x ][ y ] ] + 
	dXFrac[ y ] * mainRef[ dX[ x ][ y ] + 1 ] + 32 )  >>  6	(8-88)
	  :  0

wT[ y ] = ( dX[ x ][ y ] < refW − 1 )  ?  32  >>  ( ( y  <<  1 )  >>  nScale )  :  0	(8-89)

wL[ x ] = 0		(8-90)

wTL[ x ][ y ] = 0		(8-91)

- Otherwise, if predModeIntra is greater than or equal to INTRA_ANGULAR58, the following ordered steps apply:

- The variables dYPos[ x ], dYFrac[ x ], dYInt[ x ] and dY[ x ][ y ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dYPos[ x ] =	( ( x + 1 ) * invAngle + 2 )  >>  2
dYFrac[ x ] =	dYPos[ x ] & 63		(8-92)
dYInt[ x ] =	dYPos[ x ]  >>  6
dY[ x ][ y ] =	y + dYInt[ x ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = ( dY[ x ][ y ] < refH − 1 )  ?  sideRef[ dY[ x ][ y ] ] : 0                         (8-93)

( ( 64 − dYFrac[ x ] ) * sideRef[ dY[ x ][ y ] ] + 
	dYFrac[ x ] * sideRef[ dY[ x ][ y ] + 1 ] + 32 )  >>  6	(8-93)
	  :  0

refT[ x ][ y ] = 0		(8-94)

wT[ y ] = 0		(8-95)

wL[ x ] = ( dY[ x ][ y ] < refH − 1 )  ?  32  >>  ( ( x  <<  1 )  >>  nScale )  :  0	(8-96)

wTL[ x ][ y ] = 0		(8-97)

…

Restriction on wide angle intra prediction modes

8.2.4.2.1	General intra sample prediction

…

When one of the following conditions is true, the position-dependent prediction sample filtering process specified in clause 8.2.4.2.9 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is equal to INTRA_ANGULAR18

- predModeIntra is equal to INTRA_ANGULAR50

- predModeIntra is greater than or equal to ANGULAR2 and less than or equal to INTRA_ANGULAR10

- predModeIntra is greater than or equal to INTRA_ANGULAR58 and less than or equal to INTRA_ANGULAR66