| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0156 |
| --- | --- |

| Title: | CE4.2.4 Affine merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin, A. Robert, F. LeLéannec | Tel: Email: | franck.galpin@technicolor.com antoine.robert@technicolor.com fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of the Merge Affine mode described in JVET-K1024 [1] sub-CE 4.2.4, implemented in the JVET VTM-2.0.1. The proposed Merge Affine mode adding constructed affine temporal candidates gives an average luma BD-rate gain of -0.85%, for 11% encoder and 10% decoder complexity increase in RA configuration.

# Introduction

This contribution reports the results of the sub-CE 4.2.4b as described in JVET-K0124 [1], adding new constructed temporal affine merge candidates.

As described in JVET-K0267 [2], it was proposed to add constructed temporal candidates to the merge affine motion vector prediction. The current contribution reports the results of adding these temporal candidates on top of the common base for Merge Affine from sub-CE 4.2.1.

# Description

The implementation was done on top of the provided common base for CE4.2. In this common base, the affine merge candidates are as follows:

| Final number of candidates | Max Number of inherited candidates | Max Number of constructed candidates (4 prms) | Max Number of constructed candidates (6 prms) |
| --- | --- | --- | --- |
| 5 | 4 | 6 | 10 |

Figure 1 Corners for constructed candidates

The constructed spatial candidates for a 6 parameters model are as follows:

| Candidate number | Corner number | Corner name |
| --- | --- | --- |
| 1 | { 0, 1, 2 } | LT, RT, LB |
| 2 | { 0, 1, 3 } | LT, RT, RB |
| 3 | { 0, 2, 3 } | LT, LB, RB |
| 4 | { 1, 2, 3 } | RT, LB, RB |
| 5 | { 0, 1 } | LT, RT |
| 6 | { 0, 2 } | LT, LB |
| 7 | { 1, 2 } | RT, LB |
| 8 | { 0, 3 } | LT, RB |
| 9 | { 1, 3 } | RT, RB |
| 10 | { 2, 3 } | LB, RB |

While for a 4 parameters model, they are as follows:

| Candidate number | Corner number | Corner name |
| --- | --- | --- |
| 1 | { 0, 1 } | LT, RT |
| 2 | { 0, 2 } | LT, LB |
| 3 | { 1, 2 } | RT, LB |
| 4 | { 0, 3 } | LT, RB |
| 5 | { 1, 3 } | RT, RB |
| 6 | { 2, 3 } | LB, RB |

The new constructed temporal affine merge candidates are interlaced with the constructed spatial candidates, with a total of 16 candidates. The final number of candidates in the list is increased to 8.

| Final number of candidates | Max Number of inherited candidates | Max Number of constructed candidates (4 prms) | Max Number of constructed candidates (6 prms) |
| --- | --- | --- | --- |
| 8 | 4 | 12 | 16 |

The constructed spatial candidates for a 6 parameters model are modified as follows:

| Candidate number | Type | Corner number | Corner name |
| --- | --- | --- | --- |
| 1 | spatial | { 0, 1, 2 } | LT, RT, LB |
| 2 | temporal | { 0, 1, 2 } | LT, RT, LB |
| 3 | spatial | { 0, 1, 3 } | LT, RT, RB |
| 4 | temporal | { 0, 1, 3 } | LT, RT, RB |
| 5 | spatial | { 0, 2, 3 } | LT, LB, RB |
| 6 | temporal | { 0, 2, 3 } | LT, LB, RB |
| 7 | spatial | { 1, 2, 3 } | RT, LB, RB |
| 8 | temporal | { 1, 2, 3 } | RT, LB, RB |
| 9 | spatial | { 0, 1 } | LT, RT |
| 10 | temporal | { 0, 1 } | LT, RT |
| 11 | spatial | { 0, 2 } | LT, LB |
| 12 | temporal | { 0, 2 } | LT, LB |
| 13 | spatial | { 1, 2 } | RT, LB |
| 14 | temporal | { 1, 2 } | RT, LB |
| 15 | spatial | { 0, 3 } | LT, RB |
| 16 | temporal | { 0, 3 } | LT, RB |

While for a 4 parameters model, they are modified as follows:

| Candidate number | Type | Corner number | Corner name |
| --- | --- | --- | --- |
| 1 | spatial | { 0, 1 } | LT, RT |
| 2 | temporal | { 0, 1 } | LT, RT |
| 3 | spatial | { 0, 2 } | LT, LB |
| 4 | temporal | { 0, 2 } | LT, LB |
| 5 | spatial | { 1, 2 } | RT, LB |
| 6 | temporal | { 1, 2 } | RT, LB |
| 7 | spatial | { 0, 3 } | LT, RB |
| 8 | temporal | { 0, 3 } | LT, RB |
| 9 | spatial | { 1, 3 } | RT, RB |
| 10 | temporal | { 1, 3 } | RT, RB |
| 11 | spatial | { 2, 3 } | LB, RB |
| 12 | temporal | { 2, 3 } | LB, RB |

In addition to the test to reject pure translational candidates present in the common base, a test to reject spurious affine models is added, by checking the relative motion of the corners.

# Results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.02% | -0.68% | -1.01% | 110% | 111% |
| Class A2 | -1.15% | -1.15% | -1.25% | 112% | 113% |
| Class B | -0.75% | -0.59% | -0.75% | 111% | 109% |
| Class C | -0.61% | -0.47% | -0.48% | 110% | 109% |
| Overall | -0.85% | -0.69% | -0.83% | 111% | 110% |
| Class D | -0.90% | -0.74% | -0.81% | 111% | 118% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.42% | -0.49% | -0.53% | 117% | 116% |
| Class C | -0.40% | -0.61% | -0.32% | 110% | 109% |
| Class E | -0.76% | -0.35% | -0.39% | 111% | 113% |
| Overall | -0.50% | -0.50% | -0.42% | 113% | 113% |
| Class D | -0.62% | 0.48% | -0.43% | 109% | 108% |

| Complexity analysis of list construction |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max number of |  |  |  |  |  |  |
| Merge list size | potential candidates 4prms/6prms | candidate comparison | MV scaling | temporal candidates | Additional local buffer | Max number of memory access | Others |
| 8 | 4+12/4+16 |  |  | 6/8 |  |  |  |

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1024, 11th Meeting, Ljubljana, July 2018.

[2] F. Galpin, A. Robert, T. Poirier, F. Leleannec, “Non CE4: Virtual Temporal Affine”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0267, 11th Meeting, Ljubljana, July 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).