| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0173-v2 |
| --- | --- |

| Title: | CE9: Results of Tests 9.1.4, 9.2.4, 9.2.5, and 9.2.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Sethuraman Consulate-1, #1 Richmond Road Bangalore-560025 India | Tel: Email: | +91-80-66601000 sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_______`______________________

# Abstract

In this proposal, we summarize the results of CE9.1.4, CE9.2.4, and CE9.2.5 which study each of the 3 elements proposed in JVET-K0041 individually on top of the BMS-2.1 version of DMVR. In addition, we present the results of adding two of these elements (namely, elements studied in CEs 9.1.4 and 9.2.5) on top of the joint proposal studied under CE9.2.16. All the proposed methods meet all the CE9 requirements.  Some clarifications on the complexity calculations presented in the CE9 summary report are further elaborated in this document.

Over the anchor specified by CE9 (BMS software with VTM encoder configuration):

- CE9.1.4 which proposes use of refined MVs in neighbor coding units in top and top-right CTUs as refinement starting points for coding units within the current CTU provides an average luma BD-rate gain of -1.86% with 102% encoding time and 113% decoding time in RA configuration.

- CE9.2.4 which proposes a modified search pattern in order to limit the maximum cost evaluations per iteration shows -1.68% luma BD-rate gain with 115% decoding time in RA configuration.

- CE9.2.5 reports the results of replacing explicit half-pixel distance refinements with a parametric error surface based estimation of sub-pixel offset. -1.68% luma BD-rate gain is reported with 102% encoding time and 109% decoding time in RA configuration.

- In the test CE9.2.6, which combines elements of CE9.1.4 and CE9.2.5 on top of test CE9.2.16, an average luma BD-rate gain of -1.89% is reported with 101% encoding time and 108% decoding time in RA configuration.

# Introduction

In JVET-K0041, 3 elements related to DMVR were presented. The first element was a CTB-level data pre-fetch and processing pipeline that enabled the use of refined MVs of CUs in top and top left CTBs for DMVR to improve the coding gain compared to not using any refined MVs for refinement. The solution works with a minimum lag of 2 CTUs between vertically adjacent CTUs when working in a wavefront parallel manner.  Some of the dependency related requirements for CE9 in JVET-K1029 were derived from this reference pipeline.

The second element was a modified search pattern that was hardware friendly by ensuring that not more than 5 cost evaluations are performed in each iteration while keeping the number of dependent stages low for a given refinement range.

The third element was the use of a parametric error surface using the integer-distance cost function values that allowed up to 1/16th of a pixel accurate delta MVs to be obtained without additional pixel level computations beyond the integer distance refinement cost function evaluations.

In this proposal, each of these elements are studied (in CEs 9.1.4, 9.2.4, and 9.2.5 respectively) on top of BMS-2.1 DMVR to confirm that the gains presented in JVET-K0041 are still applicable.

A combination of multiple elements presented in the last meeting are studied in CE9.2.15 and 9.2.16 on top of the BMS-2.1 DMVR to help bound the worst-case complexity of DMVR and also to help reduce the average encoding and decoding times. In CE9.2.6, the impact of the JVET-K0041 elements on top of the combinations studied in CEs 9.2.15 and 9.2.16 are studied.

# Proposed algorithms

The three elements proposed in JVET-K0041 and studied in CEs 9.1.4, 9.2.4, 9.2.5 are summarized again here. A new combined proposal that studies the interaction of these elements with elements studied in CE9.2.16 is also presented.

## Refined MV availability rules for CTB-level pipeline with concurrency in reference data-prefetch and processing of CUs in a current CTU

In order to address the refined MV dependency issue, many proposals disabled use of refined neighbor MVs in the DMVR process completely. This had a significant impact on DMVR coding gains (close to 1% BDRATE drop). In JVET-K0041, the following rules for refined MV availability (relative to a current CTU) were laid out (for the case of lag of 2 CTUs between two vertically adjacent CTUs) which ensured concurrent data-prefetch and refinement across 2 stages of a CTB-level pipeline with wavefront parallelism, while recovering some of the lost compression gains:

- Refined MVs of neighbor CUs in top-left and top CTUs are considered as available for refinement

- Refined MVs of neighbor CUs in left CTU are considered as unavailable for refinement

- Refined MVs of neighbor CUs in current CTU are considered as unavailable for refinement

The refined MVs are used for deblocking and as TMVPs for future access units in decoding order. These rules satisfy all the minimum requirements stipulated in JVET-K1029.

In order to not use refined MVs of neighbor CU in the top-right CTU for AMVP and DMVR, one non-refined MV pair per CTB in the top-row needs to be maintained.

In CE9.1.4, the coding gains possible with this method over the BMS-2.1 DMVR method is studied.

## Parametric error surface based sub-pixel offsets estimation

The half-pixel distance refinement step in the currently adopted BMS-2.1 DMVR method requires 2 additional normative motion compensations (using 8-tap DCTIF) for each reference, one each for the horizontal half-pel distance phase and the vertical half-pel distance phase. In addition, the refinement itself requires four cost evaluations (using MR-SAD). These steps have significant computational complexity and also increase the internal buffer requirements and internal memory accesses significantly. In JVET-K0041, a parametric error surface fitted using integer distance position evaluated costs to determine 1/16th-pel accurate sub-pixel offsets with very minimal computational complexity was proposed. Also, to reduce the interpolation computations, bilinear interpolation was employed for all the refinement cost calculations and normative DCTIF MC is performed with the refined MVs.

The proposed method is summarized below:

- The parametric error surface fit is computed only if the center position is the best cost position in a given iteration.

- The center position cost and the costs at (-1,0), (0,-1), (1,0) and (0,1) positions from the center are used to fit a 2-D parabolic error surface equation of the form

Ex,y=Ax-x02+By-y02+C

where (x0, y0) corresponds to the position with the least cost and C corresponds to the minimum cost value. By solving the 5 equations in 5 unknowns, (x0, y0) is computed as:

x0=(E-1,0-E1,0)/(2(E-1,0+E1,0-2E0,0))

y0=(E0,-1-E0,1)/(2(E0,-1+E0,1-2E0,0)

(x0, y0) can be computed to any required sub-pixel precision by adjusting the precision at which the division is performed (i.e. how many bits of quotient are computed). For 1/16th-pel accuracy, just 4-bits in the absolute value of the quotient needs to be computed, which lends itself to a fast shifted subtraction based implementation of the 2 divisions required per CU.

- The computed (x0, y0) are added to the integer distance refinement MV to get the sub-pixel accurate refinement delta MV.

CE9.2.5 studies the BDRATE impact and speed-ups possible with this approach on top of the BMS-2.1 DMVR.

## Hardware implementation friendly search pattern

The search pattern adopted in BMS-2.1 DMVR requires evaluating 9 MR-SADs in the first iteration and then up to 5 MR-SADs in the second iteration in order to keep the number of dependent stages for integer distance refinement as 2 (please refer to JVET-L0029). This implies that for achieving the best timing for the refinement, up to 9 parallel MR-SAD execution units may be required in hardware. With the parametric error surface approach with the BMS-2.1 search pattern, it is possible to evaluate 5 MR-SADs in the first iteration and 8 MR-SADs in the second iteration (please refer to JVET-L0029 CE9.2.5 row).

In JVET-K0041, a modified search pattern was proposed that ensures that just 5 MR-SADs are evaluated in iteration-1 and iteration-2. In the first iteration, the center position and its left, top, right, and bottom integer distance positions are evaluated. In the second iteration, up to 5 additional MR-SADs are evaluated such that all 8-neighbor position costs around the second iteration search center are evaluated. Compared to the BMS-2.1 DMVR, this approach cannot reach the 2nd diagonal position at the end of the 2nd iteration.

In CE9.2.4, the BDRATE impact, if any, of this modified search and the complexity reduction are studied.

## Combined proposal studying CE9.1.4 and CE9.2.5 elements over CE9.2.16 elements

In CE9.2.16, many additional elements that are complementary to the three elements studied in JVET-K0041 are studied on top of BMS-2.1 DMVR (JVET-L0163). These elements have one or more of the following advantages:

- Eliminate the need to fetch additional reference samples for DMVR

- Restrict DMVR to be performed only when certain conditions are satisfied to give average computational savings and/or small coding gains

- Restrict DMVRs for small CUs to cap the worst-case complexity of DMVR

- Reduce the interpolation complexity

- Reduce the cost evaluation complexity

In CE9.2.6, the elements studied in CE9.1.4 and CE9.2.5 are studied again on top of CE9.2.16 elements.

Specifically, CE9.2.6(a) studies the impact of using refined MVs of neighbor CUs in top-left and top CTUs on top of CE9.2.16.

CE9.2.6(b) studies the impact of using refined MVs of neighbor CUs in top-left and top CUs AND the parametric error surface based sub-pixel offset estimation over CE9.2.16.

# Complexity analysis

For CEs 9.1.4, 9.2.4, and 9.2.5, the complexity analysis below is performed against the BMS-DMVR for only the elements that are introduced. For CE9.2.6, the complexity analysis is performed against the CE9.2.16 for only the additional elements introduced in this proposal.

## Complexity analysis for CE9.1.4

The use of refined MVs of CUs in top-left and top-CTUs and the use of refined MVs for deblocking and TMVP necessitate a line-buffer for storing the non-refined MVs of top-right CTU neighbor MV for use as AMVP or merge candidate. It is possible to only store the delta MV in 1/16th  pel precision w.r.t the refined MV, which will require 7-bits per component per reference. Hence, the line buffer will be 28bits x number of CTUs in row number of bits in size. However, given that DMVR is proposed to be restricted to 8x8 CU sizes and above, it is possible to store both the unrefined MVs and refined MVs in two consecutive 4x4 positions (corresponding to the bottom part of an 8x8) and avoid any extra line buffer requirements.

## Complexity analysis for CE9.2.5

As mentioned earlier, parametric error surface saves the half-pixel position motion compensated interpolations and also the cost evaluations for half-pixel distance refinement.

The cost values during iteration-1 and iteration-2 need to be maintained so that the 5 cost functions required for error surface fitting can be obtained. This requires storing a maximum of 25 cost values in a 2-D grid.

As mentioned earlier, given that the absolute value of quotient required in the divisions is only 4 bits, the division can be implemented with shifted subtraction for an estimated 20 operations per division (or 40 operations per 8x8 CU in the worst-case). With parallel compares, it is possible to realize the division in as low as 5 cycles.

## Complexity analysis for CE9.1.4

5 MR-SADs for the first iteration and 5 MR-SADs for the second iterations are required for the modified search pattern, when compared to 9 MR-SADs for the first iteration and 5 MR-SADs for the second iteration when considering only 2 dependent stages. Thus the pattern reduces the worst-case complexity for hardware.

## Complexity analysis for CE9.2.6 over CE9.2.16

The increased MV storage discussed in 3.1 above and the complexity reduction discussed in 3.2 above apply to CE9.2.16. Given that the proposal restricts DMVR to CUs of sizes 8x8 and above, all the worst-case analysis in JVET-L0029 are performed assuming 100% 8x8 CUs.

Also, CE9.2.16 offers the advantage of no increase in reference data fetch bandwidth.

Additional details can be found in JVET-L0029 and associated spreadsheet.

# Test Results

The BDRATE and running time test results are reported as per the CE requirements. BMS-2.1rc is the code base used. The results compare proposed DMVR Tool ON against the VTM-2.0 anchor (run through BMS configuration file). The tests are performed with –SIMD=SSE42.

## CE 9.1.4 Results

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.13% | -1.84% | -2.15% | 101% | 109% |
| Class A2 | -2.75% | -2.75% | -2.81% | 101% | 109% |
| Class B | -1.48% | -1.56% | -1.61% | 103% | 115% |
| Class C | -1.47% | -1.68% | -1.73% | 103% | 118% |
| Class E |  |  |  |  |  |
| Overall | -1.86% | -1.89% | -1.99% | 102% | 113% |
| Class D | -1.78% | -1.84% | -1.85% | 104% | 118% |

Table 4.1: Impact of enabling use of refined MVs of neighbor CUs in top and top-left CTUs

Thus, enabling the use of these refined MVs offers ~0.23% luma average BDRATE gain over BMS-DMVR. The increased use of DMVR increases the DecT slightly.

## CE 9.2.5 Results

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.80% | -1.61% | -1.89% | 102% | 107% |
| Class A2 | -2.38% | -2.49% | -2.60% | 101% | 107% |
| Class B | -1.38% | -1.55% | -1.60% | 103% | 110% |
| Class C | -1.44% | -1.65% | -1.74% | 102% | 112% |
| Class E |  |  |  |  |  |
| Overall | -1.68% | -1.77% | -1.90% | 102% | 109% |
| Class D | -1.83% | -2.00% | -2.01% | 103% | 113% |

Table 4.2: Parametric error surface based sub-pixel offset estimation replacing explicit half-pel distance refinement in BMS-DMVR

The parametric error surface based approach offers ~0.05% luma average BDRATE gain over BMS-DMVR. It also reduces DecT by ~7%.

## CE9.2.4 Results

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.74% | -1.52% | -1.85% | 101% | 111% |
| Class A2 | -2.32% | -2.43% | -2.56% | 102% | 112% |
| Class B | -1.32% | -1.47% | -1.49% | 104% | 117% |
| Class C | -1.39% | -1.57% | -1.70% | 104% | 120% |
| Class E |  |  |  |  |  |
| Overall | -1.63% | -1.70% | -1.83% | 103% | 115% |
| Class D | -1.72% | -1.84% | -1.89% | 105% | 122% |

Table 4.3: Replacing existing search pattern with the modified search pattern

There is no BDRATE impact when using the modified search pattern when compared to the default search pattern in BMS-DMVR.

## CE9.2.6 Results

### CE9.2.6(a)

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.11% | -1.80% | -2.11% | 101% | 109% |
| Class A2 | -2.62% | -2.67% | -2.72% | 101% | 109% |
| Class B | -1.49% | -1.53% | -1.55% | 102% | 113% |
| Class C | -1.37% | -1.55% | -1.61% | 102% | 115% |
| Class E |  |  |  |  |  |
| Overall | -1.81% | -1.82% | -1.91% | 102% | 112% |
| Class D | -1.42% | -1.54% | -1.71% | 102% | 115% |

Table 4.4: CE9.2.6(a) results (CE9.2.16 + use of refined MVs of CUs in top-left and top CUs)

When compared to the CE9.2.16 results, CE9.2.6(a) provides 0.22% average luma BDRATE gain and a 2% increase in DecT due to the increased use of DMVR to achieve the higher coding gain.

### CE9.2.6(b)

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.11% | -1.79% | -2.11% | 101% | 107% |
| Class A2 | -2.72% | -2.75% | -2.80% | 101% | 105% |
| Class B | -1.60% | -1.62% | -1.66% | 102% | 108% |
| Class C | -1.46% | -1.65% | -1.73% | 101% | 110% |
| Class E |  |  |  |  |  |
| Overall | -1.89% | -1.89% | -2.00% | 101% | 108% |
| Class D | -1.57% | -1.67% | -1.75% | 102% | 110% |

Table 4.5: CE9.2.6(b) results (CE9.2.16 + use of refined MVs of CUs in top-left and top CUs + Parametric error surface based sub-pixel offset estimation)

The addition of parametric error surface over CE9.2.6(a) offers a luma BDRATE gain of 0.08% and also reduces EncT by 1% and DecT by 4%. The two elements together offer a combined luma BDRATE gain of 0.30% and a DecT reduction by 4%.

# Conclusion

The use of refined MVs of neighbor CUs in top-left and top CTBs, studied in CE9.1.4, provides an average luma BD-rate gain of -0.23% when compared to BMS-2.1 DMVR. The parametric error surface based sub-pixel offset estimation, studied in CE9.2.5 provides significant reduction in computations and internal memory requirements and internal memory accesses. It offers -0.05% luma BD-rate gain in addition to a significant reduction in measured encoding and decoding times. The modified search pattern, studied in CE9.1.4 has no BD-rate impact, while reducing the worst-case number of MR-SADs to 10 .

The stand-alone behaviors are again retained when studied on top of CE9.2.16 which already provides reduction in complexity by combining promising candidates in CE9. On top of CE9.2.16, the use of refined MVs of neighbor CUs in top-left and top CTBs provides an additional -0.22% luma BD-rate improvement.

In test CE9.2.6(b), the parametric error surface approach (CE9.2.5) and use of refined MVs of neighbor CUs in top-left and top CTBs (CE9.1.4) are implemented on test CE9.2.16. Over the VTM2.0.1 anchor -1.89% luma BD-rate gain is reported with 1% encoding time increase and 8% decoding time increase. It is requested that the CE9.2.6(b) configuration of DMVR be considered for adoption into VVC.

# References

- S. Esenlik, Y. Chen, F. Chen, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-K1029, July 2018.

- S. Sethuraman, Jeeva Raj A., S. Kotecha, “Decoder Side MV Refinement/Derivation with CTB-level concurrency and other normative complexity reduction techniques”, JVET-K0041, July 2018.

- S. Esenlik, et. al, “CE9: Report on the results of tests CE9.2.15 and CE9.2.16”, JVET-L0163, October 2018.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).