| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0178 |
| --- | --- |

| Title: | CE 9: Refined MVs Partial Usage for Spatial (Test 9.1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen Jianhua Zheng | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents a DMVR latency solution based on BMS-2.1 with DMVR only. The DMVR latency solution determines refined mv partial usage limitation. The proposed technologies can provide X.XX% gain compared to BMS-2.1 with VTM configuration anchor.

Introduction

In current BMS, refined mv is only used for temporal case to avoid DMVR latency.

Proposed improvements

- Refined MVs Partial Usage for Spatial

In this proposed method, add the limitation of CTB line area. That is, use original mv for spatial when in current CTB line area and refined mv beyond the CTB line area can be used.

Figure 1. Refined mv Partial Usage for Spatial.

Experimental Results

The proposed method has been implemented on BMS-2.1[1], and all the tests were conducted under the common test conditions[2]. The experimental results of the refined mv partial usage for spatial compared to BMS-2.1 with VTM configuration are shown in Table1.

Table 1. Refined mv Partial Usage for Spatial

Random Access Main 10. Over BMS-2.1 - with VTM config. Y; U; V. Class A1; -2.10%; -1.79%; -2.12%. Class A2. Class B; -1.48%; -1.53%; -1.59%. Class C; -1.43%; -1.74%; -1.71%. Class E. Overall. Class D; -1.76%; -1.86%; -1.90%.

Reference

- BMS-2.1, https://vcgit.hhi.fraunhofer.de/JVET-K-CE9/VVCSoftware_BMS.

- F. Bossen,J. Boyce, X. Li, V. Seregin,K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010,Ljubljana, SI, 10–18July 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).