| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0188_v3 |
| --- | --- |

| Title: | CE9: Unidirectional Template based DMVR and its Combination with Simplified Bidirectional DMVR (Test 9.2.10 and Test 9.2.11) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution presents two solutions about motion vector refinement (MVR) based on BMS-2.1 with DMVR only. One focuses on the unidirectional template based DMVR. With simple motion information of neighboring blocks, a template is generated to refine an initial MV of the Merge mode candidate. The other is about the simplification of DMVR on BMS-2.1. It is reported that the combination of these two solutions can provide 2.26%, 0.23%, 0.67% gain for RA, LB, LP respectively compared to VTM-2.0 with around 15% encoding time and 6%~23% decoding time increase.

Introduction

In current BMS, the DMVR is only enabled for RA configuration, and has no gains for LDP or LDB configurations. Meanwhile, the DMVR in current BMS has high decoder complexity and bandwidth.

Proposed improvements

Unidirectional Template based DMVR

When this method is enabled, the initial motion vector (MV) of list0 or MV of list1 in the merge candidates will be further refined by a template matching process. As shown in Figure 1, in order to obtain a refined MV, template matching performs a SAD-based search between a template and a block with the same size as the template in a reference picture. One flag is transmitted to signal whether the MVR technique is enabled for a block using Merge mode.

As shown in Figure 2, the template is compose of two parts, i.e., the upper sub-template (TM1) and the left sub-template (TM2). The same strategy is adopted for the generation of two sub-templates. For TM1, it is obtained with the motion information of first upper PU, whose horizontal position is the same as that of current block. The width of TM1 is the same as that of the first upper PU, and its height is set as a fixed value (1 for current version).With basic motion information, i.e., MV, reference index, and inter direction, the TM1 is obtained with simple interpolation using these information. If the first upper PU is bidirectional, then the TM1 is the average of two direction. Moreover, if the first upper PU adopts intra predication mode, TM1 will be invalid and unnecessary to be constructed. Similarly, the TM2 is also acquired with the motion information of first left PU, whose vertical position is the same as that of current block. And the height of TM2 is the same as that of the first left PU, and its width is also fixed (1 for current version). Then, if it is intra coded, TM2 would be also unavailable. As a result, when both of the first PU and the first left PU are unavailable (intra coded or nonexistent), the proposed method would be invalid, and the related mode checking will be also skipped.

To avoid large complexity increase, a simple template searching strategy is adopted. The searching process has at most 4 iterations, and each iteration adopts 4 points cross search strategy to find a best point. In each iteration, its best point will be the start point of next iteration. The related issues about the implementation are given in Table I.

To avoid latency, only the refined MV of block in the above CTU row can be utilized for spatial motion prediction. Otherwise, the non-refined MV will be used for spatial motion prediction.

Fig. 1 Template matching

Fig. 2 The template generation for current block

Table I Related issues about implementation

| Initial MV signaled | YES |
| --- | --- |
| Neighboring recon. samples used | NO |
| Sub-PU mode used | NO |
| Max number of SAD calculation | 17 |
| Max. search range | 1 pixel |
| Cost function | SAD |
| Interpolation filter/tap number | Bilinear Interpolation filter ME, DCTIF/8 MC |

Simplified Bidirectional DMVR

Since the DMVR in current BMS has high decoder complexity and bandwidth, we adopt some simplifications to decrease the decoder complexity and bandwidth during the DMVR searching process. First, the bilinear interpolation filter is utilized to replace the DCTIF. Second, the block is down sampled before calculating the searching cost. Third, similar to the MV constraint in Sec 2.1, the refined MV of block in the above CTU row can be utilized for spatial motion prediction.

Experimental Results

To evaluate the coding performance of the proposed method, the BMS-2.1 [1] with VTM configuration are served as the experimental platform. Three coding configurations, namely, Random Access (RA) Main 10 Profile, Low Delay B (LDB) Main 10 Profile, and Low Delay P (LDP) Main 10 Profile specified in [2], are adopted, and all the sequences are tested across the configurations with the common test conditions. BD-rate is used to evaluate the final coding performance.

Three tests are conducted, and the details are given as followed. Anchor for is BMS-2.1 with VTM configuration.

Test 1: CE9.2.10

In Test 1, only the proposed unidirectional template is enabled. The results are given in Table I. It is reported that, compared with VTM-2.0, 0.57%, 0.23% and 0.67% coding gains are achieved on average for RA, LDB and LDP configurations, respectively. On the aspects of coding time, 11%~16% additional encoding time and 6%~9% additional decoding time are observed.

Table I Results of unidirectional template based DMVR (Test CE 9.2.10)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 with VTM Configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.36% | -0.45% | 108% | 103% |
| Class A2 | -0.73% | -0.72% | -0.83% | 109% | 105% |
| Class B | -0.56% | -0.42% | -0.44% | 111% | 107% |
| Class C | -0.53% | -0.31% | -0.50% | 113% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.57% | -0.44% | -0.54% | 111% | 106% |
| Class D | -0.63% | -0.48% | -0.44% | 115% | 109% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 with VTM Configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.47% | 0.02% | 110% | 108% |
| Class C | -0.22% | -0.22% | -0.31% | 112% | 111% |
| Class E | -0.40% | -0.30% | -0.63% | 109% | 106% |
| Overall | -0.23% | -0.35% | -0.25% | 111% | 109% |
| Class D | -0.18% | 0.03% | 0.18% | 113% | 108% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 with VTM Configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.61% | 0.16% | 0.44% | 115% | 105% |
| Class C | -0.87% | -0.57% | -0.78% | 118% | 109% |
| Class E | -0.51% | -0.29% | -0.20% | 114% | 103% |
| Overall | -0.67% | -0.20% | -0.13% | 116% | 106% |
| Class D | -1.12% | -0.13% | -0.38% | 119% | 107% |

Test 2: CE9.2.11-a

In Test 2, only the simplified DMVR is enabled. The results are given in Table II. Since DMVR is only enabled for RA configuration, the test is conducted with RA configuration only. It is reported that, compared with VTM-2.0, 1.76%, 1.81% and 1.96% coding gains are achieved on average, respectively. On the aspects of coding time, 3% additional encoding time and only 11% additional decoding time are observed.

Table II Results of simplified DMVR (Test CE 9.2.11-a)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 with VTM Configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.07% | -1.74% | -2.15% | 103% | 108% |
| Class A2 | -2.63% | -2.66% | -2.77% | 103% | 107% |
| Class B | -1.40% | -1.53% | -1.56% | 104% | 113% |
| Class C | -1.35% | -1.60% | -1.70% | 103% | 115% |
| Class E |  |  |  |  |  |
| Overall | -1.76% | -1.81% | -1.96% | 103% | 111% |
| Class D | -1.68% | -1.84% | -1.91% | 105% | 113% |

Test 3: CE9.2.11-b

In Test 3, both the proposed unidirectional template and the simplified DMVR are enabled. The results are given in Table III. It is reported that, compared with VTM-2.0, 2.26%, 2.16% and 2.37% coding gains are achieved on average for RA, LDB and LDP, respectively. On the aspects of coding time, 17% additional encoding time and only 23% additional decoding time are observed.

Table III Results of combination of two solutions (Test CE 9.2.11-b)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 with VTM Configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.51% | -2.06% | -2.45% | 113% | 115% |
| Class A2 | -3.16% | -3.02% | -3.17% | 115% | 117% |
| Class B | -1.85% | -1.87% | -1.93% | 119% | 131% |
| Class C | -1.91% | -1.97% | -2.27% | 120% | 125% |
| Class E |  |  |  |  |  |
| Overall | -2.26% | -2.16% | -2.37% | 117% | 123% |
| Class D | -2.21% | -2.18% | -2.26% | 124% | 124% |

Reference

- BMS-2.1, https://vcgit.hhi.fraunhofer.de/JVET-K-CE9/VVCSoftware_BMS.

- F. Bossen,J. Boyce, X. Li, V. Seregin,K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010,Ljubljana, SI, 10–18July 2018.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).