| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0190 |
| --- | --- |

| Title: | CE6-related: Simplification of Intra 4-Point Multiple Transforms Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An Yu-Chen Sun Jian Lou | Tel: Email: | +86-13439745728 jicheng.ajc@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes to use only DST7 for intra luma 4-point transform when the MTS_CU_flag is equal to 1, the experiments results show that there is 10% encoding time reduction with 0.0% BD-rate change.

Introduction

In addition to DCT2 which has been employed in HEVC, a Multiple Transform Selection (MTS) scheme is used for residual coding both inter and intra coded blocks in current VVC. It uses multiple selected transforms from the DCT8/DST7.

In order to control MTS scheme, separate enabling flags are specified at SPS level for intra and inter, respectively. When MTS is enabled at SPS, a coding unit (CU) level flag is signalled to indicate whether MTS is applied or not. Here, MTS is applied only for luma. The MTS CU level flag is signalled when the following conditions are satisfied.

    - Both width and height smaller than or equal to 32

    - CBF flag is equal to one

If MTS CU flag is equal to zero, then DCT2 is applied in both directions. However, if MTS CU flag is equal to one, then another two flags are additionally signaled to indicate the transform type for the horizontal and vertical directions, respectively. For intra CU, those two flags (i.e., MTS_Hor_flag and MTS_Ver_flag) are signalled when the number of non-zero coefficients is greater than two. However, for inter CU, regardless of the number of nonzero coefficients, those flags are signalled. For example, for intra CU with only 1 or 2 nonzero coefficients, DST7 is used both horizontally and vertically without signalling the additional two flags when MTS CU flag is equal to one. Transform and signalling mapping table as shown in Table 1.

Table 1 - Transform and signalling mapping table

| MTS_CU_flag | MTS_Ver_flag | MTS_Hor_flag | Intra |  | Inter |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal | Vertical | Horizontal | Vertical |
| 0 |  |  | DCT2 |  |  |  |
| 1 | 0 | 0 | DST7 | DST7 | DCT8 | DCT8 |
|  | 0 | 1 | DCT8 | DST7 | DST7 | DCT8 |
|  | 1 | 0 | DST7 | DCT8 | DCT8 | DST7 |
|  | 1 | 1 | DCT8 | DCT8 | DST7 | DST7 |

Proposed 4-point transform selection

It is proposed for intra coded luma block when the MTS_CU_flag is equal to 1, if its side length (width or height) is equal to 4 samples, then the transform for this side (width or height) will always be DST7, i.e., the DCT8 will never be used for this case, so that the flag has no need to signal.

The signaling scheme of the original MTS is modified as follows, the changed parts are highlighted.

When the MTS_CU_flag is equal to 1,

- the MTS_hor_flag

  - signaling scheme is not changed for the CU with width larger than 4;

  - is not signaled and derived as 0 (DST7) for the CU with width equal to 4;

- the MTS_ver_flag

  - signaling scheme is not changed for the CU with height larger than 4;

  - is not signaled and derived as 0 (DST7) for the CU with height equal to 4.

Experimental results

The proposed method has been implemented based on VTM2.0.1, the results are shown in Table 2. It can be seen that for all intra configuration, there is around 10% encoding time saving with 0.0% BD-rate change.

Table 2. results on VTM2.0.1

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).