| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0193 |
| --- | --- |

| Title: | CE4-related: On Affine mode restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Christophe Gisquet Patrice Onno Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents 2 modifications of the affine mode. The first one avoids the affine mode for the Merge Skip. The second one changes the block positions of the affine flag context derivation in order to have the same positions as those checked to derive the candidate of the affine Merge.

An average Luma BDR is reported compared to VTM2.1 of 0%, -0.08%, -0.16% for respectively RA, LD and LDP configurations for the first modification. An average Luma BDR is reported compared to VTM2.1 of 0%, 0.04%, 0.01% for respectively RA, LD and LDP configurations for the second modification. When both modifications are enabled the BDR results are 0%, -0.08%, -0.18% for these 3 Inter configurations.

# Problem Statement

## Affine and Skip motion compensation

The affine motion compensation prediction is particularly efficient for complex motion like rotation, zoom in, zoom out and perspective motion. The Skip mode is efficient for static content or constant motion. It seems that these modes compensate different kinds of motion and are incompatible. The results of the first proposed modification which avoid the affine mode when the current CU is skipped show that the combination of both motion compensations have no sense.

## Affine Merge candidate position and affine flag

For the affine Merge and Merge Skip only one candidate is derived. This candidate is the first candidate among the five block positions A1, B1, B0, A0, and B2 which is affine as depicted in Figure 1 (a).

The affine flag is CABAC coded with a context value. This context value is the sum of the affine flag of the 2 neighbouring blocks A2 and B3 as depicted in Figure 1 (b). So this context CtxIdx is given by the following formula:

CtxIdx = IsAffine(A2) + IsAffine(B3)

Moreover, the affine Merge or Merge Skip is enabled only when the affine Merge exists and if the CU is a square block.

Consequently, to decode or to parse a Merge or a Merge Skip 7 block positions need to be checked to decode the affine flag for the worst case.

| a | b |
| --- | --- |

Figure 1 (a) Affine Merge mode candidates positions. (b) Block position for affine flag context derivation.

# Proposed modifications

## Avoid Affine for Skip mode

The first proposed modification consists in just avoiding the affine motion compensation for the Merge Skip mode as depicted in Table 1.

This reduces the decoding and parsing complexity of the Merge Skip mode because all conditions to decode the affine flag (checking 7 block positions) are avoided. This doesn’t reduce the worst case but it reduces the decoding complexity because the Merge Skip mode is the most selected mode in Inter slices.

Moreover this modification reduces the encoding complexity by avoiding the check of the Merge Skip mode for affine Merge.

Table 1 Inter mode enabled for motion compensation type.

| INTER modes | Classical MC | Affine MC |
| --- | --- | --- |
| MERGE SKIP | X |  |
| MERGE | X | X |
| AMVP | X | X |

## Align affine context block positions with the affine Merge candidate positions

The second proposed modification aligns the block positions used to derive the affine flag context with the Merge candidate positions. The proposed block positions for the affine flag context are A1 and B1 as depicted in Figure 2 and the context index formula is:

CtxIdx = IsAffine(A1) + IsAffine(B1)

With this modification, only the affine flag value of 5 block positions needs to be checked before decoding or parsing an affine flag.

Figure 2 Proposed Block positions for Affine Flag Context.

# Experimental results

The proposed method is implemented on top of the VTM2.1 which includes bug fixes of affine motion compensation.

## Modification 1: Avoid Affine for Skip mode

Table 2 gives the results of avoiding affine for Merge Skip mode compared to VTM2.1. The proposed modification as no impact on coding efficiency for RA configurations. Yet an average coding gain of -0.08% is observed for LD configuration. This gain increases for LDP configuration with -0.16% average BDR. This proposed modification seems very efficient for Class E with an average coding gain of -0.41%

Table 2 Results of Modification 1 compared to VTM2.1

## Modification 2: Align affine context block positions with the affine Merge candidate positions

Table 3 gives the results of the second modification. There is almost no change on coding efficiency for RA and LDP configurations. For LD configuration, a small BDR loss of 0.04% is observed.

Table 3 Results of Modification 2 compared to VTM2.1

## Modification 1 + Modification 2

Table 4 gives the results of both modifications. The results are similar or little better than those obtained for modification 1 with respectively -0.08% and -0.18% BDR gain for LD and LDP configurations. As the previous results there is no impact of RA case.

Table 4 Results of Modification 1 + Modification 2compared to VTM2.1

# Conclusion

In this contribution, 2 modifications of the affine mode are proposed. The first one avoids the affine motion compensation for Merge Skip mode. This avoids an incompatibility of 2 modes and gives a small coding efficiency improvement of -0.8% and -0.16% for respectively LD and LDP configurations without impact on coding efficiency for RA case. This modification also reduces the decoding and parsing complexity by avoiding multiple checkings the neighboring blocks for the most selected mode in video coding. Moreover the encoding complexity is reduced by avoiding affine Merge Skip evaluation.

The second modification aligns the affine context with the affine merge block positions to reduce the worst case complexity of the affine flag checking to decode the affine Merge flags from 7 to 5 checks without impact on the coding efficiency.

When both modifications are enabled on top of VTM-2.1, which include bug fixes of affine motion compensation, small coding efficiency improvements of -0.8% and -0.18% for respectively LD and LDP configurations are obtained.

According to this small modification and the advantages offered for complexity, we propose to adopt both modifications for the next VTM software.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).