| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0204-v1 |
| --- | --- |

| Title: | CE3-related: disabling PDPC based on availability of reference samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes to disable PDPC for certain intra blocks based on the availability of reference samples around the block. The results show tiny gains in both complexity and coding efficiency. Although the benefit is very small, the author believes that this is a logical modification of the PDPC process that could be especially beneficial for low resolution sequences and test conditions that differ from the CTC.

# Introduction

The contribution proposes to disable PDPC automatically based on the availability of reference samples around the block.

# Disable PDPC based on availability of reference samples

PDPC is applied on top of the standard angular, DC or planar intra prediction. It consists in filtering the preliminary prediction signal obtained with angular, DC or Planar intra prediction with some of the reference samples depending on the position of the pixel in the prediction signal. The weights applied to the reference samples are position dependent, such that the weights are higher for pixels located close to the reference samples (i.e. at the upper or left border of the block) and gradually fall to zero for pixels located far from the reference samples (for example the pixels in the bottom right corner of the block). PDPC therefore shares some similarities with edge filtering.

It is proposed to disable automatically PDPC for one block when some reference samples are not available for that block. The reference samples that are considered depend on the intra prediction mode. Typically, the reference samples that are likely to be the most relevant for improving the quality of the prediction signal are checked for availability to determine if PDPC is disabled or not for the block.

Specifically, when the planar or DC intra prediction mode is used, the availability of the reference samples located above and left of the block is checked. If any of them is available, then PDPC is applied, as usual. Otherwise PDPC is disabled for the block.

When vertical intra prediction modes are used, i.e prediction modes with an index higher than 34, then the availability of the reference samples located left of the block (as illustrated in Figure 1) is checked. Indeed, since the prediction signal itself is calculated using mostly the reference samples located above the block, the reference samples left of the block are more likely to have an impact to improve the quality of the prediction when using PDPC. Therefore, if these reference samples are not available, then PDPC is disabled for the block.

Figure 1: Checking the availability of reference samples left of the block for vertical prediction directions

When horizontal intra prediction modes are used, i.e prediction modes with an index lower than 34, then the availability of the reference samples located above the block (as illustrated in Figure 2) is checked. Indeed, since the prediction signal itself is calculated using mostly the reference samples located left of the block, the reference samples above the block are more likely to have an impact to improve the quality of the prediction when using PDPC. Therefore, if these reference samples are not available, then PDPC is disabled for the block.

Figure 2: Checking the availability of reference samples above the block for horizontal prediction directions

# Results in VTM-2.0.1

The results for all intra configuration for VTM-2.0.1 are presented below. The anchor is the BMS-2.0.1 software using the VTM configuration files. The proposed modification to PDPC has been implemented in the VTM-2.0.1 software.

Figure 3: Results for all intra configuration

There is a small runtime reduction for both encoder and decoder together with a tiny gain in coding efficiency for luma. The effect is very low for high resolution sequences, because, with the CTC, there is a difference only for the blocks located at the upper and left borders of the picture. Therefore, the gain is more noticeable for low resolution sequences.

However, this idea is expected to have more benefit for use cases where small slices are relevant, such as for example using one slice per line of CTU for super low delay systems. There may also be some benefit for intra prediction in inter slices in a cyclic intra refresh scenario, or when constrained intra is used.

# Conclusion

A logical modification to the current PDPC approach is proposed that consists in disabling PDPC when the relevant reference samples are not available. The results show a tiny gain in coding efficiency together with a small decrease in runtime for low resolution sequences. It is expected to be of greater benefit for use cases different from the Common Test Conditions, for example using small slices. As it seems to be a logical complexity reducing modification, it is suggested to adopt it in VVC.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).