| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0205-v1 |
| --- | --- |

| Title: | CE12: report of CE12-1 on out-of-loop dynamic range adaptation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. François, C. Chevance, F. Hiron (Technicolor) D. Rusanovskyy, A.K. Ramasubramonian, M. Karczewicz (Qualcomm) | Tel: Email: | edouard.francois@technicolor.com dmytror@qti.qualcomm.com |
| Source: | Qualcomm, Technicolor |  |  |

_____________________________

# Abstract

The contribution reports results from CE12-1 on out-of-loop mapping for HDR content. Proposed dynamic range adaptation (DRA) is applied on input video data, Y’CbCr 4:2:0 picture (10 bits per sample) prior to coding (pre-processing). Inverse DRA is implemented after decoding loop as a post-processing (output process). The complete DRA design includes three parts: DRA mapping, chroma sample refinement, harmonization of DRA scaling with coding QP. Main change compared to the previous CE12 of JVET-11 meeting relates to the luma/chroma distribution that has been balanced in the same way as the anchor.

In the CE12-1.1, DRA is tested with the luma mapping curve used in JVET-K0298. In the CE12-1.2, DRA is tested with the luma mapping curve used in JVET-K0308. Corresponding BD-rate results, comparing to HDR anchor, are summarized in the table below, respectively for the AI and RA fixed QP configurations.

| AI fixed QP | DE100 | PSNRL | wPSNR Y | wPSNR U | wPSNR V |
| --- | --- | --- | --- | --- | --- |
| CE12-1.1 | -6.4% | -4.0% | -2.0% | -8.4% | -20.1% |
| CE12-1.2 | -7.5% | -2.9% | -2.3% | -9.5% | -21.0% |

| RA fixed QP | DE100 | PSNRL | wPSNR Y | wPSNR U | wPSNR V |
| --- | --- | --- | --- | --- | --- |
| CE12-1.1 | -8.4% | -4.8% | -2.6% | -6.0% | -24.4% |
| CE12-1.2 | -9.0% | -3.0% | -2.5% | -6.5% | -24.8% |

# Introduction

Dynamic range adaptation (DRA) was proposed in JVET-J0021 and JVET-J0022 for HDR category, and further studied in CE12. The current contribution aims at reporting results from different variants of this technology.

# Tool description

Dynamic range adaptation (DRA) is applied on input video data, Y’CbCr 4:2:0 picture (10 bits per sample) prior to coding (pre-processing). Inverse DRA is implemented after decoding loop as a post-processing (output process). The DRA design includes three parts: DRA mapping, chroma sample refinement, harmonization of DRA scaling with coding QP.

Dynamic Range Adjustment (DRA) is range-adaptive mapping of luma samples and luma-dependent scaling of chroma samples. Mapping/scaling is applied as a pre-processing and its inverse is applied at the decoder side as post-processing. In practice, in decoder, inverse DRA can be implemented using 1D look-up-tables LUTinvDraY , LUTinvDraCb and LUTinvDraCr :

Ymap = LUTinvDraY[ Yrec ]

Cxmap = ( Cxdec - 2BDchr-1 ) * LUTinvDraCx[ Yrec_coloc ] + 2BDchr-1      x=b or r

DRA scales and scalar quantizer harmonization consists in adjusting the CC-DRA scale functions to preserve a luma/chroma bit allocation assumed by the core coding design for a given QP settings introduced by chroma QP offset table (cf Table 8-10 of HEVC specification).

DRA refinement process consists in a correction of the DRA inverse mapping functions to compensate the distortion resulting from the compression that may impact the inverse DRA process. The inverse DRA LUTs are corrected by the refinement LUTs, modeled by piece-wise linear functions.

LUTinvDraY[ k ] = Min( 2Blum – 1, Max( 0, LUTrefineY[ k ] * LUTinvDraY[ k ] )

LUTinvDraCx[ k ] = LUTrefineCx[ k ] * LUTinvDraCx[ k ]

LUTrefineY, LUTrefineCb, LUTrefineCr are built from piece-wise linear (PWL) tables, signaled per slice. The PWL pivot points are equi-distant, and size is signaled in the slice header. When a table is not signaled for a component, the table previously decoded is used.

In addition, compared to the previous proposal made in JVET11 meeting, the setting of chroma QP is based on the QP given as a configuration parameter, while in previous version, it was based on the QP of the intra slices. This results in a similar luma/chroma balance behavior as the anchor, while in previous version, more bits were spent for luma.

# Performed tests

Two luma mapping curves are considered for this sub-CE:

- Luma DRA: luma mapping curve used in JVET-K0298, built from a piece-wise linear (PWL) model based on anchor dQP table, normalized to [0,1023], and mapping the signal to full range values.

- Luma Reshaping: luma mapping curve used in JVET-K0308, signalled using anchor dQP model and built from anchor dQP table, normalized to [0,1023], and mapping the signal to full range values.

The corresponding forward and inverse forward look-up-tables are depicted in the figure below.

Figure 1. Forward and inverse forward luma mapping LUTs from JVET-K0298 and JVET-K0308.

# Results

The tools described above are implemented within the BMS-2.0.1 software, configured using the macros as VTM-2.0.1 (JEM_TOOLS macros deactivated). Reference are HDR anchors as defined in the HDR CTCs. Complete simulation results are provided in the attachments to this contribution and summary for all requested metrics are provided in this document.

In the reported experiments, the DRA parameters are signalled for each IRAP picture. In addition, refinement parameters are signalled for each slice of temporal level 0.

## CE12-1.1 test results

Summary of the simulation results produced in CE12-1.1 (DRA with luma mapping curve from JVET-K0298) are provided in Table 1 and Table 2 for AI and RA fixed QP configuration.

Table 1. CE12-1.1 vs HDR anchor, AI fixed QP constraints

Table 2. CE12-1.1 vs HDR anchor, RA fixed QP constraints

## CE12-1.2 test results

Summary of the simulation results produced in CE12-1.2 (DRA with luma mapping curve from JVET-K0308) are provided in Table 3 and Table 4 for AI and RA fixed QP configuration.

Table 3. CE12-1.2 vs HDR anchor, AI fixed QP constraints

Table 4. CE12-1.2 vs HDR anchor, RA fixed QP constraints

# Analysis of results

An important aspect for results interpretation is the validity of HDR metrics. At the last JVET meeting, BoG on HDR coding discussed results of statistical analysis conducted over CfP MOS score results and reported objective results for CfP responses. Presented statistical results and discussion suggested that HDR metrics (DeltaE 100 and PSNRL100) seems to provide higher correlations with MOS scores across all HDR CfP response and rate points, in comparison to wPSNR and PSNR metrics. Due to this information, results in DeltaE100 and PSNR L100 metrics will be used in the current reporting and discussion.

Using luma mapping curve from JVET-K0298 or from JVET-K0308 results in differences in luma/chroma BD-rate gain. JVET-K0298 curve results in higher luma gain (PSNR_L 4.8% in RA) than JVET-K0308 curve (PSNR_L 3.0% in RA), while chroma is slightly improved using JVET-K0308 curve (DE100 9.0% gain instead of DE100 9.0%). Respective gains in wPSNRY are close (around 2.5% in RA). Overall, mapping curve from JVET-K0298 seems performing better.

Results consistent for all metrics and between the AI and RA configurations. The gain is also consistent in PSNR L100 and DE100 for all sequences and across all QP points. As already observed in previous CE12 from JVET11 meeting, it is probably beneficial to give a bit less bit budget to the chroma in order to favour a bit more the luma. Chroma in the anchor is indeed already of high quality, and it is more beneficial to improve luma than chroma.

# Conclusion

This contribution report results from CE12-1 tests on out-of-loop dynamic range adaptation for HDR content as proposed in JVET-J0021[1]/J0022 [2]. Simulation results show that complete method proposed to JVET provides a consistent gain of 4.0% for PSNRL, 6.4% for DE100 in AI configuration, and of 4.8% for PSNRL, 8.4% for DE100 in RA configuration.

Syntax is provided in the annex of this document (section 8).

# Patent rights declarations

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Syntax

The DRA related syntax is added in PPS() and in slice header().

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| dra_enabled_flag | u(1) |
| if (dra_enabled_flag) { |  |
| dra_num_frac_bit_scale | u(4) |
| dra_num_int_bit_scale | u(4) |
| dra_num_luma_dra_in_ranges | se(v) |
| dra_equal_ranges_flag | u(1) |
| dra_delta_luma_dqp_change_point[ 0 ] | u(10) |
| if (dra_equal_ranges_flag) { |  |
| dra_delta_val | se(v) |
| } |  |
| else { |  |
| for ( i = 1; i <= dra_num_luma_dra_in_ranges; i++ ) |  |
| { |  |
| dra_delta_luma_dqp_change_point [ i ] | u(10) |
| } |  |
| } |  |
| for ( i = 0; i <= dra_num_luma_dra_in_ranges; i++ ) |  |
| { |  |
| dra_scal_val[ i ] | u(v) |
| } |  |
| dra_scale_cb_qp_offset | u(v) |
| dra_scale_cr_qp_offset | u(v) |
| dra_base_qp | se(v) |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (dra_enabled_flag) { |  |
| dra_refine_enable_flag | u(1) |
| if (dra_refine_enable_flag) { |  |
| dra_refine_table_log2_size | u(4) |
| for ( comp = 0; comp < 3; comp++ ) { |  |
| dra_refine_comp_flag[ comp ] | u(1) |
| if (dra_refine_comp_flag[ comp ] ) { |  |
| for ( i = 0; i < draRefineTableSize; i++ ) |  |
| { |  |
| dra_refine_table_comp_value[ comp ][ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |