| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0209 |
| --- | --- |

| Title: | PCM mode with dual tree partition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Jicheng An, Jian Lou | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This document proposes PCM mode modification for separated tree partition. The proposed modification is tested on top of VTM2.0.1. In AI/RA/LDB configuration, the results show 0.02%/0.01%/-0.03% BD-rate luma difference.

Introduction

In current draft of VVC [1], PCM mode is not supported. In fact, VTM-2.0.1 will crash if dual tree partition and PCM mode are both enabled. This document proposes PCM mode modification for dual tree partition.

Description of techniques

This document proposes PCM mode modification for dual tree partition. The proposed syntax modification is:

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, cbWidth, cbHeight, treeType) |  |
| } else { |  |
| … |  |
| } |  |
| } else { |  |
| … |  |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

| pcm_sample( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) |  |
| for( i = 0; i < cbWidth* cbHeight; i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_ CHROMA) |  |
| for( i = 0; i < ( (cbWidth* cbHeight )/ ( SubWidthC * SubHeightC ) ); i++ ) |  |
| 2pcm_sample_chroma[ i ] | u(v) |
| } |  |

Simulation results

The proposed method is evaluated on top of the VTM-2.0.1 software. The PCM mode is enabled in config files.

Conclusion

This document proposes PCM mode modification for separated tree partition. The proposed modification is tested on top of VTM2.0.1. In AI/RA/LDB configuration, the results show 0.02%/0.01%/-0.03% BD-rate luma difference.

Reference

- B. Bross, J. Chen and S. Liu, "Versatile Video Coding (Draft 2)”, JVET-K1001, 2018.

Patent rights declaration(s)

Alibaba Cloud Computing Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).