| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0217 |
| --- | --- |

| Title: | Syntax and Semantics Changes of Relation Between QT/BT/TT Split Constraint Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Jianle Chen Biao Wang Anand Meher Kotra | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Syntax and semantic changes regarding to JVET-K1001-v6

…

#### 7.3.2.1 Sequence parameter set RBSP syntax

[Ed. (BB): Preliminary basic SPS, subject to further study and pending further specification development.]

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_diff_min_qt_min_cb_intra_slice | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| diff_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| diff_max_mtt_hierarchy_depth_intra_slice | ue(v) |
| sps_cclm_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_sbtmvp_enabled_flag ) |  |
| log2_sbtmvp_default_size_minus2 | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

…

#### 7.4.3.1 Sequence parameter set RBSP semantics

…

log2_min_luma_coding_block_size_minus2 plus 2 specifies the minimum luma coding block size.

The variables CtbLog2SizeY, CtbSizeY, MinCbLog2SizeY, MinCbSizeY, MinTbLog2SizeY, MaxTbLog2SizeY, PicWidthInCtbsY, PicHeightInCtbsY, PicSizeInCtbsY, PicWidthInMinCbsY, PicHeightInMinCbsY, PicSizeInMinCbsY, PicSizeInSamplesY, PicWidthInSamplesC and PicHeightInSamplesC are derived as follows:

CtbLog2SizeY = log2_ctu_size_minus2 + 2		(7-6)

CtbSizeY = 1  <<  CtbLog2SizeY		(7-7)

MinCbLog2SizeY = log2_min_luma_coding_block_size_minus2 + 2	(7-8)

MinCbSizeY = 1  <<  MinCbLog2SizeY		(7-9)

MinTbSizeY = 4		(7-10)

MaxTbSizeY = 64		(7-11)

PicWidthInCtbsY = Ceil( pic_width_in_luma_samples ÷ CtbSizeY )	(7-12)

PicHeightInCtbsY = Ceil( pic_height_in_luma_samples ÷ CtbSizeY )	(7-13)

PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY	(7-14)

PicWidthInMinCbsY = pic_width_in_luma_samples / MinCbSizeY	(7-15)

PicHeightInMinCbsY = pic_height_in_luma_samples / MinCbSizeY	(7-16)

PicSizeInMinCbsY = PicWidthInMinCbsY * PicHeightInMinCbsY	(7-17)

PicSizeInSamplesY = pic_width_in_luma_samples * pic_height_in_luma_samples	(7-18)

PicWidthInSamplesC = pic_width_in_luma_samples / SubWidthC	(7-19)

PicHeightInSamplesC = pic_height_in_luma_samples / SubHeightC	(7-20)

…

log2_min_qt_size_intra_slices_minus2 plus 2 specifies the minimum luma size of a leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I). The value of log2_min_qt_size_intra_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

MinQtLog2SizeIntraY = log2_min_qt_size_intra_slices_minus2 + 2	(7-22)

log2_min_qt_size_inter_slices_minus2 plus 2 specifies the minimum luma size of a leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 0 (B) or 1 (P). The value of log2_min_qt_size_inter_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

MinQtLog2SizeInterY = log2_min_qt_size_inter_slices_minus2 + 2	(7-23)

log2_diff_min_qt_min_cb_intra_slice specifies the difference between the minimum luma size of a leaf block resulting from quadtree splitting of a CTU and the minimum luma coding block size in slices with slice_type equal to 2 (I). The value of log2_diff_min_qt_min_cb_intra_slice shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

MinQtLog2SizeIntraY = log2_diff_min_qt_min_cb_intra_slice + MinCbLog2Size	(7-23)

log2_diff_min_qt_min_cb_inter_slice specifies the difference between the minimum luma size of a leaf block resulting from quadtree splitting of a CTU and the minimum luma coding block size in slices with slice_type equal to 0 (B) or 1 (P). The value of log2_diff_min_qt_min_cb_inter_slice shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

MinQtLog2InterSizeY = log2_diff_min_qt_min_cb_inter_slice + MinCbLog2Size	(7-24)

max_mtt_hierarchy_depth_inter_slices specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with slice_type equal to 0 (B) or 1 (P). The value of max_mtt_hierarchy_depth_inter_slices shall be in the range of 0 to CtbLog2SizeY − MinTbLog2SizeY, inclusive.

max_mtt_hierarchy_depth_intra_slices specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with slice_type equal to 2 (I). The value of max_mtt_hierarchy_depth_intra_slices shall be in the range of 0 to CtbLog2SizeY − MinTbLog2SizeY, inclusive.

diff_max_mtt_hierarchy_depth_inter_slice specifies the differnce between  maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf and the value of log2_diff_min_qt_min_cb_inter_slice in slices with slice_type equal to 0 (B) or 1 (P). The value of diff_max_mtt_hierarchy_depth_inter_slice shall be in the range of 0 to CtbLog2SizeY – log2_diff_min_qt_min_cb_inter_slice , inclusive.

MaxMttDepthInter = diff_max_mtt_hierarchy_depth_inter_slice + log2_diff_min_qt_min_cb_inter_slice

diff_max_mtt_hierarchy_depth_intra_slice specifies the differnce between maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf and the value of log2_diff_min_qt_min_cb_intra_slice in slices with slice_type equal to 2(I). The value of diff_max_mtt_hierarchy_depth_intra_slice shall be in the range of 0 to CtbLog2SizeY – log2_diff_min_qt_min_cb_intra_slice , inclusive.

MaxMttDepthIntra = diff_max_mtt_hierarchy_depth_intra_slice + log2_diff_min_qt_min_cb_intra_slice

### 7.3.3 Slice header syntax

[Ed. (BB): Preliminary basic slice header, subject to further study and pending further specification development.]

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( (slice_type == I && MaxMttDepthIntra != 0 ) ¦¦ (slice_type != I && MaxMttDepthInter != 0 ) ) { |  |
| log2_diff_max_bt_min_qt | ue(v) |
| log2_diff_max_tt_min_qt | ue(v) |
| } |  |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| if ( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

…

### 7.4.4 Slice header semantics

…

log2_diff_max_bt_min_qt specifies the difference between the maximum luma size (width or height) of a coding block that can be split using a binary split and the mimnum luma size(width or height) of a coding block can be split using a quadtree split . The value of log2_diff_max_bt_min_qt shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY, inclusive.

When log2_diff_ctu_max_bt_size is not present, the value of log2_diff_max_bt_min_qt is inferred to be equal to 0

log2_diff_max_tt_min_qt specifies the difference between the maximum luma size (width or height) of a coding block that can be split using a ternary split and the mimnum luma size(width or height) of a coding block can be split using a quadtree split . The value of log2_diff_max_bt_min_qt shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY, inclusive.

When log2_diff_ctu_max_bt_size is not present, the value of log2_diff_max_bt_min_qt is inferred to be equal to 0

The variables MinQtLog2SizeY, MaxBtLog2SizeY, MinBtLog2SizeY, MaxTtLog2SizeY, MinTtLog2SizeY, MaxBtSizeY, MinBtSizeY, MaxTtSizeY, MinTtSizeY and MaxMttDepth are derived as follows:

MinQtLog2SizeY  =  ( slice_type  = =  I )  ?  MinQtLog2SizeIntraY  :  MinQtLog2SizeInterY	(7-26)

MaxBtLog2SizeY  =  CtbLog2SizeY − log2_diff_ctu_max_bt_size	(7-27)

MaxBtLog2SizeY  =  log2_diff_max_bt_min_qt + MinQtLog2SizeY	(7-27)

MinBtLog2SizeY  =  MinCbLog2SizeY		(7-28)

MaxTtLog2SizeY  =  ( slice_type  = =  I )  ?  5  :  6	(7-29)

MaxTtLog2SizeY  =  log2_diff_max_tt_min_qt + MinQtLog2SizeY	(7-29)

MinTtLog2SizeY  =  MinCbLog2SizeY		(7-30)

MinQtSizeY  =  1  <<  MinQtLog2SizeY		(7-31)

MaxBtSizeY  =  1  <<  MaxBtLog2SizeY		(7-32)

MinBtSizeY  =  1  <<  MinBtLog2SizeY		(7-33)

MaxTtSizeY  =  1  <<  MaxTtLog2SizeY		(7-34)

MinTtSizeY  =  1  <<  MinTtLog2SizeY		(7-35)

MaxMttDepth  =  ( slice_type  = =  I )  ?  max_mtt_hierarchy_depth_intra_slices  :  
				max_mtt_hierarchy_depth_inter_slices	(7-36)

MaxMttDepth  =  ( slice_type  = =  I )  ?  MaxMttDepthIntra : MaxMttDepthInter	(7-36)

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).