| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0217 |
| --- | --- |

| Title: | Syntax and Semantics Changes of Relation Between QT/BT/TT Split Constraint Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Jianle Chen Biao Wang Anand Meher Kotra | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Syntax and semantic changes regarding to JVET-K1001-v6

…

#### 7.3.2.1 Sequence parameter set RBSP syntax

[Ed. (BB): Preliminary basic SPS, subject to further study and pending further specification development.]

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_diff_min_qt_min_cb_intra_slice | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| if (max_mtt_hierarchy_depth_inter_slices != 0 ) |  |
| log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| if (max_mtt_hierarchy_depth_intra_slices != 0 ) |  |
| log2_diff_max_bt_min_qt_intra_slice | ue(v) |
| sps_cclm_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_sbtmvp_enabled_flag ) |  |
| log2_sbtmvp_default_size_minus2 | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

…

#### 7.4.3.1 Sequence parameter set RBSP semantics

…

log2_min_luma_coding_block_size_minus2 plus 2 specifies the minimum luma coding block size.

The variables CtbLog2SizeY, CtbSizeY, MinCbLog2SizeY, MinCbSizeY, MinTbLog2SizeY, MaxTbLog2SizeY, PicWidthInCtbsY, PicHeightInCtbsY, PicSizeInCtbsY, PicWidthInMinCbsY, PicHeightInMinCbsY, PicSizeInMinCbsY, PicSizeInSamplesY, PicWidthInSamplesC and PicHeightInSamplesC are derived as follows:

CtbLog2SizeY = log2_ctu_size_minus2 + 2		(7-6)

CtbSizeY = 1  <<  CtbLog2SizeY		(7-7)

MinCbLog2SizeY = log2_min_luma_coding_block_size_minus2 + 2	(7-8)

MinCbSizeY = 1  <<  MinCbLog2SizeY		(7-9)

MinTbSizeY = 4		(7-10)

MaxTbSizeY = 64		(7-11)

PicWidthInCtbsY = Ceil( pic_width_in_luma_samples ÷ CtbSizeY )	(7-12)

PicHeightInCtbsY = Ceil( pic_height_in_luma_samples ÷ CtbSizeY )	(7-13)

PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY	(7-14)

PicWidthInMinCbsY = pic_width_in_luma_samples / MinCbSizeY	(7-15)

PicHeightInMinCbsY = pic_height_in_luma_samples / MinCbSizeY	(7-16)

PicSizeInMinCbsY = PicWidthInMinCbsY * PicHeightInMinCbsY	(7-17)

PicSizeInSamplesY = pic_width_in_luma_samples * pic_height_in_luma_samples	(7-18)

PicWidthInSamplesC = pic_width_in_luma_samples / SubWidthC	(7-19)

PicHeightInSamplesC = pic_height_in_luma_samples / SubHeightC	(7-20)

…

log2_min_qt_size_intra_slices_minus2 plus 2 specifies the minimum luma size of a leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I). The value of log2_min_qt_size_intra_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

MinQtLog2SizeIntraY = log2_min_qt_size_intra_slices_minus2 + 2	(7-22)

log2_min_qt_size_inter_slices_minus2 plus 2 specifies the minimum luma size of a leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 0 (B) or 1 (P). The value of log2_min_qt_size_inter_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

MinQtLog2SizeInterY = log2_min_qt_size_inter_slices_minus2 + 2	(7-23)

log2_diff_min_qt_min_cb_intra_slice specifies the difference between the minimum luma size of a leaf block resulting from quadtree splitting of a CTU and the minimum luma coding block size in slices with slice_type equal to 2 (I). The value of log2_diff_min_qt_min_cb_intra_slice shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

MinQtLog2SizeIntraY = log2_diff_min_qt_min_cb_intra_slice + MinCbLog2Size	(7-23)

log2_diff_min_qt_min_cb_inter_slice specifies the difference between the minimum luma size of a leaf block resulting from quadtree splitting of a CTU and the minimum luma coding block size in slices with slice_type equal to 0 (B) or 1 (P). The value of log2_diff_min_qt_min_cb_inter_slice shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

MinQtLog2InterSizeY = log2_diff_min_qt_min_cb_inter_slice + MinCbLog2Size	(7-24)

log2_diff_max_bt_min_qt_inter_slice specifies the difference between the maximum luma size (width or height) of a coding block that can be split using a binary split and the mimnum luma size(width or height) of a coding block can be split using a quadtree split in slices with slice_type equal to 0 (B) or 1 (P). The value of log2_diff_max_bt_min_qt_inter_slice shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeInterY, inclusive. When log2_diff_max_bt_min_qt_inter_slice is not present, the value of log2_diff_max_bt_min_qt_inter_slice is inferred to be equal to 0

MaxBtLog2InterSizeY  =  log2_diff_maxS_bt_min_qt_inter_slice + MinQtLog2InterSizeY

log2_diff_max_bt_min_qt_intra_slice specifies the difference between the maximum luma size (width or height) of a coding block that can be split using a binary split and the mimnum luma size(width or height) of a coding block can be split using a quadtree split in slices with slice_type equal to 2 (I) . The value of log2_diff_max_bt_min_qt_intra_slice shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When log2_diff_max_bt_min_qt_intra_slice is not present, the value of log2_diff_max_bt_min_qt_intra_slice is inferred to be equal to 0

MaxBtLog2IntraSizeY  =  log2_diff_max_bt_min_qt_intra_slice + MinQtLog2IntraSizeY

### 7.3.3 Slice header syntax

[Ed. (BB): Preliminary basic slice header, subject to further study and pending further specification development.]

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| if ( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

…

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).