| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0217 |
| --- | --- |

| Title: | Non-CE1: Relation Between QT/BT/TT Split Constraint Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Jianle Chen Biao Wang Anand Meher Kotra | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

It is proposed to signal the QT, BT and TT split size and depth limitations differentially in order to guarantee complete partitioning of the picture frame. It is asserted that the ambiguities in the VVC draft [1] are resolved with the proposed changes.

# Introduction

In the current VVC draft 2 [1] design, the relation between QT/BT/TT split constraint syntax elements are missing which will lead to the incomplete partitioning of picture problem. The proposed method provides the relations of QT/BT/TT split constraint syntax element which solves the mentioned problem.

# Problem Statement

## Problem related to MinCbSizeY, MinQtSizeY and MaxBtSizeY (MaxTtSizeY)

The width and height of the video sequence are indicated using the syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples. In the current VVC they need to be multiples of MinCbSizeY in order to ensure that the picture that the picture can be partitioned into coding blocks with minimum size. However even though pic_height_in_luma_samples and pic_width_in_luma_samples are required to be multiple of MinCbSizeY, it is still not guaranteed that a picture can be fully partitioned into blocks using the available quatree, binary tree and ternary tree splitting down to MinCbSizeY.

An example is given as follows:

Assuming

•	MinCbSizeY is equal to 4

•	MinQtSizeY is equal to 32

•	MaxBtSizeY is equal to 16

In this case if the pic_width_in_luma_samples = 132 and pic_width_in_luma_samples=132, although the width and the height of the picture are multiples of 4, it is still not possible to completely partition the picture. The reason is a parent block with a size of 32x32 cannot be partitioned using binary split or quadtree split, since if quadtree splitting is applied MinQtSizeY limitation would be violated and if binary splitting is applied MaxBtSizeY limitation would be violated. In other words it is not possible to generate a block with width or height equal to 4, which is necessary of complete partitioning of the picture. The problem is especially serious at the picture boundaries.

## Problem related to MaxBtSizeY(MaxTtSizeY) and MaxMttDepth

If MaxMttDepth is indicated to be equal to 0, MaxBtSizeY (or MaxTtSizeY) can still be present in the bitstream and can have a value greater than MinCbSizeY. In this condition creates ambiguity in the operation of the encoder and decoder, complete partitioning of a picture frame is still not possible.

# Proposed Solution

## Relation between MinCbSizeY, MinQtSizeY and MaxBtSizeY (MaxTtSizeY)

To solve the problem in section 2.1 it is proposed to:

•	Signal the indicator of minimum coding block size in the SPS as in HEVC.

•	Signal the indicator of MinQtSizeY relative to MinCbSizeY as follows:

- Signaling unsigned value log2_diff_min_qt_min_cb

- Derive MinQtLog2SizeY = log2_diff_min_qt_min_cb + MinCbLog2Size

- Derive MinQtSizeY = 1 << MinQtLog2SizeY

•	Signal the indicator of MaxBtSizeY relative to MinQtSizeY as follows:

- Signaling unsigned value log2_diff_max_bt_min_qt

- Derive MaxBtLog2SizeY = log2_diff_max_bt_min_qt + MinQtLog2Size

- Derive MaxBtSizeY = 1 << MaxBtLog2SizeY

In this solution, MaxBtSizeY is guaranteed greater than or equal to MinQtSizeY. Regarding to the example in 2.1, the MaxBtSizeY limitation would not be violated.

Note: the syntax element of log2_diff_min_qt_min_cb and log2_diff_max_bt_min_qt are seperated signaled in SPS based on slice type.

Note: the indicator of MaxTtSizeY can be signaled in the same manner as the MaxBtSizeY with the syntax element of log2_diff_max_tt_min_qt, which is noted to be hardcoded according [1].

## Relation between MaxMttDepth and MaxBtSizeY (MaxTtSizeY)

To solve the problem in section 2.3:

•	MaxBtSizeY is signalled if MaxMttDepth is not equal to 0

•	Otherwise the value of MaxBtSizeY is inferred to be equal to MinCbSizeY.

In this solution, the ambiguity between MaxBtSizeY and MaxMttDepth is solved.

Note: the indicator of MaxTtSizeY can be limited in the same manner as the indicator of MaxBtSizeY

# Syntax Modifications

Syntax modifications are provided as attachment to this contribution document.

# Conclusion

It is proposed to signal the QT, BT and TT split and size limitations differentially in order to guarantee complete partitioning of the picture frame. In the solution following aspects are proposed:

•	Signaling the indicator of minimum coding block size in SPS

•	Signaling the indicator of MinQtSizeY relative to MinCbSizeY

•	Signaling the indicator of MaxBtSizeY relative to MinQtSizeY

•

•	Signaling MaxBtSizeY (or MaxTtSizeY) only if MaxMttDepth is not equal to 0

It is asserted that the ambiguities in the VVC draft [1] are resolved with the proposed changes. It is proposed to be included in next VVC draft text.

# References

[1] “ Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, doc. JVET-K1001, July 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).