| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau SAR, CN, 3–12 Oct. 2018 | Document: JVET-L0219 |
| --- | --- |

| Title: | CE3 6.3.1: Intra mode coding with 6 modes in MPM list and Non-MPM modes coded with truncated binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Anand Meher Kotra, Semih Esenlik, Han Gao, Jianle Chen | Tel: Email: | biao.wang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a simple construction strategy to build a 6-MPM list. Only the left and above spatial neighboring blocks’ intra modes are considered, thus the decoding complexity can be reduced to a great extent when compared to the case where five spatial neighboring blocks are considered. Truncated unary coding is used for the signaling of the MPM list index. For the remaining non-MPM modes, truncated binary coding is used in order to avoid the representation gap between fixed length coding (6 bit, 64 modes) and the number of actual non-MPM modes (61 modes). Two variations of the proposal contribution are provided, one with the constraint on the line buffer for CUs along the CTU boundary and the other without. Experimental results of the proposed modification are as follows with almost no change in encoding and decoding running time:

Over VTM Anchor with constraint (AI, RA): − 0.31%, − 0.12% Luma BD rate saving.

Over VTM Anchor without constraint (AI, RA): − 0.35%, − 0.13% Luma BD rate saving.

# Introduction

This proposal tries to build a 6-MPM list with a simplified strategy. Compared to VTM 2, where 3 MPM list is used and in worst case 8 checks are required. This contribution tries to achieve more coding gain but with a limited increase in decoding complexity. The proposed solution only considers two spatial neighboring blocks: A and L as shown in Fig.1. Therefore, with this proposal only 12 checks are needed to build the 6-MPM list while significant coding gain is achieved.

# Background

VTM 2 uses a total of 67 intra prediction modes. The intra mode is signaled by dividing the available intra modes into two different lists: MPM modes list, and non-MPM modes list. The MPM modes list consists of 3 intra modes and the non-MPM modes list is comprised of the remaining 64 intra modes. The MPM mode list only considers two spatial neighbors L and A as shown in Fig. 1.

L

A

Fig. 1 Neighbour intra modes considered in VTM 2.0

The order of insertion of intra modes into the MPM list is similar with that in HEVC. The signalling of non-MPM list uses fixed length coding as 64 is aligned with power of 2. The binarization for the syntax element intra prediction mode is specified in Table 1.

| Intra prediction modes | MPM flag | Bin string |
| --- | --- | --- |
| MPM modes (3) | 1 | 0 |
|  |  | 10 |
|  |  | 110 |
| Non-MPM modes (64) | 0 | 6 bits fixed length code |

Table 1. Intra prediction mode coding in VTM2.0.1

| Intra prediction modes | MPM flag | Bin string |
| --- | --- | --- |
| MPM modes (6) | 1 | 0 |
|  |  | 10 |
|  |  | 110 |
|  |  | 1110 |
|  |  | 11110 |
|  |  | 11111 |
| Non-MPM modes (61) | 0 | Truncated binary code |

Table 2. Intra prediction mode coding in this proposal

# Proposed Method

In this proposal, the coding of MPM mode index and non-MPM modes are shown in Table 2. The MPM list is expanded to 6 intra modes, and only the left and above neighboring blocks as shown in Fig.2 are considered. Correspondingly, non-MPM list shrinks to 61 modes. If the current block’s intra prediction mode falls into the MPM list, only a mode index is needed and it is truncated unary coded. Otherwise, this intra-prediction mode is directly truncated binary coded. The order of insertion of intra modes into the MPM list is as follows:

- Insert “Left (L)”, “Above (A)”, “Planar”, “DC”.

- If there is no angular mode inserted, further insert Vertical, Horizonal, Vertical − 4, and Vertical + 4

- Otherwise if only one angular mode (labeled as Ang) has been inserted, further insert Ang − 1, and Ang+1. If Vertical mode is not equal to any of Ang − 1, Ang, and Ang + 1, then further insert Vertical, otherwise, insert Horizontal

- Otherwise (i.e., two angular modes have been inserted), use Left angular mode to derive Left − 1 and Left + 1.

  - If Left − 1 is equal to Above, further insert Left + 1 and Above − 1.

  - Otherwise if Left + 1 equals to Above, further insert Left − 1 and Above + 1

  - Otherwise (Above is not equals to either Left − 1 and Left + 1), further insert Left − 1 and Left + 1

L

A

Fig. 2 Neighbour intra modes

Furthermore, the CABAC decoding and MPM list construction is decoupled in this contribution. All bins of the index of MPM list are by-pass coded. Finally, line buffer is a concern for hardware implementation. A constraint is applied when the current CU is on the CTU boundary and its above block locates on another CTU. In such case, the above block is considered to be not available. This constrain can be switch on/off using a macro.

# Experimental Results

The table below shows the coding gain and processing time impact of the proposed solutions.

- VTM simulations

  - Anchor: VTM2.0.1

  - Test: VTM2.0.1 + Proposal

# Conclusion

The proposal reports the results for a simple 6-MPM construction method with non-MPM mode signaling using truncated binary coding. CABAC decoding is decouple from MPM list construction with by-pass coding. The proposal with line buffer constraint “ON” achieves a Luma coding gain of − 0.31% and − 0.12% on top of VTM 2.0.1 for AI and RA configurations, respectively, with no increase in encoding and decoding time.

# Reference

[1]“Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, A. Alshin et. al., doc. JVET-J0024, April 2018.

[2] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu doc. JVET-K1001, April 2018

[3] “Description of Core Experiment 3 (CE3):  Intra prediction and mode coding”, G. Van der Auwera, J. Heo, A. Filippov, doc. JVET-K1023, September 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).