| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0229 |
| --- | --- |

| Title: | CE13: Intra prediction disabled across face discontinuities (Test 2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA Cheng-Hsuan Shih, Jian-Liang Lin, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com ch.shih@mediatek.com jl.lin@mediatek.com cc.ju@mediatek.com |
| Source: | InterDigital Communications, Inc. MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 test 2.1. For intra prediction (DC, planar, and angular modes, as well as cross-component linear model), only those available reference samples that are located on the same side of the face discontinuity to which the current CU belongs are used in intra prediction. For most probable modes (MPM) in intra prediction, only the available frame packed neighbors that are also spherical neighbors are used to build the MPM list. Results based on end-to-end WS-PSNR show that HEC with intra prediction disabled across face discontinuities provides {Y, U, V} BD-rate savings of {-0.33%, 0.30%, 0.49%} and {-0.01%, -0.16%, -0.12%} over PHEC and HEC, respectively, for VTM configuration.

# Introduction

Face discontinuities in the frame packed picture may yield to seam artifacts in reconstructed viewports. One of the goals of CE13 is to study removing subjective artifacts at face boundaries using normative decoding tools. The tested coding tools include modifications to intra and inter prediction, as well as in-loop filtering and projection format adaptation. This contribution reports the coding performance of HEC with intra prediction disabled across face discontinuities, as studied in CE13 test 2.1 [1].

# Technical description

For intra prediction (DC, planar, and angular modes, as well as cross-component linear model), the use of reference samples is modified as follows. If a CU does not cross a face discontinuity, then it is determined whether each of the available reference samples is located on the same side of the face discontinuity to which the current CU belongs, and only those available reference samples that are located on the same side of the face discontinuity are used in intra prediction.

For most probable modes (MPM) in intra prediction, the construction of the MPM list is modified as follows. If a CU does not cross a face discontinuity, then it is determined if each available spatial neighbor in the frame packed picture is also a spherical neighbor, and only the available frame packed neighbors that are also spherical neighbors are used to build the MPM list.

# Simulation results

The software is implemented based on BMS-2.0.1 (with fix BMS ticket #78 and #79) and 360Lib-7.0. The encoding was done under 360° video CTC [2]. The following table only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanying Excel spreadsheet.

Results for HEC with intra prediction disabled across face discontinuities over PHEC and HEC are reported in Table 1.

Table 1. BD-rate for HEC with intra prediction disabled across face discontinuities over PHEC and HEC.

|  |  | PHEC |  |  | HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.13% | 1.02% | 0.94% | 0.10% | 0.02% | -0.21% |
|  | ChairLift | -0.21% | -0.25% | -0.13% | -0.09% | -0.49% | -0.41% |
|  | KiteFlite | -0.45% | 0.12% | 0.23% | 0.00% | -0.01% | -0.14% |
|  | Harbor | 0.02% | 1.07% | 2.01% | -0.01% | -0.44% | 0.14% |
|  | Trolley | -0.69% | 0.14% | -0.24% | -0.04% | -0.34% | -0.32% |
|  | GasLamp | -0.53% | 0.11% | 0.28% | -0.06% | -0.26% | -0.19% |
| Class S2 | Balboa | -0.12% | -0.34% | 0.44% | 0.01% | -0.37% | -0.02% |
|  | Broadway | -0.10% | 0.59% | 0.74% | -0.03% | 0.49% | 0.18% |
|  | Landing2 | -0.51% | 0.94% | 0.84% | -0.01% | -0.10% | 0.00% |
|  | BranCastle2 | -0.83% | -0.45% | -0.25% | -0.02% | -0.09% | -0.22% |
| Class S1 |  | -0.29% | 0.37% | 0.51% | -0.02% | -0.25% | -0.19% |
| Class S2 |  | -0.39% | 0.19% | 0.44% | -0.01% | -0.02% | -0.02% |
| Overall |  | -0.33% | 0.30% | 0.49% | -0.01% | -0.16% | -0.12% |
| Enc Time[%] |  | 100% |  |  | - |  |  |
| Dec Time[%] |  | 98% |  |  | 99% |  |  |

BD-rate savings over PHEC and HEC are around {-0.33%, 0.30%, 0.49%} and {-0.01%, -0.16%, -0.12%}, respectively. The complexity of HEC with intra prediction disabled across face discontinuities is similar to that of PHEC and HEC.

# Conclusion

This contribution reports simulation result of CE13 test 2.1, which evaluates HEC with intra prediction disabled across face discontinuities. The results show BD-rate savings of {-0.33%, 0.30%, 0.49%} and {-0.01%, -0.16%, -0.12%} over PHEC and HEC, respectively, for VTM configuration.

# References

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-K1033, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).