| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0238 |
| --- | --- |

| Title: | AHG8: Chroma sample location type support for 360Lib |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The 360Lib supports projection format conversion between different chroma formats. However, when the 4:2:0 chroma format is used, several issues have been identified in the 360Lib and 360° CTC related to the chroma sample location type:

- With the configuration files used in the CTC, chroma sample location type 2 is assumed for the input and output, whereas the 360° video sequences in CTC are in type-0.

- The cross-component linear model (CCLM) tool in VTM-2.0 assumes chroma sample location type 0 when downsampling the reconstructed chroma samples. If the input video has a chroma sample location type different from the one assumed in CCLM mode, an average coding loss in the range of 1% to 2% is observed on the chroma components (JVET-L0239).

- The input and output chroma sample location types cannot be specified independently.

- Only chroma sample location types 0 and 2 are supported, but not always correctly.

- Input/output using chroma sample location type 0 is only supported by means of resampling between type-0 and type-2.

- Only WS-PSNR supports different chroma location types, other metrics only support type-0.

- When a multi-face project format such as the cubemap (CMP) or a cubemap-based projection formats is used, different rotations are applied to different faces, causing the chroma sample location type to be inconsistent among different faces within a frame packed picture.

This contribution proposes to fix these problems by modifying the 360Lib software and 360Lib configuration files as follow:

- Change the default chroma sample location type to type-0.

- Add parameters to specify the chroma sample location types of the input and output independently.

- Add support for all four chroma sample location types in 360Lib.

- Modify the mapping functions in 360Lib such that mathematically correct mapping and resampling is performed between all chroma sample location types in a single step.

- Modify the metric calculations in 360Lib such that mathematically correct calculation is performed considering the chroma types of the two inputs.

- Modify the 360Lib software such that multi-face project format has a consistent chroma sample location type within the entire frame packed picture.

It is reported that, with the proposed modification to the 360Lib, the average coding performance can be improved by -0.35% and -0.26% for the U and V components, respectively, based on end-to-end WS-PSNR for PERP encoding. For HEC encoding, the average coding gain is -1.47% and -1.03%for the U and V components, respectively.

# Problem statement

## Chroma sample location type

Three different chroma formats are commonly used when encoding video sequences: 4:4:4, 4:2:2, and 4:2:0. In the 4:2:0 chroma format, the chroma planes are down-sampled by a factor 2 in both horizontal and vertical directions compared to the luma plane. The chroma sample location type defines the position of chroma samples with respect to luma samples. Figure 1 depicts the location of chroma samples in 4:2:0 chroma format for chroma sample types 0-3. In HEVC, the chroma sample location type may be signaled in the video usability information (VUI) parameters using the syntax elements chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field. At the decoder side, the video player may exploit these syntax elements when converting the decoded picture format to the display format.

Figure 1. Location of chroma samples in 4:2:0 chroma format.

The actual chroma sample location type of the input video sequence should be considered in projection format conversion. The sequences provided by InterDigital were converted from the RGB 4:4:4 output of the Kolor stitching software to YCbCr 4:2:0 using the HDRTools [1]. The configuration files used for this conversion specify to use chroma sample type 0. Thus, the sequences provided by InterDigital are in chroma sample location type 0 format. For the GoPro sequences, they have been generated following a similar workflow, using either FFmpeg or HDRTools with the default settings, and therefore are believed to have chroma sample location type 0 as well.

## Chroma format support issues in 360Lib-7.0

360Lib uses the type-2 chroma sample location when the internal chroma format is set to 4:2:0 [2]. In the chroma sample location type 2, the luma and chroma sampling grids are aligned (except for a factor of 2 in each dimension in terms of resolution). Thus, when performing projection format conversion for the chroma components, the chroma sampling grid is first scaled up by a factor of 2 to align with the luma sampling grid before conversion, and is scaled down by a factor of 2 after conversion. If the input format uses chroma sample location type 0, it can be resampled to chroma sample location type 2 for the internal representation. The output is obtained by resampling the internal representation to chroma sample location type 0. However, the resampling is disabled by default and the chroma sample location type is set to type-2 by default. Resampling is an additional conversion step, which may reduce the fidelity of the chroma components. As explained in Section 1.1, the 360° video sequences in CTC are using the chroma sample location type 0. Thus, setting the default chroma sample location type to type-2 causes a mismatch between the actual relation between luma and chroma samples in the original content and the chroma definition used in 360Lib.

Furthermore, in 360Lib-7.0, the location of chroma samples is not defined in the frame packed picture, but in the individual face coordinate system (uv-coordinates). Thus, if a face is rotated in the frame packed picture, the chroma samples of that face are not aligned with the top left luma sample. For example, considering the default cube layout used in the 360Lib configuration files, the bottom (face # 3) and back (face #1) faces are rotated by 180° and 270° counter-clockwise, respectively, as depicted in Figure 2 (a). Within each face, chroma sample location type 2 is used with respect to the uv-coordinates. However, because of rotation, the chroma samples in the frame packed picture that correspond to the bottom and back faces are no longer of type 2, because they are not aligned with the top left luma samples anymore. Instead, the chroma samples are aligned with the bottom right and top right luma samples for the bottom and back faces, respectively. Figure 2 (b) depicts the location of chroma samples if chroma sample location type 2 is used in the frame packed picture for all faces.

| (a) | (b) |
| --- | --- |

Figure 2. Location of chroma samples in a cube-based frame packed picture with chroma sample location type 2: 360Lib output (a) and expected output (b).

## Cross-component linear model prediction in VTM-2.0

To reduce the cross-component redundancy, a cross-component linear model (CCLM) prediction mode is used in the VTM-2.0 [3] for intra prediction. In the CCLM mode, the chroma samples are predicted based on the reconstructed luma samples of the same CU by using a linear model as follows:

predCi, j=α·recL'i, j+ βpredCi,j=α·recL'i,j+β

where predCi, jpredCi,j represents the predicted chroma samples in a CU and recLi, jrecLi,j represents the downsampled reconstructed luma samples of the same CU. Parameters αα and ββ are derived by minimizing the regression error between the neighbouring reconstructed luma and chroma samples around the current block. Figure 3 shows the location of the left and above samples and the sample of the current block involved in the CCLM mode.

Figure 3. Locations of the samples used for the derivation of α and β.

As depicted in Figure 3, the CCLM prediction in VTM assumes chroma sample location type 0, since it is the most commonly used in legacy content in ITU-R BT.601 or ITU-R BT.709 container. Figure 4 depicts the location of chroma samples in a cube-based frame packed picture with chroma sample location type 0 that should ideally be produced by the 360Lib for encoding with VTM-2.0. If an input video with a chroma sample location type that is different from type 0 is encoded by VTM-2.0, the downsampling filter used in CCLM in VTM-2.0 may cause a shift between the actual chroma samples in the input video and the down-sampled reconstructed luma samples used for prediction. The performance of CCLM prediction may be reduced when the chroma sample location type of the input video doesn’t match with the downsampling filters used in CCLM [4].

Figure 4. Location of chroma samples in a cube-based frame packed picture with chroma sample type 0.

# Proposed fix

To resolve these problems, the following actions are proposed:

- Change the default chroma sample location type to type-0 in CTC.

- Add parameters to specify the chroma sample location types of the input and output independently.

- Add support for all four chroma sample location types.

- Modify the mapping functions such that mathematically correct mapping and resampling is performed between all chroma sample location types in a single step.

- Modify the metric calculations such that mathematically correct calculation is performed considering the chroma types of the two inputs.

- Modify the mapping process of multi-face project formats to ensure that the chroma sample location type of all faces are consistent within a frame packed picture.

In 360Lib, for each sample within the destination face, the following mapping is used to convert from the destination geometry (ud, vd, faceIdd) to the source geometry (us, vs, faceIds):

ud = ud * (1 << getComponentScaleX(chId))

vd = vd * (1 << getComponentScaleY(chId))

(x, y, z) = destination->map2DTo3D(ud, vd, faceIdd)

(us, vs, faceIds) = source->map3DTo2D(x, y, z)

us = us / (1 << getComponentScaleX(chId))

vs = vs / (1 << getComponentScaleY(chId))

To account for the shift introduced by the chroma sample location type and face rotation, the mapping functions can be modified as follows:

ud = ud * (1 << getComponentScaleX(chId)) + ∆ud

vd = vd * (1 << getComponentScaleY(chId)) + ∆vd

(x, y, z) = destination->map2DTo3D(ud, vd, faceIdd)

(us, vs, faceIds) = source->map3DTo2D(x, y, z)

us = (us - ∆us) / (1 << getComponentScaleX(chId))

vs = (vs - ∆vs) / (1 << getComponentScaleY(chId))

where the (chroma) offsets ∆u and ∆v are computed using the following algorithm:

Input: chId, faceId, chroma_format_idc, chroma_sample_loc_type Output: ∆u and ∆v begin if chId != 0 && chroma_format_idc == 1 ∆u = chroma_sample_loc_type = = 0 ¦¦ chroma_sample_loc_type = = 2 ? 0 : 0.5 ∆v = chroma_sample_loc_type = = 2 ¦¦ chroma_sample_loc_type = = 3 ? 0 : 0.5 switch getRotation(faceId) case 0 skip case 90 swap(∆u, ∆v) ∆u = 1 - ∆u case 180 ∆u = 1 - ∆u ∆v = 1 - ∆v case 270 swap(∆u, ∆v) ∆v = 1 - ∆v otherwise Error: not supported else ∆u = 0 ∆v = 0 end

The metrics are modified in the same way when computing weights based on the coordinates of chroma samples in the reference and test pictures (e.g., in WS-PSNR) or when the reference and test pictures have different projection format and/or resolution (e.g., in cross-format S-PSNR).

# Simulation results

The software is implemented based on BMS-2.0.1 and 360Lib-7.0. The encoding was done under 360° video CTC [5]. The following tables report the end-to-end WS-PSNR and end-to-end WS-PSNR-NN results.

For the anchor, the 360Lib-7.0 was used for conversion from original 8K/6K ERP to coded 4K PERP/HEC and for conversion from decoded 4K PERP/HEC to reconstructed 8K/6K ERP. The end-to-end metrics were measured using the modified 360Lib using chroma sample type 0 for the reference and test sequences. For the test, the modified 360Lib was used for all conversions and metrics calculations, also using chroma sample type 0 for the reference and test sequences.

Results for PERP and HEC are reported in Table 1 and Table 2, respectively.

Table 1. BD-rate for PERP.

|  |  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.01% | -0.24% | -0.77% | -0.01% | -0.23% | -0.76% |
|  | ChairLift | 0.00% | -1.53% | 0.00% | 0.00% | -1.51% | 0.04% |
|  | KiteFlite | 0.00% | -0.33% | -0.07% | -0.02% | -0.35% | -0.10% |
|  | Harbor | -0.01% | -0.38% | -0.88% | 0.01% | -0.40% | -0.90% |
|  | Trolley | 0.01% | -0.41% | -0.26% | 0.00% | -0.39% | -0.22% |
|  | GasLamp | -0.03% | -0.32% | -0.43% | -0.06% | -0.28% | -0.47% |
| Class S2 | Balboa | 0.03% | 0.15% | -0.10% | 0.04% | 0.13% | -0.10% |
|  | Broadway | -0.02% | -0.03% | 0.18% | -0.01% | -0.03% | 0.18% |
|  | Landing2 | 0.04% | -0.14% | -0.04% | 0.01% | -0.16% | -0.02% |
|  | BranCastle2 | -0.02% | -0.30% | -0.19% | -0.02% | -0.31% | -0.19% |
| Class S1 |  | -0.01% | -0.54% | -0.40% | -0.01% | -0.52% | -0.40% |
| Class S2 |  | 0.01% | -0.08% | -0.04% | 0.00% | -0.09% | -0.03% |
| Overall |  | 0.00% | -0.35% | -0.26% | -0.01% | -0.35% | -0.26% |

Table 2. BD-rate for HEC.

|  |  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.03% | -0.93% | -0.54% | 0.03% | -0.83% | -0.50% |
|  | ChairLift | -0.09% | -5.90% | -0.16% | -0.08% | -5.91% | -0.15% |
|  | KiteFlite | -0.02% | -0.86% | -1.60% | -0.06% | -0.82% | -1.60% |
|  | Harbor | -0.07% | -0.70% | -0.89% | -0.08% | -0.62% | -0.90% |
|  | Trolley | 0.00% | -0.88% | -0.58% | -0.01% | -0.80% | -0.56% |
|  | GasLamp | -0.04% | -1.59% | -3.74% | -0.06% | -1.54% | -3.74% |
| Class S2 | Balboa | 0.04% | -0.60% | -0.61% | 0.07% | -0.57% | -0.58% |
|  | Broadway | -0.01% | -0.39% | -0.45% | -0.05% | -0.33% | -0.41% |
|  | Landing2 | -0.02% | -0.47% | 0.08% | -0.04% | -0.43% | 0.12% |
|  | BranCastle2 | -0.01% | -2.35% | -1.80% | 0.00% | -2.35% | -1.81% |
| Class S1 |  | -0.03% | -1.81% | -1.25% | -0.04% | -1.75% | -1.24% |
| Class S2 |  | 0.00% | -0.95% | -0.70% | 0.00% | -0.92% | -0.67% |
| Overall |  | -0.02% | -1.47% | -1.03% | -0.03% | -1.42% | -1.01% |

Simulation results show coding gains for the chroma components when using the proper chroma sample location type during projection format conversion. For PERP encoding, the average coding gain is -0.35% and -0.26% for the U and V components, respectively. The coding gain reaches up to -1.53% for the ChairLift sequences on the U component. For HEC encoding, the average coding gain is -1.47% and -1.03% for the U and V components, respectively, based on end-to-end WS-PSNR. The gain for HEC is larger than for PERP because the back and bottom faces are rotated, as shown in Figure 2. The coding gain reaches up to -5.90% for the ChairLift sequences on the U component. The gain is mostly due to the use of type-0 instead of type-2 for encoding, as the CCLM downsampling filters assume type-0.

# Conclusion

This contribution proposes to fix chroma sample location issues in 360Lib software and configuration files as follow:

- Change the default chroma sample location type to type-0.

- Add parameters to specify the chroma sample location types of the input and output independently.

- Add support for all four chroma sample location types.

- Modify the mapping functions such that mathematically correct mapping and resampling is performed between all chroma sample location types in a single step.

- Modify the metric calculations such that mathematically correct calculation is performed considering the chroma types of the two inputs.

- Modify the mapping process of multi-face project formats to ensure that the chroma sample location type of all faces are consistent within a frame packed picture.

It is reported that, with the proposed modification to the 360Lib, the average coding performance can be improved by -0.35% and -0.26% for the U and V components, respectively, based on end-to-end WS-PSNR for PERP encoding. For HEC encoding, the average coding gain is -1.47% and -1.03%for the U and V components, respectively. It is therefore suggested to include this software fix in the next release of 360Lib and to update the configuration files for the 360 video CTC.

# References

- HDRTools, https://gitlab.com/standards/HDRTools

- Y. Ye, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 7”, JVET-K1004, July 2018, Ljubljana, SI.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2)”, JVET-K1002, July 2018, Ljubljana, SI.

- P. Hanhart, Y. He, Y. Ye, “CE3-related: Enabling different chroma sample location types in CCLM”, JVET-L0239, October 2018, Macao, CN.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).