| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0242-v1 |
| --- | --- |

| Title: | AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yingbin Wang, Zhenzhong Chen, Yiming Li Wuhan University Liang Zhao, Shan Liu, Xiang Li Tencent | Tel: Email: | zzchen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents modifications of dense residual convolutional network based in-loop filter (DRNLF), which is proposed in JVET-K0391, in terms of network structure, training materials, and training process. The current VTM software employs multiple in-loop filters including deblocking filter (DF), sample adaptive offset (SAO) and adaptive loop filter (ALF). In this contribution, the proposed DRNLF is introduced as an additional filter placed between DF and SAO. Simulation results report X%, X%, X% BD-rate savings for luma, and both chroma components compared with VTM2.0.1 under AI configuration.

Introduction

In [1], a dense residual convolutional network based in-loop filter (DRNLF) is applied as an additional filter in VTM software, which provides impressive coding performance improvement. However, the employ of DRNLF introduces great computational costs. In this contribution, a simplified network structure is employed for the trade off between computational efficiency and performance. Besides, the network is trained on dataset in YUV color spaces with a loss function designed to balance the coding gain between different components.

Technical description

Proposed Decoding Scheme

In current VTM software, multiple in-loop filters including deblocking filter (DF), sample adaptive offset (SAO) and adaptive loop filter (ALF) are employed. Similar to [1], the proposed DRNLF is introduced into VTM as an additional filter placed after DF, and before SAO and ALF, as shown in Fig.1. The reconstructed frames are firstly filtered by DF before processed by DRN. Then SAO and ALF is employed to further reduce the artifacts. The decision of whether to apply proposed filter or not is given based the rate-distortion optimization (RDO) on block level.

Fig. 1 Proposed decoding scheme in VTM

DRNLF

In this section, the network structure of DRN is described firstly and then training details are provided.

Network Structure

The network structure of DRN is shown in Fig.2, where N denotes the number of dense residual unit (DRU) and M denotes the kernel size of the convolutional layer. In this contribution, some modifications are applied to DRNLF for the trade off between computational efficiency and performance. Firstly, the 3x3 convolutional layers outside the global identity skip connection are removed. And normalized QP map is concatenated with the reconstructed frame as the inputs to DRN. In addition, 3x3 depth-wise separable convolutional (DSC) layer is employed in each DRU. The number of the model parameters are 68K (which are 810K in [1]).

Fig. 2 Network structure of DRN.

Training details

DIV2K dataset [2] is utilized to generate the training sets and validating set for the proposed network. There are 800 images in its training set and 100 images in its validation set. The network is trained on YUV color space directly. The images in DIV2K dataset, which is in RGB color space, are converted to YUV420 and then compressed by VTM-2.0.1 [3] using different QPs under All Intra (AI) configuration. The degraded images and its corresponding QP are used as the input to the network. And the images before compression are used as ground truth. The degraded images are generated by compressing the high-quality images. Let  denote the image with compression artifacts, and  denote the corresponding high quality image,  denote the parameters of the proposed DRN. The proposed DRN can be represented by a function  that try to remove the artifacts in  and restore it to . The goal of the training is to estimate the optimal  that minimizes the following loss:

Where  denotes mean square error (MSE), and  is the weight of different component. In this contribution, the weights of YUV are set to 10, 1, 1, respectively.

Results

Conclusions

This contribution presents modifications of DRNLF in terms of network structure, training materials, and training process for the trade off between computational efficiency and performance.

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.

Reference

- Yingbin Wang, Zhenzhong Chen, Yiming Li, Liang Zhao, Shan Liu, Xiang Li, “Dense Residual Convolutional Neural Network based In-Loop Filter,” JVET-K0391, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- DIV2K, https://data.vision.ee.ethz.ch/cvl/DIV2K/.

- VTM 2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-2.0.1