| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0244-v1 |
| --- | --- |

| Title: | CE9.2.13: MVD-based Early-skip Method and Switchable MC Filters for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Yu Han, Han Huang, Yan Zhang, Chao-Hsiung Hung, Wei-Jung Chien2, Marta Karczewicz3 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 {chunchic1, wchien2, martak3}@ qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the 11th JVET meeting, the Core Experiment 9 was established in JVET-K1029 to study coding tools in the category of decoder-side motion vector derivation (DMVR). Motion vector difference based early-skip method and switchable motion compensation (MC) filters are studied in the context of CE9. The former skips the DMVR process adaptively when the motion vector difference (MVD) between the selected candidate and any previous ones in the same merge list falls within a pre-defined threshold value. The latter applies 2-tap bilinear filters to interpolate sample values that are used for DMVR search within the search range. According to the distance between the original MVs and refined ones, shorter filters instead of 8-tap DCTIF can be applied to final motion compensation. Experiments were conducted on top of the CE9 anchors (i.e. BMS-2.1rc1 w/ VTM cfg.) for Random Access case. Performance results are summarized as follows.

MVD-based vs. VTM: (Y) -1.66%, (U) -1.73%, (V) -1.88%, (Enc.) 102%, (Dec.) 118%.

Switch MC  vs. VTM: (Y) -1.68%, (U) -1.56%, (V) -1.57%, (Enc.) 102%, (Dec.) 113%.

DMVR        vs. VTM: (Y) -1.63%, (U) -1.73%, (V) -1.85%, (Enc.) xxx%, (Dec.) xxx%.

# Introduction and Problem Statement

Decoder-side Motion Vector Refinement (DMVR) [1] is a bi-prediction technique for Merge blocks with two initially signalled motion vectors (MV) that can be further refined by using bilateral matching prediction. During the matching process of DMVR, the sample values on the search region are interpolated by using 8-tap DCTIF. After the matching process is done, the refined MVs are used for motion compensation in the prediction stage and boundary strength calculation in deblock filter. Due to a latency issue at the decoder side [1][2], original MVs are used for spatial prediction for subsequent coding blocks, instead of refined MVs. DMVR can be switched on and off adaptively without signalling overhead. Its MV refinement process operates only when the following conditions are all satisfied.

- The coding unit is a Merge block.

- Uni-directional, ATMVP, STMVP, affine and merge candidates using IC mode are all excluded.

- Only one of (POCref0 – POCcur) and (POCref1 – POCcur) values is negative.

In the next section, a MVD-based approached is proposed for DMVR to adaptively skip its refinement process when the motion vector difference (MVD) between the selected candidate and any previous ones in the same merge list falls within a pre-defined threshold.

In DMVR, motion compensation is performed twice, one for bilateral matching within a given search range and the other for final prediction signals of the prediction block. The former always breaks the worst-case memory bandwidth limit of VTM, since extra pixels need to be fetched from the decoded picture buffer in order to interpolate all the samples values in the search area. The latter would break this limit from time to time, depending on whether the refined MVs point to a different integer grid as the original MVs do on a given reference picture. To fix this issue, in the next section, shorter interpolation filters are proposed to reduce the memory bandwidth requirement. The worst-case memory bandwidth limit of VTM can thus be guaranteed.

# Technical Description

## MVD-based Early-skip Method

An additional constraint is imposed on DMVR to confine its MV refinement process. Thus, DMVR is conditionally disabled when the below condition is satisfied.

- The MV difference between the selected merge candidate and any of the previous ones in the same merge list is less than a pre-defined threshold (that is, ¼-, ½- and 1-pixel-wide intervals for CUs with less than 64 pixels, less than 256 pixels and at least 256 pixels, respectively).

## Bilinear Interpolation for Search Region

In this proposal, 2-tap bilinear filters are applied as instead of 8-tap DCTIF to generate the sample values within the search range. Mean-removed SAD is used as the distortion metrics during the bilateral matching process.

## Switchable Interpolation Filter for Final Motion Compensation

In this proposal, interpolation filters for final motion compensation is switchable between the 8-tap DCTIF filters and alternative shorter filters. The selection of interpolation filters is a two-step process:

1.) computing the distance between the integer-grid coordinate of an original MV and that of its refined MV;

2.) determining the MC filter length based on the maximum value out of vertical and horizontal distance values computed from Step 1.

The values of filter coefficients for switchable filters are specified in Appendix.

### Selection of Luma Filter Length

The aforementioned two-step process can be formulated as a function of the coordinates pointed respectively to by refined MVs and original MVs, as follows:

| (dx, dy) = ¦ (vx << s, vy << s) – (vx’ << s, vy’ << s) ¦, | (1) |
| --- | --- |
| d = max(dx, dy), | (2) |

where s denotes the MV storage precision (i.e. 4 for 1/16-pel precision, 2 for 1/4-pel precision), (vx, vy) and (vx0’, vy0’) represent respectively for the original MV and refined MV pointing to the same reference picture in a reference list, (dx, dy) are the distance values computed respectively for horizontal and vertical directions, and d will then be used to determine the length of interpolation filters, as follows:

- Configuration A: 8-tap filters for d=0 and 2-tap filters for d>0.

- Configuration B: 8-tap filters for d=0 and 4-tap filters for d>0.

- Configuration C: 8-tap filters for d=0, 6-tap filters for d=1 and 4-tap filters for d>1.

It is noted that filter length is determined independently for L0 prediction and L1 prediction.

In addition, another configuration is formed to relax the restriction that the filter lengths for horizontal interpolation and vertical interpolation must be identical. Therefore, the distance value of dx and dy are taken into consideration separately as instead of the d in Eq. (2). The following configuration is specified to test switchable interpolation filters between 8-tap and 4-tap filters:

- Configuration D: 8-tap and 4-tap filters for horizontal interpolation when dx=0 and dx>0, respectively, and 8-tap and 4-tap filters for vertical interpolation when dy=0 and dy>0, respectively.

### Selection of Chroma Filter Length

The same two-step process mentioned in Sec. 2.3.1 is applied for chroma except for the filter assignment. According to the chroma sampling format, the s in Eq. (1) should be redefined, as follows:

| (cx, cy) = ¦ (vx << sx, vy << sy) – (vx’ << sx, vy’ << sy) ¦, | (3) |
| --- | --- |
| c = max(cx, cy), | (4) |

where (sx, sy) are defined as (sx + 1, sy + 1), (sx + 1, sy) and (sx, sy) for YUV 4:2:0, 4:2:2 and 4:4:4 formats, respectively. With (cx, cy) and c in Eq. (3) and Eq. (4), interpolation filter length for final Chroma motion compensation can be determined. Below specifies the length of Chroma interpolation filters associated with the aforementioned configurations A to D in Sec. 2.3.1.

- Configuration A: 4-tap filters for c=0 and 2-tap filters for c>0.

- Configuration B: 4-tap filters for c=0 and 2-tap filters for c>0.

- Configuration C: 4-tap filters for c=0 and 2-tap filters for c>0.

- Configuration D: 4-tap and 2-tap filters for horizontal interpolation when cx=0 and cx>0, respectively, and 4-tap and 2-tap filters for vertical interpolation when cy=0 and cy>0, respectively.

# Experimental Results

The experiments for DMVR with the proposed testing configurations were conducted using the BMS-2.1rc1 software (5b93362a62b8e8a54f0a12069cd729c94defe7e8) [3] and the common test conditions of CE9 [4][5] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchors were generated using the BMS-2.1rc1 software with VTM configurations and SIMD=SSE42, respectively. Macro settings specified in the BMS-2.1rc1 software are not changed. Parallel encoding and decoding were applied to Random Access case for anchors and tests.

There are 5 tests were conducted and reports, including:

- Test 1: MVD-based early-skip method;

- Test 2-1: Configuration A + bilinear interpolation for search + Test 1;

- Test 2-2: Configuration B + bilinear interpolation for search + Test 1;

- Test 2-3: Configuration C + bilinear interpolation for search + Test 1;

- Test 2-4: Configuration D + bilinear interpolation for search + Test 1.

Performance results generated on top of the VTM anchors were summarized in Table 1 and Table 2.

- Performance results of CE9.2.13 Test 1 and DMVR relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE 9.2.13 Test 1 vs. VTM Anchor |  |  |  |  | DMVR vs. VTM Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.79% | -1.59% | -1.96% | 102% | 114% | -1.76% | -1.57% | -1.92% | xx% | xx% |
| Class A2 | -2.29% | -2.45% | -2.56% | 103% | 116% | -2.32% | -2.45% | -2.52% | xx% | xx% |
| Class B | -1.40% | -1.49% | -1.52% | 102% | 120% | -1.32% | -1.48% | -1.53% | xx% | xx% |
| Class C | -1.43% | -1.60% | -1.74% | 101% | 119% | -1.39% | -1.62% | -1.68% | xx% | xx% |
| Overall | -1.66% | -1.73% | -1.88% | 102% | 118% | -1.63% | -1.73% | -1.85% | xx% | xx% |
| Class D | -1.65% | -1.74% | -1.77% | 102% | 120% | -1.74% | -1.75% | -1.84% | xx% | xx% |

- BD-rate results and processing time ratios of CE9.2.13 Test 2 relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 2-1 vs. VTM Anchor |  |  |  |  | Test 2-2 vs. VTM Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.70% | -1.67% | -1.85% | 102% | 111% | -1.87% | -1.53% | -1.56% | 102% | 109% |
| Class A2 | -1.76% | -1.93% | -1.89% | 102% | 112% | -2.27% | -1.83% | -1.57% | 101% | 109% |
| Class B | -1.05% | -1.16% | -1.16% | 102% | 114% | -1.41% | -1.14% | -1.03% | 101% | 113% |
| Class C | -0.92% | -1.10% | -1.05% | 102% | 115% | -1.35% | -1.17% | -1.17% | 101% | 115% |
| Overall | -1.29% | -1.40% | -1.41% | 102% | 113% | -1.66% | -1.36% | -1.28% | 101% | 112% |
| Class D | -0.93% | -1.01% | -0.97% | 102% | 114% | -1.51% | -1.23% | -1.00% | 101% | 115% |
|  | Test 2-3 vs. VTM Anchor |  |  |  |  | Test 2-4 vs. VTM Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.81% | -1.56% | -1.60% | 102% | 111% | -1.86% | -1.61% | -1.78% | 102% | 111% |
| Class A2 | -2.24% | -1.85% | -1.71% | 102% | 111% | -2.30% | -2.10% | -2.01% | 101% | 110% |
| Class B | -1.38% | -1.15% | -1.09% | 102% | 115% | -1.43% | -1.36% | -1.33% | 102% | 114% |
| Class C | -1.34% | -1.17% | -1.16% | 102% | 116% | -1.39% | -1.38% | -1.38% | 101% | 116% |
| Overall | -1.63% | -1.38% | -1.34% | 102% | 114% | -1.68% | -1.56% | -1.57% | 102% | 113% |
| Class D | -1.53% | -1.29% | -1.20% | 102% | 116% | -1.60% | -1.56% | -1.46% | 101% | 115% |

# Conclusion

This contribution firstly presented a MVD-threshold-based constraint imposed on DMVR and was tested in the context of CE 9. On top of it, bilinear filters for DMVR search and shorter interpolation filters for final MC are conducted to reduce the worst-case memory bandwidth requirement of DMVR. From the best configuration of these CE tests, it is reported that the BD-rate variation introduced from the proposed method is less than 0.1% when compared with the original DMVR. The reported decoding runtime is lower than that of the original DMVR. In view of these, we would recommend the committee to consider integrating the proposed technique into the next release of both BMS and VTM software and next version of VVC working draft.

# References

- S. Esenlik, I. Krasnov and et. al., “CE9: DMVR with Bilateral Matching (Test 2.9),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0217, July 2018.

- X. Chen, Semih Esenlik and J. Zheng, “CE9: Simplification of DMVR (Test 9.1.1),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0199, July 2018.

- BMS-2.1rc1 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tree/5b93362a62b8e8 a54f0a12069cd729c94defe7e8

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1010, July 2018.

- S. Esenlik, Y.-W. Chen and F. Chen, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1029, July 2018.

- S. Esenlik, Y.-W. Chen and F. Chen, “CE9: Summary Report on Decoder Side MV Derivation,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0029, Oct. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Filter Coefficients of Shorter Filters

## Luma Filters

The 8-tap Luma filters used in this CE are exactly the same as the 8-tap DCTIF in VTM and BMS. Assuming the coefficients values of an 8-tap DCTIF filter is [c0, c1, c2, c3, c4, c5, c6, c7]. The corresponding 6-tap and 4-tap filter are specified as follows:

- 6-tap filter: [c0 + c1, c2, c3, c4, c5, c6 + c7]

- 4-tap filter: [c0 + c1 + c2, c3, c4, c5 + c6 + c7]

The 2-tap Luma filter is the same as bilinear filter. The actual coefficient values of 2-tap, 4-tap and 6-tap Luma filters are specified as follows:

| 2-tap | 4-tap | 6-tap |
| --- | --- | --- |
| { 64, 0, } { 60, 4, } { 56, 8, } { 52, 12, } { 48, 16, } { 44, 20, } { 40, 24, } { 36, 28, } { 32, 32, } { 28, 36, } { 24, 40, } { 20, 44, } { 16, 48, } { 12, 52, } { 8, 56, } { 4, 60, } | { 0, 64, 0, 0 } { -2, 63, 4, -1 } { -4, 62, 8, -2 } { -6, 60, 13, -3 } { -7, 58, 17, -4 } { -8, 52, 26, -6 } { -7, 47, 31, -7 } { -8, 45, 34, -7 } { -8, 40, 40, -8 } { -7, 34, 45, -8 } { -7, 31, 47, -7 } { -6, 26, 52, -8 } { -4, 17, 58, -7 } { -3, 13, 60, -6 } { -2, 8, 62, -4 } { -1, 4, 63, -2 } | { 0, 0, 64, 0, 0, 0 } { 1, -3, 63, 4, -2, 1 } { 1, -5, 62, 8, -3, 1 } { 2, -8, 60, 13, -4, 1 } { 3, -10, 58, 17, -5, 1 } { 3, -11, 52, 26, -8, 2 } { 2, -9, 47, 31, -10, 3 } { 3, -11, 45, 34, -10, 3 } { 3, -11, 40, 40, -11, 3 } { 3, -10, 34, 45, -11, 3 } { 3, -10, 31, 47, -9, 2 } { 2, -8, 26, 52, -11, 3 } { 1, -5, 17, 58, -10, 3 } { 1, -4, 13, 60, -8, 2 } { 1, -3, 8, 62, -5, 1 } { 1, -2, 4, 63, -3, 1 } |

## Chroma Filters

The 4-tap Chroma filters used in this CE are exactly the same as those in VTM and BMS. The 2-tap Chroma filter is the same as bilinear filter. The actual coefficient values of 2-tap Chroma filters are specified as follows:

2-tap

{ 64, 0 } { 62, 2 } { 60, 4 } { 58, 6 } { 56, 8 } { 54, 10 } { 52, 12 } { 50, 14 } { 48, 16 } { 46, 18 } { 44, 20 } { 42, 22 } { 40, 24 } { 38, 26 } { 36, 28 } { 34, 30 } { 32, 32 } { 30, 34 } { 28, 36 } { 26, 38 } { 24, 40 } { 22, 42 } { 20, 44 } { 18, 46 } { 16, 48 } { 14, 50 } { 12, 52 } { 10, 54 } { 8, 56 } { 6, 58 } { 4, 60 } { 2, 62 }